3GPP TSG-RAN WG4 # 110	R4-2401165

Athens, Greece, February 26 – March 1, 2024

Agenda item:	8.23.4.1

Source:	MediaTek Inc.

Title:	Simulation results of NR MIMO evolution for downlink

Document for:	Discussion

# Introduction

In RAN#101, the status report of RAN1 led Rel-18 WI of NR MIMO evolution for downlink and uplink lists the current progress [1]. The latest WF of WI of NR MIMO evolution for downlink and uplink lists several open items with initial proposals [2]. This discussion document is showing simulation results for PMI prediction codebook, CJT codebook and DMRS enhancements.

# Simulation parameters and results

In this chapter we are going to discuss simulation parameters in Chapter 2.1, simulation results in Chapter 2.2 and simulation analysis in Chapter 2.3.

# 2.1 Rel-16/17 Type-II codebook refinement

# 2.1.1 Simulation parameters

In this chapter simulation configuration of PMI prediction tests are shown in Table 1. The configuration follows initial proposals given in WF [2]. We have simulated 3 different parameter configurations to evaluate PMI prediction performance.

| Channel | TDLA30-30, TDLA30-50 and TDLA30-100 |
| --- | --- |
| Antenna correlation | XP medium |
| CSI-RS antenna ports | 16 with (N1, N2) = (4, 2), (O1, O2) = (4, 4) |
| Aperiodic CSI-RS period | 2ms |
| CQI/RI/PMI delay | 8ms |
| paramCombination-Doppler-r18 | 7 (L=4, pυ=1/2, β=1/2) |
| Rank | 2 |
| MCS | 13 and 20 |
| K, N4, Q, m, d | Left figures: K=4, N4=1, Q=1, m=d=2 (Rel-18 with prediction only) Right figures: K=4, N4=4, Q=2, m=d=2 (Rel-18 with prediction and report compression) Reference in all figures: K=1, N4=1, Q=1, m=d=8 (Rel-16 reference) Reference in all figures: Type-I CB with random WB-i1 and SB-i2 (Random PMI reference) |
| delay | Prediction delay applied to Rel-18 N4={1, 4} simulations: 5 (match delta parameter) or 8 (true PMI apply timing delay) |

Note1: No PDSCH allocation in CSI-RS slots to keep constant PDSCH code rate
Note2: K is prediction filter length

Table 1: Simulation configuration of PMI prediction

# 2.1.2 Simulation results

In this chapter the simulation results of 3 different parameter combinations are shown in Figures 1 to 6.

Figure 1: Simulation results of PMI prediction using N4={1, 4} with MCS13 Rank2 in TDLA30-30.

Figure 2: Simulation results of PMI prediction using N4={1, 4} with MCS13 Rank2 in TDLA30-50.

Figure 3: Simulation results of PMI prediction using N4={1, 4} with MCS13 Rank2 in TDLA30-100.

Figure 4: Simulation results of PMI prediction using N4={1, 4} with MCS20 Rank2 in TDLA30-30.

Figure 5: Simulation results of PMI prediction using N4={1, 4} with MCS20 Rank2 in TDLA30-50.

Figure 6: Simulation results of PMI prediction using N4={1, 4} with MCS20 Rank2 in TDLA30-100.

| Test | prediction delay 8 | prediction delay 5 | Rel-16 |
| --- | --- | --- | --- |
| MCS-13, K=4, N4=1, TDLA-30ns-30Hz | 3.1 | 2.7 | 2.3 |
| MCS-13, K=4, N4=4, TDLA-30ns-30Hz | 2.6 | 2.5 | 2.3 |
| MCS-13, K=4, N4=1, TDLA-30ns-50Hz | 2.4 | 2.3 | 2.4 |
| MCS-13, K=4, N4=4, TDLA-30ns-50Hz | 2.4 | 2.3 | 2.4 |
| MCS-13, K=4, N4=1, TDLA-30ns-100Hz | 2.3 | 2.4 | 2.4 |
| MCS-13, K=4, N4=4, TDLA-30ns-100Hz | 2.3 | 2.3 | 2.4 |

Table 2: Simulation results (gamma values) of PMI prediction with MCS13 Rank 2.

| Test | prediction delay 8 | prediction delay 5 | Rel-16 |
| --- | --- | --- | --- |
| MCS-20, K=4, N4=1, TDLA-30ns-30Hz | 2.6 | 2.3 | 2.0 |
| MCS-20, K=4, N4=4, TDLA-30ns-30Hz | 2.2 | 2.1 | 2.0 |
| MCS-20, K=4, N4=1, TDLA-30ns-50Hz | 2.0 | 1.8 | 2.0 |
| MCS-20, K=4, N4=4, TDLA-30ns-50Hz | 1.9 | 1.9 | 2.0 |
| MCS-20, K=4, N4=1, TDLA-30ns-100Hz | 2.0 | 2.0 | 2.0 |
| MCS-20, K=4, N4=4, TDLA-30ns-100Hz | 2.0 | 2.0 | 2.0 |

Table 3: Simulation results (gamma values) of PMI prediction with MCS20 Rank 2.

# 2.1.3 Simulation analysis

We have simulated the following configurations for performance comparison purpose

- Rel-18 with prediction only: K=4, N4=1, Q=1, m=d=2

- Rel-18 with prediction and report compression: K=4, N4=4, Q=2, m=d=2

- Rel-16 reference: K=1, N4=1, Q=1, m=d=8

- Random PMI reference: Type-I CB with random WB-i1 and SB-i2

Rel-18 solution are simulated with 2 prediction timing assumptions matching either configured delta parameter or matching true PMI apply timing. Our expectation of feasible test is where Rel-18 solution can outperform Rel-16 and random PMI references with reasonable margin and justify usage of new feature in such scenario.

We can do several observations from these simulations.

Observation #1: We do not see gains over Rel-16 reference in TDLA30-50 and TDLA30-100 channels.

Observation #2: We see significant gains over Rel-16 reference in TDLA30-30 when N4=1.

Observation #3: We see minor gains over Rel-16 reference in TDLA30-30 when N4=4.

Observation #4: We see feasible gamma values in TDLA30-30 when with both N4=1 and N4=4.

Observation #5: We see prediction timing matching PMI apply timing outperforming prediction timing based on signalled delta parameter.

# 2.2 Enhancements of CSI acquisition for CJT targeting FR1 and up to 4 TRPs

# 2.2.1 Simulation parameters

In this chapter simulation configuration of CJT tests are shown in Table 4. The configuration follows initial proposals given in WF [2].

| NTRP (Number of TRPs) | 2 |  |
| --- | --- | --- |
| Channel | TDLA30-10 |  |
| Antenna correlation | XP high |  |
| K (numberOfCSI-RS-Resources) | 2 |  |
| CSI-RS periodicity | 5 slots (No PDSCH allocation in CSI-RS slots) |  |
| CSI-RS antenna ports | 8 with (N1, N2) = (4, 1), (O1, O2) = (4, 1) |  |
| codebookMode | Mode2 |  |
| paramCombination-CJT-L-r18 | 7 ({4, 4}) | 4 ({2, 2}) |
| paramCombination-CJT-r18 | 4 (pυ=1/4 and 1/8, β=1/2) and 7 (pυ=1/2 and 1/2, β=1/2) | 1 (pυ=1/8, β=1/4) |
| typeII-CJT-RI-Restriction-r18 | 0001 and 0010 (Rank1 and Rank2) |  |
| MCS | 13 and 20 |  |

Table 4: Simulation configuration of CJT

# 2.2.2 Simulation results

In this chapter the simulation results of 3 different CJT parameter combinations, single TRP reference and random PMI are shown in Figures 7 to 10. Corresponding gamma values are collected in Tables 5 - 6.

Figure 7: Simulation results of 4Rx CJT with MCS13 and MCS20 Rank1 in TDLA30-10 XP high.

Figure 8: Simulation results of 4Rx CJT with MCS13 and MCS20 Rank2 in TDLA30-10 XP high.

Figure 9: Simulation results of 2Rx CJT with MCS13 and MCS20 Rank1 in TDLA30-10 XP high.

Figure 10: Simulation results of 2Rx CJT with MCS13 and MCS20 Rank2 in TDLA30-10 XP high.

|  | Gamma values (TDLA-30-10, 4Rx, (N1,N2)=(4,1), (O1,O2)=(4,1)) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Rank-1 MCS13 |  |  |  | Rank-1 MCS20 |  |  |  | Rank-2 MCS13 |  |  |  | Rank-2 MCS20 |  |  |  |
|  | eType-2 CJT {4,4} 4 | eType-2 CJT {4,4} 7 | eType-2 CJT {2,2} 1 | 8-Tx Rel-16 eType2 | eType-2 CJT {4,4} 4 | eType-2 CJT {4,4} 7 | eType-2 CJT {2,2} 1 | 8-Tx Rel-16 eType2 | eType-2 CJT {4,4} 4 | eType-2 CJT {4,4} 7 | eType-2 CJT {2,2} 1 | 8-Tx Rel-16 eType2 | eType-2 CJT {4,4} 4 | eType-2 CJT {4,4} 7 | eType-2 CJT {2,2} 1 | 8-Tx Rel-16 eType2 |
| γ | 5.67 | 5.60 | 4.73 | 3.13 | 3.80 | 3.77 | 3.35 | 2.55 | 6.58 | 7.11 | 3.85 | 2.52 | 4.67 | 4.99 | 2.96 | 2.24 |

Table 5: Gamma values of CJT 4Rx tests for different configurations.

|  | Gamma values (TDLA-30-10, 2Rx, (N1,N2)=(4,1) , (O1,O2)=(4,1)) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Rank-1 MCS13 |  |  |  | Rank-1 MCS20 |  |  |  | Rank-2 MCS13 |  |  |  | Rank-2 MCS20 |  |  |  |
|  | eType-2 CJT {4,4} 4 | eType-2 CJT {4,4} 7 | eType-2 CJT {2,2} 1 | 8-Tx Rel-16 eType2 | eType-2 CJT {4,4} 4 | eType-2 CJT {4,4} 7 | eType-2 CJT {2,2} 1 | 8-Tx Rel-16 eType2 | eType-2 CJT {4,4} 4 | eType-2 CJT {4,4} 7 | eType-2 CJT {2,2} 1 | 8-Tx Rel-16 eType2 | eType-2 CJT {4,4} 4 | eType-2 CJT {4,4} 7 | eType-2 CJT {2,2} 1 | 8-Tx Rel-16 eType2 |
| γ | 7.71 | 7.37 | 5.78 | 3.56 | 4.88 | 4.79 | 3.68 | 2.80 | 8.39 | 9.86 | 4.17 | 2.63 | 5.95 | 6.66 | 3.37 | 2.30 |

Table 6: Gamma values of CJT 2Rx tests for different configurations.

# 2.2.3 Simulation analysis

We have simulated 3 different CJT parameter combinations, single TRP reference and random PMI. We can do several observations from these results.

Observation #6: We see significant gains of paramCombination-CJT-L-r18 of value 7 compared to value 4.

Observation #7: We see comparable performance of paramCombination-CJT-r18 values 7 and 4 in Rank1.

Observation #8: We see significant gains of paramCombination-CJT-r18 of value 7 compared to value 4 in Rank2.

Observation #9: We see higher gamma values on MCS13 compared to MCS20.

Observation #10: We see higher gamma values on Rank2 compared to Rank1.

# 2.3 Larger number of orthogonal DMRS ports for downlink MU-MIMO

# 2.3.1 Simulation parameters

Simulation parameters follow configuration of existing demodulation tests defined in TS38.101-4 [3] except Rel-18 enhanced DMRS type is configured.

# 2.3.2 Simulation results

In this chapter the demodulation performance of legacy and enhanced DMRS performance is compared in many existing demodulation tests. Simulation results are shown in Tables X to X.

| FR1 FDD 4RX Test | Performance loss of Rel-18 DMRS compared to Rel-15 DMRS |
| --- | --- |
| Test 1-1 | < 0.3dB |
| Test 1-3 | < 0.1dB |
| Test 2-1 | < 0.1dB |
| Test 3-1 | < 0.1dB |
| Test 4-1 | < 0.1dB |

Table X: Enhanced DMRS performance effect in FR1 FDD 4RX tests.

| FR1 TDD 4RX Test | Performance loss of Rel-18 DMRS compared to Rel-15 DMRS |
| --- | --- |
| Test 1-1 | < 0.3dB |
| Test 1-3 | < 0.1dB |
| Test 2-1 | < 0.1dB |
| Test 3-1 | < 0.1dB |
| Test 4-1 | < 0.1dB |

Table X: Enhanced DMRS performance effect in FR1 TDD 4RX tests.

| FR2 Test | Performance loss of Rel-18 DMRS compared to Rel-15 DMRS |
| --- | --- |
| Test 1-1 | < 0.2dB |
| Test 1-3 | < 0.3dB |
| Test 1-4 | < 0.2dB |
| Test 2-2 | < 0.2dB |
| Test 2-3 | < 0.1dB |

Table X: Enhanced DMRS performance effect in FR2 tests.

# 2.3.3 Simulation analysis

We have compared legacy and enhanced DMRS performance in many existing demodulation tests to understand if we can expect similar performance or not. From simulation results we can do following general observation.

Observation #11: In our simulations we see maximum performance losses less than 0.3dB. Therefore, we see that reusing old values with possible additional margin is sufficient.

# Conclusion

In this paper we provided the view on the MIMO evolution for downlink. The following observations are made:

Observations of predicted PMI

Observation #1: We do not see gains over Rel-16 reference in TDLA30-50 and TDLA30-100 channels.

Observation #2: We see significant gains over Rel-16 reference in TDLA30-30 when N4=1.

Observation #3: We see minor gains over Rel-16 reference in TDLA30-30 when N4=4.

Observation #4: We see feasible gamma values in TDLA30-30 when with both N4=1 and N4=4.

Observation #5: We see prediction timing matching PMI apply timing outperforming prediction timing based on signalled delta parameter.

Observations of CJT

Observation #6: We see significant gains of paramCombination-CJT-L-r18 of value 7 compared to value 4.

Observation #7: We see comparable performance of paramCombination-CJT-r18 values 7 and 4 in Rank1.

Observation #8: We see significant gains of paramCombination-CJT-r18 of value 7 compared to value 4 in Rank2.

Observation #9: We see higher gamma values on MCS13 compared to MCS20.

Observation #10: We see higher gamma values on Rank2 compared to Rank1.

Observations of enhanced DMRS

Observation #11: In our simulations we see maximum performance losses less than 0.3dB. Therefore, we see that reusing old values with possible additional margin is sufficient.

# Reference

- RP-231717, “Status Report: NR MIMO evolution for downlink and uplink”, Samsung

- R4-2321141, “WF on [109][327] NR_MIMO_evo_DL_UL_demod”, Samsung

- 3GPP TS 38.101-4 V18.2.0 (2023-12), “User Equipment (UE) radio transmission and reception; Part 4: Performance requirements (Release 18)”