# 3GPP TSG-RAN WG4 Meeting #110								   		R4-2401192

# Athens, GR, 26 Feb – 01 Mar, 2024

Source: 	Xiaomi

Title: 	Discussion on RRM requirements for NTN deployment in above 10 GHz bands

Agenda Item:	8.18.6.1

Document for:	Discussion

# 1 Introduction

In last RAN4#109 meeting, discussion on RRM requirements for Network verified UE location was conducted and a way forward was agreed in [1]. In this contribution, we provide our further consideration of the remaining open issue for SL unlicensed operation based on the progress in last meeting.

- Discussion

### Issue 1-3: Further relaxation of Te_NTN for PRACH No agreement ### Issue 1-6: Te_NTN for 60kHz and 120kHz Agreement: - UL 60kHz SCS: - 13 Ts for all cases with 120kHz/240kHz SSB - UL 120kHz SCS: - Case 1 and case 2: 7.5 Ts - FFS for the applicable side condition on case 2 - Case 3: Higher than 7.5 Ts, FFS for the exact value ### Issue 1-7: NTA-offset Agreement: - RAN4 to define the exact value of NTA,offset for NR NTN band above 10 GHz. - Option 1: the value of NTA-offset defined in Table 7.1.2-2 for FR2 - Other options are not precluded.

Based on the discussion in last meeting, Te_NTN for UL 120KHz SCS in case 3 is still FFS.

According to the equation agreed in RAN4#108-bis meeting, Tg = 0.5*Tcp – (Td + Tp + Tr + Ta + Tf + Tm), the maximum error budget for UL 120KHz SCS for GNSS position error and satellite position error can be calculated in following table with the following components:

Td is taken from the timing error limit defined for TN in section 7.1.2 in TS 38.133;

Tr follows the step size of TA command, 16∙642μ∙Tc, fine in section 4.2 in TS 38.213;

Ta is taken from the timing advance adjustment accuracy defined for TN in section 7.3.2.2 in TS 38.133;

For Tf, we tend to think there is no need to consider it as Tf represents timing drift due to a frequency offset, which is a measure in frequency domain.

| SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | CP length (Tcp) | Tr (TAC resolution error) | Ta (TA adjustment accuracy error) | Td (UE downlink synchronization error) | Tp = Te_GNSS + Te_SAT | GNSS position error + serving-satellite position error |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 120 | 60 | 36*64*Tc | 2*64*Tc | 2*64*Tc | 3.5*64*Tc | 10.5*64*Tc | 52m |
|  | 120 | 18*64*Tc | 1*64*Tc | 0.5*64*Tc | 3.5*64*Tc | 4*64*Tc | 20m |
| 240 | 60 | 36*64*Tc | 2*64*Tc | 2*64*Tc | 3*64*Tc | 11*64*Tc | 54m |
|  | 120 | 18*64*Tc | 1*64*Tc | 0.5*64*Tc | 3*64*Tc | 4.5*64*Tc | 22m |

Based on the calculation, it is observed that the max error budget for GNSS position error (X) and serving-satellite position error (Y) is around 55m for 60 KHz UL SCS case and around 20m for 120 KHz UL SCS case. From our perspective, it is kind of a challenge for UE if the serving-satellite position error (Y) is set as below 20m for 120 KHz UL SCS case. However, RAN4 has agreed to define the Te_NTN requirement for 120 KHz UL SCS case in case 3, i.e., mobile UE for GSO, according to last meeting discussion. And the exact value is proposed to be relaxed, i.e. higher than 7.5 Ts which is the agreed Te_NTN value for 120 KHz UL SCS in case 1 and 2.

Considering Te_NTN= Td+Tp, we are fine with a higher number as some of the components in the above calculation are derived with a margin. We suggest to use Te_NTN=10 Ts for 120KHz UL SCS in case 3.

Proposal 1: It is suggested to define Te_NTN=10 Ts for 120KHz UL SCS in case 3.

In last meeting, it is also FFS on whether the value of NTA,offset defined in Table 7.1.2-2 for FR2 can be reused for NTA,offset for NR NTN band above 10 GHz. The initial motivation to define NTA,offset is to avoid the situation that interference occur between uplink and downlink. In our understanding, the value of NTA,offset is related to BS ramping down and up time and BS synchronization accuracy. Generally, from BS side, no new requirement was introduced. We think it is reasonable to reuse the value of NTA,offset defined in Table 7.1.2-2 for FR2 for NTA,offset for NR NTN band above 10 GHz.

Proposal 2: The value of NTA,offset defined in Table 7.1.2-2 for FR2 can be reused for NTA,offset for NR NTN band above 10 GHz.

- Conclusion

Proposal 1: It is suggested to define Te_NTN=10 Ts for 120KHz UL SCS in case 3.

Proposal 2: The value of NTA,offset defined in Table 7.1.2-2 for FR2 can be reused for NTA,offset for NR NTN band above 10 GHz.

- Reference

- R4-2321362, WF on NTN RRM requirements, Qualcomm Incorporated