3GPP TSG-RAN WG4 Meeting #110	R4-2401226

Athens, Greece, Feb 26 – Mar 1, 2024

Agenda item:	8.14.3.3

Source: 	Qualcomm Incorporated

Title: 	Simulation results for RedCap positioning with frequency hopping

Document for:	Information

# Introduction

RAN4 agreed on simulation assumptions for RedCap positioning measurements with frequency hopping [1]. This paper includes simulation results for the following measurements:

- UE Rx-Tx time difference

- RSTD

# Simulation results for UE Rx-Tx time difference

| FR | Total PRS BW (PRBs) | SCS (kHz) | sample rate (Tc) | Num. samples | Num symbols | Accuracy [90%] (Tc) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = 0 dB | SINR = -6 dB |
| FR1 | 272 | 15 | 32 | 2 | 4 | 19 | 51 |
|  | 272 | 30 | 16 | 2 | 4 | 10 | 23 |
|  | 132 | 60 | 16 | 2 | 4 | 10 | 25 |
| FR2 | 272 | 60 | 8 | 2 | 4 | 5 | 13 |
|  | 272 | 120 | 4 | 2 | 4 | 3 | 7 |

Table 1: UE Rx-Tx simulation results for 2 Rx in AWGN with reduced number of samples.

| FR | Total PRS BW (PRBs) | SCS (kHz) | sample rate (Tc) | Num. samples | Num symbols | Accuracy [90%] (Tc) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = -3 dB | SINR = -13 dB |
| FR1 | 272 | 15 | 32 | 4 | 4 | 38 | 92 |
|  | 272 | 30 | 16 | 4 | 4 | 19 | 46 |
|  | 132 | 60 | 16 | 4 | 4 | 18 | 47 |
| FR2 | 272 | 60 | 8 | 4 | 4 | 9 | 24 |
|  | 272 | 120 | 4 | 4 | 4 | 5 | 12 |

Table 2: UE Rx-Tx simulation results for 2 Rx in AWGN with 4 samples.

| FR | Total PRS BW (PRBs) | SCS (kHz) | sample rate (Tc) | Num. samples | Num symbols | Accuracy [90%] (Tc) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = 0 dB | SINR = -6 dB |
| FR1 | 272 | 15 | 32 | 2 | 4 | 19 | 51 |
|  | 272 | 30 | 16 | 2 | 4 | 10 | 23 |
|  | 132 | 60 | 16 | 2 | 4 | 10 | 26 |

Table 3: UE Rx-Tx simulation results for 1 Rx in AWGN with reduced number of samples.

| FR | Total PRS BW (PRBs) | SCS (kHz) | sample rate (Tc) | Num. samples | Num symbols | Accuracy [90%] (Tc) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = -3 dB | SINR = -13 dB |
| FR1 | 272 | 15 | 32 | 4 | 4 | 38 | 95 |
|  | 272 | 30 | 16 | 4 | 4 | 18 | 48 |
|  | 132 | 60 | 16 | 4 | 4 | 21 | 54 |

Table 4: UE Rx-Tx simulation results for 1 Rx in AWGN with 4 samples.

# Simulation results for RSTD

| FR | Total PRS BW (PRBs) | SCS (kHz) | sample rate (Tc) | Num. samples | Num symbols | Accuracy [5%, 95%] (Tc) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINRref = -3 dB, SINRi = -6 dB |
| FR1 | 272 | 15 | 32 | 2 | 4 | ±37 |
|  | 272 | 30 | 16 | 2 | 4 | ±18 |
|  | 132 | 60 | 16 | 2 | 4 | ±20 |
| FR2 | 272 | 60 | 8 | 2 | 4 | ±10 |
|  | 272 | 120 | 4 | 2 | 4 | ±5 |

Table 5: RSTD simulation results for 2 Rx in AWGN with reduced number of samples.

| FR | Total PRS BW (PRBs) | SCS (kHz) | sample rate (Tc) | Num. samples | Num symbols | Accuracy [5%, 95%] (Tc) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINRref = -6 dB, SINRi = -13 dB |
| FR1 | 272 | 15 | 32 | 4 | 4 | ±56 |
|  | 272 | 30 | 16 | 4 | 4 | ±30 |
|  | 132 | 60 | 16 | 4 | 4 | ±28 |
| FR2 | 272 | 60 | 8 | 4 | 4 | ±14 |
|  | 272 | 120 | 4 | 4 | 4 | ±7 |

Table 6: RSTD simulation results for 2 Rx in AWGN with 4 samples.

| FR | Total PRS BW (PRBs) | SCS (kHz) | sample rate (Tc) | Num. samples | Num symbols | Accuracy [5%, 95%] (Tc) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINRref = -3 dB, SINRi = -6 dB |
| FR1 | 272 | 15 | 32 | 2 | 4 | ±37 |
|  | 272 | 30 | 16 | 2 | 4 | ±17 |
|  | 132 | 60 | 16 | 2 | 4 | ±20 |

Table 7: RSTD simulation results for 1 Rx in AWGN with reduced number of samples.

| FR | Total PRS BW (PRBs) | SCS (kHz) | sample rate (Tc) | Num. samples | Num symbols | Accuracy [5%, 95%] (Tc) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINRref = -6 dB, SINRi = -13 dB |
| FR1 | 272 | 15 | 32 | 4 | 4 | ±56 |
|  | 272 | 30 | 16 | 4 | 4 | ±32 |
|  | 132 | 60 | 16 | 4 | 4 | ±31 |

Table 8: RSTD simulation results for 1 Rx in AWGN with 4 samples.

# References

[1] R4-2314460, Simulation assumptions for RedCap positioning with FH, RAN4#108