3GPP TSG-RAN4 Meeting #110		R4-2401232

# Athens, Greece, Feb 26 – Mar 1, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev |  | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to core requirements for Sidelink positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm, Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2/19/2024 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Core requirements for SL positioning measurements have not been completed: there are IE names and other text in [], editor’s notes for maintenance issues, some parameters were left FFS/TBD, and the start of the measurement period was not defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the names of SLPP IEs - Update the definition of number of samples. - Update the definition of processing time and sample duration for the measurement period requirements. - Add applicability conditions for measurement period based on UE capabilities. - Define the starting point of the measurement period. - For SL PRS-RSRP(P) the measurement period requirement of the primary SL positioning measurement applies. - Editorial corrections and remove editor’s notes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRM core requirements for SL positioning would be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12A.2, 12A.3, 12A.4, 12A.5, 12A.6, 12A.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------START OF CHANGE----------------------------

## 12A.2	SL-RSTD measurements

### 12A.2.1	Introduction

The requirements in clause 12A.2 shall apply provided the UE has received SL-TDOA-RequestLocationInformation- message from LMF or another UE via SLPP [37] requesting the UE to measure and report SL RSTD measurements defined in TS 38.215 [4] based on SL-PRS.

### 12A.2.2	Requirements Applicability

The requirements in clause 12A.2 apply for periodic, aperiodic, and triggered RSTD measurements, provided:

-	SL RSTD related side conditions given in clause TBD for FR1 are fulfilled, for a corresponding Band.

### 12A.2.3	Measurement Capability

UE SL RSTD measurement capability is as indicated by the UE in SL-TDOA-ProvideCapabilities, according to TS 38.355 [37].

### 12A.2.4	Measurement Reporting Requirements

The measurement reporting delay is defined as the time between the moment when the measurement report is triggered and the moment when the UE starts to transmit the measurement report over the air interface.

For UE reporting to LMF, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PSSCH with subframe or slot or subslot duration.

For UE reporting to another UE, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the STCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the transmitted STCH. The delay uncertainty is: 2 x TTISTCH where TTISTCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PSSCH with subframe or slot or subslot duration.

This measurement reporting delay excludes any delay caused by no SL resources for UE to send the measurement report.

The reported SL RSTD measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clauses TBD.

The SL RSTD measurements performed and reported according to this section shall meet the SL RSTD measurement accuracy requirements in clause TBD, for each measured SL-PRS resource.

### 12A.2.5	Measurements Period Requirements

When the physical layer receives last of SL-TDOA-ProvideAssistanceData message and SL-TDOA-RequestLocationInformation message from LMF or another UE via SLPP [37], the UE shall be able to perform SL RSTD measurements, defined in TS 38.215 [4], during the measurement period TSL RSTD,Total defined as:

TSL RSTD,Total=s=1STSL RSTD,effect,s  ,

where

S is the number of samples per SL PRS resource, defined below:

S = 1 for SL-PRS BW ≥ 48 PRBs,

S = 4 for SL-PRS BW<48 PRBs,

TSL RSTD,effect,s is defined as:

TSL RSTD,effect,s=ts+1-ts , for s<S, provided that TSL RSTD,effect,s≥Tdur,s+ΔSLproc , where ts+1 and ts are the beginning of the first slot corresponding to the next and current SL-PRS samples, respectively, in which the UE is configured to receive SL-PRS for performing the SL-RSTD measurement,

TSL RSTD,effect,s=Tdur,s+ΔSLproc , for s=S,

Tdur,s is the duration of SL-PRS sample s of the SL RSTD measurement and it is equal to the duration of one slot,

ΔSLproc=[TBDcomponent 4 of FG 41-1-1] is the processing time.

The measurement period TSL RSTD,Total applies to each individual SL PRS resource measured by a UE provided the number of active SL PRS resources indicated in [component 2 of FG 41-1-1] and the maximum number of slots containing active SL PRS resources indicated in [component 3 of FG 41-1-1] are not exceeded. If either capability is exceeded while the UE is performing SL RSTD measurements, the measurement period can be longer. For each SL PRS resource, the measurement period TSL RSTD,Total starts at the start of the slot containing the first sample of the SL PRS resource.

If the synchronization reference source of the UE changes while the UE is performing the SL RSTD measurement, then the UE shall restart the SL RSTD measurement after the synchronization reference source change and the measurement period can be longer.

TSL RSTD,restart=K+1*TSL RSTD, Total

----------------------END OF CHANGE----------------------------

----------------------START OF CHANGE----------------------------

## 12A.3	SL-RSRP measurements

### 12A.3.1	Introduction

The requirements in clause 12A.3 shall apply provided the UE has received SL-TDOA-RequestLocationInformation, SL-TOA-RequestLocationInformation, SL-RTT-RequestLocationInformation or SL-AoA-RequestLocationInformation message from LMF or another UE via SLPP [37] requesting the UE to measure and report SL PRS-RSRP measurements defined in TS 38.215 [4] based on SL-PRS.

### 12A.3.2	Requirements Applicability

The requirements in clause 12A.3 apply for periodic, aperiodic, and triggered SL PRS-RSRP measurements, provided:

-	SL PRS-RSRP related side conditions given in clause TBD for FR1 are fulfilled, for a corresponding Band.

### 12A.3.3	Measurement Capability

UE SL PRS-RSRP measurement capability is as indicated by the UE in [sl-TDOA-ProvideCapabilities, sl-RTT-ProvideCapabilities, sl-AOA-ProvideCapabilities, or sl-TOA-ProvideCapabilities], according to TS 38.355 [37].

### 12A.3.4	Measurement Reporting Requirements

The measurement reporting delay is defined as the time between the moment when the measurement report is triggered and the moment when the UE starts to transmit the measurement report over the air interface.

For UE reporting to LMF, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the DCCH or SCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH or SCCH. The delay uncertainty is: 2 x TTIDCCH/SCCH where TTIDCCH/SCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration.

For UE reporting to another UE, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the STCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the transmitted STCH. The delay uncertainty is: 2 x TTISTCH where TTISTCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PSSCH with subframe or slot or subslot duration.

This measurement reporting delay excludes any delay caused by no SL resources for UE to send the measurement report.

The reported SL PRS-RSRP measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clauses TBD.

The SL PRS-RSRP measurements performed and reported according to this section shall meet the SL PRS-RSRP measurement accuracy requirements in clause TBD, for each measured SL-PRS resource.

### 12A.3.5	Measurements Period Requirements

When SL PRS-RSRP measurements are requested via SL-TDOA-RequestLocationInformation the measurement period requirement in clause 12A.2.5 applies.

When SL PRS-RSRP measurements are requested via SL-RTT-RequestLocationInformation the measurement period requirement in clause 12A.4.5 applies.

When SL PRS-RSRP measurements are requested via SL-AoA-RequestLocationInformation the measurement period requirement in clause 12A.6.5 applies.

When SL PRS-RSRP measurements are requested via SL-TOA-RequestLocationInformation the measurement period requirement in clause 12A.7.5 applies.

TSL PRS-RSRP,Total

TSL PRS-RSRP,Total=s=1STSL PRS-RSRP,effect,s

S

S

TSL PRS-RSRP,effect,s=

ts+1-tsTSL PRS-RSRP,effect,s≥Tdur,s+ΔSLprocts+1ts

TSL PRS-RSRP,effect,s=Tdur,s+ΔSLproc ,

Tdur,s

ΔSLproc=[TBD]

TSL PRS-RSRP,Total

----------------------END OF CHANGE----------------------------

----------------------START OF CHANGE----------------------------

## 12A.4	SL-Rx-Tx measurements

### 12A.4.1	Introduction

The requirements in clause 12A.4 shall apply provided the UE has received SL-RTT-RequestLocationInformation from LMF or another UE via SLPP [37] requesting the UE to measure and report SL Rx-Tx time difference measurements defined in TS 38.215 [4] based on SL-PRS.

### 12A.4.2	Requirements Applicability

The requirements in clause 12A.4 apply for [periodic,] aperiodic, and triggered SL Rx-Tx time difference measurements, provided:

-	SL Rx-Tx time difference related side conditions given in clause [TBD] for FR1 are met for a corresponding Band.

-	The actual time difference between the corresponding SL-PRS transmission and reception used to derive the measurement is no larger than [160] ms.

### 12A.4.3	Measurement Capability

SL Rx-Tx time difference measurement capability is as indicated by the UE in SL-RTT-ProvideCapabilities according to TS 38.355 [37].

### 12A.4.4	Measurement Reporting Requirements

The measurement reporting delay is defined as the time between the moment when the measurement report is triggered and the moment when the UE starts to transmit the measurement report over the air interface.

For UE report to LMF, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PSSCH with subframe or slot or subslot duration.

For UE report to another UE, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the STCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the sidelink STCH. The delay uncertainty is: 2 x TTISTCH where TTISTCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PSSCH with subframe or slot or subslot duration.

The measurement reporting delay excludes any delay caused by no SL resources for UE to send the measurement report.

The reported SL Rx-Tx time difference measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clause [TBD].

The SL Rx-Tx time difference measurements performed and reported according to this section shall meet the SL Rx-Tx time difference measurement accuracy requirements in clause [TBD], for each measured SL-PRS resource.

### 12A.4.5	Measurement Period Requirements

When the physical layer receives SL-RTT-ProvideAssistanceData message and SL-RTT-RequestLocationInformation message from LMF or another UE via SLPP [37], the UE shall be able to perform SL Rx-Tx time difference measurements, defined in TS 38.215 [4], during TSL RxTx,total defined as:

TSL RxTx,total=s=1STSL RxTx,effect,s+Tuncertain

Where,

S is the number of samples for a single SL Rx-Tx measurement, defined below:

S = 1 for SL-PRS BW ≥ 48 PRBs,

S = 4 for SL-PRS BW< 48 PRBs

TSL RxTx,effect,s is defined as:

TSL RxTx,effect,s=ts+1-ts, for s<S, provided that TSL RxTx,effect,s≥Tdur,s+ΔSLproc , where ts and ts+1 are the start of the s-th and (s+1)-th slot, respectively, where UE is configured to measure SL-PRS.

TSL RxTx,effect,s=Tdur,s+ΔSLproc , for s=S,

Tdur,s is the duration of the s-th sample, and it is equal to the duration of one slot

ΔSLproc=[TBDcomponent 4 of FG 41-1-1] is the processing time.

Tuncertain is defined as below:

If the UE reports the transmission timestamp of a SL PRS as defined in TS 38.215 [4], and the SL PRS transmission occurs after the SL PRS reception used to derive the measurement, Tuncertain is the additional time delay from the SL PRS reception until the actual SL PRS transmission.

Otherwise, Tuncertain=0.

Editor’s note: The SL PRS measurement period for measurement on SL-PRS for multiple UEs is FFS.

The measurement period TSL RxTx,total applies to each individual SL PRS resource measured by a UE provided the number of active SL PRS resources indicated in [component 2 of FG 41-1-1] and the maximum number of slots containing active SL PRS resources indicated in [component 3 of FG 41-1-1] are not exceeded. If either capability is exceeded while the UE is performing SL Rx-Tx measurements, the measurement period can be longer. For each SL PRS resource, the measurement period TSL RxTx,total starts at the start of the slot containing the first sample of the SL PRS resource.

If the synchronization reference source of the UE changes while the measuring UE is performing the SL Rx-Tx time difference measurement, then the UE shall restart the SL Rx-Tx time difference measurement and

TSL RxTx,restart=K+1*TSL RxTx, Total

the measurement period can be longer.

----------------------END OF CHANGE----------------------------

----------------------START OF CHANGE----------------------------

## 12A.5	SL-RSRPP measurements

### 12A.5.1	Introduction

The requirements in clause 12A.5 shall apply provided the UE has received SL-TDOA-RequestLocationInformation, SL-TOA-RequestLocationInformation, SL-RTT-RequestLocationInformation or SL-AoA-RequestLocationInformation from LMF or another UE via SLPP requesting the UE to measure and report SL PRS-RSRPP measurements defined in TS 38.215 [4].

### 12A.5.2	Requirements Applicability

The requirements in clause 12A.5 apply for [periodic and] triggered SL PRS-RSRPP measurements, provided:

-	SL PRS-RSRPP related side conditions given in clause [TBD] for FR1 are met for a corresponding Band.

### 12A.5.3	Measurement Capability

SL PRS-RSRPP measurement capability is as indicated by the UE in [NR-SL-RSRPP-ProvideCapabilities] according to TS 38.355 [TBD].

### 12A.5.4	Measurement Reporting Requirements

The measurement reporting delay is defined as the time between the moment when the measurement report is triggered and the moment when the UE starts to transmit the measurement report over the air interface.

For UE report to LMF, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PSSCH with subframe or slot or subslot duration.

For UE report to another UE, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the STCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the sidelink STCH. The delay uncertainty is: 2 x TTISTCH where TTISTCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PSSCH with subframe or slot or subslot duration.

This measurement reporting delay excludes any delay caused by no SL resources for UE to send the measurement report.

The reported SL PRS-RSRPP measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clauses [TBD].

The SL PRS-RSRPP measurements performed and reported according to this section shall meet the SL PRS-RSRPP measurement accuracy requirements in clause [TBD], for each measured SL-PRS resource.

### 12A.5.5	Measurement Period Requirements

When SL PRS-RSRPP measurements are requested via SL-TDOA-RequestLocationInformation the measurement period requirement in clause 12A.2.5 applies.

When SL PRS-RSRPP measurements are requested via SL-RTT-RequestLocationInformation the measurement period requirement in clause 12A.4.5 applies.

When SL PRS-RSRPP measurements are requested via SL-AoA-RequestLocationInformation the measurement period requirement in clause 12A.6.5 applies.

When SL PRS-RSRPP measurements are requested via SL-TOA-RequestLocationInformation the measurement period requirement in clause 12A.7.5 applies.

----------------------END OF CHANGE----------------------------

----------------------START OF CHANGE----------------------------

## 12A.6	SL-AoA measurements

### 12A.6.1	Introduction

The requirements in clause 12A.6.5 shall apply provided the UE has received SL-AoA-RequestLocationInformation from LMF or another UE via SLPP requesting the UE to measure and report SL AoA measurements defined in TS 38.215 [4].

### 12A.6.2	Requirements Applicable

The requirements in clause 12A.6 apply for periodic, aperiodic, and triggered SL AoA measurements, provided:

-	SL AoA related side conditions given in clause [TBD] for FR1 are met for a corresponding Band.

### 12A.6.3	Measurement Capability

SL AoA measurement capability is as indicated by the UE in SL-AoA-ProvideCapabilities according to TS 38.355 [37].

### 12A.6.4	Measurement Reporting Requirements

The measurement reporting delay is defined as the time between the moment when the measurement report is triggered and the moment when the UE starts to transmit the measurement report over the air interface.

For UE reporting to LMF, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PSSCH with subframe or slot or subslot duration.

For UE reporting to another UE, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the STCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the sidelink STCH. The delay uncertainty is: 2 x TTISTCH where TTISTCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PSSCH with subframe or slot or subslot duration.

The measurement reporting delay excludes any delay caused by no SL resources or no SL-PRS resources for UE to send the measurement report.

The reported SL AoA measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clauses [TBD].

[The SL AoA measurements performed and reported according to this section shall meet the SL AoA measurement accuracy requirements in clause [TBD], for each measured SL-PRS resource.]

Editor’s note: FFS whether to define accuracy requirements for SLAoA.

### 12A.6.5	Measurement Period Requirements

When the physical layer receives SL-AoA-ProvideAssistanceData message and SL-AoA-RequestLocationInformation message from LMF or another UE via SLPP, the UE shall be able to measure SL AoA measurements, defined in TS 38.215 [4], during TSL AoA,total defined as:

TSL AoA,total=s=1STSL AoA, effect,s ,

where,

S is the number of samples for the SL AoA measurement, defined as below:

S = 1 for SL-PRS BW ≥ 48 PRBs,

S = 4 for L-PRS BW,48 PRBs,

TSL AoA,effect,s is defined as below,

TSL AoA,effect,s=ts+1-ts, for s<S, provided that TSL AoA,effect,s≥Tdur,s+TSLproc , where ts and ts+1 are the start of the s-th and (s+1)-th slot, respectively, where UE is configured to measure SL-PRS,

TSL AoA,effect,s=Tdur,s+ΔSLproc , for s=S,

Tdur,s is the duration of SL-PRS sample s of the SL AoA measurement, and it is equal to the duration of one slot

ΔSLproc=[TBDcomponent 4 of FG 41-1-1] is the processing time.

The measurement period TSL AoA,total applies to each individual SL PRS resource measured by a UE provided the number of active SL PRS resources indicated in [component 2 of FG 41-1-1] and the maximum number of slots containing active SL PRS resources indicated in [component 3 of FG 41-1-1] are not exceeded. If either capability is exceeded while the UE is performing SL AoA measurements, the measurement period can be longer. For each SL PRS resource, the measurement period TSL AoA,total starts at the start of the slot containing the first sample of the SL PRS resource.

The requirements in clause 12A.6 do not apply if the SL-PRS configuration given by [NR-SL-PRS-AssistanceData exceeds] exceeds any of the UE measurement capabilities given by [NR-SL-PRS-ResourcesCapability in NR-SL-AOA-ProvideCapabilities].

If the synchronization reference source of the UE changes while the UE is performing the SL AoA measurement, then the UE shall continue and complete the on-going SL AoA measurements while meeting the measurement period requirement in this clause[ and the accuracy requirements in clause [TBD]].

----------------------END OF CHANGE----------------------------

----------------------START OF CHANGE----------------------------

## 12A.7	SL-RTOA measurements

### 12A.7.1	Introduction

The requirements in clause 12A.7.5 shall apply provided the UE has received SL-TOA-RequestLocationInformation from LMF or another UE via SLPP requesting the UE to measure and report SL RTOA measurements defined in TS 38.215 [4].

### 12A.7.2	Requirements Applicable

The requirements in clause 12A.7 apply for periodic, aperiodic, and triggered SL RTOA measurements, provided:

-	SL RTOA related side conditions given in clause [TBD] for FR1 are met for a corresponding Band.

### 12A.7.3	Measurement Capability

SL RTOA measurement capability is as indicated by the UE in SL-TOA-ProvideCapabilities according to TS 38.355 [37].

### 12A.7.4	Measurement Reporting Requirements

The measurement reporting delay is defined as the time between the moment when the measurement report is triggered and the moment when the UE starts to transmit the measurement report over the air interface.

For UE reporting to LMF, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PSSCH with subframe or slot or subslot duration.

For UE reporting to another UE, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the STCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the sidelink STCH. The delay uncertainty is: 2 x TTISTCH where TTISTCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PSSCH with subframe or slot or subslot duration.

The measurement reporting delay excludes any delay caused by no SL resources or no SL-PRS resources for UE to send the measurement report.

The reported SL RTOA measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clauses [TBD].

[The SL RTOA measurements performed and reported according to this section shall meet the SL RTOA measurement accuracy requirements in clause [TBD], for each measured SL-PRS resource.]

Editor’s note: FFS whether to define accuracy requirements for SL RTOA.

### 12A.7.5	Measurement Period Requirements

When the physical layer receives SL-TOA-ProvideAssistanceData message from SL-TOA-RequestLocationInformation message from LMF or another UE via SLPP, the UE shall be able to measure SL RTOA measurements, defined in TS 38.215 [4], during TSL RTOA,total defined as:

TSL RTOA,total=s=1STSL RTOA, effect,s ,

where,

S is the number of samples for the SL RTOA measurement, defined as below:

S = 1 for SL-PRS BW ≥ 48 PRBs,

S = 4 for SL-PRS BW < 48 PRBs,

TSL RTOA,effect,s is defined as below,

TSL RTOA,effect,s=ts+1-ts, for s<S, provided that TSL RTOA,effect,s≥Tdur,s+TSLproc , where ts and ts+1 are the start of the s-th and (s+1)-th slot, respectively, where UE is configured to measure SL-PRS,

TSL RTOA,effect,s=Tdur,s+ΔSLproc , for s=S,

Tdur,s is the duration of SL-PRS sample s of the SL RTOA measurement, and it is equal to the duration of one slot

ΔSLproc=[TBDcomponent 4 of FG 41-1-1] is the processing time.

The measurement period TSL RTOA,total applies to each individual SL PRS resource measured by a UE provided the number of active SL PRS resources indicated in [component 2 of FG 41-1-1] and the maximum number of slots containing active SL PRS resources indicated in [component 3 of FG 41-1-1] are not exceeded. If either capability is exceeded while the UE is performing SL RTOA measurements, the measurement period can be longer. For each SL PRS resource, the measurement period TSL RTOA,total starts at the start of the slot containing the first sample of the SL PRS resource.

The requirements in clause 12A.6 do not apply if the SL-PRS configuration given by [NR-SL-PRS-AssistanceData exceeds] exceeds any of the UE measurement capabilities given by [NR-SL-PRS-ResourcesCapability in NR-SL-RTOA-ProvideCapabilities].

If the synchronization reference source changes at the measuring UE, while the UE is performing the SL RTOA measurements, the UE shall restart the SL RTOA measurements after the synchronization reference source change and the measurement period can be longer.

----------------------END OF CHANGE----------------------------