3GPP TSG-RAN WG4 Meeting #110 					                  R4-2401234

Athens, GR, 26 Feb - 01 Mar, 2024

Source:	China Telecom

Title:	Discussion on combination of HST and RRM relaxation

Agenda Item:	12.1.1

Document for:	Discussion

- Introduction

In RAN4 #109, the discussion on combination of HST and RRM relaxation was triggered by RAN2 LS [1], and the options for further discussion are captured in the WF [2]. This contribution presents our views.

- Discussion

The following agreements were reached in RAN4 #109 [2]:

RAN4 confirms that the configuration of both HST and relaxed RRM measurements criteria is supported from RAN4 perspective.

Companies are encouraged to further discuss the following options on whether and how to clarify UE measurement behavior when both HST and relaxed RRM measurement criteria are enabled:

- Option 1: clarify that UE is allowed to enable relaxed measurement when both HST and low mobility evaluation are enabled. This has no negative impact on UE in high mobility mode, since the relaxed measurement criteria are unlikely to be met.

- Option 2: clarify that existing RRM requirements are not applicable if both HST and relaxed RRM measurements criteria are enabled.

Firstly, from operator’s perspective, we agree it is useful to support the configuration of both HST and relaxed RRM measurements criteria. In the real deployments, the BSs along the railway can be deployed to serve the UEs in the high-speed train, the UEs in the train station as well as the normal UEs outside the train & the train station, thus it is typical to have non-HST UEs under the coverage of HST BSs.

Regarding the two options, we think option 2 with modified wording is sufficient, i.e., no RAN4 requirements are not applicable if both HST and relaxed RRM measurements criteria are enabled. With this, in addition to clarify the existing RAN4 requirement, it also means that RAN4 is not planning to define new requirements if both HST and relaxed RRM measurements criteria are enabled.

Proposal: Clarify that no RAN4 requirements are not applicable if both HST and relaxed RRM measurements criteria are enabled.

- Conclusion

The contribution discussed the combination of HST and RRM relaxation, with the following proposal:

Proposal: Clarify that no RAN4 requirements are not applicable if both HST and relaxed RRM measurements criteria are enabled.

- References

- R4-2318022, LS on combination of HST and RRM relaxation, RAN2

- R4-2321415, WF on combination of HST and RRM relaxation, Apple