3GPP TSG-RAN WG4 Meeting # 110                                   R4-2401238

# Athens, Greece, February 26 – March 01, 2024

Agenda item:	8.5.1.2

Source:	China Telecom

Title:	Discussion on UE capabilities for Case 2 NCSG and concurrent MG

Document for:	Discussion

- Introduction

In RAN4 #109 meeting, UE capabilities for pre-configured MGs, multiple concurrent MGs and NCSG were discussed and the WF [1] was approved.

In this paper, we present our views about UE capabilities for Case 2 NCSG and concurrent MG.

- Discussion

Feature list for Case 2 NCSG and concurrent MG

Status in the WF [1]:

Issue 1-1-1: Rel-18 NR UE features for NR_MG_enh2 WI for Objective 1: < Way Forward > - The following feature list will be used as a starting for further discussion in the next meeting Index; Feature group; Components; Prerequisite feature groups; Need for the gNB to know if the feature is supported; Applicable to the capability signalling exchange between UEs (V2X WI only)”.; Consequence if the feature is not supported by the UE; Type; Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Capability interpretation for mixture of FDD/TDD and/or FR1/FR2; Note; Mandatory/Optional. [32-1]; Concurrent gap with Pre-MG in a FR; Support of multiple per-UE (or per-FR) measurement gap patterns with at least one per-UE (or per-FR) Pre-MG. Detail in Clause [9.1.x.2] of TS 38.133.; 19-3-x and 19-2 x = 1 or 2; Yes; No; Network should not configure concurrent gap with Pre-MG; Per UE; No; No; N.A; Optional with capability signalling. [32-2]; 2 Pre-MG configuration with simultaneous activation/deactivation; Support configurations of 2 Pre-MG with simultaneous activation/deactivation in the same FR.; 32-1; Yes; No; Unknown activation time for simultaneous Pre-MG is expected; Per UE; No; No; N.A; Optional with capability signalling. [32-3]; Dynamic collision; Support the RRM requirements when the activation/deactivation delay of Pre-MG overlaps the other measurement gap with lower priority or Pre-MG; 32-1; Yes; No; UE is not expected to meet the requirements; Per UE; No; No; N.A; Optional with capability signalling. [32-4]; Concurrent gap with NCSG in a FR; Support of multiple per-UE (or per-FR) measurement gap patterns with at least one per-UE (or per-FR) NCSG. Detail in clause [9.1.y.2] of TS 38.133.; 19-1 and 19-2; Yes; No; Network should not configure concurrent gap with NCSG; Per UE; No; No; N.A; Optional with capability signalling. [32-5]; 2 NCSG configuration in a FR; Support configurations of 2 NCSG in the same FR; 32-4; Yes; No; Network should not configure 2 NCSG in the same FR; Per UE; No; No; N.A; Optional with capability signalling.

For concurrent gap with NCSG in a FR, it was agreed to consider a new capability for concurrent gaps with NCSG in the same FR in previous meeting.

For two NCSGs configuration in a FR, as for whether to consider an additional capability for NCSG + NCSG in the same FR, since the parallel measurement upon NCSGs has been agreed not to be supported in Rel-18, in our views, there is no need to consider an additional capability for NCSG + NCSG in the same FR.

Observation 1: For concurrent gap with NCSG in a FR, it was agreed to consider a new capability for concurrent gaps with NCSG in the same FR in previous meeting.

Observation 2: For two NCSGs configuration in a FR, as for whether to consider an additional capability for NCSG + NCSG in the same FR, since the parallel measurement upon NCSGs has been agreed not to be supported in Rel-18, there is no need to consider an additional capability for NCSG + NCSG in the same FR.

Proposal 1: Feature list for Case 2 NCSG and concurrent MG:

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [32-4] | Concurrent gap with NCSG in a FR | Support of multiple per-UE (or per-FR) measurement gap patterns with at least one per-UE (or per-FR) NCSG. Detail in clause [9.1.y.2] of TS 38.133. | 19-1 and 19-2 | Yes | No | Network should not configure concurrent gap with NCSG | Per UE | No | No | N.A |  | Optional with capability signalling |

- Conclusion

The contribution presents our views about RRM performance requirements for pre-configured MGs, multiple concurrent MGs and NCSG, with the following observations and proposals:

Observation 1: For concurrent gap with NCSG in a FR, it was agreed to consider a new capability for concurrent gaps with NCSG in the same FR in previous meeting.

Observation 2: For two NCSGs configuration in a FR, as for whether to consider an additional capability for NCSG + NCSG in the same FR, since the parallel measurement upon NCSGs has been agreed not to be supported in Rel-18, there is no need to consider an additional capability for NCSG + NCSG in the same FR.

Proposal 1: Feature list for Case 2 NCSG and concurrent MG:

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [32-4] | Concurrent gap with NCSG in a FR | Support of multiple per-UE (or per-FR) measurement gap patterns with at least one per-UE (or per-FR) NCSG. Detail in clause [9.1.y.2] of TS 38.133. | 19-1 and 19-2 | Yes | No | Network should not configure concurrent gap with NCSG | Per UE | No | No | N.A |  | Optional with capability signalling |

- References

- R4-2321640, WF for NR_MGE-2_part1, MediaTek inc., RAN WG4 Meeting #109, November 2023.