3GPP TSG-RAN4 Meeting #110	R4-2401302

# Athens, Greece,  Feb 26th 2024 - Mar 1st 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 4101 | rev | - | Current version: | 17.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_redcap-Core) Correction to RedCap core requirements_R17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap-Core |  |  |  |  |  | Date: |  |  | 2024-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - FR2 Rx beam sweeping scaling factor N is not considered in FR2 RedCap inter-frequency blind HO requirements. We suggest align with legacy FR2-1 HO requirements, i.e. N = 8 for Tsearch. - Interruption requirements for CGI measurement w/ autonomous gap specified in cl 8.2.2.2.14 is referred in 9.11A.2. However, RedCap UE only support single carrier operation. The clause referred here should be the scheduling restriction reuqirements in cl 9.11A.4. - CGI reading requirements for 1Rx RedCap only applies in FR1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - FR2 Rx beam sweeping scaling factor N = 8 is considered in FR2 RedCap inter-frequency blind HO requirements. - Wrong reference in cl 9.11A.2 is corrected. - CGI reading requirements for 1Rx RedCap in FR2 is removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Spec is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1D.1.3, 9.11A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

6.1D.1.3.2	Interruption time

The interruption time is the time between end of the last TTI containing the RRC command on the old PDSCH and the time the UE starts transmission of the new PRACH, excluding the RRC procedure delay.

When intra-frequency or inter-frequency handover is commanded, the interruption time shall be less than Tinterrupt

Tinterrupt = Tsearch + TIU + Tprocessing  + T∆ + Tmargin ms

Where:

Tsearch is the time required to search the target cell which depends on whether the target cell is already known when the handover command is received by the UE.

-	If the target cell is a known intra-frequency cell, Tsearch = 0ms. If the target cell is an unknown intra-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = 8*Trs ms.

-	If the target cell is a known inter-frequency cell, then

if the measured SSB is the target SSB for HO of the target cell, Tsearch = 0ms;

if the measured SSB and the target SSB for HO belong to the same NR target cell and fulfil the following conditions, Tsearch = Trs ms:

    - CD-SSB in initial DL BWP is the measured SSB and NCD-SSB in RedCap specific DL BWP is the target SSB for HO

    - NCD-SSB in RedCap specific DL BWP is the measured SSB and CD-SSB in initial DL BWP is the target SSB for HO

    - Both measured SSB and the target SSB for HO are NCD-SSBs within different RedCap specific DL BWPs

Otherwise, the target cell is an unknown inter-frequency cell. In this case, if the target cell Es/Iot≥-2 dB, then Tsearch = 24* Trs ms.

Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times.

T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = Trs for both known and unknown target cell.

Tprocessing is time for UE processing. Tprocessing can be up to 20ms.

Tmargin is time for SSB post-processing. Tmargin can be up to 2ms.

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [3].

Trs is the SMTC periodicity of the target NR cell if the UE has been provided with an SMTC configuration for the target cellin the handover command, otherwise,

-	Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing as NCD-SSB indicated by nonCellDefiningSSB-r17 if the first active DL BWP included in handover command is configured with nonCellDefiningSSB-r17, otherwise, as CD-SSB indicated by absoluteFrequencySSB in frequencyInfoDL in handover command.

-	If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs=5ms assuming the SSB transmission periodicity is 5ms. There is no requirement if the SSB transmission periodicity is not 5ms. If the UE has been provided with higher layer in TS 38.331 [2] signaling of smtc2 prior to the handover command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell.

In FR2, the target cell is known if it has been meeting the following conditions:

-	At least one of the SSBs measured belongs to the same NR target cell,

-	During the last 5 seconds before the reception of the handover command:

-	the UE has sent a valid measurement report for the target cell and

-	at least one of the SSBs measured which belongs to the same NR target cell remains detectable according to the cell identification requirements as described in clause 9.2B for intra-frequency measurements and clause 9.3B for inter-frequency measurements,

-	The reference SSB of the NR target cell also remains detectable during the handover delay according to the cell identification requirements are described in clause 9.2B for intra-frequency handover and clause 9.3B for inter-frequency handover.

Otherwise, it is unknown.

<End of modified section 1>

<Start of modified section 2>

9.11A.2	CGI identification of an NR cell with autonomous gaps

The UE shall identify and report the CGI of a known NR target cell when requested by the network for the purpose of reportCGI. Only one cell is provided to the UE with cellForWhichToReportCGI for identifying the CGI.The UE may make autonomous gaps in both downlink reception and uplink transmission for receiving MIB and SIB1 message according to clause 5.5.3 of TS 38.331 [2]. Note that a UE is not required to use autonomous gap if useAutonomousGaps is set to false. If autonomous gaps are used for measurement with the purpose of reportCGI, regardless of whether DRX is used or not, the UE shall be able to identify a new CGI of NR cell within:

Tidentify_CGI_redcap = (TMIB_redcap + T SIB1_redcap) ms

Where:

For 2 Rx RedCap UE:

TMIB_redcap is the time period used to acquire MIB message. TMIB_redcap = 6 * TSMTC ms for target cell carrier frequency on FR1 and TMIB_redcap = 25 * TSMTC ms for target cell carrier frequency on FR2.

TSIB1_redcap is the time period used to acquire SIB1 message. TSIB1_redcap = 6 * TRMSI-scheduling ms.

For 1 Rx RedCap UE:

TMIB_redcap is the time period used to acquire MIB message. TMIB_redcap = 6 * TSMTC ms for target cell carrier frequency on FR1.

TSIB1_redcap is the time period used to acquire SIB1 message. TSIB1_redcap = 12 * TRMSI-scheduling ms.

Where TSMTC is the SMTC periodicity configured for the target cell measurement, and TRMSI-scheduling is

-	the periodicity with which the SIB1 is actually transmitted by the NR target cell when SSB and RMSI CORESET multiplexing pattern is 1

-	the maximum between the periodicity with which the SIB1 is actually transmitted by the NR target cell and TSMTC when SSB and RMSI CORESET multiplexing pattern is 2 or 3.

The requirement for identifying the CGI of an NR cell within Tidentify_CGI_redcap is applicable when no DRX is used as well as when any of the DRX cycles specified in TS 38.331 [2] is used.

Within the time Tidentify_CGI_redcap, over which the UE identifies the CGI of an NR cell, the UE shall fulfil interruption requirements specified in,

-	Clause 9.11A.4if the UE is configured with SA operation mode.

In the requirement a cell is known if,

-	During the last 5 seconds for FR1 or 3 seconds for FR2 before the reception of the report CGI command:

-	The UE has sent a valid L3-RSRP measurement report with SSB index for the target cell and

-	During MIB decoding at least reported SSBs remains detectable according to the cell identification conditions specified in clauses 9.2B or 9.3B of TS 38.133, and

-	During SIB1 decoding the SSB used for MIB decoding remains detectable according to the cell identification conditions specified in clauses 9.2B or 9.3B of TS 38.133, and

-	During MIB decoding, the SSB for MIB decoding remains detectable with SNR ≥-3dB

-	During SIB1 decoding, the PDSCH for SIB1 decoding remains detectable with SNR ≥-3dB

<End of modified section 2>