3GPP TSG-RAN WG4 Meeting #110	R4-2401374

Athens, Greece, Feb 26th – Mar 1st, 2024

Title: 	Discussion on RRM test cases for NR NCR-MT

Source: 	Huawei, HiSilicon

Agenda item:	8.20.5

Document for:	Discussion

Introduction

In RAN4, the RRM core requirements for NR NCR-MT have been defined in TS38.106. In this contribution, we will provide the discussion on RRM test cases for NR NCR-MT in R18.

Discussion

In R18, the following RRM core requirements have been defined in [1] for NR NCR-MT:

- RRC Re-establishment - Random access - Timing requirements - Radio Link Monitoring - Link recovery procedure

The above RRM requirements for NCR-MT are reused from the corresponding RRM requirements for legacy UE or IAB-MT. Similarly, how to define RRM test cases for NCR-MT also can be derived from the principles of the existing RRM test cases for legacy UE or IAB-MT. In current RRC re-establishment test cases for legacy UE or IAB-MT, the RRC re-establishment delay is verified via the moment when the UE starts to send PRACH preambles. So, the behaviour of triggering random access procedure also can be verified in RRC re-establishment tests. So, there is no need to define a new test for verifying random access procedure. The current test case list for IAB-MT can be used as baseline for verifying the functionalities of NCR-MT.

Proposal 1: The following test case list are suggested to verify the RRM requirements of NR NCR-MT:

| No | Requirements | Type of Test | Basic test configuration |
| --- | --- | --- | --- |
| 1 | RRC Re-establishment | RRC Re-establishment for NR NCR-MT | Developed based on the existing NR IAB-MT tests for in TS38.106 |
| 2 | Transmit Timing | NR NCR-MT transmit timing tests for FR1 | Developed based on the existing NR IAB-MT transmit timing tests for FR1 in TS38.106 |
| 3 |  | NR NCR-MT transmit timing tests for FR2 | Developed based on the existing NR IAB-MT transmit timing tests for FR2 in TS38.106 |
| 4 | Radio Link Monitoring | NR NCR-MT SSB based RLM out-of-sync test for FR1 | Developed based on the existing NR IAB-MT SSB based RLM out-of-sync test for FR1 in TS38.106 |
| 5 |  | NR NCR-MT SSB based RLM in-sync test for FR1 | Developed based on the existing NR IAB-MT SSB based RLM in-sync test for FR1 |
| 6 |  | NR NCR-MT CSI-RS based RLM out-of-sync test for FR1 | Developed based on the existing NR IAB-MT CSI-RS based RLM out-of-sync test for FR1 in TS38.106 |
| 7 |  | NR NCR-MT CSI-RS based RLM in-sync test for FR1 | Developed based on the existing NR IAB-MT CSI-RS based RLM in-sync test for FR1 in TS38.106 |
| 8 |  | NR NCR-MT SSB based RLM out-of-sync test for FR2 | Developed based on the existing NR IAB-MT SSB based RLM out-of-sync test for FR2 in TS38.106 |
| 9 |  | NR NCR-MT SSB based RLM in-sync test for FR2 | Developed based on the existing NR IAB-MT SSB based RLM in-sync test for FR2 in TS38.106 |
| 10 |  | NR NCR-MT CSI-RS based RLM out-of-sync test for FR2 | Developed based on the existing NR IAB-MT CSI-RS based RLM out-of-sync test for FR2 in TS38.106 |
| 11 |  | NR NCR-MT CSI-RS based RLM in-sync test for FR2 | Developed based on the existing NR IAB-MT CSI-RS based RLM in-sync test for FR2 in TS38.106 |
| 12 | Link recovery procedure | NR NCR-MT SSB based BFD and link recovery test for FR1 | Developed based on the existing NR IAB-MT SSB based BFD and link recovery test for FR1 in TS38.106 |
| 13 |  | NR NCR-MT CSI-RS based BFD and link recovery test for FR1 | Developed based on the existing NR IAB-MT CSI-RS based BFD and link recovery test for FR1 in TS38.106 |
| 14 |  | NR NCR-MT SSB based BFD and link recovery test for FR2 | Developed based on the existing NR IAB-MT SSB based BFD and link recovery test for FR2 in TS38.106 |
| 15 |  | NR NCR-MT CSI-RS based BFD and link recovery test for FR2 | Developed based on the existing NR IAB-MT CSI-RS based BFD and link recovery test for FR2 in TS38.106 |

Conclusions

In this paper we provided the RRM test case list for verifying RRM requirements on NCR-MT.

Proposal 1: The following test case list are suggested to verify the RRM requirements of NR NCR-MT:

| No | Requirements | Type of Test | Basic test configuration |
| --- | --- | --- | --- |
| 1 | RRC Re-establishment | RRC Re-establishment for NR NCR-MT | Developed based on the existing NR IAB-MT tests for in TS38.106 |
| 2 | Transmit Timing | NR NCR-MT transmit timing tests for FR1 | Developed based on the existing NR IAB-MT transmit timing tests for FR1 in TS38.106 |
| 3 |  | NR NCR-MT transmit timing tests for FR2 | Developed based on the existing NR IAB-MT transmit timing tests for FR2 in TS38.106 |
| 4 | Radio Link Monitoring | NR NCR-MT SSB based RLM out-of-sync test for FR1 | Developed based on the existing NR IAB-MT SSB based RLM out-of-sync test for FR1 in TS38.106 |
| 5 |  | NR NCR-MT SSB based RLM in-sync test for FR1 | Developed based on the existing NR IAB-MT SSB based RLM in-sync test for FR1 |
| 6 |  | NR NCR-MT CSI-RS based RLM out-of-sync test for FR1 | Developed based on the existing NR IAB-MT CSI-RS based RLM out-of-sync test for FR1 in TS38.106 |
| 7 |  | NR NCR-MT CSI-RS based RLM in-sync test for FR1 | Developed based on the existing NR IAB-MT CSI-RS based RLM in-sync test for FR1 in TS38.106 |
| 8 |  | NR NCR-MT SSB based RLM out-of-sync test for FR2 | Developed based on the existing NR IAB-MT SSB based RLM out-of-sync test for FR2 in TS38.106 |
| 9 |  | NR NCR-MT SSB based RLM in-sync test for FR2 | Developed based on the existing NR IAB-MT SSB based RLM in-sync test for FR2 in TS38.106 |
| 10 |  | NR NCR-MT CSI-RS based RLM out-of-sync test for FR2 | Developed based on the existing NR IAB-MT CSI-RS based RLM out-of-sync test for FR2 in TS38.106 |
| 11 |  | NR NCR-MT CSI-RS based RLM in-sync test for FR2 | Developed based on the existing NR IAB-MT CSI-RS based RLM in-sync test for FR2 in TS38.106 |
| 12 | Link recovery procedure | NR NCR-MT SSB based BFD and link recovery test for FR1 | Developed based on the existing NR IAB-MT SSB based BFD and link recovery test for FR1 in TS38.106 |
| 13 |  | NR NCR-MT CSI-RS based BFD and link recovery test for FR1 | Developed based on the existing NR IAB-MT CSI-RS based BFD and link recovery test for FR1 in TS38.106 |
| 14 |  | NR NCR-MT SSB based BFD and link recovery test for FR2 | Developed based on the existing NR IAB-MT SSB based BFD and link recovery test for FR2 in TS38.106 |
| 15 |  | NR NCR-MT CSI-RS based BFD and link recovery test for FR2 | Developed based on the existing NR IAB-MT CSI-RS based BFD and link recovery test for FR2 in TS38.106 |

Reference

- R4-2321649, “Big CR to TS 38.106 on RRM core requirements for NR network-controlled repeaters”, ZTE Corporation, Ericsson, Nokia