3GPP TSG-RAN WG4 Meeting #110	R4-2401405

# Athens, Greece, February 26th – March 1st 2024

Source:	Ericsson

Title:	Simulation results for less than 5MHz BS demodulation requirements

Agenda item:	8.10.6.2

Document for:	Information

# Introduction

According to the WF in RAN4#109 [1], some simulation assumptions are agreed for initial simulations.

In this contribution, simulation results are provided to check the performance difference between 5MHz and 3MHz for PUSCH.

# Simulation results

## 2.1	Normal PUSCH

Table 2.1-1 Simulation parameters for normal PUSCH

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | Pos0, pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain | PUSCH mapping type | A |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | 3MHz: 12PRB, 15PRB |
| assignment | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL pattern. |  |  |

Table 2.1-2 Simulation results for normal PUSCH

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | MCS | Additional DM-RS position | SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 5MHz 25RB | 3MHz 12RB | 3MHz 15RB |
| 1 | 2 | Normal | TDLB100-400 Low | 70 % | 2 | Pos0 | -0.24 | 0.29 | 0.30 |
|  |  |  |  |  |  | Pos1 | -5.03 | -4.50 | -4.76 |
|  |  | Normal | TDLC300-100 Low | 70 % | 16 | Pos0 | 9.71 | 8.55 | 8.69 |
|  |  |  |  |  |  | Pos1 | 7.89 | 7.71 | 7.48 |
|  |  | Normal | TDLA30-10 Low | 70 % | 20 | Pos0 | 9.61 | 9.26 | 9.41 |
|  |  |  |  |  |  | Pos1 | 9.36 | 9.18 | 9.15 |

## 2.2 	HST PUSCH

Table 2.2-1 Simulation parameters for HST PUSCH

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos2 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | 0 |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain | PUSCH mapping type | A |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | 3MHz: 12 PRB 5MHz: 25 PRB |
| assignment | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL pattern. |  |  |

Table 2.2-2 Simulation results for HST PUSCH

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions (Annex G) | Fraction of maximum throughput | MCS | Additional DM-RS position |  | SNR(dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | 5MHz 25RB | 3MHz 12RB |
| 1 | 1 | Normal | HST Scenario 3-NR500 | 70 % | 2 | pos2 | -3.60 |  | -3.45 |
|  |  | Normal | HST Scenario 1-NR500 | 70 % | 2 | pos2 | -3.75 |  | -3.58 |
|  | 2 | Normal | HST Scenario 1-NR500 | 70 % | 16 | pos2 | 6.37 |  | 6.19 |
|  |  | Normal | HST Scenario 3-NR500 | 70 % | 2 | pos2 | -6.58 |  | -6.41 |
|  |  | Normal | HST Scenario 3-NR500 | 70 % | 16 | pos2 | 7.41 |  | 7.17 |

Table 2.2-3 Simulation results for HST PUSCH with fading channel

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions (Annex G) | Fraction of maximum throughput | MCS | Additional DM-RS position | SNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 5MHz 25RB | 3MHz 12RB | 3MHz 15RB |
| 1 | 2 | Normal | TDLC300-600 | 70 % | 2 | pos2 | -5.13 | -4.18 | -4.44 |

## 2.3 	UL TA

Table 2.3-1 Simulation parameters for UL TA

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Uplink-downlink allocation for TDD |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U |
| Channel bandwidth |  | 15 kHz SCS: 3Mhz |
| MCS |  | 16 |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | DM-RS position (l0) | 2 |
|  | Additional DM-RS position | pos2 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
| Time domain resource assignment | DM-RS sequence generation | NID0=0, nSCID =0 for moving UE NID0=1, nSCID =1 for stationary UE |
|  | PUSCH mapping type | Both A |
|  | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | 3 MHz CBW/15kHz SCS: 6 RB for each UE 5 MHz CBW/15kHz SCS:12 RB for each UE |
|  | Starting PRB index | Moving UE: 0 Stationary UE: 6 for 3MHz, 12 for 5MHz |
|  | Frequency hopping | Disabled |
| SRS resource allocation | Slots in which sounding RS is transmitted (Note 1) | For FDD: slot #1 in radio frames For TDD: last symbol in slot #3 in radio frames for 15KHz |
|  | SRS resource allocation | 15 kHz SCS: CSRS = 5, BSRS =0, for 20 RB |
| NOTE 1. The transmission of SRS is optional. And the transmission comb and SRS periodicity are configured as KTC = 2, and TSRS = 10 for 15 kHz SCS, TSRS = 20 for 30 kHz SCS respectively. |  |  |

Table 2.3-2 Scenario parameters for UL TA

| Parameter | Scenario Z |
| --- | --- |
| Channel model | Stationary UE: AWGN Moving UE: AWGN |
| UE speed | 500 km/h |
| CP length | Normal |
| A | 15 kHz: 10 s |
|  | 15 kHz: 0.18 s-1 |

Table 2.3-3 Simulation results for UL TA

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Mapping type | SCS [kHz] | Moving propagation conditions and correlation matrix (Annex G) | MCS | SNR [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 5MHz 12RB | 3MHz 6RB |  |
| 1 | 2 | Normal | A | 15 | Scenario Z | 16 | 5.4 |  | 5.6 |

# Conclusions

In the previous sections we made the following observations:

Based on the discussion in the previous sections we propose the following:

No table of figures entries found.

# References

[1]	R4-2321064, Way Forward for NR_FR1_lessthan_5MHz_BW_demod, Nokia, Nokia Shanghai Bell