3GPP TSG-RAN WG4 Meeting #110 	R4-2401409

# Athens, Greece,  26th February - 1st March, 2024

Source:	Ericsson

Title:	Simulation results for Rel-18 NR PRACH repetitions demodulation requirements

Agenda item:	8.19.2

Document for:	Information

# Introduction

In RAN4#108bis meeting, companies kicked off the discussion on NR PRACH Rel-18 enhancement for BS demodulation requirements. Some initial simulation results were delivered in RAN#109 meeting for format B4 [1]. Based on the WF [2], further results are delivered in this meeting.

# Simulation result

## 2.1	FR2-1 PRACH repetition

Table 2.1-1: Test parameters for PRACH repetition demodulation requirement

| PRACH | PRACH SCS | Time error tolerance |  |
| --- | --- | --- | --- |
| preamble | (kHz) | AWGN | TDLA30-300 |
| B4, A2, C2 | 60 | 0.13 us | 0.28 us |
|  | 120 | 0.07 us | 0.22 us |

Table 2.1-2: PRACH repetition missed detection requirements for Normal Mode, 60 kHz SCS in FR2-1

| Number | Number of | Propagation | Frequency |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| of TX antennas | demodulation branches | conditions and correlation matrix (Annex G) | offset | Repetition | SNR |  |  |
|  |  |  |  |  | Format B4 | Format A2 | Format C2 |
| 1 | 2 | AWGN | 0 | 2 | -22.2 | -17.9 | -17.8 |
|  |  | TDLA30-300 Low | 4000 Hz | 2 |  | -10.3 | -10.6 |

Table 2.1-3: PRACH repetition missed detection requirements for Normal Mode, 120 kHz SCS in FR2-1

| Number | Number of | Propagation | Frequency |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| of TX antennas | demodulation branches | conditions and correlation matrix (Annex G) | offset | Repetition | SNR |  |  |
|  |  |  |  |  | Format B4 | Format A2 | Format C2 |
| 1 | 2 | AWGN | 0 | 2 | -22.2 | -17.7 | -17.7 |
|  |  | TDLA30-300 Low | 4000 Hz | 2 | -12.8 | -10.6 | -10.9 |

## 2.2	FR1 PRACH repetition

Table 2.2-1: Test parameters for PRACH repetition demodulation requirement

| PRACH | PRACH SCS | Time error tolerance |
| --- | --- | --- |
| preamble | (kHz) | TDLC300-100 |
| B4, A2, C2 | 15 | 2.03 us |
|  | 30 | 1.77 us |

Table 2.2-2: PRACH repetition missed detection requirements for Normal Mode, 15 kHz SCS in FR1

| Number | Number of | Propagation | Frequency |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| of TX antennas | demodulation branches | conditions and correlation matrix (Annex G) | offset | Repetition | SNR |  |  |
|  |  |  |  |  | Format B4 | Format A2 | Format C2 |
| 1 | 2 | TDLC300-100 Low | 400 Hz | 2 | -15.7 | -11.3 | -11.6 |

Table 2.2-3: PRACH repetition missed detection requirements for Normal Mode, 30 kHz SCS in FR1

| Number | Number of | Propagation | Frequency |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| of TX antennas | demodulation branches | conditions and correlation matrix (Annex G) | offset | Repetition | SNR |  |  |
|  |  |  |  |  | Format B4 | Format A2 | Format C2 |
| 1 | 2 | TDLC300-100 Low | 400 Hz | 2 | -16.2 | -11.4 | -11.3 |

# Conclusions

Simulation results for Rel-18 NR PRACH enhancement demodulation requirements are delivered.

# References

[1] R4-2319311, “Simulation results for Rel-18 NR PRACH repetitions demodulation requirements”, Ericsson, RAN4#109, Chicago, USA

[2] R4-2321061, “WF on NR_cov_enh2_demod”, China Telecom, Nokia, RAN4#109, Chicago, USA