3GPP TSG-RAN WG4 Meeting #110	R4-2401543
Athens, GR, 26 Feb – 01 Mar, 2024

Agenda item:	8.11.1.1

Source:	vivo, CAICT, Huawei, HiSilcon, Xiaomi, OPPO, MediaTek

Title:	Performance metric for Coherent UL MIMO

Document for:	Approval

1	Introduction

It was agreed that the RAN4 will focus on performance metric discussion for coherent UL-MIMO with a goal to select a single metric as baseline in Rel-18 [1]. The corresponding comparison criteria to assist in down-selection should be discussed and decided.

This contribution provides views on how to handle these two different performance metrics.

2	Discussion

The non-coherent UL-MIMO TRP test method has been defined in TR 38.870 v18.0.0, section 7.4.3.3, fixed TPMI index 2 is configured, the performance metric is TRP.

The TxD TRP test method has been defined in TR 38.870 v18.0.0, section 7.4.3.2, with 2Tx configured, the performance metric is TRP.

The common test procedure for coherent UL MIMO has been defined in section 7.4.3.3, with 2 or 4 TPMI index based on UE declaration. There are two options for performance metric: Option 1, averaging TRPs; Option 2, Max EIRPs.

To make the decision on the selection of a proper performance metric for coherent UL-MIMO, we propose to consider the following comparison criteria:

- Testing time

- Performance metric consistency

- Regulatory impacts (FR1 TRP concept)

- Statistical properties (typical case, or ideal case)

- Alignment with other SDO conclusions for 2Tx

- OEM antenna design

- Operator network deployment

- Correlation effects removal

The comparison of two options has been summarized in Table 1:

Table 1: Performance metric selection based on comparison criteria

| Comparison criteria | Option 1, averaging TRPs | Option 2, Max EIRPs |
| --- | --- | --- |
| 1. Testing time | - Good Save 50%~30% testing time | - Bad |
| 2. Performance metric consistency | - Good Aligned with other 2Tx test cases (non-coherent UL MIMO and TxD TRP) defined in TR 38.870 Aligned with 1Tx test cases, the advantages of single-layer UL MIMO performance can be estimated. | - Bad not comparable with 1Tx and other 2Tx test cases |
| 3. Regulatory impacts | - Good Aligned with FR1 regulatory radiated power measurement | - Bad |
| 4. Statistical properties | - Good The averaged performance is close to the statistical performance of UE at a typical case in the field | - Bad The max performance metric assumes a perfect network scheduling with ideal (smartest) algorithm of different gNB, which is a corner case SRS-based TPMI has been precluded |
| 5.Alignment with Other SDOs | - Good Aligned with CTIA spec and initial CCSA conclusions for 2Tx test case | - Bad New metric, not adopted in any SDO |
| 6.OEM antenna design | - Good Antenna design to meet typical performance | - Bad Antenna design assumes a perfect gNB indication, but this may never happen in real scenario |
| 7.Operator network deployment | - Good Typical UE performance for coverage budget estimation to guide network deployment | - Bad Performance of ideal case, can not be used for real network deployment |
| 8.Correlation effects removal | - Good Averaging method can cancel the TRP deviations introduced by the correlation effects in theory | - Bad No consideration on antenna correlation effects on 2-TX TRP |

Proposal 1: Select option 1 (averaging TRPs) as baseline performance metric for coherent UL MIMO.

3 Conclusion

In this paper, we provide our views on concluding performance metric for coherent UL MIMO.

Proposal 1: Select option 1 (averaging TRPs) as baseline performance metric for coherent UL MIMO.

4	References

- R4-2321099, WF on NR_FR1_TRP_TRS_enh, vivo, 3GPP TSG-RAN WG4 Meeting #109

- R4-2305701, Further discussion on FR1 1 Layer UL-MIMO 2Tx SISO OTA TRP, MediaTek Inc, vivo, CAICT, 3GPP TSG-RAN WG4 Meeting #106bis-e

- R4-2313775, On Phase Impacts on Single-Layer UL MIMO TRP Measurements and 2Tx Test Mode, Keysight Technologies, 3GPP TSG-RAN4 Meeting #108

- R4-2316319, On TxD and Single-Layer UL MIMO TRP Measurements, Keysight Technologies

- R4-2311672, Multi-TPMI TRP time estimation, Rohde & Schwarz, Keysight Technologies

- CTIA OTA Test Plan

- Discussion outcome for CCSA 1484.9 Measurement method for radiated RF power and receiver performance of wireless device Part 9:5G NR wireless device (Sub-6GHz)

- R4-2313628, 2Tx SISO OTA TRP Deviation and ECC, MediaTek Inc, 3GPP TSG-RAN WG4 Meeting #108, August 2023

- R4-2320175, Discussion on 2TX test methodology, CAICT, RAN4#109

- R4-2319915, Discussion on single-layer UL MIMO test methodology, OPPO

- R4-2320246, Discussion on TRP test method for Single-Layer UL-MIMO, Google

- R4-2320379, Discussion on FR1 2Tx TRP test method, Qualcomm

- R4-2318834, On Single-Layer UL MIMO for non-coherent/coherent UEs, Keysight Technologies