3GPP TSG-RAN WG4 Meeting #110                                                            R4-2401546

Athens, Greece, 26 Feb. – 01 Mar. 2024

Agenda Item:	8.13.2.2

Source:	Ericsson

Title:	Simulation results for phase II

Document for:	Information

# 1	Introduction

After last RAN4 #109 meeting’s discussion, there were several parameter configurations and assumptions agreed that should be used for further simulation campaign for the phase II evaluations. A way forward was approved to carry all the agreements and open issues [1].

In this contribution, we shared our simulation results including modulation order blind detection studies based on the agreed parameter configurations and assumptions for information.

# 2	Discussion

## 2.1 Assumptions

In this section, we summarized the simulation assumptions that we used for our evaluations.

Following assumptions are assumed:

- Maximum transmission bandwidth configuration for all UEs

- Assume all scheduled DMRS ports have same QCL assumptions.

- Assume the scrambling ID for DMRS sequence is the same for the target UE the co-scheduled UE(s)

- Random precoder for rank 1+1, orthogonal precoder for rank 2+2

- Same CDM group for rank 1+1, different CDM groups for rank 2+2

The following table 2.1-1 is showing the general parameter configurations, which are basically using what were agreed in the previous meetings.

Table 2.1-1 General parameter configurations

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | Target UE | Co-scheduled UE |
| Number |  |  | 1 | 1 |
| Channel Bandwidth/SCS |  | MHz/kHz | 10/15 for FDD |  |
| Duplex mode |  |  | FDD |  |
| MCS |  |  | 13/17 | QPSK/16QAM |
| Allocation for interference UE and target UE | Rank allocation |  | 1 | 1 |
|  |  |  | 2 | 2 |
|  | DMRS port allocation |  | For rank 1+1: Port 1000 For rank 2+2: Port 1000 and 1001 | For rank 1+1: Port 1001 For rank 2+2: Port 1002 and 1003 |
| MIMO configuration |  |  | 2T2R, 4T4R with different correlations for verification |  |
| CDM groups without data |  |  | Same for rank 1+1 / different for rank 2+2, 1+1+1 |  |
| Precoding model | Target UE |  | Random precoding with Single panel Type 1 per PRB bundling size per slot | - For case with rank 1+1: Random - For case with rank 2+2: Orthogonal |
| PDSCH configuration | Mapping type |  | Type A |  |
|  | Starting symbol (S) |  | 2 |  |
|  | Length (L) |  | 12 |  |
|  | PRB bundling size |  | 2 |  |
|  | PRB bundling type |  | Static |  |
| PDSCH DMRS configuration | DMRS Type |  | DMRS Type 1 |  |
|  | Number of additional DMRS |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |
| Propagation conditions |  |  | TDLC300-100 + TDLA30-10 |  |

## 2.2 Test cases and results summary

We’ve evaluated the following test cases and summarized the results as follows.

Table 2.2-1 Simulation results for modulation order blind detection

| Test num | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate |  | Propagation condition | Rank (target UE + co-scheduled UE) | Correlation matrix and antenna configuration | Reference value SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Target UE | Co-scheduled UE |  |  |  | RML |
| Rank 1+1 |  |  |  |  |  |  |  |
| 1 | 10 / 15 | MCS13 | QPSK | TDLC300-100 | 1+1 | 2x2, ULA medium | 13.6 |
| 2 | 10 / 15 | MCS13 | QPSK | TDLC300-100 | 1+1 | 2x2, ULA low | 10.6 |
| 3 | 10 / 15 | MCS17 | 16QAM | TDLC300-100 | 1+1 | 2x2, ULA medium | 23.2 |
| 4 | 10 / 15 | MCS17 | 16QAM | TDLC300-100 | 1+1 | 2x2, ULA low | 18.1 |
| 5 | 10 / 15 | MCS13 | QPSK | TDLC300-100 | 1+1 | 2x4, ULA medium | 11.8 |
| 6 | 10 / 15 | MCS13 | QPSK | TDLA-30-10 | 1+1 | 2x4,ULA low | 4.2 |
| 7 | 10 / 15 | MCS17 | 16QAM | TDLC300-100 | 1+1 | 2x4, ULA medium | 22.1 |
| 8 | 10 / 15 | MCS17 | 16QAM | TDLA-30-10 | 1+1 | 2x4. ULA low | 8.5 |
| Rank 2+2 |  |  |  |  |  |  |  |
| 9 | 10 / 15 | MCS13 | QPSK | TDLA30-10 | 2+2 | 4x4, ULA Low | 10.3 |
| 10 | 10 / 15 | MCS13 | QPSK | TDLA30-10 | 2+2 | 4x4, XP medium | N/A |
| 11 | 10 / 15 | MCS17 | 16QAM | TDLA30-10 | 2+2 | 4x4, ULA Low | 17.3 |
| 12 | 10 / 15 | MCS17 | 16QAM | TDLA30-10 | 2+2 | 4x4, XP medium | N/A |

Table 2.2-2 Simulation results for DCI index 1-5 (without MOBD)

| Test num | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate |  | Propagation condition | Rank (target UE + co-scheduled UE) | Correlation matrix and antenna configuration | Reference value SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Target UE | Co-scheduled UE |  |  |  | RML |
| Rank 1+1 |  |  |  |  |  |  |  |
| 13 | 10 / 15 | MCS13 | QPSK | TDLC300-100 | 1+1 | 2x2, ULA medium | 13.7 |
| 14 | 10 / 15 | MCS13 | QPSK | TDLC300-100 | 1+1 | 2x2, ULA low | 10.6 |
| 15 | 10 / 15 | MCS13 | QPSK | TDLC300-100 | 1+1 | 2x4, ULA medium | 11.8 |
| 16 | 10 / 15 | MCS13 | QPSK | TDLA-30-10 | 1+1 | 2x4. ULA low | 4.1 |
| Rank 2+2 |  |  |  |  |  |  |  |
| 17 | 10 / 15 | MCS13 | QPSK | TDLA30-10 | 2+2 | 4x4, ULA Low | 10.3 |
| 18 | 10 / 15 | MCS13 | QPSK | TDLA30-10 | 2+2 | 4x4, XP medium | 11.6 |
| 19 | 10 / 15 | MCS17 | 16QAM | TDLA30-10 | 2+2 | 4x4, ULA Low | 16.9 |
| 20 | 10 / 15 | MCS17 | 16QAM | TDLA30-10 | 2+2 | 4x4, XP medium | 18.9 |

# 3	Summary

In this contribution, we shared our simulation results based on the parameter configurations and assumptions listed in the WF of last RAN4 #109 meeting.

# References

[1]	R4-2321114: WF on the advanced receiver for MU-MIMO scenario, China Telecom, RAN4 #109, Chicago, US.

# Annex

Summary of simulation assumptions for alignment [1]:

|  | For DCI index 1~5 | For BD MO evaluation |
| --- | --- | --- |
| Antenna config/ Rank of paired UEs | 2x2: 1+1 2x4: 1+1 4x4: 2+2 | 2x2: 1+1 2x4: 1+1 4x4: 2+2 |
| Precoder selection | Orthogonal for 2+2 Random for 1+1 (Interested companies can provide results for rank 1+1 ULA medium cases with orthogonal precoding) | Orthogonal for 2+2 Random for 1+1 (Interested companies can provide results for rank 1+1 ULA medium cases with orthogonal precoding) |
| Channel and antenna correlation | For 1+1 with 2T2R: - TDLC300-100, ULA medium and ULA low For 1+1 with 2T4R: - TDLA30-10, ULA low - TDLC300-100, ULA medium For 2+2 with 4T4R: - TDLA30-10, ULA Low - TDLA30-10, XP medium | For 1+1 with 2T2R: - TDLC300-100, ULA medium and ULA low For 1+1 with 2T4R: - TDLA30-10, ULA low - TDLC300-100, ULA medium For 2+2 with 4T4R: - TDLA30-10, ULA Low - TDLA30-10, XP medium |
| MCS(target UE)+Modulation order co-UE | 1+1: MCS13+QPSK 2+2: MCS13+QPSK 2+2: MCS17+16QAM | 1+1: MCS13+QPSK 1+1: MCS17+16QAM 2+2: MCS13+QPSK 2+2: MCS17+16QAM |
| Allocation of co-UE | Full FDRA | Full FDRA |