3GPP TSG-RAN WG4 Meeting #110                                                       R4-2401550

Athens, Greece, 26th Feb. – 1st Mar. 2024

Agenda Item:	8.27.2

Source:	Ericsson

Title:	On PDCCH demodulation requirements for DSS enhancement

Document for:	Discussion

# 1	Introduction

After last RAN4 #109 meeting, most of the parameter assumptions have been agreed for doing evaluations. According to the WF [1], followings are agreed assumptions:

| Parameters | Values |
| --- | --- |
| Duplex mode | FDD |
| SCS | 15 kHz |
| Bandwidth | 10 MHz |
| Channel model | TDLA30-10 and TDLC300-100 |
| Antenna configuration | 2x2, 2x4, 4x2, 4x4 |
| CRS | 4 port CRS |
| DCI payload (excluding CRC) | 39 bits |
| Interleaving | Non-interleaved |
| Precoding | Precoder cycling per REG bundle |
| REG bundle size | 6 |
| Shift index | 0 |
| CORSET duration | 2 (2nd symbol and 3rd symbol) |
| CORSET RB | 48 |
| AL | 4, 8 |
| - Note 1: The bandwidth of NR CORESET is within the bandwidth of LTE. - Note 2: gNB punctures the PDCCH data and DMRS REs overlapped with LTE CRS. Note 3: PDCCH channel estimation is assumed to use only the clean PDCCH symbol. |  |

In this contribution, we shared our views on the left open issues and gave our proposals on how to design the NR PDCCH test cases.

# 2	Discussion

## 2.1 Parameter assumption

Antenna configuration

During last meeting’s discussion, companies have different views on the antenna configuration assumption. Following two options have been taken into account:

Antenna configuration Way forward: - Option 1: 2x2, 2x4, 4x2, 4x4 - Option 2: 4x2, 4x4 only

Since we assumed 4 ports LTE CRS, it is also straightforward to consider 4Tx for NR PDCCH, plus RAN1’s studies considered 4Tx for evaluation as well. In 38.101-4, only 4Tx has been considered for both LTE and NR for minimum requirements for PDSCH with inter-cell CRS interference (5.2.2.1.19).

However, previously, only 2Tx has been considered for NR PDCCH requirements (5.3.2 and 5.3.3). It is also preferable to cover more scenarios with 2Tx. Thus, we slightly prefer option 1 but can accept option 2 to consider 4Tx only.

Proposal 1: option 1 and option 2. If consider option1, need to distribute them into different test cases.

Channel bandwidth

In last meeting, companies agreed on considering 10MHz/15kHz for channel bandwidth and SCS. For TDD, however, there are two candidate options:

Channel bandwidth Agreement: - For FDD, consider 10MHz/15kHz Way forward: - For TDD, - Option 1: 10MHz/15kHz - Option 2: 20MHz/15kHz

RAN1 has used 20MHz for studies and evaluations. As far as we know, in 38.101-4, requirement for CRS interference mitigation under NR-LTE coexistence scenario (5.2.2.2.19) and requirement for inter-cell CRS interference cancellation (5.2.2.2.20) both considered 20MHz Bandwidth for TDD as we recall some operators mentioned they observed sever interference over that bandwidth. Therefore, we propose to consider option2: 20MHz/15kHz for channel bandwidth and SCS.

Proposal 2: For TDD scenario, consider option 2: 20MHz/15kHz for channel bandwidth and SCS.

Aggregation level

Aggregation level 4 and 8 were agreed for evaluation purpose.

Aggregation level Agreement: - 4, 8 (for evaluation purpose)

According to our simulations [2], results for AL=4 and AL=8 are reasonable for defining PDCCH requirements. Figure 2.1-1 and 2.1-2 showed the PDCCH performance of AL=4 and AL=8:

Figure 2.1-1 AL=4 PDCCH performance

Figure 2.1-2 AL=8 PDCCH performance

Besides, as discussed in the last RAN4 #109 meeting, there is no need to consider more aggregation levels for further evaluations. Therefore, we propose to consider only AL=4 and AL=8 for defining PDCCH requirements.

Proposal 3: Consider only AL=4 and AL=8 for defining PDCCH requirements.

Channel model

As for the channel model, for fulfilling the test coverage, we propose to consider both TDLA30-10 and TDLC300-100 for defining PDCCH requirements. To avoid duplicating the test, we also propose to distribute two channel models to different test cases.

Proposal 4: Consider both TDLA30-10 and TDLC300-100 for defining requirements, plus distribute them into different test cases.

## 2.2 Proposed Test cases

Followings are the test cases that we propose for defining PDCCH requirements:

Proposal 5: Consider test cases in table 2.2-1 for defining PDCCH requirements.

Table 2.2-1 Simulation results

| Test case | SCS(KHz)/CBW (MHz) | CORESET RB | Payload | AL | Propagation condition | Antenna configuration | SNR @1%BLER |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15/10 | 48 | 39 | 4 | TDLA30-10 | 2x2 Low |  |
| 2 | 15/10 | 48 | 39 | 8 | TDLC300-100 | 2x2 Low |  |
| 3 | 15/10 | 48 | 39 | 4 | TDLA30-10 | 2x4 Low |  |
| 4 | 15/10 | 48 | 39 | 8 | TDLC300-100 | 2x4 Low |  |
| 5 | 15/10 | 48 | 39 | 4 | TDLA30-10 | 4x2 Low |  |
| 6 | 15/10 | 48 | 39 | 8 | TDLC300-100 | 4x2 Low |  |
| 7 | 15/10 | 48 | 39 | 4 | TDLA30-10 | 4x4 Low |  |
| 8 | 15/10 | 48 | 39 | 8 | TDLC300-100 | 4x4 Low |  |

# 3	Summary

In this contribution, we shared our views on the left open issues and gave our proposals on how to design the test case. Our proposals are summarized as follows:

Proposal 1: Option 1 and option 2. If consider option1, need to distribute them into different test cases.

Proposal 2: For TDD scenario, consider option 2: 20MHz/15kHz for channel bandwidth and SCS.

Proposal 3: Consider only AL=4 and AL=8 for defining PDCCH requirements.

Proposal 4: Consider both TDLA30-10 and TDLC300-100 for defining requirements, plus distribute them into different test cases.

Proposal 5: Consider test cases in table 2.2-1 for defining PDCCH requirements.

# References

[1]	R4-2321187: WF for Enhancement of Dynamic Spectrum Sharing demodulation requirements, RAN4 #109, Chicago, US.

[2]	R4-2401551: Simulation results for Rel-18 DSS enhancement demodulation, Ericsson, RAN4 #110, Athens, GR.