3GPP TSG-RAN WG4 Meeting # 110	R4-2401589

Athens, GR, Feb 26 – Mar 01, 2024

Agenda Item: 8.3.3

Source: MediaTek Inc.

Title: Discussion on test cases list for multi-Rx UEs

Document for: Discussion

# Introduction

According to WF [1], there’re several open issues about test cases for multi-RX. In this paper, we provide our views to this topic.

# Discussion

## 2.1 Accuracy requirements and test cases for multi-RX

In last meeting, there’re several issues in WF [1] related to RRM test cases for multi-RX.

#### Issue 1-4: Test case(s) for fast beam sweeping - Companies are encouraged to bring analysis on test case(s) for fast beam sweeping #### Issue 1-5: Test case(s) for scheduling restriction - Companies are encouraged to bring analysis on test case(s) for scheduling restriction #### Issue 1-6: Test case(s) for measurement restriction - Companies are encouraged to bring analysis on test case(s) for measurement restriction #### Issue 1-7: Test case(s) for TCI state switching - Companies are encouraged to bring analysis on test case(s) for TCI state switching #### Issue 1-8: Test case(s) for group-based beam reporting (GBBR) - Companies are encouraged to bring analysis on test case(s) for group-based beam reporting #### Issue 1-9: List of test case(s) for multi-Rx in Rel-18 - Companies are encouraged to bring analysis on full list of test case(s) for mulit-Rx WI ### Accuracy requirements #### Issue 1-10: Accuracy requirements for multi-Rx in Rel-18 - Proposals - Option 1: - The legacy accuracy requirements for L1-RSRP measurement apply for L1-RSRP measurements with group-based beam reporting. - Option 2: - The legacy accuracy requirements in section 10.1.20 of TS 38.133 apply for L1-RSRP measurements under multi-rx operation, with a clarification that multi-rx chain L1-RSRP accuracy requirements apply for FR2-1. - Option 3: - The legacy accuracy requirements in section 10.1.20 of TS 38.133 apply for L1-RSRP measurements under multi-rx operation, with a clarification that multi-rx chain L1-RSRP accuracy requirements apply for FR2-1. - No new accuracy requirements section is created for L1-RSRP measurements under multi-rx operation.

For issue 1-10, we support option 3 to reuse the legacy accuracy requirement in clause 10.1.20 of TS 38.133. The only extra thing to do is probably adding a clarification for multi-RX chain L1-RSRP accuracy requirement and apply for FR2-1.

Proposal 1: Legacy accuracy requirements in the clause 10.1.20 of TS 38.133 apply to multi-Rx UE.

In our understanding, it is not necessary to create new test cases for individual issues 1-4 to 1-8. Considering that Group-based beam reporting (GBBR) would be a prerequisite of many other enhanced requirements, there is no need to add a dedicated new test case only for GBBR. Instead, the GBBR measurement delay (and accuracy) can be checked under other test cases. On the other hand, since we never have test case for measurement restriction, we do not prefer to introduce corresponding test cases. Therefore, we suggest the following test cases list:

- Test case 1: GBBR and faster beam sweeping for issue 1-4 and 1-8.

- Test case 2: GBBR and TCI state switching for issue 1-7 and 1-8.

- Test case 3: Scheduling restriction

Proposal 2: RAN4 to consider below test cases list for multi-RX:

- Test case 1: GBBR and faster beam sweeping

- Test case 2: GBBR and TCI state switching

- Test case 3: Scheduling restriction

## 2.2 Test case 1: GBBR and fast beam sweeping

For test case 1, we suggest merging test cases of GBBR and faster beam sweeping, since these requirements could be tested together as shown in Figure 1. We can take the legacy L1-RSRP test case as baseline for the configurations and test requirements. The following configurations and requirements are suggested:

- In Figure 1, T1=[5s], T2=[2s], SCS=120 kHz, L1-RSRP reporting period=320 slot.

  - Before test,

    - UE is connected to Cell 1 (PCell) on radio channel 1 (PCC) without multi-RX operation.

    - UE report faster beam sweeping capability with reduced beam number. (We use N=4 as an example for the following calculation).

  - During T1, UE shall send L1-RSRP non-GBBR report for SSB#0 in every L1-measurement period

    - L1-measurement period = max(TReport, ceil(M(1)*P(3)*N(8))*TSSB) = 480ms

      - TSSB=20ms

      - TReport =320 slots.

  - T2 starts from TE transmitting non-zero power for SSB#1. The duration of T2 is the time for UE to prepare the L1-RSRP GBBR for SSB#0 (with RSRP0) and SSB 1 (with RSRP1) which may require additional SSB detection time.

  - T3 starts from TE changes the power level of SSB#0

    - UE shall send L1-RSRP report configured with groupBasedBeamReporting-r17 for SSB #0 and SSB #1 in every L1-measurement period

      - YGBBR = L1-measurement period(480ms) + TReport(40ms) = 520ms.

        - L1-measurement period = max(TReport, ceil(M(1)*P(3)*N(8))*TSSB) = 480ms.

          - TSSB=20ms,

          - TReport =320 slots(40ms).

        - YGBBR is the longest duration allowed for UE to reflect the changed RSRP (from RSRP0 to RSRP0’) in L1-RSRP GBBR.

    - UE shall send L1-RSRP report not configured with groupBasedBeamReporting-r17 for SSB#0 every L1-measurement period

      - Ynon-GBBR = L1-measurement period(240ms) + TReport(40ms) = 280ms.

      - L1-measurement period = max(TReport, ceil(M(1)*P(3)*N(4))*TSSB) = 240ms.

        - TSSB=20ms,

        - TReport =320slot(40ms).

      - Ynon-GBBR is the longest duration allowed for UE to reflect the changed RSRP (from RSRP0 to RSRP0’) in L1-RSRP non-GBBR.

    - UE does not indicate multi-RX fallback in UE assistance information.

- There’re two AoAs (AoA0 and AoA1) for different SSB index (SSB #0 and #1)

  - Different RSRP level (RSRP0 and RSRP1) for SSB #0 (AoA1) and SSB #1(AoA2).

- The reported L1-RSRP value shall include the Rx beamforming gain in the range of [-10 to +20 dB].

Figure 1. Test case for L1-RSRP Rel-17 GBBR and faster beam sweeping for non-GBBR

From above, we suggest the principle of test case 1 in the following proposal:

Proposal 3: RAN4 to consider the principle in test case 1 (GBBR and fast beam sweeping):

- Reuse the configuration and test requirement of legacy L1-RSRP test case as baseline.

  - T1=[5s], T2=[1.24s], T3=[2s], SCS=120 kHz, TSSB=20ms, TReport =320 slots.

- There’re two AoAs for different SSB#0 (AoA#1) and SSB#1 (AoA#2).

  - Three RSRP level (RSRP0, RSRP0’ and RSRP1) to reflect the RSRP value within L1-RSRP GBBR and L1-RSRP non-GBBR:

    - Before T3, the SSB#0 is transmitted with RSRP0.

    - After T3, the SSB#0 is transmitted with RSRP0’.

    - After T2, the SSB#1 is transmitted with RSRP1.

- Test requirements are set on the time duration to reflect the RSRP change within L1-RSRP GBBR and L1-RSRP non-GBBR during T3.

  - YGBBR is the max allowed time to reflect the RSRP change in L1-RSRP GBBR.

  - Ynon-GBBR is the max allowed time to reflect the RSRP change in L1-RSRP non-GBBR.

- During T2 and T3, UE does not indicate multi-RX fallback in UE assistance information.

- Before test, UE is connected to Cell 1 (PCell) on radio channel 1 (PCC) without multi-RX operation.

## 2.3 Test case 2: GBBR and TCI state switching

For test case 2, we suggest merging test cases of GBBR and dual TCI states switching since these procedures could be tested continuously shown in Figure 2. We take the legacy test cases of L1-RSRP and single TCI state switching as baseline for the configuration and test requirements. The following configuration and requirements are suggested:

- In Figure 2, T1=[2s], T2=[1.28s], T3=[0.2s], SCS=120 kHz, L1-RSRP reporting period=320 slot.

  - The test scenario is from single TCI (TCI state 0) to dual TCI (TCI states 1 and 2).

    - This scenario can cover the scope of dual TCI state switching. To avoid duplicated test scope, we don’t consider the scenarios of dual TCI to dual TCI or dual TCI to single TCI.

  - Before test,

    - UE is connected to Cell 1 (PCell) on radio channel 1 (PCC) without multi-RX operation.

    - UE is configured with 2 different TCI states for Cell 1, PDCCH TCI state 1 (QCL’d to SSB #1) and TCIstate 2 (QCL’d to SSB #2).

    - UE is indicated with TCI state 0 as the active PDCCH TCI state.

    - tci-PresentInDCI is not configured in the PDSCH configuration, i.e., TCI state for the PDSCH is identical to the PDCCH TCI state. (Skip DCI based TCI switching for PDSCH)

  - During T1, UE send L1-RSRP non-GBBR report.

  - T2 starts from TE transmitting non-zero power for SSB#1 and SSB#2. The duration of T2 is the time for UE to prepare L1-RSRP Rel-17 GBBR for SSB#1 and SSB#2 (with the same RSRP for simplicity) which may require additional SSB detection time.

  - T3 starts from TE sending the dual TCI states switch command at slot #n. UE is activated with dual TCI states with delay THARQ+3ms after slot #n.

- There’re three AoAs (AoA0, AoA1 and AoA2) for different SSB index (SSB #0, #1 and #2)

  - Different RSRP levels (RSRP0 and RSRP1/2) for SSB0 (AoA1) and SSB1/2 (AoA2/3).

Figure 2. Test case for L1-RSRP Rel-17 GBBR and dual TCI states switching

From above, we suggest the principle of test case 2 in the following proposal:

Proposal 4: RAN4 to consider the principle in test case 2 (GBBR and dual TCI states switching):

- Reuse the configuration and test requirement of legacy L1-RSRP and single TCI state switching test case as baseline.

  - T1=[2s], T2=[1.24s], T3=[0.2s], SCS=120 kHz, TSSB=20ms, TReport =320 slots.

  - Before test,

    - UE is connected to Cell 1 (PCell) on radio channel 1 (PCC) without multi-RX operation.

    - UE is indicated in TCI state 0 as the active PDCCH TCI state.

    - UE is additionally configured with 2 different TCI states for Cell 1, PDCCH TCI state 1 (QCL’d to SSB#1) and TCI state 2 (QCL’d to SSB#2).

    - tci-PresentInDCI is not configured in the PDSCH configuration, i.e. TCI state for the PDSCH is identical to the PDCCH TCI state.

- There’re three AoAs (AoA0, AoA1 and AoA2) for different SSB indices (SSB 0, 1 and 2)

  - Different RSRP level (RSRP0 and RSRP1/2) for SSB#0 (AoA#2/#3).

    - During the whole testing, the SSB#0 is transmitted with RSRP0.

    - After T2, TE start to transmit the SSB#1 and SSB#2 with non-zero power (RSRP1/2).

- The test scenario is from single TCI (TCI state 0) to dual TCI (TCI states 1 and 2).

  - To avoid duplicated test scope, we don’t consider the scenarios of dual TCI to dual TCI or dual TCI to single TCI.

## 2.4 Test case 3: Scheduling restriction

For test case 3, we reuse legacy test case as a starting point but only consider CSI-RS based L1 measurement since the relaxation for scheduling restriction is only for CSI-RS in multi-RX.

In legacy, RAN4 only consider scheduling restriction test cases in RLM in which the purpose is to verify that the NR UE correctly receives PDCCH scheduled on the symbols right before RLM symbols. Here, since UE is able to simultaneously receive both PDCCH and CSI-RS on the same OFDM symbol. We need to create such a scenario in the test case to verify the UE behaviour. From TS38.214, we can see some limitations for CSI-RS and PDCCH on the same OFDM symbol, as captured below. The principle is that CSI-RS and CORESET of PDCCH are allowed to overlap on the same OFDM symbol as long as their PRBs are not overlapping.

#### TS 38.214 5.1.6.1 CSI-RS reception procedure The CSI-RS defined in Clause 7.4.1.5 of [4, TS 38.211], may be used for time/frequency tracking, CSI computation, L1-RSRP computation, L1-SINR computation, mobility, and tracking during fast SCell activation. For a CSI-RS resource associated with a NZP-CSI-RS-ResourceSet with the higher layer parameter repetition set to 'on', the UE shall not expect to be configured with CSI-RS over the symbols during which the UE is also configured to monitor the CORESET, while for other NZP-CSI-RS-ResourceSet configurations, if the UE is configured with a CSI-RS resource and a search space set associated with a CORESET in the same OFDM symbol(s), the UE may assume that the CSI-RS and a PDCCH DM-RS transmitted in all the search space sets associated with CORESET are quasi co-located with 'typeD', if 'typeD' is applicable. If the CORESET is activated with two TCI states, UE may assume that the first TCI state of the CORESET as the default QCL assumption for the CSI-RS. This also applies to the case when CSI-RS and the CORESET are in different intra-band component carriers, if 'typeD' is applicable. Furthermore, the UE shall not expect to be configured with the CSI-RS in PRBs that overlap those of the CORESET in the OFDM symbols occupied by the search space set(s).

Therefore, the following proposal is suggested:

Proposal 5: RAN4 to consider below conditions in scheduling restriction test case for multi-RX:

- PDCCHs and CSI-RSs are overlapped on the same OFDM symbol with disjoint PRB allocation.

- To consider either RLM or L1-RSRP scheduling restriction for test case.

In addition, since there is no test case for measurement restriction in legacy, we do not prefer to consider test cases for measurement restriction

Proposal 6: Do not introduce test cases for measurement restriction for multi-RX.

# Summary

In this paper, the discussion of R18 multi-Rx chains is provided. We have the following proposal:

Proposal 1: Legacy accuracy requirements in the clause 10.1.20 of TS 38.133 apply to multi-Rx UE.

Proposal 2: RAN4 to consider below test cases list for multi-RX:

- Test case 1: Group-based beam reporting (GBBR) and fast beam sweeping

- Test case 2: Group-based beam reporting and TCI state switching

- Test case 3: Scheduling restriction

Proposal 3: RAN4 to consider the principle in test case 1 (GBBR and fast beam sweeping):

- Reuse the configuration and test requirement of legacy L1-RSRP test case as baseline.

  - T1=[5s], T2=[1.24s], T3=[2s], SCS=120 kHz, TSSB=20ms, TReport =320 slots.

- There’re two AoAs for different SSB#0 (AoA#1) and SSB#1 (AoA#2).

  - Three RSRP level (RSRP0, RSRP0’ and RSRP1) to reflect the RSRP value within L1-RSRP GBBR and L1-RSRP non-GBBR:

    - Before T3, the SSB#0 is transmitted with RSRP0.

    - After T3, the SSB#0 is transmitted with RSRP0’.

    - After T2, the SSB#1 is transmitted with RSRP1.

- Test requirements are set on the time duration to reflect the RSRP change within L1-RSRP GBBR and L1-RSRP non-GBBR during T3.

  - YGBBR is the max allowed time to reflect the RSRP change in L1-RSRP GBBR.

  - Ynon-GBBR is the max allowed time to reflect the RSRP change in L1-RSRP non-GBBR.

- During T2 and T3, UE does not indicate multi-RX fallback in UE assistance information.

  - Before test, UE is connected to Cell 1 (PCell) on radio channel 1 (PCC) without multi-RX operation.

Proposal 4: RAN4 to consider the principle in test case 2 (GBBR and dual TCI states switching):

- Reuse the configuration and test requirement of legacy L1-RSRP and single TCI state switching test case as baseline.

  - T1=[2s], T2=[1.24s], T3=[0.2s], SCS=120 kHz, TSSB=20ms, TReport =320 slots.

  - Before test,

    - UE is connected to Cell 1 (PCell) on radio channel 1 (PCC) without multi-RX operation.

    - UE is indicated in TCI state 0 as the active PDCCH TCI state.

    - UE is additionally configured with 2 different TCI states for Cell 1, PDCCH TCI state 1 (QCL’d to SSB#1) and TCI state 2 (QCL’d to SSB#2).

    - tci-PresentInDCI is not configured in the PDSCH configuration, i.e. TCI state for the PDSCH is identical to the PDCCH TCI state.

- There’re three AoAs (AoA0, AoA1 and AoA2) for different SSB indices (SSB 0, 1 and 2)

  - Different RSRP level (RSRP0 and RSRP1/2) for SSB#0 (AoA#2/#3).

    - During the whole testing, the SSB#0 is transmitted with RSRP0.

    - After T2, TE start to transmit the SSB#1 and SSB#2 with non-zero power (RSRP1/2).

- The test scenario is from single TCI (TCI state 0) to dual TCI (TCI states 1 and 2).

  - To avoid duplicated test scope, we don’t consider the scenarios of dual TCI to dual TCI or dual TCI to single TCI.

Proposal 5: RAN4 to consider below conditions in scheduling restriction test case for multi-RX:

- PDCCHs and CSI-RSs are overlapped on the same OFDM symbol with disjoint PRB allocation.

- To consider either RLM or L1-RSRP scheduling restriction for test case.

Proposal 6: Do not introduce test cases for measurement restriction for multi-RX.

# Reference

- R4-2321602 WF on core maintenance and performance part, Vivo, Ericsson RAN4 #109