3GPP TSG-RAN WG4 Meeting # 110	R4-2401675

# Athens, GR, 26 Feb – 01 Mar, 2024

Title: 	Simulation results for advanced receiver for MU-MIMO

Source: 	Huawei, HiSilicon

Agenda item:	8.13.2.2

Document for:	Discussion

Background

In last meeting, a WF [1] on the receiver for MU-MIMO scenario was approved. This contribution provides our views on simulation results.

- Discussions

The simulation assumptions are captured in Table 2-1:

Table 2-1: Simulation assumptions

| Parameter |  | Unit | Target UE | Co-scheduled UE |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  | FDD, TDD |  |
| SCS |  | kHz | FDD:15 TDD:30 |  |
| Bandwidth |  | MHz | FDD:10 TDD:40 |  |
| TDD pattern |  |  | 7D1S2U, S=6D4G4U |  |
| Active DL BWP index |  |  | 1 |  |
| PDSCH configuration | Mapping type |  | Type A |  |
|  | k0 |  | 0 |  |
|  | Starting symbol (S) |  | 2 |  |
|  | Length (L) |  | 12 |  |
|  | PDSCH aggregation factor |  | 1 |  |
|  | PRB bundling type |  | Static |  |
|  | PRB bundling size |  | 2 |  |
|  | Resource allocation type |  | Type 0 |  |
|  | RBG size |  | Config2 |  |
|  | VRB-to-PRB mapping type |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |  |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |  |
|  | Number of additional DMRS |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |
|  | Antenna ports indexes |  | Rank1+1: 1000 Rank2+2:1000, 1001 | Rank1+1:1001 Rank 2+2:1000,1001 |
|  | Number of PDSCH DMRS CDM group(s) without data |  | Rank1+1: 1 Rank2+2: 2 | Rank1+1: 1 Rank2+2: 2 |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination or codebook Index, chosen from section 5.2.2.2.1 of TS 38.214 [12]. | Random PMI selection: Single Panel Type I, Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination or codebook Index, chosen from section 5.2.2.2.1 of TS 38.214 [12].Any column of precoder matrix is not equal to any column of precoder matrix of Target UE Orthogonal PMI selection: Select the precoder to ensure any column of precoder is orthogonal to any column of precoder for the target PDSCH |
| MU-MIMO Beamforming Model |  |  | As specified in B.4.2 in 38.101-4 |  |
| Receiver |  |  | R-ML with modulation order detection R-ML without modulation order detection MMSE-IRC |  |
| Modulation order estimation assumption |  |  | Per PRG detection with range from QPSK to 256QAM |  |
| Number of HARQ Processes |  |  | FDD:4 TDD:8 | N/A |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 | N/A |
| Note 1: The DMRS scrambling ID is same for both target UE and Co-scheduled UE. |  |  |  |  |

We provide our simulation results for agreed cases in Table 2-2 to Table 2-5

Table 2-2: Summary of simulation results without modulation order detection for FDD

| Rank for target + Co-UE | MIMO | Precoder selection | Channel Model | Antenna correlation | MCS for the target UE (MCS Table 1) | Modulation order for the co-scheduled UE | R-ML SNR@70% maxTP(dB) | MMSE-IRC SNR@70% maxTP(dB) | Gain R-ML vs MMSE-IRC (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1+1 | 2T2R | Random | TDLC300-100 | ULA medium | 13 | QPSK | 13.7 | 22.1 | 8.4 |
|  |  |  |  | ULA Low |  |  | 10.8 | 15.0 | 4.2 |
|  | 2T4R |  | TDLA30-10 | ULA Low |  |  | 5.9 | 7.4 | 1.5 |
|  |  |  | TDLC300-100 | ULA medium |  |  | 12.9 | 25.0 | 12.1 |
|  | 2T2R | Orthogonal | TDLC300-100 | ULA medium |  |  | 13.0 | 19.4 | 6.4 |
|  | 2T4R |  |  |  |  |  | 12.1 | 20.5 | 8.4 |
| 2+2 | 4T4R | Orthogonal | TDLA30-10 | ULA Low |  |  | 9.5 | 12.1 | 2.6 |
|  |  |  |  | XP medium |  |  | 10.5 | 13.9 | 3.4 |
|  |  |  |  | ULA Low | 17 | 16QAM | 15.1 | 16.4 | 1.3 |
|  |  |  |  | XP medium |  |  | 16.8 | 18.4 | 1.6 |

Table 2-3: Summary of simulation results without modulation order detection for TDD

| Rank for target + Co-UE | MIMO | Precoder selection | Channel Model | Antenna correlation | MCS for the target UE (MCS Table 1) | Modulation order for the co-scheduled UE | R-ML SNR@70% maxTP(dB) | MMSE-IRC SNR@70% maxTP(dB) | Gain R-ML vs MMSE-IRC (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1+1 | 2T2R | Random | TDLC300-100 | ULA medium | 13 | QPSK | 14.3 | 23.7 | 9.4 |
|  |  |  |  | ULA Low |  |  | 11.1 | 16.3 | 5.2 |
|  | 2T4R |  | TDLA30-10 | ULA Low |  |  | 6.1 | 7.6 | 1.5 |
|  |  |  | TDLC300-100 | ULA medium |  |  | 13.6 | 27.7 | 14.1 |
|  | 2T2R | Orthogonal | TDLC300-100 | ULA medium |  |  | 13.4 | 20.4 | 7.0 |
|  | 2T4R |  |  |  |  |  | 12.5 | 22.3 | 9.8 |
| 2+2 | 4T4R | Orthogonal | TDLA30-10 | ULA Low |  |  | 9.6 | 12.3 | 2.7 |
|  |  |  |  | XP medium |  |  | 10.6 | 14.1 | 3.5 |
|  |  |  |  | ULA Low | 17 | 16QAM | 15.4 | 16.6 | 1.2 |
|  |  |  |  | XP medium |  |  | 17.0 | 18.7 | 1.7 |

Table 2-4: Summary of simulation results for case with modulation order estimation for FDD

| Rank for target + Co-UE | MIMO | Precoder selection | Channel Model | Antenna correlation | MCS for the target UE (MCS Table 1) | Modulation order for the co-scheduled UE | R-ML SNR(dB)@70% max TP | MMSE-IRC SNR(dB)@70% max TP | Gain R-ML vs MMSE-IRC (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1+1 | 2T2R | Random | TDLC300-100 | ULA medium | 13 | QPSK | 14.5 | 22.1 | 7.6 |
|  |  |  |  | ULA Low |  |  | 11.2 | 15.0 | 3.8 |
|  | 2T4R |  | TDLA30-10 | ULA Low |  |  | 6.0 | 7.4 | 1.4 |
|  |  |  | TDLC300-100 | ULA medium |  |  | 14.6 | 25.0 | 10.4 |
|  | 2T2R |  | TDLC300-100 | ULA medium | 17 | 16QAM | 22.7 | N/A | N/A |
|  |  |  |  | ULA Low |  |  | 17.7 | 24.3 | 6.6 |
|  | 2T4R |  | TDLA30-10 | ULA Low |  |  | 10.8 | 10.8 | 0.0 |
|  |  |  | TDLC300-100 | ULA medium |  |  | 22.7 | N/A | N/A |
|  | 2T2R | Orthogonal | TDLC300-100 | ULA medium | 13 | QPSK | 13.6 | 19.4 | 5.8 |
|  | 2T4R |  |  |  |  |  | 13.5 | 20.5 | 7.0 |
|  | 2T2R |  |  |  | 17 | 16QAM | 20.9 | 26.1 | 5.2 |
|  | 2T4R |  |  |  |  |  | 20.9 | 27.9 | 7.0 |
| 2+2 | 4T4R |  | TDLA30-10 | ULA Low | 13 | QPSK | 10.6 | 12.1 | 1.5 |
|  |  |  |  | XP medium |  |  | 10.7 | 13.9 | 3.2 |
|  |  |  |  | ULA Low | 17 | 16QAM | 15.9 | 16.4 | 0.5 |
|  |  |  |  | XP medium |  |  | 17.0 | 18.4 | 1.4 |

Table 2-5: Summary of simulation results for case with modulation order estimation for TDD

| Rank for target + Co-UE | MIMO | Precoder selection | Channel Model | Antenna correlation | MCS for the target UE (MCS Table 1) | Modulation order for the co-scheduled UE | R-ML SNR(dB)@70% max TP | MMSE-IRC SNR(dB)@70% max TP | Gain R-ML vs MMSE-IRC (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1+1 | 2T2R | Random | TDLC300-100 | ULA medium | 13 | QPSK | 15.2 | 23.7 | 8.5 |
|  |  |  |  | ULA Low |  |  | 11.5 | 16.3 | 4.8 |
|  | 2T4R |  | TDLA30-10 | ULA Low |  |  | 6.1 | 7.6 | 1.5 |
|  |  |  | TDLC300-100 | ULA medium |  |  | 15.9 | 27.7 | 11.8 |
|  | 2T2R |  | TDLC300-100 | ULA medium | 17 | 16QAM | 24.4 | N/A | N/A |
|  |  |  |  | ULA Low |  |  | 18.4 | N/A | N/A |
|  | 2T4R |  | TDLA30-10 | ULA Low |  |  | 11.1 | 11.0 | -0.1 |
|  |  |  | TDLC300-100 | ULA medium |  |  | 25.4 | N/A | N/A |
|  | 2T2R | Orthogonal | TDLC300-100 | ULA medium | 13 | QPSK | 14.1 | 20.4 | 6.3 |
|  | 2T4R |  |  |  |  |  | 14.3 | 22.3 | 8.0 |
|  | 2T2R |  |  |  | 17 | 16QAM | 22.1 | 28.8 | 6.7 |
|  | 2T4R |  |  |  |  |  | 22.5 | 32.4 | 9.9 |
| 2+2 | 4T4R |  | TDLA30-10 | ULA Low | 13 | QPSK | 10.6 | 12.3 | 1.7 |
|  |  |  |  | XP medium |  |  | 10.6 | 14.1 | 3.5 |
|  |  |  |  | ULA Low | 17 | 16QAM | 15.8 | 16.6 | 0.8 |
|  |  |  |  | XP medium |  |  | 17.0 | 18.7 | 1.7 |

- Conclusion

This contribution provides our simulation results for advanced receiver for MU-MIMO

- Reference

[1]    R4-2321114 WF on the advanced receiver for MU-MIMO scenario. China Telcom, Apple