3GPP TSG-RAN WG4 Meeting # 110	R4-2401680

# Athens, GR, 26 Feb – 01 Mar, 2024

Title: 	Discussions on Rel-18 eDSS performance requirements

Source: 	Huawei, HiSilicon

Agenda item:	8.27.2

Document for:	Discussion

Background

In last meeting, a WF on enhancement of DSS demodulation requirements was approved and many open issues were left. In this contribution, we provide our views on these open issues and test setup.

- Discussions

UE receiver assumption

RAN4 have reached following agreements for UE receiver assumption:

Agreement: - Leave the receiver design to implementation for evaluation purposes. - Further check submitted results. If there is a large variation in results, then discuss receiver assumptions.

We proposed LLR punctured which is a well-known and simple algorithm to against the effect of the CRS interference, but finally comprised to check the performance based on the companies’ implementation and discuss the receiver details if there is large variation. But we didn’t discuss how to define the “large variation”,  we propose to use 2.5dB as the threshold to judge the alignment. I.e. If span of any evaluation cases (test 1~8) is larger than 2.5dB, RAN4 should discuss and unify the receiver assumption.

Proposal 1: If span of any evaluation test case is larger than 2.5dB, RAN4 should discuss and unify the receiver assumption.

Antenna configuration

We have following options for antenna configuration:

Way forward: - Option 1: 2x2, 2x4, 4x2, 4x4 - Option 2: 4x2, 4x4 only

We propose to align the Tx number for CRS and NR PDCCH. Since 4 CRS ports has been agreed, 2Tx should be dropped. Option 1 means different REs have different number of TXs, which may cause potential test setup complexity. Type I single panel codebook can be used.

Proposal 2:  Only consider 4x2 and 4x4.

Channel bandwidth

We have following options for channel bandwidth:

Agreement: - For FDD, consider 10MHz/15kHz Way forward: - For TDD, - Option 1: 10MHz/15kHz - Option 2: 20MHz/15kHz

We prefer option 1 which is aligned with Rel-15 DSS test setup.

Proposal 3:  Consider 10MHz/15kHz for TDD.

Aggregation level

We have following options for Aggregation level:

Agreement: - 4, 8 (for evaluation purpose)

We propose to choose both for requirements definition

Proposal 4: Choose AL4 and 8 for requirements definition.

Channel model

We have following options for channel model:

Agreement: - TDLA30-10 and TDLC300-100 - Discuss whether to down-select in the next meeting

We propose to use TDLA30-10 for AL4 and TDLC300-100 for AL8

Proposal 5: Choose TDLA30-10 for AL4 and TDLC300-100 for AL8.

- Simulation results

This section provides corresponding simulation results based on evaluation cases in WF[1] with simulation assumptions in Appendix.

Figure 2-1: BLER-SNR curve for PDCCH

Table 2-1: PDCCH simulation results for eDSS

| Test case | SCS(KHz)/CBW (MHz) | CORESET RB | Payload | AL | Propagation condition | Antenna configuration | SNR(dB)@ 1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15/10 | 48 | 39 | 4 | TDLA30-10 | 2x2 Low | 1.5 |
| 2 | 15/10 | 48 | 39 | 8 | TDLC300-100 | 2x2 Low | -3.4 |
| 3 | 15/10 | 48 | 39 | 4 | TDLA30-10 | 2x4 Low | -3.6 |
| 4 | 15/10 | 48 | 39 | 8 | TDLC300-100 | 2x4 Low | -7.0 |
| 5 | 15/10 | 48 | 39 | 4 | TDLA30-10 | 4x2 Low | -0.3 |
| 6 | 15/10 | 48 | 39 | 8 | TDLC300-100 | 4x2 Low | -4.1 |
| 7 | 15/10 | 48 | 39 | 4 | TDLA30-10 | 4x4 Low | -4.2 |
| 8 | 15/10 | 48 | 39 | 8 | TDLC300-100 | 4x4 Low | -7.2 |

We observe for case 4 and 8 (4Rx, AL8), the required SNR is lower than -6dB.

Observation 1: For case 4 and 8 (4Rx, AL8), the required SNR is lower than -6dB.

- Conclusion

Proposal 1: If span of any evaluation test case is larger than 2.5dB, RAN4 should discuss and unify the receiver assumption.

Proposal 2:  Only consider 4x2 and 4x4.

Proposal 3:  Consider 10MHz/15kHz for TDD

Proposal 4: Choose AL4 and 8 for requirements definition

Proposal 5: Choose TDLA30-10 for AL4 and TDLC300-100 for AL8.

Observation 1: For case 4 and 8 (4Rx, AL8), the required SNR is lower than -6dB.

- Reference

[1]    R1-2321139  WF for Enhancement of Dynamic Spectrum Sharing demodulation requirements. Ericsson

- Appendix

Table 5-1: Simulation assumptions

| Parameters | Values |
| --- | --- |
| Duplex mode | FDD |
| SCS | 15 kHz |
| Bandwidth | 10 MHz |
| Channel model | TDLA30-10 and TDLC300-100 |
| Antenna configuration | 2x2, 2x4, 4x2, 4x4 |
| CRS | 4 port CRS |
| DCI payload (excluding CRC) | 39 bits |
| Interleaving | Non-interleaved |
| Precoding | Precoder cycling per REG bundle |
| REG bundle size | 6 |
| Shift index | 0 |
| CORSET duration | 2 (2nd symbol and 3rd symbol) |
| CORSET RB | 48 |
| AL | 4, 8 |
| - Note 1: The bandwidth of NR CORESET is within the bandwidth of LTE. - Note 2: gNB punctures the PDCCH data and DMRS REs overlapped with LTE CRS. Note 3: PDCCH channel estimation is assumed to use only the clean PDCCH symbol. |  |