3GPP TSG-RAN WG4 Meeting # 110	R4-2401682

# Athens, GR, 26 Feb – 01 Mar, 2024

Title: 	Discussions on UE performance for Non-collocated CA

Source: 	Huawei, HiSilicon

Agenda item:	8.7.4

Document for:	Discussion

Background

In last meeting, a WF on NonCol_intraB_ENDC_NR_CA Demod [1] was agreed. In this contribution we provide our views on remaining open issue.

Discussions

The only remaining open issue is whether to confirm MCS4+MCS21 for requirements definition:

Agreement: MCS pair - Assume 3% TxEVM for the purpose of simulation for MCS4 + MCS21 to complete the requirement this week and close the WI. The MCS pair and SNR value corresponding to the higher MCS will be in square bracket, subject to reviewing the simulation results.

We update our simulation results as follows:

Table 2-1: Simulation assumptions

|  | Type 2 UE NR-CA demodulation requirements |
| --- | --- |
| Received power difference | 25 dB |
| Received time difference | 33 µs |
| Channel model | AWGN (TS38.101-4 B.1.1) |
| Maximum number of HARQ transmission | 1 |
| Signal/Noise setting | Set both Es and Noc to set SNR test points |
| Throughput measurement procedure | Measure both carriers |
| Antenna configuration | 2x2 for both carriers |
| Rank configuration | Rank1 for carrier with lower power Rank2 for carrier with higher power |
| MCS Table | MCS Table 1 for lower CC MCS Table 2 for stronger CC |
| Tx EVM | 6% for weaker CC (QPSK) 3% for stronger CC (256QAM) |
| Test metric | 70% of the maximum throughput |

Table 2-2: Simulation results for weaker CC

| MCS | SNR(dB)@70% of max TP (Ideal) | SNR(dB)@70% of max TP (Impairment) |
| --- | --- | --- |
| 4 | -4.3 | -3.3 |
| 5 | -3.4 | -2.4 |

Table 2-3: Simulation results for stronger CC

| MCS | SNR(dB)@70% of max TP (Ideal) | SNR(dB)@70% of max TP (Impairment) |
| --- | --- | --- |
| 21 | 19.5 | 20.5 |
| 22 | 20.4 | 21.4 |
| 23 | 22.1 | 23.1 |
| 24 | 22.6 | 23.6 |
| 25 | 24.4 | 25.4 |

- Based on our simulation results, the SNR difference for MCS4 (Weaker CC)+MCS21 (Stronger CC) is 23.8dB, so we propose to confirm MCS pair agreed in last meeting.

Proposal 1: Confirm MCS4 + MCS21 for requirements definition.

Conclusion

In this paper we provide our views on UE performance for Non-collocated CA. The proposal are:

Proposal 1: Confirm MCS4 + MCS21 for requirements definition.

Reference

[1]    R4-2321052  WF for NonCol_intraB_ENDC_NR_CA_Demod. Ericsson