3GPP TSG-RAN WG4 Meeting #110	R4-2401689

# Athens, Greece, 26th Feb - 1st Mar, 2024

Title: 	Simulation results on FR2 UL 256QAM performance requirements

Source: 	Huawei, HiSilicon

Agenda item:	8.2.3.1

Document for:	Discussion

Background

During RAN4#109 meeting, Way Forward [1] for 256 QAM BS Demod was approved. In this contribution, we provide our updated simulation results for FR2 UL 256QAM demodulation performance.

Simulation

| PTRS | SCS/BW | DMRS | Ideal | Impairment |
| --- | --- | --- | --- | --- |
| Enabled | 60kHz 50MHz | 1+1 | 16.8 | 19.4 |
|  |  | 1+0 | 17.1 | 19.9 |
|  | 120kHz 50MHz | 1+1 | 16.2 | 18.7 |
|  |  | 1+0 | 16.6 | 19.3 |
|  | 120kHz 200MHz | 1+1 | 16.2 | 19 |
|  |  | 1+0 | 16.4 | 19.4 |
| Disabled | 60kHz 50MHz | 1+1 | 16.2 | 18.7 |
|  | 120kHz 50MHz | 1+1 | 15.5 | 18 |
|  | 120kHz 200MHz | 1+1 | 15.5 | 18.2 |

Conclusion

In this contribution, we provide our updated simulation results for FR2 UL 256QAM demodulation performance.

Reference

R4-2321060, Way Forward for [109][317] NR_RF_FR2_req_Ph3_Demod, Moderator (Nokia, Nokia Shanghai Bell)