3GPP TSG-RAN WG4 Meeting #110	R4-2401695

# Athens, Greece, 26th Feb - 1st Mar, 2024

Title: 	Discussion on BS demodulation requirements for further coverage enhancements

Source: 	Huawei, HiSilicon

Agenda item:	8.19.2

Document for:	Discussion

Background

During RAN4#109 meeting, WF [1] on NR_cov_enh2_demod was approved. In this contribution, we share our views about BS demodulation requirements for further coverage enhancements.

Discussion

Multiple PRACH transmission requirements

Coverage of frequency range

- FR2-1 agreed, and with priority. FR1 as FFS.

Considering that the enhancements of PRACH are targeting for FR2 based on the latest WID, we prefer to not cover FR1 for multiple PRACH transmission requirements.

Do not cover FR1 for multiple PRACH transmission requirements.

Sub Carrier Spacing

- If FR1 is agreed to be introduced with PRACH repetition, 15 kHz and 30 kHz should be included. - For FR2-1 agreed 120 kHz, 60 kHz is FFS.

For the SCS, we propose to not consider 60kHz in FR2-1 for PRACH requirements, since that there is no any deployment in the real network currently.

Do not consider 60kHz in FR2-1 for PRACH requirements.

Proposals

In this contribution, we discuss on BS demodulation requirements for further coverage enhancements. Our observations and proposals are:

- Do not cover FR1 for multiple PRACH transmission requirements.

Do not consider 60kHz in FR2-1 for PRACH requirements.

Reference

R4-2321061, WF on NR_cov_enh2_demod, RAN4#109, China Telecom, Nokia