3GPP TSG-RAN WG4 Meeting #110	R4-2401706

# Athens, Greece, 26th Feb - 1st Mar, 2024

Title: 	Discussion on demodulation performance requirements for mobile IAB

Source: 	Huawei, HiSilicon

Agenda item:	8.25.6

Document for:	Discussion

Background

During RAN4#109 meeting, WF [1] for NR mobile IAB demod was approved. In this contribution, we share our views about demodulation performance requirements for mobile IAB.

Discussion

IAB-MT

How to define demodulation requirements for mIAB-MT

Agreement: (agreed online) Reuse applicable UE demodulation requirements in 38.101-4, and further discuss how to choose corresponding test cases.

From our understanding, for the cases selection to be reused from UE demodulation requirements in 38.101-4, the principle can be followed that one case with larger Doppler per physical channel can be selected.

FR1 PDSCH conducted

For FR1 PDSCH conducted test for mIAB, there is three candidate cases with higher Doppler can be reused from all the legacy UE PDSCH requirements, i.e. Test 1-1, 1-2 and 1-4 in Table 5.2.3.2.1-3 of TS 38.101-4. To reduce the test effort, we propose to reuse Test 1-1 in Table 5.2.3.2.1-3 of TS 38.101-4.

Table 5.2.3.2.1-3: Minimum performance for Rank 1 Test num.; Reference channel; Bandwidth (MHz) / Subcarrier spacing (kHz); Modulation format and code rate; TDD UL-DL pattern; Propagation condition; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNR (dB). 1-1; R.PDSCH.2-1.1 TDD; 40 / 30; QPSK, 0.30; FR1.30-1A; TDLB100-400; 2x4, ULA Low; 70; -4.1. 1-2; R.PDSCH.2-1.2 TDD; 40 / 30; QPSK, 0.30; FR1.30-1; TDLC300-100; 2x4, ULA Low; 70; -2.7. 1-3; R.PDSCH.2-4.1 TDD; 40 / 30; 256QAM, 0.82; FR1.30-1; TDLA30-10; 2x4, ULA Low; 70; 21.6. 1-4; R.PDSCH.2-2.1 TDD; 40 / 30; 16QAM, 0.48; FR1.30-1; TDLC300-100; 2x4, ULA Low; 30; -1.2. 1-5; R.PDSCH.2-5.1 TDD; 40 / 30; QPSK, 0.30; FR1.30-2; TDLA30-10; 2x4, ULA Low; 70; -3.8. 1-6; R.PDSCH.2-6.1 TDD; 40 / 30; QPSK, 0.30; FR1.30-3; TDLA30-10; 2x4, ULA Low; 70; -3.6. 1-7; R.PDSCH.2-10.1 TDD; 40 / 30; 16QAM, 0.48; FR1.30-1; HST-1000; 1x4; 70; 3.4. 1-8; R.PDSCH.2-11.1 TDD; 40 / 30; QPSK, 0.30; FR1.30-5; TDLB100-400; 2x4, ULA Low; 70; -4.0. 1-9; R.PDSCH.2-12.1 TDD; 40 / 30; QPSK, 0.30; FR1.30-6; TDLB100-400; 2x4, ULA Low; 70; -4.0. 1-10; R.PDSCH.2-10.2 TDD; 40 / 30; 16QAM, 0.48; FR1.30-1; TDLC300-1200; 2x4; 70; 5.8. 1-11; R.PDSCH.2-10.3 TDD; 40 / 30; 64QAM, 0.43; FR1.30-1; HST-1667; 1x4; 70; 6.8. 1-12; R.PDSCH.2-25.1 TDD; 40 / 30; 1024QAM, 0.79; FR1.30-1; TDLD30-5; 2x4, ULA Low; 70; 26.3.

- For FR1 PDSCH conducted test for mIAB, reuse Test 1-1 in Table 5.2.3.2.1-3 of TS 38.101-4.

FR1 PDSCH radiated

For FR1 PDSCH radiated test for mIAB, there is three candidate cases with higher Doppler can be reused from all the legacy UE PDSCH requirements, i.e. Test 1-1, 1-2 and 1-4 in Table 5.2.2.2.1-3 of TS 38.101-4. To reduce the test effort, we propose to reuse Test 1-1 in Table 5.2.2.2.1-3 of TS 38.101-4.

Table 5.2.2.2.1-3: Minimum performance for Rank 1 Test num.; Reference channel; Bandwidth (MHz) / Subcarrier spacing (kHz); Modulation format and code rate; TDD UL-DL pattern; Propagation condition; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNR (dB). 1-1; R.PDSCH.2-1.1 TDD; 40 / 30; QPSK, 0.30; FR1.30-1A; TDLB100-400; 2x2, ULA Low; 70; -1.1. 1-2; R.PDSCH.2-1.2 TDD; 40 / 30; QPSK, 0.30; FR1.30-1; TDLC300-100; 2x2, ULA Low; 70; 0.2. 1-3; R.PDSCH.2-4.1 TDD; 40 / 30; 256QAM, 0.82; FR1.30-1; TDLA30-10; 2x2, ULA Low; 70; 25.3. 1-4; R.PDSCH.2-2.1 TDD; 40 / 30; 16QAM, 0.48; FR1.30-1; TDLC300-100; 2x2, ULA Low; 30; 1.6. 1-5; R.PDSCH.2-5.1 TDD; 40 / 30; QPSK, 0.30; FR1.30-2; TDLA30-10; 2x2, ULA Low; 70; -0.9. 1-6; R.PDSCH.2-6.1 TDD; 40 / 30; QPSK, 0.30; FR1.30-3; TDLA30-10; 2x2, ULA Low; 70; -0.8. 1-7; R.PDSCH.2-10.1 TDD; 40 / 30; 16QAM, 0.48; FR1.30-1; HST-1000; 1x2; 70; 6.4. 1-8; R.PDSCH.2-11.1 TDD; 40 / 30; QPSK, 0.30; FR1.30-5; TDLB100-400; 2x2, ULA Low; 70; -1.0. 1-9; R.PDSCH.2-12.1 TDD; 40 / 30; QPSK, 0.30; FR1.30-6; TDLB100-400; 2x2, ULA Low; 70; -1.1. 1-10; R.PDSCH.2-10.2 TDD; 40 / 30; 16QAM, 0.48; FR1.30-1; TDLC300-1200; 2x2; 70; 9.5. 1-11; R.PDSCH.2-10.3 TDD; 40 / 30; 64QAM, 0.43; FR1.30-1; HST-1667; 1x2; 70; 9.6. 1-12; R.PDSCH.2-25.1 TDD; 40 / 30; 1024QAM, 0.79; FR1.30-1; TDLD30-5; 2x2, ULA Low; 70; 29.4.

- For FR1 PDSCH radiated test for mIAB, reuse Test 1-1 in Table 5.2.2.2.1-3 of TS 38.101-4.

FR2 PDSCH radiated

For FR2 PDSCH radiated test for mIAB, there is three candidate cases with higher Doppler can be reused from all the legacy UE PDSCH requirements, i.e. Test 1-1, 1-2 and 1-3 in Table 7.2.2.2.1-3 of TS 38.101-4. To reduce the test effort, we propose to reuse Test 1-3 in Table 7.2.2.2.1-3 of TS 38.101-4.

Table 7.2.2.2.1-3: Minimum performance for Rank 1 (FRC) for FR2-1 Test num.; Reference channel; Bandwidth (MHz) / Subcarrier spacing (kHz); Modulation and code rate; TDD UL-DL pattern; Propagation condition; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNRBB (dB). 1-1; R.PDSCH.5-1.1 TDD; 100 / 120; QPSK, 0.30; FR2.120-1A; TDLC60-300; 2x2 ULA Low; 70; -0.4. 1-2; R.PDSCH.5-2.1 TDD; 100 / 120; 16QAM, 0.48; FR2.120-1; TDLA30-300; 2x2 ULA Low; 30; 1.7. 1-3; R.PDSCH.5-3.1 TDD; 100 / 120; 64QAM, 0.46; FR2.120-1; TDLA30-300; 2x2 XPL Medium; 70; 12.4. 1-4; R.PDSCH.5-10.1 TDD; 50 / 120; 256QAM 0.67; FR2.120-1; TDLD30-75; 2x2 ULA Low; 70; 20.2.

- For FR2 PDSCH radiated test for mIAB, reuse Test 1-3 in Table 7.2.2.2.1-3 of TS 38.101-4.

FR1 PDCCH conducted

For FR1 PDCCH conducted test for mIAB, there is two candidate cases with higher Doppler can be reused from all the legacy UE PDCCH requirements, i.e. Test 2 in Table 5.3.3.2.1-1 of TS 38.101-4 and Test 1 in Table 5.3.3.2.2-1 of TS 38.101-4. To reduce the test effort, we propose to reuse Test 2 in Table 5.3.3.2.1-1 of TS 38.101-4.

Table 5.3.3.2.1-1: Minimum performance for PDCCH with 30 kHz SCS Test number; Bandwidth (MHz); CORESET RB; CORESET duration; Aggregation level; Reference Channel; Propagation Condition; Antenna configuration and correlation Matrix; Reference value. Pm-dsg (%); SNR (dB). 1; 40; 102; 1; 2; R.PDCCH. 2-1.1 TDD; TDLA30-10; 1x4 Low; 1; 2.1. 2; 40; 102; 1; 4; R.PDCCH. 2-1.2 TDD; TDLC300-100; 1x4 Low; 1; -0.9. 3; 40; 48; 2; 16; R.PDCCH. 2-2.1 TDD; TDLA30-10; 1x4 Medium A; 1; -3.6. Table 5.3.3.2.2-1: Minimum performance for PDCCH with 30 kHz SCS Test number; Bandwidth (MHz); CORESET RB; CORESET duration; Aggregation level; Reference Channel; Propagation Condition; Antenna configuration and correlation Matrix; Reference value. Pm-dsg (%); SNR (dB). 1; 40; 90; 1; 8; R.PDCCH. 2-1.3 TDD; TDLC300-100; 2x4 Low; 1; -4.3.

For FR1 PDCCH conducted test for mIAB, reuse Test 2 in Table 5.3.3.2.1-1 of TS 38.101-4.

FR1 PDCCH radiated

For FR1 PDCCH radiated test for mIAB, there is two candidate cases with higher Doppler can be reused from all the legacy UE PDCCH requirements, i.e. Test 2 in Table 5.3.2.2.1-1 of TS 38.101-4 and Test 1 in Table 5.3.2.2.2-1 of TS 38.101-4. To reduce the test effort, we propose to reuse Test 2 in Table 5.3.2.2.1-1 of TS 38.101-4.

Table 5.3.2.2.1-1: Minimum performance for PDCCH with 30 kHz SCS Test number; Bandwidth (MHz); CORESET RB; CORESET duration; Aggregation level; Reference Channel; Propagation Condition; Antenna configuration and correlation Matrix; Reference value. Pm-dsg (%); SNR (dB). 1; 40; 102; 1; 2; R.PDCCH. 2-1.1 TDD; TDLA30-10; 1x2 Low; 1; 7.0. 2; 40; 102; 1; 4; R.PDCCH. 2-1.2 TDD; TDLC300- 100; 1x2 Low; 1; 3.0. 3; 40; 48; 2; 16; R.PDCCH. 2-2.1 TDD; TDLC300- 100; 1x2 Low; 1; -3.8. Table 5.3.2.2.2-1: Minimum performance for PDCCH with 30 kHz SCS Test number; Bandwidth (MHz); CORESET RB; CORESET duration; Aggregation level; Reference Channel; Propagation Condition; Antenna configuration and correlation Matrix; Reference value. Pm-dsg (%); SNR (dB). 1; 40; 90; 1; 8; R.PDCCH. 2-1.3 TDD; TDLC300-100; 2x2 Low; 1; -1.2.

For FR1 PDCCH radiated test for mIAB, reuse Test 2 in Table 5.3.2.2.1-1 of TS 38.101-4.

FR2 PDCCH radiated

For FR2 PDCCH radiated test for mIAB, there is only one case with higher Doppler can be reused from all the legacy UE PDCCH requirements, i.e. Test 1-2 in Table 7.3.2.2.1-1 of TS 38.101-4. So we propose to reuse Test 1-2 in Table 7.3.2.2.1-1 of TS 38.101-4.

Table 7.3.2.2.1-1: Minimum performance requirements with 120 kHz SCS for FR2-1 Test number; Bandwidth (MHz); CORESET RB; CORESET duration; Aggregation level; Reference Channel; Propagation Condition; Antenna configuration and correlation Matrix; Reference value. Pm-dsg (%); SNRBB (dB). 1-1; 100; 60; 1; 2; R.PDCCH. 5-1.1 TDD; TDLA30-75; 1x2 Low; 1; 6.4. 1-2; 100; 60; 1; 4; R.PDCCH. 5-1.2 TDD; TDLA30-300; 1x2 Low; 1; 3.0. Table 7.3.2.2.2-1: Minimum performance requirements with 120 kHz SCS for FR2-1 Test number; Bandwidth (MHz); CORESET RB; CORESET duration; Aggregation level; Reference Channel; Propagation Condition; Antenna configuration and correlation Matrix; Reference value. Pm-dsg (%); SNRBB (dB). 2-1; 100; 60; 1; 8; R.PDCCH. 5-1.3 TDD; TDLA30-75; 2x2 Low; 1; 0.1. 2-2; 100; 60; 2; 16; R.PDCCH. 5-2.1 TDD; TDLA30-75; 2x2 Low; 1; -3.0.

For FR2 PDCCH radiated test for mIAB, reuse Test 1-2 in Table 7.3.2.2.1-1 of TS 38.101-4.

Test case scope

Agreement: (agreed online) Introduce mIAB-MT demodulation requirements for PDSCH and PDCCH. FFS on CSI reporting: Option 1: Apply IAB-MT CSI requirement. Option 2: Reuse applicable legacy UE CSI requirements.

There are only cases with very low mobility for the legacy UE CSI requirements, even higher Doppler is considered for PDSCH and PDCCH cases. We think such cases are enough to verify whether CSI reporting is accurate. Also to reduce the simulation effort, we propose to reuse CSI requirements for mIAB from legacy UE CSI requirements.

Reuse CSI requirements for mIAB from legacy UE CSI requirements.

Antenna configuration for conducted test

- Agreement (agreed online) - 4Rx agreed as the start point. - FFS on 1 or 2 TX.

Based on discussion in clause 2.1.1, we propose to consider only 2Tx for PDSCH conducted test and only 1Tx for PDCCH conducted test.

Consider only 2Tx for PDSCH conducted test and only 1Tx for PDCCH conducted test.

Antenna configuration for OTA test

Agreement: - 2Rx antenna configuration for OTA. - For PDSCH, 2Tx antenna configuration for OTA - For PDCCH, further discussion on 1Tx and/or 2Tx

Based on discussion in clause 2.1.1, we propose to consider only 1Tx for PDCCH OTA test.

Consider only 1Tx for PDCCH OTA test.

How to add mIAB-MT requirement to current specification

- Proposals - Option 1: Add a new section in TS 38.174 to state that demodulation performance requirements for mIAB-MT. (QC) - Option 2: Other options are not precluded.

For the specification drafting, from the readability and future compatibility point of view, we prefer to add new clause 8.2.2.1.3 and 8.2.2.2.3, as following:

Add new clauses 8.2.2.1.3 and 8.2.2.2.3 for mIAB-IMT performance requirements.

IAB-DU

- Proposals - Option 1: Reuse legacy IAB-DU requirements with TDLA30-10 channel condition. (Ericsson) - Option 2: Other options are not precluded. - Recommended WF - Companies to discuss based on test cases defined for IAB-DU in 38.174.

Based on the WID, both onboard and surrounding UEs are regarded as candidate users for the IAB-DU. That means we cannot exclude the users with different mobility than IAB-DU.

The work on Mobile IAB in Rel-18 should focus on the scenario of mobile-IAB-nodes mounted on vehicles providing 5G coverage/capacity enhancement to onboard and/or surrounding UEs.

The work on Mobile IAB in Rel-18 should focus on the scenario of mobile-IAB-nodes mounted on vehicles providing 5G coverage/capacity enhancement to onboard and/or surrounding UEs.

In the existing Rel-16 IAB-DU demodulation performance requirements, the mobility has already been considered. So we propose to reuse all legacy IAB-DU requirements for the mIAB-DU.

Reuse all legacy IAB-DU requirements for the mIAB-DU.

Proposals

In this contribution, we discuss on demodulation performance requirements for mobile IAB. Our observations and proposals are:

- For FR1 PDSCH conducted test for mIAB, reuse Test 1-1 in Table 5.2.3.2.1-3 of TS 38.101-4.

- For FR1 PDSCH radiated test for mIAB, reuse Test 1-1 in Table 5.2.2.2.1-3 of TS 38.101-4.

- For FR2 PDSCH radiated test for mIAB, reuse Test 1-3 in Table 7.2.2.2.1-3 of TS 38.101-4.

For FR1 PDCCH conducted test for mIAB, reuse Test 2 in Table 5.3.3.2.1-1 of TS 38.101-4.

For FR1 PDCCH radiated test for mIAB, reuse Test 2 in Table 5.3.2.2.1-1 of TS 38.101-4.

For FR2 PDCCH radiated test for mIAB, reuse Test 1-2 in Table 7.3.2.2.1-1 of TS 38.101-4.

Reuse CSI requirements for mIAB from legacy UE CSI requirements.

Consider only 2Tx for PDSCH conducted test and only 1Tx for PDCCH conducted test.

Consider only 1Tx for PDCCH OTA test.

Add new clauses 8.2.2.1.3 and 8.2.2.2.3 for mIAB-IMT performance requirements.

- The work on Mobile IAB in Rel-18 should focus on the scenario of mobile-IAB-nodes mounted on vehicles providing 5G coverage/capacity enhancement to onboard and/or surrounding UEs.

Reuse all legacy IAB-DU requirements for the mIAB-DU.

Reference

R4-2321144, WF for [109][329] NR_mobile_IAB_demod, RAN4#109, Ericsson