3GPP TSG-RAN WG4 Meeting #110	R4-2401708

# Athens, Greece, 26th Feb - 1st Mar, 2024

Title: 	Discussion on UE CSI reporting requirements for NR FR2 multi-Rx chain DL reception

Source: 	Huawei, HiSilicon

Agenda item:	8.3.4.3

Document for:	Discussion

Background

During RAN4#109 meeting, WF [1] on UE demodulation and CSI requirements for FR2 multi-Rx DL reception was approved. In this contribution, we share our views about UE CSI reporting requirements for NR FR2 multi-Rx chain DL reception.

Discussion

Performance metric

Based on our simulation results, we can see that there is much gain at 90% of the maximum throughput comparing to 70% of the maximum throughput. Also considering 90% of the maximum throughput is the same value as legacy requirements, we propose to select 90% of the maximum throughput for FR2 multi-Rx PMI reporting requirements. Considering implementation margin, we prefer to select γ = 1.3 as the test metric.

Select 90% of the maximum throughput for FR2 multi-Rx PMI reporting requirements.

Select γ = 1.3 as the test metric.

Reference channel

- <way forward> - Option 1: Use reference channels as presented in Table 2.3-1 for FR2 multi-Rx PMI reporting requirements. - Option 2: Reuse the reference channel from TS38.101-4, section 6.3.2.1.7 with adaptation to 4 ports as starting point. Configure the codebook for 1 panel.

Considering that we have two cases with different PTRS configuration, two reference channels should be defined. We propose to use reference channels as following Table 2.2-1 for FR2 multi-Rx PMI reporting requirements. For the Option 2, the reference channel in section 6.3.2.1.7 is configured with FR1 so it cannot be used.

Table 2.2-1 Reference channels for FR2 multi-Rx PMI reporting requirements

| Parameter | Unit | 1 PTRS port | 2 PTRS port |
| --- | --- | --- | --- |
| Channel bandwidth | MHz | 100 | 100 |
| Subcarrier spacing | kHz | 120 | 120 |
| Allocated resource blocks | PRBs | 66 | 66 |
| Number of consecutive PDSCH symbols |  | 12 | 12 |
| Allocated slots per 2 frames |  | 63 | 63 |
| MCS table |  | 64QAM | 64QAM |
| MCS index |  | 13 | 13 |
| Modulation |  | 16QAM | 16QAM |
| Target Coding Rate |  | 0.48 | 0.48 |
| Number of MIMO layers |  | 2 | 2 |
| Number of DMRS REs (Note 3) |  | 24 | 24 |
| Overhead for TBS determination |  | 6 | 12 |
| Information Bit Payload per Slot |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | Bits | N/A | N/A |
| For CSI-RS Slot i, if mod(i,5) =1 for i from {0,…,159} | Bits | N/A | N/A |
| For Slot i = 80 | Bits | 28680 | 27144 |
| For Slot i, if mod(i, 5) = {0,2} for i from {1,…,79,82,…,159} | Bits | 28680 | 27144 |
| Transport block CRC per Slot |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | Bits | N/A | N/A |
| For CSI-RS Slot i, if mod(i,5) =1 for i from {0,…,159} | Bits | N/A | N/A |
| For Slot i = 80 | Bits | 24 | 24 |
| For Slot i, if mod(i, 5) = {0,2} for i from {1,…,79,82,…,159} | Bits | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | CBs | N/A | N/A |
| For CSI-RS Slot i, if mod(i,5) =1 for i from {0,…,159} | CBs | N/A | N/A |
| For Slot i = 80 | CBs | 4 | 4 |
| For Slot i, if mod(i, 5) = {0,2} for i from {1,…,79,82,…,159} | CBs | 4 | 4 |
| Binary Channel Bits Per Slot |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | Bits | N/A | N/A |
| For CSI-RS Slot i, if mod(i,5) =1 for i from {0,…,159} | Bits | N/A | N/A |
| For Slot i = 80 | Bits | 57648 | 55008 |
| For Slot i, if mod(i, 5) = {0,2} for i from {1,…,79,82,…,159} | Bits | 60720 | 58080 |
| Max. Throughput averaged over 2 frames | Mbps | 90.342 | 90.342 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data |  |  |  |

Use reference channels as shown in Table 2.2-1 for FR2 multi-Rx PMI reporting requirements.

Proposals

In this contribution, we discuss on UE CSI reporting requirements for NR FR2 multi-Rx chain DL reception. Our observations and proposals are:

- Select 90% of the maximum throughput for FR2 multi-Rx PMI reporting requirements.

Select γ = 1.3 as the test metric.

Use reference channels as shown in Table 2.2-1 for FR2 multi-Rx PMI reporting requirements.

Table 2.2-1 Reference channels for FR2 multi-Rx PMI reporting requirements

| Parameter | Unit | 1 PTRS port | 2 PTRS port |
| --- | --- | --- | --- |
| Channel bandwidth | MHz | 100 | 100 |
| Subcarrier spacing | kHz | 120 | 120 |
| Allocated resource blocks | PRBs | 66 | 66 |
| Number of consecutive PDSCH symbols |  | 12 | 12 |
| Allocated slots per 2 frames |  | 63 | 63 |
| MCS table |  | 64QAM | 64QAM |
| MCS index |  | 13 | 13 |
| Modulation |  | 16QAM | 16QAM |
| Target Coding Rate |  | 0.48 | 0.48 |
| Number of MIMO layers |  | 2 | 2 |
| Number of DMRS REs (Note 3) |  | 24 | 24 |
| Overhead for TBS determination |  | 6 | 12 |
| Information Bit Payload per Slot |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | Bits | N/A | N/A |
| For CSI-RS Slot i, if mod(i,5) =1 for i from {0,…,159} | Bits | N/A | N/A |
| For Slot i = 80 | Bits | 28680 | 27144 |
| For Slot i, if mod(i, 5) = {0,2} for i from {1,…,79,82,…,159} | Bits | 28680 | 27144 |
| Transport block CRC per Slot |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | Bits | N/A | N/A |
| For CSI-RS Slot i, if mod(i,5) =1 for i from {0,…,159} | Bits | N/A | N/A |
| For Slot i = 80 | Bits | 24 | 24 |
| For Slot i, if mod(i, 5) = {0,2} for i from {1,…,79,82,…,159} | Bits | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | CBs | N/A | N/A |
| For CSI-RS Slot i, if mod(i,5) =1 for i from {0,…,159} | CBs | N/A | N/A |
| For Slot i = 80 | CBs | 4 | 4 |
| For Slot i, if mod(i, 5) = {0,2} for i from {1,…,79,82,…,159} | CBs | 4 | 4 |
| Binary Channel Bits Per Slot |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | Bits | N/A | N/A |
| For CSI-RS Slot i, if mod(i,5) =1 for i from {0,…,159} | Bits | N/A | N/A |
| For Slot i = 80 | Bits | 57648 | 55008 |
| For Slot i, if mod(i, 5) = {0,2} for i from {1,…,79,82,…,159} | Bits | 60720 | 58080 |
| Max. Throughput averaged over 2 frames | Mbps | 90.342 | 90.342 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data |  |  |  |

Reference

R4-2321140, WF on UE demodulation and CSI requirements for FR2 multi-Rx DL reception, RAN4#109, Qualcomm