3GPP TSG-RAN WG4 Meeting #110	R4-2401710

# Athens, Greece, 26th Feb - 1st Mar, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | draft | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on Minimum requirements and FRC definition for sDCI SDM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR2_multiRX_DL-Perf |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduce minimum requirements and FRC definition for sDCI SDM. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For introducing minimum requirements and FRC definition for sDCI SDM, add clause 7.2.2.2.5, update clause 9.2A.1.1, A.3.2.2.5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be inconsist between specification and RAN4 agreements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.2.2.5 (New clause), 9.2A.1.1, A.3.2.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | New clause: 7.2.2.2.5 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R4-2321205 |  |  |  |  |  |  |  |  |

<START OF THE CHANGE 1>

##### 7.2.2.2.5	Minimum requirements for PDSCH Single-DCI based SDM scheme for multi-Rx reception

The performance requirements are specified in Table 7.2.2.2.5-3, with the addition of test parameters in Table 7.2.2.2.5-2 and the downlink physical channel setup according to Annex C.5.1.

The test purposes are specified in Table 7.2.2.2.5-1.

Table 7.2.2.2.5-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH performance with Single-DCI based SDM scheme for multi-Rx reception. | 1-1, 1-2 |

Table 7.2.2.2.1-2: Test Parameters

| Parameter |  |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  |  | TCI State #1 |  |
|  |  | CORESETPoolIndex |  |  | 0 |  |
| CSI-RS for tracking |  | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resources 1,2,3,4 | k0=1 for CSI-RS resources 5,6,7,8 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 | l0 = 6 for CSI-RS resources 5 and 7 l0 = 10 for CSI-RS resources 6 and 8 |
|  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 | 1 for CSI-RS resource 5,6,7,8 |
|  |  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4,5,6,7,8 |  |
|  |  | Density |  |  | 3 |  |
|  |  | CSI-RS periodicity |  | Slots | 160 |  |
|  |  | CSI-RS offset |  | Slots | 80 for CSI-RS resources 1 and 2 81 for CSI-RS resources 3 and 4 | 80 for CSI-RS resources 5 and 6 81 for CSI-RS resources 7 and 8 |
|  |  | QCL info |  |  | TCI state #0 |  |
| CSI-RS for beam refinement |  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | k0=0 for CSI-RS resource 1,2 | k0=1 for CSI-RS resource 3,4 |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | l0 = 8 for CSI-RS resource 1 l0 = 9 for CSI-RS resource 2 | l0 = 8 for CSI-RS resource 3 l0 = 9 for CSI-RS resource 4 |
|  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2 | 1 for CSI-RS resource 3,4 |
|  |  | CDM Type |  |  | 'No CDM' for CSI-RS resource 1,2 | 'No CDM' for CSI-RS resource 3,4 |
|  |  | Density (ρ) |  |  | 3 for CSI-RS resource 1,2 | 3 for CSI-RS resource 3,4 |
|  |  | CSI-RS periodicity |  |  | 160 for CSI-RS resource 1,2 | 160 for CSI-RS resource 3,4 |
|  |  | CSI-RS offset |  |  | 0 for CSI-RS resource 1,2 | 0 for CSI-RS resource 3,4 |
|  |  | Repetition |  |  | ON | ON |
|  |  | QCL info |  |  | TCI state #1 | TCI state #2 |
| Duplex mode |  |  |  |  | TDD |  |
| Active DL BWP index |  |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  |  | Type A |  |
|  | k0 |  |  |  | 0 |  |
|  | Starting symbol (S) |  |  |  | 1 |  |
|  | Length (L) |  |  |  | Specific to each Reference channel as defined in A.3.2.2 |  |
|  | PRB bundling type |  |  |  | Static |  |
|  | PRB bundling size |  |  |  | 2 |  |
|  | Resource allocation type |  |  |  | Type 1 |  |
|  | RBG size |  |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  |  | 1000 | 1002 |
|  | TCI state |  |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  |  | 1 |  |
| TCI State #1 | Type 1 QCL information |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration | N/A |
|  |  |  | QCL Type |  | Type A | N/A |
|  | Type 2 QCL information |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration | N/A |
|  |  |  | QCL Type |  | Type D | N/A |
| TCI State #2 | Type 1 QCL information |  | CSI-RS resource |  | N/A | CSI-RS resource 5 from 'CSI-RS for tracking’ configuration |
|  |  |  | QCL Type |  | N/A | Type A |
|  | Type 2 QCL information |  | CSI-RS resource |  | N/A | CSI-RS resource 5 from 'CSI-RS for tracking’ configuration |
|  |  |  | QCL Type |  | N/A | Type D |
| PTRS configuration | Frequency density (KPT-RS) |  |  |  | 2 | 2 for Test 1-2, Disabled for Test 1-1 |
|  | Time density (LPT-RS) |  |  |  | 1 | 1 for Test 1-2, Disabled for Test 1-1 |
|  | Resource Element Offset |  |  |  | 2 | 3 for Test 1-2, N/A for Test 1-1 |
| Resource allocation |  |  |  |  | Full-overlapping |  |
| Timing offset of the second TRxP from the first TRxP |  |  |  | us | -0.0625 |  |
| Frequency offset of the second TRxP from the first TRxP |  |  |  | Hz | 600 |  |
| Number of HARQ Processes |  |  |  |  | 8 |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |  |
| Precoding configuration |  |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity. |  |
| Note 1: PDSCH transmission is done from both TRxPs (PDSCH Layer 0 is transmitted from TRxP #1 and PDSCH layer 1 is transmitted from TRxP #2) |  |  |  |  |  |  |

Table 7.2.2.2.1-3: Minimum performance

|  | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNRBB (dB) |
| 1-1 | R.PDSCH.5-16.1 TDD | 100 / 120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-35 | 2x4 XPL Low | 70 | [12.4] |
| 1-2 | R.PDSCH.5-16.2 TDD | 100 / 120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-35 | 2x4 XPL Low | 70 | [12.4] |

<END OF THE CHANGE 1>

<START OF THE CHANGE 2>

#### 9.2A.1.1	NR CA between FR1 and FR2-2

The performance requirements for SCell on FR2-2 band are specified in Table 9.2A.1-5. The test parameters for SCell are specified in Table 7.2.2.2.1-2 with additional change of PDSCH resource allocation type specified in Table 9.2A.1-1:

Table 9.2A.1-1: PDSCH resource allocation type

| Test numer | Allocation Type | Start RB | LRBs |
| --- | --- | --- | --- |
| 1-1 | Type 1 | 98 | 66 |
| 1-2 |  |  |  |
| 1-3 |  |  |  |
| 1-4 |  |  |  |
| 1-6 | ` | 24 | 20 |
| 1-5Note1 | Type 0 | N/A | N/A |
| Note 1: Full BWP is allocated for PDSCH |  |  |  |

The test parameters for PCell in Table 9.2A.1-3 and the downlink physical channel setup according to Annex C.3.1. In this test, Pcell is in FR1 and Scell is in FR2-2 and only requirements for Scell should be verified.

The test purposes are specified in Table 9.2A.1-2.

Table 9.2A.1-2: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH mapping Type A normal performance in FR2-2 Scell CC in CA between FR1 and FR2-2 under 2 receive antenna conditions and with different channel models, MCSs and number of MIMO layers | 1-1,1-2,1-3,1-4,1-5,1-6 |

Table 9.2A.1-3: Test parameters for PCell

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Bandwidth |  | MHz | 40 |
| Subcarrier spacing |  | kHz | 30 |
| TDD pattern |  |  | FR1.30-1 |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Dmrs-AdditionalPosition |  | pos1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |
| PUCCH format for HARQ-ACK feedback |  |  | PUCCH format 3 |

Table 9.2A.1-4: K1 values for each CC

| Cells | K1 values |
| --- | --- |
| Pcell CC | {8,7,6,5,4,3,2} |
| SCell CC with 120kHz | {8,8,8,8,7,7,7,7} |
| SCell CC with 480kHz (Note 1) | {8,8,8,7} |
| Note 1: One k1 value applies for 4 TBs scheduledby one DCI |  |

Table 9.2A.1-5: Minimum performance for Rank 1 (FRC) in Scell CC

| Test num | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of max through-put (%) | SNRBB (dB) |
| 1-1 | R.PDSCH.5-13.1 TDD | 400 / 120 | QPSK, 0.30 | FR2.120-1 | TDLA30-650 | 2x2 ULA Low | 70 | 0.9 |
| 1-2 | R.PDSCH.5-14.1 TDD | 400 / 120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-200 | 2x2 ULA Low | 70 | 9.0 |
| 1-3 | R.PDSCH.5-14.1 | 400 / 120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-650 | 2x2 ULA Low | 30 | 2.7 |
| 1-4 | R.PDSCH.5-15.1 TDD | 400 / 120 | 64QAM, 0.43 | FR2.120-1 | TDLD30-200 | 2x2 ULA Low | 70 | 11.6 |
| 1-5 | R.PDSCH.8-3.1 TDD | 400 / 480 | QPSK, 0.30 | FR2.480-1 | TDLA10-200 | 2x2 ULA Low | 70 | 1.0 |
| 1-6 | R.PDSCH.8-4.1 TDD | 400 / 480 | 16QAM 0.48 | FR2.480-1 | TDLD10-200 | 2x2 ULA Low | 70 | 8.3 |

<END OF THE CHANGE 2>

<START OF THE CHANGE 3>

#### A.3.2.2.5	Reference measurement channels for SCS 120 kHz FR2

<Unchanged part skipped>

Table A.3.2.2.5-13: Reference measurement channels for FR1+FR2-2 CA with 120 kHz SCS (QPSK)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.5-13.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 100 |  |  |  |  |
| Subcarrier spacing | kHz | 120 |  |  |  |  |
| Allocated resource blocks | PRBs | 66 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} |  | N/A |  |  |  |  |
| For Slot i=0,80,81 |  | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} |  | 9 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠0,80,81 |  | 13 |  |  |  |  |
| Allocated slots per 2 frames |  | 29 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 4 |  |  |  |  |
| Modulation |  | QPSK |  |  |  |  |
| Target Coding Rate |  | 0.30 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} |  | N/A |  |  |  |  |
| For Slot i=0,80,81 |  | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} |  | 12 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠0,80,81 |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 6 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} | Bits | N/A |  |  |  |  |
| For Slot i=0,80,81 |  | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} | Bits | 3624 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠0,80,81 | Bits | 5504 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} | Bits | N/A |  |  |  |  |
| For Slot i=0,80,81 |  | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} | Bits | 16 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠0,80,81 | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} | CBs | N/A |  |  |  |  |
| For Slot i=0,80,81 |  | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} | CBs | 1 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠0,80,81 | CBs | 1 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} | Bits | N/A |  |  |  |  |
| For Slot i=0,80,81 | Bits | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} | Bits | 12210 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠0,80,81 |  | 18282 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 7.229 |  |  |  |  |
| Note 1: Slot i is slot index per 160 slots |  |  |  |  |  |  |

Table A.3.2.2.5-14: Reference measurement channels for FR1+FR2-2 CA with 120 kHz SCS (16QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.5-14.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 100 |  |  |  |  |
| Subcarrier spacing | kHz | 120 |  |  |  |  |
| Allocated resource blocks | PRBs | 66 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} |  | N/A |  |  |  |  |
| For Slot i=0,80,81 |  | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} |  | 9 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠80,81 |  | 13 |  |  |  |  |
| Allocated slots per 2 frames |  | 29 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 13 |  |  |  |  |
| Modulation |  | 16QAM |  |  |  |  |
| Target Coding Rate |  | 0.48 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} |  | N/A |  |  |  |  |
| For Slot i=0,80,81 |  | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} |  | 12 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠0,80,81 |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 6 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} | Bits | N/A |  |  |  |  |
| For Slot i=0,80,81 |  | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} | Bits | 11272 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠0,80,81 | Bits | 17424 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} | Bits | N/A |  |  |  |  |
| For Slot i=0,80,81 |  | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} | Bits | 24 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠0,80,81 | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} | CBs | N/A |  |  |  |  |
| For Slot i=0,80,81 |  | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} | CBs | 2 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠0,80,81 | CBs | 3 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} | Bits | N/A |  |  |  |  |
| For Slot i=0,80,81 | Bits | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} | Bits | 24420 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠0,80,81 |  | 36564 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 22.804 |  |  |  |  |
| Note 1: Slot i is slot index per 160 slots |  |  |  |  |  |  |

Table A.3.2.2.5-15: Reference measurement channels for FR1+FR2-2 CA with 120 kHz SCS (64QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.5-15.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 100 |  |  |  |  |
| Subcarrier spacing | kHz | 120 |  |  |  |  |
| Allocated resource blocks | PRBs | 66 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} |  | N/A |  |  |  |  |
| For Slot i=0,80,81 |  | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} |  | 9 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠0,80,81 |  | 13 |  |  |  |  |
| Allocated slots per 2 frames |  | 29 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 17 |  |  |  |  |
| Modulation |  | 64QAM |  |  |  |  |
| Target Coding Rate |  | 0.43 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} |  | N/A |  |  |  |  |
| For Slot i=0,80,81 |  | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} |  | 12 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠0,80,81 |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 6 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} | Bits | N/A |  |  |  |  |
| For Slot i=0,80,81 |  | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} | Bits | 15112 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠0,80,81 | Bits | 23568 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} | Bits | N/A |  |  |  |  |
| For Slot i=0,80,81 |  | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} | Bits | 24 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠0,80,81 | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} | CBs | N/A |  |  |  |  |
| For Slot i=0,80,81 |  | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} | CBs | 2 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠0,80,81 | CBs | 3 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i mod 40= {4,9,10,…,39} | Bits | N/A |  |  |  |  |
| For Slot i=0,80,81 | Bits | N/A |  |  |  |  |
| For Slots i mod 40= {3,8} | Bits | 36630 |  |  |  |  |
| For Slot i mod 40= {0,1,2,5,6,7} and i≠0,80,81 |  | 54846 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 30.791 |  |  |  |  |
| Note 1: Slot i is slot index per 160 slots |  |  |  |  |  |  |

Table A.3.2.2.5-16 Reference measurement channels for PDSCH Single-DCI based SDM scheme for FR2 multi-Rx reception

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.5-16.1 TDD | R.PDSCH.5-16.2 TDD |  |  |  |
| Channel bandwidth | MHz | 100 | 100 |  |  |  |
| Subcarrier spacing | kHz | 120 | 120 |  |  |  |
| Allocated resource blocks | PRBs | 66 | 66 |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | 9 | 9 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} |  | 13 | 13 |  |  |  |
| Allocated slots per 2 frames |  | 127 | 127 |  |  |  |
| MCS table |  | 64QAM | 64QAM |  |  |  |
| MCS index |  | 17 | 17 |  |  |  |
| Modulation |  | 64QAM | 64QAM |  |  |  |
| Target Coding Rate |  | 0.43 | 0.43 |  |  |  |
| Number of MIMO layers |  | 2 | 2 |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | 24 | 24 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} |  | 24 | 24 |  |  |  |
| Overhead for TBS determination |  | 6 | 12 |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 26632 | 24576 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | Bits | 43032 | 40976 |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 24 | 24 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | Bits | 24 | 24 |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | CBs | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | CBs | 4 | 3 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | CBs | 6 | 5 |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A |  |  |  |
| For Slots i = 80, 81 | Bits | 95580 | 91224 |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 63756 | 60984 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,79,82,…,159} | Bits | 100188 | 95832 |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 247.0132 | 233.9576 |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

<END OF THE CHANGE 3>