3GPP TSG-RAN4 Meeting #110	R4-2401743

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0493 | rev | - | Current version: | 17.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_unlic-Perf) Correction of configuration for NR-U CQI reporting requirements (R17) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-Perf |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | NR-U CQI reporting test verifies the reported CQI indexes for SCell on band with shared spectrum access. This means CSI report should be configured in SCell. However the current specification only configures CSI report in PCell only. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the CSI report configuration in SCell. - Remove CSI reporting configuration from PCell, because CSI report is not necessary in PCell Remove ‘]’ from SNR in Table 6.2A.3.1.2-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test equipment does not know how to configure CSI reporting in SCell. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.3.1.2, 6.2A.4.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.521-4 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change ------------------------------------------------------------

##### 6.2A.3.1.2	Minimum requirement for CQI reporting for SCell on band with shared spectrum access

The purpose of the requirements is to verify that the reported CQI values are in accordance with the CQI definition given in TS 38.214 [12] for Scell on band with shared spectrum access. For each downlink transmission duration the transmission power offset is randomly chosen between [0, +6] dB and 2 sets of CQI reports are obtained for each transmission power offset. The reporting accuracy of CQI under AWGN condition is determined by the reporting variance and BLER performance using the transport format indicated by the reported CQI median for each power offset. To account for sensitivity of the input SNR the reporting definition is considered to be verified if the reporting accuracy is met for at least one of two SNR levels separated by an offset of 1 dB.

For the parameters specified in Table 6.2A.3.1.2-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	For each transmission power offset the reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than 90% of the time.

b)	For each transmission power offset, if the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than 0.1. For each transmission power offset, if the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to 0.1.

c)	The absolute difference in median CQI for each of transmission power offset shall be ≥ 2.

The test parameters for configuring the PCell are specified in Table 6.2A.3.1.2-2, but requirements are only applicable to SCell on band with shared spectrum access.

Table 6.2A.3.1.2-1: CQI reporting test parameters for SCell on band with shared spectrum access

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 20 |  |
| Subcarrier spacing |  | kHz | 30 |  |
| Duplex Mode |  |  | TDD |  |
| Downlink Transmission Model |  |  | As specified in Annex B.5 |  |
| Downlink Transmission Model Parameters | Downlink period | ms | 5 |  |
|  | LBT failure probability (pLBT) |  | 0.25 |  |
|  | Downlink transmission duration values set | slot | {4,6,7} |  |
|  | Occupied OFDM symbols in slot other than the last slot of the downlink duration | symbols | 14 |  |
|  | Occupied OFDM symbols in the last slot of the downlink duration | symbols | 14 |  |
| TDD UL-DL pattern |  |  | FR1.30-7 |  |
| SNR |  | dB | 8 | 9 |
| Es for power offset 1 |  | dBm/Hz | -112 |  |
| Es for power offset 2 |  | dBm/Hz | -106 |  |
| Propagation channel |  |  | AWGN |  |
| Antenna configuration |  |  | 2×2 with static channel specified in Annex B.1 |  |
| Beamforming Model |  |  | As specified in Annex B.4.1 |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  | 4 |  |
|  | CDM Type |  | FD-CDM2 |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,4 |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 9 |  |
|  | CSI-RS periodicity and offset | slot | 10/1 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |  |
|  | Number of CSI-RS ports (X) |  | 2 |  |
|  | CDM Type |  | FD-CDM2 |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 3,(6,-) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 3 |  |
|  | NZP CSI-RS-timeConfig periodicity and offset | slot | Not configured |  |
|  | aperiodicTriggeringOffset |  | 0 |  |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |  |
|  | CSI-IM RE pattern |  | 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4, 9) |  |
|  | CSI-IM timeConfig periodicity and offset | slot | Not configured |  |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |  |
|  | Codebook Mode |  | 1 |  |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |
|  | CodebookSubsetRestriction |  | 010000 |  |
|  | RI Restriction |  | N/A |  |
| ReportConfigType |  |  | Aperiodic |  |
| CQI-table |  |  | Table 2 |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForChannelMeasurements |  |  | configured |  |
| timeRestrictionForInterferenceMeasurements |  |  | configured |  |
| cqi-FormatIndicator |  |  | Wideband |  |
| pmi-FormatIndicator |  |  | Wideband |  |
| Sub-band Size |  | RB | 8 |  |
| Csi-ReportingBand |  |  | 1111111 |  |
| CSI-Report periodicity and offset |  | slot | Not configured |  |
| aperiodicTriggeringOffset |  |  | 7 |  |
| CSI request |  |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |  |
| reportTriggerSize |  |  | 1 |  |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |
| Physical channel for CSI report |  |  | PUSCH |  |
| CQI/RI/PMI delay |  | ms | 9.5 |  |
| Maximum number of HARQ transmission |  |  | 1 |  |
| Measurement channel |  |  | As specified in Table A.4-2, TBS.2-8 |  |

Table 6.2A.3.1.2-2: Configuration parameters for PCell

| Parameter | Unit | Test 1 |
| --- | --- | --- |
| Bandwidth | MHz | 20 |
| Subcarrier spacing | kHz | 30 |
| Duplex Mode |  | TDD |
| TDD UL-DL pattern |  | FR1.30-1 |
| Propagation channel |  | AWGN |
| Antenna configuration |  | 2×2 with static channel specified in Annex B.1 |
| Beamforming Model |  | As specified in Annex B.4.1 |

### 6.2A.4	4RX requirements

#### 6.2A.4.1	CQI reporting definition under AWGN conditions

##### 6.2A.4.1.1	Minimum requirement for CQI reporting for SCell on band with shared spectrum access

The purpose of the requirements is to verify that the reported CQI values are in accordance with the CQI definition given in TS 38.214 [12] for Scell on band with shared spectrum access. For each downlink transmission duration the transmission power offset is randomly chosen between [0, +6] dB and 2 sets of CQI reports are obtained for each transmission power offset. The reporting accuracy of CQI under AWGN condition is determined by the reporting variance and BLER performance using the transport format indicated by the reported CQI median for each power offset. To account for sensitivity of the input SNR the reporting definition is considered to be verified if the reporting accuracy is met for at least one of two SNR levels separated by an offset of 1 dB.

For the parameters specified in Table 6.2A.4.1.1-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	For each transmission power offset the reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than 90% of the time.

b)	For each transmission power offset, if the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than 0.1. For each transmission power offset, if the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to 0.1.

c)	The absolute difference in median CQI for each of transmission power offset shall be ≥ 2.

The test parameters for configuring the PCell are specified in Table 6.2A.4.1.1-2, but requirements are only applicable to SCell on band with shared spectrum access.

Table 6.2A.4.1.1-1: CQI reporting test parameters for SCell on band with shared spectrum access

| Parameter |  |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 20 |  |
| Subcarrier spacing |  |  | kHz | 30 |  |
| Duplex Mode |  |  |  | TDD |  |
| Downlink Transmission Model |  |  |  | As specified in Annex B.5 |  |
| Downlink Transmission Model Parameters | Downlink period |  | ms | 5 |  |
|  | LBT failure probability (pLBT) |  |  | 0.25 |  |
|  | Downlink transmission duration values set |  | slot | {4,6,7} |  |
|  | Occupied OFDM symbols in slot other than the last slot of the downlink duration |  | symbols | 14 |  |
|  | Occupied OFDM symbols in the last slot of the downlink duration |  | symbols | 14 |  |
| TDD UL-DL pattern |  |  |  | FR1.30-7 |  |
| SNR |  |  | dB | 5 | 6 |
| Es for power offset 1 |  |  | dBm/Hz | -112 |  |
| Es for power offset 2 |  |  | dBm/Hz | -106 |  |
| Propagation channel |  |  |  | AWGN |  |
| Antenna configuration |  |  |  | 2×4 with static channel specified in Annex B.1 |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |
| ZP CSI-RS configuration |  | CSI-RS resource Type |  | Periodic |  |
|  |  | Number of CSI-RS ports (X) |  | 4 |  |
|  |  | CDM Type |  | FD-CDM2 |  |
|  |  | Density (ρ) |  | 1 |  |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,4 |  |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 9 |  |
|  |  | CSI-RS periodicity and offset | slot | 10/1 |  |
| NZP CSI-RS for CSI acquisition |  | CSI-RS resource Type |  | Aperiodic |  |
|  |  | Number of CSI-RS ports (X) |  | 2 |  |
|  |  | CDM Type |  | FD-CDM2 |  |
|  |  | Density (ρ) |  | 1 |  |
|  |  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  | Row 3,(6,-) |  |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 3 |  |
|  |  | NZP CSI-RS-timeConfig periodicity and offset | slot | Not configured |  |
|  |  | aperiodicTriggeringOffset |  | 0 |  |
| CSI-IM configuration |  | CSI-IM resource Type |  | Aperiodic |  |
|  |  | CSI-IM RE pattern |  | 0 |  |
|  |  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4, 9) |  |
|  |  | CSI-IM timeConfig periodicity and offset | slot | Not configured |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |
|  |  | Codebook Mode |  | 1 |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |
|  |  | CodebookSubsetRestriction |  | 010000 |  |
|  |  | RI Restriction |  | N/A |  |
| ReportConfigType |  |  |  | Aperiodic |  |
| CQI-table |  |  |  | Table 2 |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForChannelMeasurements |  |  |  | configured |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | configured |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |
| Sub-band Size |  |  | RB | 8 |  |
| Csi-ReportingBand |  |  |  | 1111111 |  |
| CSI-Report periodicity and offset |  |  | slot | Not configured |  |
| aperiodicTriggeringOffset |  |  |  | 7 |  |
| CSI request |  |  |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |  |
| reportTriggerSize |  |  |  | 1 |  |
| CSI-AperiodicTriggerStateList |  |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |
| Physical channel for CSI report |  |  |  | PUSCH |  |
| CQI/RI/PMI delay |  |  | ms | 9.5 |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |
| Measurement channel |  |  |  | As specified in Table A.4-2, TBS.2-8 |  |

Table 6.2A.4.1.1-2: Configuration parameters for PCell

| Parameter | Unit | Test 1 |
| --- | --- | --- |
| Bandwidth | MHz | 20 |
| Subcarrier spacing | kHz | 30 |
| Duplex Mode |  | TDD |
| TDD UL-DL pattern |  | FR1.30-1 |
| Propagation channel |  | AWGN |
| Antenna configuration |  | 2×4 with static channel specified in Annex B.1 |
| Beamforming Model |  | As specified in Annex B.4.1 |

------------------------------------------------------------- End of change ------------------------------------------------------------