3GPP TSG-RAN4 Meeting #110	R4-2401747

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.108 | CR | 0020 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (LTE_NBIOT_eMTC_NTN_req-Perf) CR: Completion of eMTC SAN demodulation requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIoT_eMTC_NTN_req-Perf |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | eMTC SAN demodulation requirements are in [] |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove [] from eMTC SAN demodulation requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | eMTC SAN demodulation requirements cannot be set. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS36.108 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change ------------------------------------------------------------

## 8.2	Performance requirements for PUSCH

### 8.2.1	Requirements for PUSCH supporting coverage enhancement

For the parameters specified in Table 8.2.1-1 the throughput shall be equal to or larger than the fraction of maximum throughput stated in the Tables 8.2.1-2 and 8.2.1-3 at the given SNR.

Table 8.2.1-1 Test Parameters for PUSCH

| Parameter | unit | Mode A | Mode B |
| --- | --- | --- | --- |
| Maximum number of HARQ transmissions |  | 4 | 2 |
| RV sequences |  | 0, 2, 3, 1, 0, 2, 3, 1 | FDD: 0, 0, 0, 0, 2, 2, 2, 2, 3, 3, 3, 3, 1, 1, 1, 1 |
| Frequency hopping |  | OFF | OFF |
| Frequency hopping interval | subframes | N/A | N/A |
| Duration of PUSCH segment transmission (Nsegmentprecompensation) | subframes | 8 | 256 |
| Note 1: Guard period shall be created according to TS36.211 [9], 5.2.5 |  |  |  |

Table 8.2.1-2 Minimum requirements for PUSCH, 1.4 MHz Channel Bandwidth for Mode A, 1Tx

| Number of TX antennas | Number of RX antennas | CE Mode | Number of PUSCH repetitions | Propagation conditions and correlation matrix (Annex C) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Mode A | 8 | NTN-TDLA100-5 | A5-2 | 70% | -3.1 |
| 1 | 1 | Mode A | 8 | NTN-TDLC5-5 | A5-2 | 70% | -3.4 |
| 1 | 2 | Mode A | 8 | NTN-TDLA100-5 | A5-2 | 70% | -7.3 |
| 1 | 2 | Mode A | 8 | NTN-TDLC5-5 | A5-2 | 70% | -7.3 |

Table 8.2.1-3 Minimum requirements for PUSCH, 1.4 MHz Channel Bandwidth for Mode B, 1Tx

| Number of TX antennas | Number of RX antennas | CE Mode | Number of PUSCH repetitions | Propagation conditions and correlation matrix (Annex C) | FRC (Annex A) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Mode B | 256 | NTN-TDLA100-5 | A5-1 | 70% | -11.6 |
| 1 | 1 | Mode B | 256 | NTN-TDLC5-5 | A5-1 | 70% | -13.3 |
| 1 | 2 | Mode B | 256 | NTN-TDLA100-5 | A5-1 | 70% | -14.5 |
| 1 | 2 | Mode B | 256 | NTN-TDLC5-5 | A5-1 | 70% | -15.5 |

## 8.3	Performance requirements for PUCCH

### 8.3.1	PUCCH performance requirements for coverage enhancement

#### 8.3.1.1	DTX to ACK performance

The DTX to ACK requirement is valid for any number of receive antennas, for all frame structures and for any channel bandwidth.

##### 8.3.1.1.1	Minimum requirement

The DTX to ACK probability, i.e. the probability that ACK is detected when nothing is sent per PUCCH transmission, shall not exceed 1% per PUCCH transmission. A PUCCH transmission may take multiple subframes due to PUCCH transmission repetition. The performance measure is defined as follows:

ProbPUCCH DTX→ACK bits=#false ACK bits#PUCCH DTX×#ACK/NAK bits≤10-2

where:

-	#(false ACK bits) denotes the number of detected ACK bits per PUCCH transmission.

-	#(ACK/NACK bits) denotes the number of encoded bits per PUCCH transmission.

-	#(PUCCH DTX) denotes the number of DTX occasions per PUCCH transmission.

#### 8.3.1.2	ACK missed detection requirements for single user PUCCH format 1a

The ACK missed detection probability is the probability of not detecting an ACK when an ACK is sent.

##### 8.3.1.2.1	Minimum requirements

The ACK missed detection probability shall not exceed 1% at the SNR given in table 8.3.1.2.1-1 for 1Tx.

Table 8.3.1.2.1-1 Minimum requirements for single user PUCCH format 1a, 1Tx

| Number of TX antennas | Number of RX antennas | Channel Bandwidth | Cyclic Prefix | Propagation conditions and correlation matrix (Annex C) | Repetitions | Duration of PUCCH segment transmission (subframes) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1.4MHz | Normal | NTN-TDLA100-5 | 8 | 8 | -2.6 |
| 1 | 2 | 1.4MHz | Normal | NTN-TDLA100-5 | 8 | 8 | -8.2 |
| Note 1 Frequency Hopping: OFF Note 2 Guard period shall be created according to TS36.211 [9], 5.2.5. |  |  |  |  |  |  |  |

## 8.4	Performance requirements for PRACH

### 8.4.1	PRACH False alarm probability

The false alarm requirement is valid for any number of receive antennas, for all frame structures and for any channel bandwidth.

The false alarm probability is the conditional total probability of erroneous detection of the preamble (i.e. erroneous detection from any detector) when input is only noise.

#### 8.4.1.1	Minimum requirement

The false alarm probability shall be less than or equal to 0.1%.

### 8.4.2	PRACH detection requirements

The probability of detection is the conditional probability of correct detection of the preamble when the signal is present. There are several error cases – detecting different preamble than the one that was sent, not detecting a preamble at all or correct preamble detection but with the wrong timing estimation. For NTN-TDLA100-5, a timing estimation error occurs if the estimation error of the timing of the strongest path is larger than 2.08us. The strongest path for the timing estimation error refers to the strongest path.

The test preambles for coverage enhancement are listed in table A.6-1.

#### 8.4.2.1	Minimum requirements

The probability of detection shall be equal to or exceed 99% for the SNR levels listed in Table 8.4.2.1-1.

The requirements for coverage enhancement (Table 8.4.2.1-1) are only valid for the satellite access node supporting coverage enhancement.

Table 8.4.2.1-1 PRACH missed detection requirements (PRACH frequency hopping OFF)

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex C) | Frequency offset | Number of Repetitions | SNR [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Burst format 0 | Burst format 1 | Burst format 2 | Burst format 3 |
| 1 | 1 | NTN-TDLA100-5 | 270Hz | 8 | -11.0 | -10.6 | - | - |
| 1 | 1 | NTN-TDLA100-5 | 270Hz | 16 | - | - | -16.1 | -16.1 |
| 1 | 2 | NTN-TDLA100-5 | 270Hz | 8 | -16.3 | -16.1 | - | - |
| 1 | 2 | NTN-TDLA100-5 | 270Hz | 16 | - | - | -20.1 | -20.1 |
| Note 1: Under fading channels, the PRACH detection performance may be significantly different with different PRACH Configuration Indexes. The requirements in this table are defined based on the simulation results with PRACH Configuration Indexes (3, 19, 35, 51) for Format 0, Format 1, Format 2, and Format 3 respectively. |  |  |  |  |  |  |  |  |

------------------------------------------------------------- End of change ------------------------------------------------------------