3GPP TSG-RAN WG4 Meeting #110	R4-2401751

Athens, Greece, 26 February – 1 March 2024

Agenda Item:	8.7.4

Source:	Ericsson

Title:	UE demodulation requirements for non-colocated NR-CA deployment scenario

Document for:	Discussion

# 1	Introduction

RAN4#109 agreed with the test configuration of the UE demodulation requirements for intra-band non-colocated EN-DC/NR-CA deployments [1], and also agreed with the big CR to define Type 2 UE NonCol NR-CA PDSCH demodulation requirements [2]. Table 1 summarizes the test parameters and Table 2 list the MCS pair configured for each cell.

This contribution provides our simulation results for PDSCH demodulation requirements for Type 2 UE to complete the Type 2 UE demodulation requirements.

Table 1	Test parameters for Type 2 UE NR-CA demodulation requirements

|  | Type 2 UE NR-CA demodulation requirements |
| --- | --- |
| Received power difference | 25 dB |
| Received time difference | 33 µs |
| Channel model | AWGN (TS38.101-4 B.1.1) |
| Maximum number of HARQ transmission | 1 |
| Signal/Noise setting | Set both Es and Noc to set SNR test points |
| Throughput measurement procedure | Measure both carriers |
| Antenna configuration | 2x2 for both carriers |
| MCS table and MCS index | See Table 2 |
| Test metric | 70% of the maximum throughput |

Table 2	MCS pair for Type 2 UE NR-CA demodulation requirements.

|  | Rank | MCS table | MCS index |
| --- | --- | --- | --- |
| Carrier with lower power | Rank 1 | MCS table 1 | MCS4 (QPSK, 0.30) |
| Carrier with higher power | Rank 2 | MCS table 2 | MCS21 (256QAM, 0.69) |

# 2	Simulation results

Table 3 summarizes our simulation results for Type 2 UE demodulation requirements. We propose to consider our impairment results to finalize the Type 2 UE demodulation requirements.

Table 3	Simulation results.

| Carrier | MCS table | MCS index | Rank | Alignment results | Impairment results |
| --- | --- | --- | --- | --- | --- |
| Carrier with lower power | Table 1 | 4 | 1 | -4.1 | -2.1 |
| Carrier with higher power | Table 2 | 21 | 2 | 19.7 | 21.7 |

# References

- R4-2321052, “WF on [109][319] NonCol_intraB_ENDC_NR_CA_Demod”, Ericsson.

- R4-2318352, “BigCR for 38.101-4: Type 2 UE NonCol NR-CA PDSCH demodulation requirements”, Nokia, Nokia Shanghai Bell.