3GPP TSG-RAN WG4 Meeting #110	R4-2401867

# Athens, Greece, February 26 – March 1, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on UE feature list for NR_MG_enh2 WI

Agenda item:	8.5.1

Document for:	Discussion

Introduction

This paper presents our view on RRM aspects related to further enhancements of measurement gaps for Rel-18 [1]. In particular, it presents our view on the UE feature list for both objectives of the NR_MG_enh2 work item as discussed at RAN4 #109 [2] and lists corresponding proposals.

Discussion

At RAN4 #109, a starting point for the UE feature list for both objectives of the NR_MG_enh2 work item was defined [2], replicated below.

- Sub-topic 1-1: feature list for NR_MG_enh2 WI Issue 1-1-1: Rel-18 NR UE features for NR_MG_enh2 WI for Objective 1: < Way Forward > - The following feature list will be used as a starting for further discussion in the next meeting Index; Feature group; Components; Prerequisite feature groups; Need for the gNB to know if the feature is supported; Applicable to the capability signalling exchange between UEs (V2X WI only)”.; Consequence if the feature is not supported by the UE; Type; Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Capability interpretation for mixture of FDD/TDD and/or FR1/FR2; Note; Mandatory / Optional. [32-1]; Concurrent gap with Pre-MG in a FR; Support of multiple per-UE (or per-FR) measurement gap patterns with at least one per-UE (or per-FR) Pre-MG. Detail in Clause [9.1.x.2] of TS 38.133.; 19-3-x and 19-2 x = 1 or 2; Yes; No; Network should not configure concurrent gap with Pre-MG; Per UE; No; No; N.A; Optional with capability signalling. [32-2]; 2 Pre-MG configuration with simultaneous activation / deactivation; Support configurations of 2 Pre-MG with simultaneous activation / deactivation in the same FR.; 32-1; Yes; No; Unknown activation time for simultaneous Pre-MG is expected; Per UE; No; No; N.A; Optional with capability signalling. [32-3]; Dynamic collision; Support the RRM requirements when the activation / deactivation delay of Pre-MG overlaps the other measurement gap with lower priority or Pre-MG; 32-1; Yes; No; UE is not expected to meet the requirements; Per UE; No; No; N.A; Optional with capability signalling. [32-4]; Concurrent gap with NCSG in a FR; Support of multiple per-UE (or per-FR) measurement gap patterns with at least one per-UE (or per-FR) NCSG. Detail in clause [9.1.y.2] of TS 38.133.; 19-1 and 19-2; Yes; No; Network should not configure concurrent gap with NCSG; Per UE; No; No; N.A; Optional with capability signalling. [32-5]; 2 NCSG configuration in a FR; Support configurations of 2 NCSG in the same FR; 32-4; Yes; No; Network should not configure 2 NCSG in the same FR; Per UE; No; No; N.A; Optional with capability signalling. Issue 1-1-2: Rel-18 NR UE features for NR_MG_enh2 WI for Objective 2: < Way Forward > - The following feature list will be used as the starting point for further discussion in the next meeting Index; Feature group; Components; Prerequisite feature groups; Need for the gNB to know if the feature is supported; Applicable to the capability signalling exchange between UEs (V2X WI only)”.; Consequence if the feature is not supported by the UE; Type; Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Capability interpretation for mixture of FDD/TDD and/or FR1/FR2; Note; Mandatory / Optional. [32-6]; Need for interruption; UE capability to indicate whether interruption is needed when UE reports “no-gap” in NeedForGaps InfoNR; nr-NeedForGap-Reporting-r16; yes; no; Network cannot know whether the UE can repport NeedFor Interruption InfoNR; Per UE; No; No; N.A; Optional with capability signalling. [32-7]; [Inter-RAT EUTRAN measurements without gap and outside active DL BWP]; 1. Support of requirements of inter-RAT EUTRAN measurements outside active DL BWP without gap with or without interruption. FFS: Whether this could be coupled or related to the capability of EMW supporting.; Yes; NA; UE behaviour of supporting inter-RAT EUTRAN measurements without gap is known to network; [Per UE]; No; No; N.A; Optional with capability signalling. [32-8]; Inter-RAT EUTRAN measurement without gap [and within active DL BWP]; Support of inter-RAT EUTRAN measurements without gap when CRS is contained within UE’s active DL BWP; [32-9]; Yes; No; Measurement gap will be needed for inter-RAT EUTRAN measurements; Per UE; No; FR1 only; N.A; Optional with capability signalling. [32-9]; Effective measurement window for inter-RAT EUTRAN measurements; Support configuration of effective measurement window for inter-RAT EUTRAN measurements, including offset, duration and periodicity.; [32-7 or 32-8]; Yes; No; Undefined UE measurement behavior and when to allow scheduling restriction; Per UE; No; No; N.A; - A bitmap for 6 effective measurement window (EMW) patterns defined in TS 38.133. - TBD which patterns(s) are mandatory supported; Optional with capability signalling. [32-10]; Simultaneous RxDataCRS-DiffNumerology; Support concurrent inter-RAT measurement on EUTRAN cell in non-DSS with CRS contained within UE’s active DL BWP and PDCCH or PDSCH reception from the serving cell with a different numerology; 32-8; Yes; No; scheduling restriction is applicable; Per UE; No; FR1 only; N.A; Optional with capability signalling. Issue 1-1-3: Rel-18 LTE UE features for NR_MG_enh2 WI: < Way Forward > - The following feature list will be used as the starting point for further discussion in the next meeting Index; Feature group; Components; Prerequisite feature groups; Need for the gNB to know if the feature is supported; Applicable to the capability signalling exchange between UEs (V2X WI only)”.; Consequence if the feature is not supported by the UE; Type; Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Capability interpretation for mixture of FDD/TDD and/or FR1/FR2; Note; Mandatory / Optional. [x-y]; Inter-RAT NR measurement without gap; Support of inter-RAT NR measurements without gap with or without interruption; Yes; NA; The UE does not support inter-RAT NR measurements without gap with or without interruption for performing inter-RAT NR measurement without gap; Per UE; No; No; NA; # RAN2 implemented already as interRAT-NeedForIntr # NR-r18.; Optional with capability signalling.

UE feature list for Objective 1

Following modifications and additions are needed to the UE feature list for objective 1, depicted in red.

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory / Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [32-1] | Concurrent gap with Pre-MG in a FR | Support of multiple per-UE (or per-FR) measurement gap patterns with at least one per-UE (or per-FR) Pre-MG. Detail in clause [9.1.x.2] of TS 38.133. | 19-3-x and 19-2 x = 1 or 2 | Yes | No | Network should not configure concurrent gap with Pre-MG | Per UE | No | No | N.A |  | Optional with capability signalling |
| [32-2] | 2 Pre-MG configuration with simultaneous activation / deactivation | Support configurations of 2 Pre-MG with simultaneous activation / deactivation in the same FR. | 32-1 | Yes | No | Unknown activation time for simultaneous Pre-MG is expected | Per UE | No | No | N.A |  | Optional with capability signalling |
| [32-3] | Dynamic collision | Support the RRM requirements when the activation / deactivation delay of Pre-MG overlaps the other measurement gap with lower priority or Pre-MG | 32-1 | Yes | No | UE is not expected to meet the requirements | Per UE | No | No | N.A |  | Optional with capability signalling |
| [32-4] [32-2] | Concurrent gap with NCSG in a FR | Support of multiple per-UE (or per-FR) measurement gap patterns with at least one per-UE (or per-FR) NCSG. Detail in clause [9.1.y.2] of TS 38.133. | 19-1 and 19-2 | Yes | No | Network should not configure concurrent gap with NCSG | Per UE | No | No | N.A |  | Optional with capability signalling |
| [32-5] | 2 NCSG configuration in a FR | Support configurations of 2 NCSG in the same FR | 32-4 | Yes | No | Network should not configure 2 NCSG in the same FR | Per UE | No | No | N.A |  | Optional with capability signalling |

The definition of two optional capabilities for Case 1 and 1 optional capability for Case 2 will lead to an undesirable complexity increase for the network, as it needs to take into account 4 different UE behaviors for Case 1 and 2 different UE behaviors for Case 2. Thus, for the combination of Case 1 + Case2, there are 8 different UE behaviours, which is undesirable for this feature.

Moreover, the gap combination Pre-MG + Pre-MG was defined as part of Case 1 requirements. Hence support of simultaneous Pre-MG (de)activation and support of dynamic collisions, if the Pre-MG (de)activation overlaps with the other MG occasion, is mandatory and no optional capabilities are needed. Similarly, for the gap combination NCSG + NCSG, it was defined as part of Case 2 requirements. Thus, it is proposed to remove UE capability indices 32-2, 32-3 and 32-5, as depicted above.

Adopt the proposed UE feature list for objective 1 in R4-2321640 with removal of UE capability indices 32-2, 32-3 and 32-5.

UE feature list for Objective 2 for NR UE features

Following modifications and additions are needed to the UE feature list for objective 2 related to NR UE features, depicted in red.

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory / Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [32-6] | Need for interruption | UE capability to indicate whether interruption is needed when UE reports “no-gap” in NeedForGaps InfoNR | nr-NeedForGap-Reporting-r16 | yes | no | Network cannot know whether the UE can repport NeedFor Interruption InfoNR | Per UE | No | No | N.A |  | Optional with capability signalling |
| [32-7] | [Inter-RAT EUTRAN measurements without gap and outside active DL BWP] | 1. Support of requirements of inter-RAT EUTRAN measurements outside active DL BWP without gap with or without interruption. FFS: Whether this could be coupled or related to the capability of EMW supporting. | eutra-NeedForGapNCSG-Reporting-r17 | Yes | NA | UE behaviour of supporting inter-RAT EUTRAN measurements without gap is known to network | [Per UE] | No | No | N.A |  | Optional with capability signalling |
| [32-8] | Inter-RAT EUTRAN measurement without gap [and within active DL BWP] | Support of inter-RAT EUTRAN measurements without gap when CRS is contained within UE’s active DL BWP | [32-9] | Yes | No | Measurement gap will be needed for inter-RAT EUTRAN measurements | Per UE | No | FR1 only | N.A |  | Optional with capability signalling |
| [32-9] | Effective measurement window for inter-RAT EUTRAN measurements | Support configuration of effective measurement window for inter-RAT EUTRAN measurements, including offset, duration and periodicity. | [32-7 or 32-8] | Yes | No | Undefined UE measurement behavior and when to allow scheduling restriction | Per UE | No | No | N.A | - A bitmap for 6 effective measurement window (EMW) patterns defined in TS 38.133. - TBD which patterns(s) are mandatory supported | Optional with capability signalling |
| [32-10] | Simultaneous RxDataCRS-DiffNumerology | Support concurrent inter-RAT measurement on EUTRAN cell in non-DSS with CRS contained within UE’s active DL BWP and PDCCH or PDSCH reception from the serving cell with a different numerology | 32-8 | Yes | No | scheduling restriction is applicable | Per UE | No | FR1 only | N.A |  | Optional with capability signalling |

Adopt the proposed UE feature list for objective 2 for NR UE features in R4-2321640 with the following change to UE capability index 32-7: add eutra-NeedForGapNCSG-Reporting-r17 as prerequisite feature group.

UE feature list for Objective 2 for LTE UE features

No modifications and additions are needed to the UE feature list for objective 2 related to LTE UE features.

Adopt the proposed UE feature list for objective 2 for LTE UE features in R4-2321640.

Conclusion

This paper has presented our views on UE feature list for objectives 1 and 2 of the NR_MG_enh2 work item. Resulting from this discussion, we make following proposals:

- Adopt the proposed UE feature list for objective 1 in R4-2321640 with removal of UE capability indices 32-2, 32-3 and 32-5.

Adopt the proposed UE feature list for objective 2 for NR UE features in R4-2321640 with the following change to UE capability index 32-7: add eutra-NeedForGapNCSG-Reporting-r17 as prerequisite feature group.

Adopt the proposed UE feature list for objective 2 for LTE UE features in R4-2321640.

- References

- RP-232440, “Revised WID: Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps”, Intel, MediaTek

- R4-2321640, “WF for NR_MGE-2_part1”, Moderator (MediaTek inc.)

- R4-2321632, “WF on NR_MG_enh2_part2”, Intel Corporation