3GPP TSG RAN WG4 Meeting #107	R4-2401883

Athens, Greece, 26th February – 1st March 2024

Source:		Rohde & Schwarz

Title:	Discussion on 5G broadcast UE RF limits

Agenda Item:		6.1.8

Document for:	Discussion

# Introduction

Some of the UE RF requirements for LTE based 5G broadcast are still brackets. In the last meeting some measurement results were presented in R4-2321734 [1]. In this contribution we present our measurement results for ACS and Reference Sensitivity.

# Discussion

## Test Setup

For the test, we measured the performance of a UE in the Reference Sensitivity and ACS TCs in band n71 and used one our R&S TS8980FTA-3A Conformance Test System in combination with our R&S Contest software. The common parameters for the test are shown below:

- Band: n71

- Configured Bandwidth: 10 MHz

- Wanted signal center frequency: Mid frequency according to 38.508-1 (634.5 MHz)

- SCS: 15 kHz

- Allocated RBs: 30, 35, 40 RBs according to chapter 5.6H in TS 36.101 [2] for 6, 7, 8 MHz respectively

- Allocation centered in the 10 MHz bandwidth

Figure 1 6 MHz allocation

Figure 2 7 MHz allocation

Figure 3 8 MHz Allocation

## Reference Sensitivity Test

As a first step we checked the Reference Sensitivity value for the device under test to establish a reference wanted signal level for the ACS test later. For this the DL power was lowered until the measured throughput was starting to degrade.

For all three allocations, the throughput starts to degrade at around -100 dBm (see picture below) indicating that the UE reaches its performance limits.

These values are below the ones specified in TS 36.101 [2] for the LTE based 5G broadcast, so it can be assumed that the UE under test would also pass the PMCH test.

Observation 1: The Reference Sensitivity Level observed during the test is around -100 dBm, which is lower than the required level from the PMCH specification.

## ACS Test

On the Transmission side, 5G Broadcast Network Planning determines the overall network performance and capacity in terms of available bitrate and targeted coverage. The network capacity itself highly depends on the targeted SNR. On the receiving side, ACS is the ability to reject these unwanted signals and maintain a high signal-to-noise ratio (SNR) for the desired signal. Therefore, ACS is an important parameter for the optimal 5G Broadcast Network Planning.

Observation 2: ACS is an important parameter with respect to network planning.

For testing the ACS behavior, the NR FR1 Test Case 7.5 (from TS 38.521-1 [3]) was performed. The signal levels of the wanted and interfering signals were modified to match the requirements from chapter 7.5.1H of 36.101 [3] as shown below. As Refsens level a level of -100 dBm was assumed.

The interfering signal consisted of a 5 MHz NR signal as defined for the NR FR1 test case in TS 38.521-1 [4]. The interfering signal frequency was also modified to match the offset specified in the table above. The following table summarizes the settings for the ACS test, the screenshots show the corresponding settings in the test software:

|  | 6 MHz | 7 MHz | 8 MHz |
| --- | --- | --- | --- |
| Allocated RBs | 30 | 35 | 40 |
| Offset RBs | 11 | 9 | 6 |
| Center 10 MHz carrier (MHz) | 634,50 | 634,50 | 634,50 |
| Upper End of Mid allocation (MHz) | 637,20 | 637,74 | 638,10 |
| Lower End of Mid allocation (MHz) | 631,80 | 631,44 | 630,90 |
| Middle of Mid Allocation (MHz) | 634,50 | 634,59 | 634,50 |
| Interferer for Mid allocation | 628,9875 | 628,5825 | 627,9975 |
| DL Level (dBm) | -86 | -86 | -86 |
| Interferer Level (dBm) | -58,5 | -57 | -56 |

Table 1 Settings for ACS Test

With these settings the Test Case passed for all bandwidths and achieved 100% throughput (see below screenshot from the test program).

Observation 3: The DUT shows no degradation in ACS performance when using the test parameters from the specification.

# Proposals

In this contribution, we present our measurement results for the UE requirements for LTE based 5G broadcast and make the following observations.

Observation 1: The Reference Sensitivity Level observed during the test is around -100 dBm, which is lower than the required level from the PMCH specification.

Observation 2: ACS is an important parameter with respect to network planning.

Observation 3: The DUT shows no degradation in ACS performance when using the test parameters from the specification.

# References

- R4-2321734, “Further considerations on ACS for 5G terrestrial broadcast”, Apple Inc., RAN4#109, November 2023

- TS 36.101 v18.4.0

- TS 38.521-1 v18.1.0