3GPP TSG-RAN WG4 Meeting # 110	R4-2401903

Athens, GR, Feb 26 – Mar 01, 2024

Agenda Item: 8.21.2.1

Source: MediaTek Inc.

Title: Discussion on R18 MIMO for L1-RSRP measurement

Document for: Discussion

- Introduction

According to WF [1], RAN4 had some agreements in the last meeting while some issues were discussed without conclusion yet. In this meeting, this WI is divided into three agenda items to be discussed: (1) RRM requirements impacts, (2) Timing requirements for UL multi-DCI multi-TRP with two TAs, (3) Unified TCI framework. The discussion in this paper focus on the “RRM requirements impacts on L1-RSRP measurement when RTD > CP”.

- Discussion

In the following section, the requirement of L1-RSRP measurement when RTD > CP is discussed.

2.1 The requirement of L1-RSRP measurement

last RAN4 meeting reached the following agreement on the requirement of L1-RSRP measurement when RTD > CP.

Issue 3-1-11: L1-RSRP measurement when RTD>CP Agreement: - Define requirements for FR1 for intra/inter-cell. - For FR1, if UE supports RTD>CP, reuse Rel-17 L1-RSRP measurement period in 9.13 for a cell with different PCI from serving cell, remove the RTD is smaller than CP side condition. Reuse L1-RSRP measurement period in 9.5 for serving cell. - If UE does not support RTD>CP, for RTD is larger than CP, no requirements. - FFS on scheduling/measurement restriction for different SCS and TDD.

Regarding SSB based L1-RSRP measurement when RTD>CP, from the point of UE behaviour, the requirements defined here should be aligned with R18 LTM. In R18 LTM, the requirements for multiple cells in multiple frequency layers are also discussed. If there are multiple frequency layers, the measurement delay of intra-frequency neighbour cell is scaled by # of frequency layers in FR1. In our understanding, here we only need to consider two cells in mTRP on a single frequency layer, otherwise the requirements should be updated.

Proposal 1: Make it clear in spec that L1-RSRP measurement requirements defined for mTRP when RTD>CP are only applicable to single carrier scenario.

In R18 LTM, for UE capable of RTD>CP, scheduling restriction is extended by one more symbol before and after SSB symbols, and measurement restriction is allowed on the SSB symbols and one more symbol before/after SSB symbols. From the point of measurement, UE behaviour would be the same as R18 LTM. We suggest following the agreement in R18 LTM here.

Proposal 2: For UE capable of RTD>CP, scheduling restriction is extended by one more symbol before and after SSB symbols, and measurement restriction is allowed on the SSB symbols and one more symbol before/after SSB symbols.

- Summary

In this paper, the discussion of R18 MIMO is provided. We have the following observations and proposals:

Proposal 1: Make it clear in spec that L1-RSRP measurement requirements defined for mTRP when RTD>CP are only applicable to single carrier scenario.

Proposal 2: For UE capable of RTD>CP, scheduling restriction is extended by one more symbol before and after SSB symbols, and measurement restriction is allowed on the SSB symbols and one more symbol before/after SSB symbols.

- Reference

[1] R4-2317371, WF on NR_MIMO_evo_DL_UL WI, Samsung, RAN#109