3GPP TSG-RAN4 Meeting #110	R4-2401909

# Athens, GR,  26th Feb. - 1st Mar., 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_Mob_enh2-Core) DraftCR for L1-RSRP measurement on 38.133 R18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Measurement restriction and scheduling restriction in different clauses are not consistent. - Some agreements are not captured - No requirements, if the actual RTD of serving cell and neighbor cell is larger than CP for UE incapable of RTD>CP - For inter-f L1-RSRP measurement with MG, M= 2 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M= 4 - Measurement period for inter-frequency L1-RSRP measurement without gap when UE incapable of RTD>CP - Known cell condition for inter-frequency L1-RSRP measurement is not correct. - There are some typos, brackets, and editor notes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Align measurement restriction and scheduling restriction in different clauses - Capture the agreements missing - Correct known cell condition for inter-frequency L1-RSRP measurement - Correct the typos. Remove the brackets and editor notes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Measurement restriction and scheduling restriction in different clauses are not consistent. - The requirements for L1-RSRP measurement in LTM are not complete or correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.3, 8.1.3.3, 8.5.2.3, 8.5.3.3, 8.5.5.3, 8.5.6.3, 8.18.2.3, 8.18.3.3, 8.18.5.3, 9.8.5, 9.5.4.1, 9.5.5.2, 9.14, 9.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of change 1>>

8.1.2.3	Measurement restrictions for SSB based RLM

The UE is required to be capable of measuring SSB for RLM without measurement gaps. The UE is required to perform the SSB measurements with measurement restrictions as described in the following scenarios.

For FR1, when the SSB for RLM is in the same OFDM symbol as CSI-RS for RLM, BFD, CBD or L1-RSRP measurement,

-	If SSB and CSI-RS have same SCS, UE shall be able to measure the SSB for RLM without any restriction;

-	If SSB and CSI-RS have different SCS,

-	If UE supports simultaneousRxDataSSB-DiffNumerology, UE shall be able to measure the SSB for RLM without any restriction;

-	If UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both SSB for RLM and CSI-RS. Longer measurement period for SSB based RLM is expected, and no requirements are defined

For FR2, when the SSB for RLM measurement on one CC is in the same OFDM symbol as CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same CC or different CCs in the same band, UE is required to measure one of but not both SSB for RLM and CSI-RS. Longer measurement period for SSB based RLM is expected, and no requirements are defined.

For FR2, when the SSB for RLM measurement on one CC is in the same or adjacent OFDM symbol as SSB from cell with additional PCI for BFD, CBD or L1-RSRP measurement on the same CC or different CCs in the same band, UE is required to measure one of but not both SSBs. Longer measurement period for SSB based RLM is expected, and no requirements are defined.

For FR2, for UE incapable of [capability of measurement with RTD>CP] and for UE capable of [capability of measurement with RTD>CP], when the SSB for RLM measurement on one CC fully or partially overlapps with the OFDM symbol as SSB from candidate LTM neighbor cell for intra-frequency L1-RSRP measurement or inter-frequency L1-RSRP measurement without gap in the same band, UE is required to measure one of but not both SSBs. Longer measurement period for SSB based RLM is expected, and no requirements are defined.

For FR2, there is no measurement restriction allowed when the network configures mixed numerology between SSB for RLM measurement on one FR2 band and CSI-RS for RLM, BFD, CBD, L1-RSRP or L1-SINR measurement on the other FR2 band, provided that UE is capable of independent beam management on this FR2 band pair.

<< End of change 1>>

<< Start of change 2>>

8.1.3.3	Measurement restrictions for CSI-RS based RLM

The SSB mentioned in this clause can be associated with either the serving cell PCI or a PCI different from serving cell PCI.

The UE is required to be capable of measuring CSI-RS for RLM without measurement gaps. The UE is required to perform the CSI-RS measurements with measurement restrictions as described in the following clauses.

For both FR1 and FR2, when the CSI-RS for RLM is in the same OFDM symbol as SSB for RLM, BFD, CBD or L1-RSRP measurement, UE is not required to receive CSI-RS for RLM in the PRBs that overlap with an SSB.

For FR1, when the SSB for RLM, BFD, CBD, or L1-RSRP measurement is within the active BWP and has same SCS than CSI-RS for RLM, the UE shall be able to perform CSI-RS measurement without restrictions.

For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement is within the active BWP and has different SCS than CSI-RS for RLM, the UE shall be able to perform CSI-RS measurement with restrictions according to its capabilities:

-	If the UE supports simultaneousRxDataSSB-DiffNumerology the UE shall be able to perform CSI-RS for RLM measurement without restrictions.

-	If the UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both CSI-RS for RLM and SSB. Longer measurement period for CSI-RS based RLM is expected, and no requirements are defined.

For FR1, when the CSI-RS for RLM is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement, UE shall be able to measure the CSI-RS for RLM without any restriction.

For FR2, when the CSI-RS for RLM measurement on one CC is in the same OFDM symbol as SSB for RLM, BFD, or L1-RSRP measurement on the same CC or different CCs in the same band, or in the same symbol as SSB for CBD measurement on the same CC or different CCs in the same band when beam failure is detected, UE is required to measure one of but not both CSI-RS for RLM and SSB. Longer measurement period for CSI-RS based RLM is expected, and no requirements are defined.

For UE incapable of [capability of measurement with RTD>CP] and for UE capable of [capability of measurement with RTD>CP],

-	For both FR1 and FR2, when the CSI-RS for RLM measurement fully or partially overlapps with the OFDM symbol as SSB from candidate LTM neighbor cell for intra-frequency L1-RSRP measurement or inter-frequency L1-RSRP measurement without gap, UE is not required to receive CSI-RS for RLM measurement in the PRBs that overlap with an SSB.

-	For FR1, when the CSI-RS for RLM measurement fully or partially overlapps with the OFDM symbol as SSB from candidate LTM neighbor cell for intra-frequency L1-RSRP measurement or inter-frequency L1-RSRP measurement without gap, if CSI-RS and SSB have different SCS and UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both CSI-RS for RLM measurement and SSB. Longer measurement period for CSI-RS based RLM is expected, and no requirements are defined.

-	For FR2, when the CSI-RS for RLM measurement on one CC fully or partially overlapps with the OFDM symbol as SSB from candidate LTM neighbor cell for intra-frequency L1-RSRP measurement or inter-frequency L1-RSRP measurement without gap in the same band, UE is required to measure one of but not both CSI-RS for RLM measurement and SSB. Longer measurement period for CSI-RS based RLM is expected, and no requirements are defined.

For FR2, when the CSI-RS for RLM measurement on one CC is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same CC or different CCs in the same band,

-	In the following cases, UE is required to measure one of but not both CSI-RS for RLM and the other CSI-RS. Longer measurement period for CSI-RS based RLM is expected, and no requirements are defined.

-	The CSI-RS for RLM or the other CSI-RS in a resource set configured with repetition ON, or

-	The other CSI-RS is configured in q1 and beam failure is detected, or

-	The two CSI-RS-es are not QCL-ed w.r.t. QCL-TypeD, or the QCL information is not known to UE,

-	Otherwise, UE shall be able to measure the CSI-RS for RLM without any restriction.

For FR2-1 for PCell, there is no measurement restriction allowed for UE supporting [TBD - multi-rx capability], and the UE is required to measure both the CSI-RS for RLM and the other CSI-RS for RLM, BFD, CBD or L1-RSRP measurement, while meeting requirements in clause 8.1.3.2, provided the following conditions are met:

-	Both CSI-RSs are not in any CSI-RS resource set with repetition ON, and

-	[The two CSI-RS resources and both PDSCHs are overlapped on the same OFDM symbol, and]

-	One CSI-RS has same QCL source as the active TCI state of one PDSCH, and the other CSI-RS has same QCL source as the active TCI state of the other PDSCH, and

-	Resources of the active TCI states for the two PDSCHs have been reported as a resource group in Rel-17 group-based RSRP report.

Editor’s note: FFS how to capture UE is activated with multi-Rx operation.

<< End of change 2>>

<< Start of change 3>>

8.5.2.3	Measurement restriction for SSB based beam failure detection

The UE is required to be capable of measuring SSB for BFD without measurement gaps. The UE is required to perform the SSB measurements with measurement restrictions as described in the following scenarios.

For FR1, when the SSB for BFD measurement is in the same OFDM symbol as CSI-RS for RLM, BFD, CBD or L1-RSRP measurement,

-	If SSB and CSI-RS have same SCS, UE shall be able to measure the SSB for BFD measurement without any restriction;

-	If SSB and CSI-RS have different SCS,

-	If UE supports simultaneousRxDataSSB-DiffNumerology, UE shall be able to measure the SSB for BFD measurement without any restriction;

-	If UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both SSB for BFD measurement and CSI-RS. Longer measurement period for SSB based BFD measurement is expected, and no requirements are defined.

For FR2, when the SSB for BFD measurement on one CC is in the same OFDM symbol as CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same CC or different CCs in the same band, UE is required to measure one of but not both SSB for BFD measurement and CSI-RS. Longer measurement period for SSB based BFD measurement is expected, and no requirements are defined.

For FR2, when the SSB for BFD measurement on one CC is in the same or adjacent OFDM symbol as SSB from cell with additional PCI for L1-RSRP measurement on the same CC or different CCs in the same band, UE is required to measure one of but not both SSBs. Longer measurement period for SSB based BFD is expected, and no requirements are defined.

For FR2, for UE incapable of [capability of measurement with RTD>CP] and for UE capable of [capability of measurement with RTD>CP], when the SSB for BFD measurement on one CC fully or partially overlapps with the OFDM symbol as SSB from candidate LTM neighbor cell for intra-frequency L1-RSRP measurement or inter-frequency L1-RSRP measurement without gap in the same band, UE is required to measure one of but not both SSBs. Longer measurement period for SSB based BFD is expected, and no requirements are defined.

For FR2, if the network configures same or mixed numerology between SSB for BFD measurement on one FR2 band and CSI-RS for RLM, BFD, CBD, L1-RSRP or L1-SINR measurement on the other FR2 band, UE shall be able to perform the related SSB based measurements in one band without any measurement restrictions on the other band, provided that UE is capable of independent beam management on this FR2 band pair.

<< End of change 3>>

<< Start of change 4>>

8.5.3.3	Measurement restrictions for CSI-RS beam failure detection

The SSB mentioned in this clause can be associated with either the serving cell PCI or a PCI different from serving cell PCI.

The UE is required to be capable of measuring CSI-RS for BFD without measurement gaps. The UE is required to perform the CSI-RS measurements with measurement restrictions as described in the following scenarios.

For both FR1 and FR2, when the CSI-RS for BFD measurement is in the same OFDM symbol as SSB for RLM, BFD, CBD or L1-RSRP measurement, UE is not required to receive CSI-RS for BFD measurement in the PRBs that overlap with an SSB.

For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement is within the active BWP and has same SCS than CSI-RS for BFD measurement, the UE shall be able to perform CSI-RS measurement without restrictions.

For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement is within the active BWP and has different SCS than CSI-RS for BFD measurement, the UE shall be able to perform CSI-RS measurement with restrictions according to its capabilities:

-	If the UE supports simultaneousRxDataSSB-DiffNumerology the UE shall be able to perform CSI-RS measurement without restrictions.

-	If the UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both CSI-RS for BFD measurement and SSB. Longer measurement period for CSI-RS based BFD measurement is expected, and no requirements are defined.

For FR1, when the CSI-RS for BFD measurement is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement, UE shall be able to measure the CSI-RS for BFD measurement without any restriction.

For FR2, when the CSI-RS for BFD measurement on one CC is in the same OFDM symbol as SSB for RLM, BFD or L1-RSRP measurement on the same CC or different CCs in the same band, or in the same symbol as SSB for CBD measurement on the same CC or different CCs in the same band when beam failure is detected, UE is required to measure one of but not both CSI-RS for BFD measurement and SSB. Longer measurement period for CSI-RS based BFD measurement is expected, and no requirements are defined.

For UE incapable of [capability of measurement with RTD>CP] and for UE capable of [capability of measurement with RTD>CP],

-	For both FR1 and FR2, when the CSI-RS for BFD measurement fully or partially overlapps with the OFDM symbol as SSB from candidate LTM neighbor cell for intra-frequency L1-RSRP measurement or inter-frequency L1-RSRP measurement without gap, UE is not required to receive CSI-RS for BFD measurement in the PRBs that overlap with an SSB.

-	For FR1, when the CSI-RS for BFD measurement fully or partially overlapps with the OFDM symbol as SSB from candidate LTM neighbor cell for intra-frequency L1-RSRP measurement or inter-frequency L1-RSRP measurement without gap, if CSI-RS and SSB have different SCS and UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both CSI-RS for BFD measurement and SSB. Longer measurement period for CSI-RS based BFD is expected, and no requirements are defined.

-	For FR2, when the CSI-RS for BFD measurement on one CC fully or partially overlapps with the OFDM symbol as SSB from candidate LTM neighbor cell for intra-frequency L1-RSRP measurement or inter-frequency L1-RSRP measurement without gap in the same band, UE is required to measure one of but not both CSI-RS for BFD measurement and SSB. Longer measurement period for CSI-RS based BFD is expected, and no requirements are defined.

For FR2, when the CSI-RS for BFD measurement on one CC is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same CC or different CCs in the same band,

-	In the following cases, UE is required to measure one of but not both CSI-RS for BFD measurement and the other CSI-RS. Longer measurement period for CSI-RS based BFD measurement is expected, and no requirements are defined.

-	The CSI-RS for BFD measurement or the other CSI-RS in a resource set configured with repetition ON, or

-	The other CSI-RS is configured in set  and beam failure is detected, or

-	The two CSI-RS-es are not QCL-ed w.r.t. QCL-TypeD, or the QCL information is not known to UE,

-	Otherwise, UE shall be able to measure the CSI-RS for BFD measurement without any restriction.

For FR2-1, when the first CSI-RS for BFD measurement on PCell is in the same OFDM symbol as the second CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same PCell, the UE supporting [multi-Rx capability] is required to measure both the first and the second CSI-RSs without measurement restrictions, provided the following conditions are met:

-	Both CSI-RSs are not in any CSI-RS resource set with repetition ON, and

-	One CSI-RS has same QCL source as the active TCI state of one PDSCH, and the other CSI-RS has same QCL source as the active TCI state of the other PDSCH, and

-	the CSI-RS configured for BFD and [at least one of the PDSCHs] are on the same OFDM symbols, and

-	Resources of the active TCI states for the two PDSCHs have been reported as a resource group in Rel-17 group-based RSRP report.

-	[UE is activated with multi-Rx operation]

Editor’s note 1: FFS the condition CSI-RS(s) and PDSCH(s) are overlapped on the same OFDM symbol(s).

Editor’s note 2: FFS how to capture UE is activated with multi-Rx operation.

<< End of change 4>>

<< Start of change 5>>

8.5.5.3	Measurement restriction for SSB based candidate beam detection

For FR1, when the SSB for CBD measurement is in the same OFDM symbol as CSI-RS for RLM, BFD, CBD or L1-RSRP measurement,

-	If SSB and CSI-RS have same SCS, UE shall be able to measure the SSB for CBD measurement without any restrictions;

-	If SSB and CSI-RS have different SCS-es,

-	If UE supports simultaneousRxDataSSB-DiffNumerology, UE shall be able to measure the SSB for CBD measurement without any restriction;

-	If UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both SSB for CBD measurement and CSI-RS. Longer measurement period for SSB based CBD measurement is expected, and no requirements are defined.

For FR2, when the SSB for CBD measurement on one CC is in the same OFDM symbol as CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same CC or different CCs in the same band, UE is required to measure one of but not both SSB for CBD measurement and CSI-RS. Longer measurement period for SSB based CBD measurement is expected, and no requirements are defined.

For FR2, when the SSB for CBD measurement on one CC is in the same OFDM symbol as SSB from cell with additional PCI for BFD or L1-RSRP measurement on the same CC or different CCs in the same band, UE is required to measure one of but not both SSBs. Longer measurement period for SSB based CBD measurement is expected, and no requirements are defined.

For FR2, for UE incapable of [capability of measurement with RTD>CP] and for UE capable of [capability of measurement with RTD>CP], when the SSB for CBD measurement on one CC fully or partially overlapps with the OFDM symbol as SSB from candidate LTM neighbor cell for intra-frequency L1-RSRP measurement or inter-frequency L1-RSRP measurement without gap in the same band, UE is required to measure one of but not both SSBs. Longer measurement period for SSB based CBD measurement is expected, and no requirements are defined.

For FR2, if network configures same or mixed numerology between SSB for CBD measurement on one FR2 band and CSI-RS for RLM, BFD, CBD, L1-RSRP or L1-SINR measurement on the other FR2 band, UE shall be able to perform the related SSB based measurements in one band without any measurement restrictions in the other band,  provided that UE is capable of independent beam management on this FR2 band pair.

<< End of change 5>>

<< Start of change 6>>

8.5.6.3	Measurement restriction for CSI-RS based candidate beam detection

The SSB mentioned in this clause can be associated with either the serving cell PCI or a PCI different from serving cell PCI.

For both FR1 and FR2, when the CSI-RS for CBD measurement is in the same OFDM symbol as SSB for RLM, BFD, CBD or L1-RSRP measurement, UE is not required to receive CSI-RS for CBD measurement in the PRBs that overlap with an SSB.

For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement is within the active BWP and has same SCS than CSI-RS for CBD measurement, the UE shall be able to perform CSI-RS based CBD measurement without restrictions.

For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement is within the active BWP and has different SCS than CSI-RS for CBD measurement, the UE shall be able to perform CSI-RS based CBD measurement with restrictions according to its capabilities:

-	If the UE supports simultaneousRxDataSSB-DiffNumerology the UE shall be able to perform CSI-RS based CBD measurement for without restrictions.

-	If the UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both CSI-RS for CBD measurement and SSB. Longer measurement period for CSI-RS based CBD measurement is expected, and no requirements are defined.

For FR1, when the CSI-RS for CBD measurement is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement, UE shall be able to measure the CSI-RS for CBD measurement without any restriction.

For FR2, when the CSI-RS for CBD measurement on one CC is in the same OFDM symbol as SSB for RLM, BFD, CBD or L1-RSRP measurement on the same CC or different CCs in the same band, UE is required to measure one of but not both CSI-RS for CBD measurement and SSB. Longer evaluation period for CSI-RS based CBD measurement is expected, and no requirements are defined.

For UE incapable of [capability of measurement with RTD>CP] and for UE capable of [capability of measurement with RTD>CP],

-	For both FR1 and FR2, when the CSI-RS for CBD measurement fully or partially overlapps with the OFDM symbol as SSB from candidate LTM neighbor cell for intra-frequency L1-RSRP measurement or inter-frequency L1-RSRP measurement without gap, UE is not required to receive CSI-RS for CBD measurement in the PRBs that overlap with an SSB.

-	For FR1, when the CSI-RS for CBD measurement fully or partially overlapps with the OFDM symbol as SSB from candidate LTM neighbor cell for intra-frequency L1-RSRP measurement or inter-frequency L1-RSRP measurement without gap, if CSI-RS and SSB have different SCS and UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both CSI-RS for CBD measurement and SSB. Longer measurement period for CSI-RS based CBD is expected, and no requirements are defined.

-	For FR2, when the CSI-RS for CBD measurement on one CC fully or partially overlapps with the OFDM symbol as SSB from candidate LTM neighbor cell for intra-frequency L1-RSRP measurement or inter-frequency L1-RSRP measurement without gap in the same band, UE is required to measure one of but not both CSI-RS for CBD measurement and SSB. Longer measurement period for CSI-RS based CBD is expected, and no requirements are defined.

For FR2, when the CSI-RS for CBD measurement on one CC is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same CC or different CCs in the same band, UE is required to measure one of but not both CSI-RS for CBD measurement and the other CSI-RS. Longer evaluation period for CSI-RS based CBD measurement is expected, and no requirements are defined.

For FR2-1, when the first CSI-RS for CBD measurement on PCell is in the same OFDM symbol as the second CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same PCell, the UE supporting [multi-Rx capability] is required to measure both the first and the second CSI-RSs without measurement restrictions, while meeting requirements in clause 8.5.6.2, provided the following conditions are met,

-	Both CSI-RSs are not in any CSI-RS resource set with repetition ON, and

-	One CSI-RS has same QCL source as the active TCI state of one PDCCH or PDSCH, and the other CSI-RS has same QCL source as the active TCI state of the other PDCCH or PDSCH, and

-	[The CSI-RS configured for CBD and both PDSCHs are on the same OFDM symbols, and]

-	Resources of the active TCI states for the two PDCCHs, or two PDSCHs have been reported as a resource group in Rel-17 group-based RSRP report.

Editor’s note 1: FFS the condition CSI-RS(s) and PDSCH(s) are overlapped on the same OFDM symbol(s).

Editor’s note 2: FFS how to capture UE is activated with multi-Rx operation.

<< End of change 6>>

<< Start of change 7>>

8.18.2.3	Measurement restriction for SSB based beam failure detection

The UE is required to be capable of measuring SSB for BFD without measurement gaps. The UE is required to perform the SSB measurements with measurement restrictions as described in the following scenarios.

For FR1, when the SSB for BFD measurement is in the same OFDM symbol as CSI-RS for RLM, BFD, CBD or L1-RSRP measurement,

-	If SSB and CSI-RS have same SCS, UE shall be able to measure the SSB for BFD measurement without any restriction;

-	If SSB and CSI-RS have different SCS,

-	If UE supports simultaneousRxDataSSB-DiffNumerology, UE shall be able to measure the SSB for BFD measurement without any restriction.

-	If UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both SSB for BFD measurement and CSI-RS. Longer measurement period for SSB based BFD measurement is expected, and no requirements are defined.

For FR2, when the SSB for BFD measurement on one CC is in the same OFDM symbol as CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same CC or different CCs in the same band, UE is required to measure one of but not both SSB for BFD measurement and CSI-RS. Longer measurement period for SSB based BFD measurement is expected, and no requirements are defined. When the SSB and CSI-RS for BFD measurements are from different sets q0,0 and q0,1, UE shall be able to perform measure both SSB and CSI-RS for BFD measurements.

For FR2, when the SSB for BFD measurement on one CC is in the same or adjacent OFDM symbol as SSB with a different PCI for RLM, CBD or L1-RSRP measurement on the same CC or different CCs in the same band, UE is required to measure one of but not both SSBs with the two different PCIs. Longer measurement period for SSB based BFD is expected, and no requirements are defined.

For FR2, if the network configures same or mixed numerology between SSB for BFD measurement on one FR2 band and CSI-RS for RLM, BFD, CBD, L1-RSRP or L1-SINR measurement on the other FR2 band, UE shall be able to perform the related SSB based measurements in one band without any measurement restrictions on the other band, provided that UE is capable of independent beam management on this FR2 band pair.

For FR2, for UE incapable of [capability of measurement with RTD>CP] and for UE capable of [capability of measurement with RTD>CP], when the SSB for BFD measurement in sets q0,1 or q1,1 fully or partially overlapps with the OFDM symbol as SSB for intra-frequency L1-RSRP measurement or inter-frequency L1-RSRP measurement without gap on candidate LTM neighbour cell in the same band, UE is required to measure one of but not both SSBs. Longer measurement period for SSB based BFD is expected, and no requirements are defined.

<< End of change 7>>

<< Start of change 8>>

8.18.3.3	Measurement restrictions for CSI-RS beam failure detection

The SSB mentioned in this clause can be associated with either the serving cell PCI or a PCI different from serving cell PCI.

The UE is required to be capable of measuring CSI-RS for BFD without measurement gaps. The UE is required to perform the CSI-RS measurements with measurement restrictions as described in the following scenarios.

For both FR1 and FR2, when the CSI-RS for BFD measurement is in the same OFDM symbol as SSB for RLM, BFD, CBD or L1-RSRP measurement, UE is not required to receive CSI-RS for BFD measurement in the PRBs that overlap with an SSB.

For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement is within the active BWP and has same SCS than CSI-RS for BFD measurement, the UE shall be able to perform CSI-RS measurement without restrictions.

For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement is within the active BWP and has different SCS than CSI-RS for BFD measurement, the UE shall be able to perform CSI-RS measurement with restrictions according to its capabilities:

-	If the UE supports simultaneousRxDataSSB-DiffNumerology the UE shall be able to perform CSI-RS measurement without restrictions.

-	If the UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both CSI-RS for BFD measurement and SSB. Longer measurement period for CSI-RS based BFD measurement is expected, and no requirements are defined.

For FR1, when the CSI-RS for BFD measurement is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement, UE shall be able to measure the CSI-RS for BFD measurement without any restriction.

For FR2, when the CSI-RS for BFD measurement on one CC is in the same OFDM symbol as SSB for RLM, BFD or L1-RSRP measurement on the same CC or different CCs in the same band, or in the same symbol as SSB for CBD measurement on the same CC or different CCs in the same band when beam failure is detected, UE is required to measure one of but not both CSI-RS for BFD measurement and SSB. Longer measurement period for CSI-RS based BFD measurement is expected, and no requirements are defined.

For UE incapable of [capability of measurement with RTD>CP] and for UE capable of [capability of measurement with RTD>CP],

-	For both FR1 and FR2, when the CSI-RS for BFD measurement fully or partially overlapps with the OFDM symbol as SSB from candidate LTM neighbor cell for intra-frequency L1-RSRP measurement or inter-frequency L1-RSRP measurement without gap, UE is not required to receive CSI-RS for BFD measurement in the PRBs that overlap with an SSB.

-	For FR1, when the CSI-RS for BFD measurement fully or partially overlapps with the OFDM symbol as SSB from candidate LTM neighbor cell for intra-frequency L1-RSRP measurement or inter-frequency L1-RSRP measurement without gap, if CSI-RS and SSB have different SCS and UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both CSI-RS for BFD measurement and SSB. Longer measurement period for CSI-RS based BFD is expected, and no requirements are defined.

-	For FR2, when the CSI-RS for BFD measurement on one CC fully or partially overlapps with the OFDM symbol as SSB from candidate LTM neighbor cell for intra-frequency L1-RSRP measurement or inter-frequency L1-RSRP measurement without gap in the same band, UE is required to measure one of but not both CSI-RS for CBD measurement and SSB. Longer measurement period for CSI-RS based BFD is expected, and no requirements are defined.

For FR2, when the CSI-RS for BFD measurement on one CC is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same CC or different CCs in the same band,

-	In the following cases, UE is required to measure one of but not both CSI-RS for BFD measurement and the other CSI-RS. Longer measurement period for CSI-RS based BFD measurement is expected, and no requirements are defined.

-	The CSI-RS for BFD measurement or the other CSI-RS in a resource set configured with repetition ON, or

-	The other CSI-RS is configured in two sets q1,0 and q1,1 and beam failure is detected, or

-	The two CSI-RS-es are not QCL-ed w.r.t. QCL-TypeD, or the QCL information is not known to UE,

-	Otherwise, UE shall be able to measure the CSI-RS for BFD measurement without any restriction.

For FR2-1, for PCell, there is no measurement restriction allowed for UE supporting [TBD - multi-rx capability], and the UE is required to measure both the CSI-RS for BFD and the other CSI-RS for RLM, BFD, CBD or L1-RSRP measurement, provided the following conditions are met:

-	Both CSI-RSs are not in any CSI-RS resource set with repetition ON

-	[FFS: The two CSI-RS resources are overlapped on the same OFDM symbol.]

-	[The two CSI-RS resources and both PDSCH are overlapped on the same OFDM symbol.]

Editor’s note: FFS whether CSI-RS need to overlap with PDSCH for measurement restriction

-	The CSI-RS and both of the PDSCHs are on the same OFDM symbol(s).

-	One CSI-RS has same QCL source as the active TCI state of one PDSCH, and the other CSI-RS has same QCL source as the active TCI state of the other PDSCH

-	Resources of the active TCI states for the two PDSCHs have been reported as a resource group in Rel-17 group-based RSRP report.

-	[FFS how to capture UE is activated with multi-Rx operation]

When two CSI-RSs for BFD measurements are from different sets q0,0 and q0,1, UE shall be able to perform measure both CSI-RSs for BFD measurements.

<< End of change 8>>

<< Start of change 9>>

8.18.5.3	Measurement restriction for SSB based candidate beam detection

For FR1, when the SSB for CBD measurement is in the same OFDM symbol as CSI-RS for RLM, BFD, CBD or L1-RSRP measurement,

-	If SSB and CSI-RS have same SCS, UE shall be able to measure the SSB for CBD measurement without any restrictions.

-	If SSB and CSI-RS have different SCS-es,

-	If UE supports simultaneousRxDataSSB-DiffNumerology, UE shall be able to measure the SSB for CBD measurement without any restriction.

-	If UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both SSB for CBD measurement and CSI-RS. Longer measurement period for SSB based CBD measurement is expected, and no requirements are defined.

For FR2, when the SSB for CBD measurement on one CC is in the same OFDM symbol as CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same CC or different CCs in the same band, UE is required to measure one of but not both SSB for CBD measurement and CSI-RS. Longer measurement period for SSB based CBD measurement is expected, and no requirements are defined. When the SSB and CSI-RS for CBD measurements are from different sets q1,0 and q1,1, UE shall be able to measure both SSB and CSI-RS for CBD measurements.

For FR2, when the SSB for CBD measurement on one CC is in the same OFDM symbol as SSB with a different PCI for RLM, BFD or L1-RSRP measurement on the same CC or different CCs in the same band, UE is required to measure one of but not both SSBs with the two different PCIs. Longer measurement period for SSB based CBD measurement is expected, and no requirements are defined.

For FR2, if network configures same or mixed numerology between SSB for CBD measurement on one FR2 band and CSI-RS for RLM, BFD, CBD, L1-RSRP or L1-SINR measurement on the other FR2 band, UE shall be able to perform the related SSB based measurements in one band without any measurement restrictions in the other band, provided that UE is capable of independent beam management on this FR2 band pair.

For FR2, for UE incapable of [capability of measurement with RTD>CP] and for UE capable of [capability of measurement with RTD>CP], when the SSB for CBD measurement in sets q0,1 or q1,1 fully or partially overlapps with the OFDM symbol as SSB for intra-frequency L1-RSRP measurement or inter-frequency L1-RSRP measurement without gap on candidate LTM neighbour cell on the same band, UE is required to measure one of but not both SSBs. Longer measurement period for SSB based CBD is expected, and no requirements are defined.

<< End of change 9>>

<< Start of change 10>>

8.18.6.3	Measurement restriction for CSI-RS based candidate beam detection

The SSB mentioned in this clause can be associated with either the serving cell PCI or a PCI different from serving cell PCI.

For both FR1 and FR2, when the CSI-RS for CBD measurement is in the same OFDM symbol as SSB for RLM, BFD, CBD or L1-RSRP measurement, UE is not required to receive CSI-RS for CBD measurement in the PRBs that overlap with an SSB.

For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement is within the active BWP and has same SCS than CSI-RS for CBD measurement, the UE shall be able to perform CSI-RS based CBD measurement without restrictions.

For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement is within the active BWP and has different SCS than CSI-RS for CBD measurement, the UE shall be able to perform CSI-RS based CBD measurement with restrictions according to its capabilities:

-	If the UE supports simultaneousRxDataSSB-DiffNumerology the UE shall be able to perform CSI-RS based CBD measurement for without restrictions.

-	If the UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both CSI-RS for CBD measurement and SSB. Longer measurement period for CSI-RS based CBD measurement is expected, and no requirements are defined.

For FR1, when the CSI-RS for CBD measurement is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement, UE shall be able to measure the CSI-RS for CBD measurement without any restriction.

For FR2, when the CSI-RS for CBD measurement on one CC is in the same OFDM symbol as SSB for RLM, BFD, CBD or L1-RSRP measurement on the same CC or different CCs in the same band, UE is required to measure one of but not both CSI-RS for CBD measurement and SSB. Longer evaluation period for CSI-RS based CBD measurement is expected, and no requirements are defined.

For FR2, when the CSI-RS for CBD measurement on one CC is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same CC or different CCs in the same band, UE is required to measure one of but not both CSI-RS for CBD measurement and the other CSI-RS. Longer evaluation period for CSI-RS based CBD measurement is expected, and no requirements are defined. When two CSI-RSs for CBD measurements are from different sets q1,0 and q1,1, UE shall be able to measure both CSI-RSs for CBD measurements.

For UE incapable of [capability of measurement with RTD>CP] and for UE capable of [capability of measurement with RTD>CP],

-	For both FR1 and FR2, when the CSI-RS for CBD measurement fully or partially overlapps with the OFDM symbol as SSB from candidate LTM neighbor cell for intra-frequency L1-RSRP measurement or inter-frequency L1-RSRP measurement without gap, UE is not required to receive CSI-RS for CBD measurement in the PRBs that overlap with an SSB.

-	For FR1, when the CSI-RS for CBD measurement fully or partially overlapps with the OFDM symbol as SSB from candidate LTM neighbor cell for intra-frequency L1-RSRP measurement or inter-frequency L1-RSRP measurement without gap, if CSI-RS and SSB have different SCS and UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both CSI-RS for CBD measurement and SSB. Longer measurement period for CSI-RS based CBD is expected, and no requirements are defined.

-	For FR2, when the CSI-RS for CBD measurement on one CC fully or partially overlapps with the OFDM symbol as SSB from candidate LTM neighbor cell for intra-frequency L1-RSRP measurement or inter-frequency L1-RSRP measurement without gap in the same band, UE is required to measure one of but not both CSI-RS for CBD measurement and SSB. Longer measurement period for CSI-RS based CBD is expected, and no requirements are defined.

For FR2-1, for PCell, there is no measurement restriction allowed for UE supporting [TBD - multi-rx capability], and the UE is required to measure both the CSI-RS for CBD and the other CSI-RS for RLM, BFD, CBD or L1-RSRP measurement, provided the following conditions are met:

-	Both CSI-RSs are not in any CSI-RS resource set with repetition ON, and

-	[FFS: The two CSI-RS resources are overlapped on the same OFDM symbol, and]

-	[The two CSI-RS resources and both PDSCH are overlapped on the same OFDM symbol, and]

-	The CSI-RS and both of the PDSCHs are on the same OFDM symbol(s), and

-	One CSI-RS has same QCL source as the active TCI state of one PDSCH, and the other CSI-RS has same QCL source as the active TCI state of the other PDSCH, and

-	Resources of the active TCI states for the two PDSCHs have been reported as a resource group in Rel-17 group-based RSRP report.

Editor’s note 1: FFS whether CSI-RS need to overlap with PDSCH for measurement restriction.

Editor’s note 2: [FFS how to capture UE is activated with multi-Rx operation.]

<< End of change 10>>

<< Start of change 11>>

9.8.5	Measurement restriction for L1-SINR measurement

The UE is required to be capable of measuring L1-SINR without measurement gaps. The UE is required to perform the SSB and CSI-RS/CSI-IM measurements with measurement restrictions as described in the following clauses.

9.8.5.1	Measurement restriction if SSB configured for L1-SINR Measurement

For FR1, when the SSB configured as CMR for L1-SINR measurement is in the same OFDM symbol as CSI-RS for RLM, BFD, CBD, L1-RSRP or L1-SINR measurement,

-	If SSB and CSI-RS have same SCS, UE shall be able to measure the SSB for L1-SINR measurement without any restriction;

-	If SSB and CSI-RS have different SCS,

-	If UE supports simultaneousRxDataSSB-DiffNumerology, UE shall be able to measure the SSB for L1-SINR measurement without any restriction;

-	If UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both SSB for L1-SINR measurement and CSI-RS. Longer measurement period for SSB based L1-SINR measurement is expected, and no requirements are defined.

For FR2, when the SSB configured as CMR for L1-SINR measurement on one CC is in the same OFDM symbol as CSI-RS for RLM, BFD, CBD, L1-RSRP or L1-SINR measurement on the same CC or different CCs in the same band, UE is required to measure one of but not both SSB for L1-SINR measurement and CSI-RS. Longer measurement period for SSB based L1-RSRP measurement is expected, and no requirements are defined.

For FR2, there is no measurement restriction allowed when the network configures mixed numerology between SSB configured as CMR for L1-SINR measurement on one FR2 band and CSI-RS for RLM, BFD, CBD, L1-RSRP or L1-SINR measurement on the other FR2 band, provided that UE is capable of independent beam management on this FR2 band pair.

For FR2, for UE incapable of [capability of measurement with RTD>CP] and for UE capable of [capability of measurement with RTD>CP], when the SSB configured as CMR for L1-SINR measurement on one CC fully or partially overlapps with the OFDM symbol as SSB from candidate LTM neighbor cell for intra-frequency L1-RSRP measurement or inter-frequency L1-RSRP measurement without gap in the same band, UE is required to measure one of but not both SSB for L1-SINR measurement and SSB for L1-RSRP measurement on neighbor cell. Longer measurement period for SSB based L1-RSRP measurement is expected, and no requirements are defined.

9.8.5.2	Measurement restriction if CSI-RS configured for L1-SINR measurement

For both FR1 and FR2, when the CSI-RS configured for L1-SINR measurement is in the same OFDM symbol as SSB for RLM, BFD, CBD, L1-RSRP or L1-SINR measurement, UE is not required to receive CSI-RS for L1-SINR measurement in the PRBs that overlap with an SSB.

For FR1, when the SSB for RLM, BFD, CBD, L1-RSRP or L1-SINR measurement is within the active BWP or outside the active BWP for UE supporting [FG53-1], provided that the SSB is within the configured UE-specific CBW, and has same SCS than CSI-RS configured for L1-SINR measurement, the UE shall be able to perform CSI-RS measurement without restrictions.

For FR1, when the SSB for RLM, BFD, CBD, L1-RSRP or L1-SINR measurement is within the active BWP or outside the active BWP for UE supporting [FG53-1], provided that the SSB is within the configured UE-specific CBW, and has different SCS than CSI-RS configured for L1-SINR me

-	If the UE supports simultaneousRxDataSSB-DiffNumerology, UE shall be able to perform CSI-RS measurement without restrictions.

-	If the UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both CSI-RS for L1-SINR measurement and SSB. Longer measurement period for CSI-RS based L1-SINR measurement is expected, and no requirements are defined.

For FR1, when the CSI-RS configured for L1-SINR measurement is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD, L1-RSRP or L1-SINR measurement, UE shall be able to measure the CSI-RS for L1-SINR measurement without any restriction.

For FR2, when the CSI-RS configured for L1-SINR measurement on one CC is in the same OFDM symbol as SSB for RLM, BFD, L1-RSRP or L1-SINR measurement on the same CC or different CCs in the same band, or in the same symbol as SSB for CBD measurement on the same CC or different CCs in the same band when beam failure is detected, UE is required to measure one of but not both CSI-RS for L1-SINR measurement and SSB. Longer measurement period for CSI-RS based L1-SINR measurement is expected, and no requirements are defined.

For FR2, when the CSI-RS configured for L1-SINR measurement on one CC is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD, L1-RSRP or L1-SINR measurement on the same CC or different CCs in the same band,

-	In the following cases, UE is required to measure one of but not both CSI-RS for L1-SINR measurement and the other CSI-RS. Longer measurement period for CSI-RS based L1-SINR measurement is expected, and no requirements are defined.

-	The CSI-RS for L1-SINR measurement or the other CSI-RS in a resource set configured with repetition ON, or

-	The CSI-RS or the other CSI-RS is configured as dedicated IMR for L1-SINR computation with SSB as CMR, or

-	The other CSI-RS is configured in q1 and beam failure is detected, or

-	The two CSI-RS-es are not QCL-ed w.r.t. QCL-TypeD, or the QCL information is not known to UE,

-	Otherwise, UE shall be able to measure the CSI-RS configured for L1-SINR measurement without any restriction.

For UE incapable of [capability of measurement with RTD>CP] and for UE capable of [capability of measurement with RTD>CP],

-	For both FR1 and FR2, when the CSI-RS configured for L1-SINR measurement fully or partially overlapps with the OFDM symbol as SSB from candidate LTM neighbor cell for intra-frequency L1-RSRP measurement or inter-frequency L1-RSRP measurement without gap, UE is not required to receive CSI-RS for L1-SINR measurement in the PRBs that overlap with an SSB.

-	For FR1, when the CSI-RS configured for L1-SINR measurement fully or partially overlapps with the OFDM symbol as SSB from candidate LTM neighbor cell for intra-frequency L1-RSRP measurement or inter-frequency L1-RSRP measurement without gap, if CSI-RS and SSB have different SCS and UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both CSI-RS for L1-SINR measurement and SSB. Longer measurement period for CSI-RS based L1-SINR measurement is expected, and no requirements are defined.

-	For FR2, when the CSI-RS configured for L1-SINR measurement on one CC fully or partially overlapps with the OFDM symbol as SSB from candidate LTM neighbor cell for intra-frequency L1-RSRP measurement or inter-frequency L1-RSRP measurement without gap in the same band, UE is required to measure one of but not both CSI-RS for L1-SINR measurement and SSB. Longer measurement period for CSI-RS based L1-SINR measurement is expected, and no requirements are defined.

For FR2, when the UE is indicated with two TCI states for simultaneous PDSCH reception, when the CSI-RS configured for L1-SINR measurement on one CC is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD, L1-RSRP or L1-SINR measurement on the same CC or different CCs in the same band, UE shall be able to measure the CSI-RS configured for L1-SINR measurement without any restriction under the following conditions:

-	Both CSI-RSs are not in any CSI-RS resource set with repetition ON,

-	[The two CSI-RS resources and both PDSCHs are overlapped on the same OFDM symbol,]

-	[One CSI-RS has same QCL source as the active TCI state of one PDSCH, and the other CSI-RS has same QCL source as the active TCI state of the other PDSCH,]

-	[Resources of the active TCI states for the two PDSCHs have been reported as a resource group in Rel-17 group-based RSRP report.]

Editor’s note: FFS how to capture UE is activated with multi-Rx operation.

9.8.5.3	Measurement restriction if CSI-IM configured for L1-SINR measurement

For both FR1 and FR2, when the CSI-IM configured for L1-SINR measurement is in the same OFDM symbol as SSB for RLM, BFD, CBD, L1-RSRP or L1-SINR measurement, UE is not required to measure CSI-IM for L1-SINR measurement in the PRBs that overlap with an SSB.

For FR1, UE shall be able to measure the CSI-IM configured for L1-SINR measurement without any restriction.

For FR2, when the CSI-IM configured for L1-SINR measurement on one CC is in the same OFDM symbol as SSB for RLM, BFD, L1-RSRP or L1-SINR measurement on the same CC or different CCs in the same band, or in the same symbol as SSB for CBD measurement on the same CC or different CCs in the same band when beam failure is detected, UE is required to measure one of but not both CSI-IM for L1-SINR measurement and SSB. Longer measurement period for L1-SINR measurement is expected, and no requirements are defined.

For FR2, when the CSI-IM configured for L1-SINR measurement on one CC is in the same OFDM symbol as the CSI-RS for RLM, BFD, CBD, L1-RSRP or L1-SINR measurement on the same CC or different CCs in the same band,

-	In the following cases, UE is required to measure one of but not both CSI-IM for L1-SINR measurement and CSI-RS. Longer measurement period for L1-SINR measurement is expected, and no requirements are defined.

-	The CSI-RS in a resource set configured with repetition ON, or

-	The CSI-IM or the CSI-RS is configured as dedicated IMR for L1-SINR computation with SSB as CMR, or

-	The CSI-RS is configured in q1 and beam failure is detected, or

-	The CMR for L1-SINR measurement and the CSI-RS are not QCL-ed w.r.t. QCL-TypeD, or the QCL information is not known to UE,

-	Otherwise, UE shall be able to measure the CSI-IM configured for L1-SINR measurement without any restriction.

For UE incapable of [capability of measurement with RTD>CP] and for UE capable of [capability of measurement with RTD>CP],

-	For both FR1 and FR2, when the CSI-IM configured for L1-SINR measurement fully or partially overlapps with the OFDM symbol as SSB from candidate LTM neighbor cell for intra-frequency L1-RSRP measurement or inter-frequency L1-RSRP measurement without gap, UE is not required to receive CSI-IM for L1-SINR measurement in the PRBs that overlap with an SSB.

-	For FR2, when the CSI-IM configured for L1-SINR measurement on one CC fully or partially overlapps with the OFDM symbol as SSB from candidate LTM neighbor cell for intra-frequency L1-RSRP measurement or inter-frequency L1-RSRP measurement without gap in the same band, UE is required to measure one of but not both CSI-RS for L1-SINR measurement and SSB. Longer measurement period for CSI-IM based L1-SINR measurement is expected, and no requirements are defined.

<< End of change 11>>

<< Start of change 12>>

#### 9.5.4.1	SSB based L1-RSRP Reporting

When groupBasedBeamReporting-r17 is not configured, the UE shall be capable of performing L1-RSRP measurements based on the configured SSB resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB.

When groupBasedBeamReporting-r17 is configured, the UE shall be capable of performing L1-RSRP measurements based on the two configured SSB resource sets for L1-RSRP, and the UE physical layer shall be capable of reporting group-based L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB.

When there is no intra-frequency L1-RSRP measurement on LTM neighbor cell(s) to measure, the value of TL1-RSRP_Measurement_Period_SSB is defined in Table 9.5.4.1-1 for FR1, the value of TL1-RSRP_Measurement_Period_SSB is defined in Table 9.5.4.1-2 for FR2 when highSpeedMeasFlagFR2-r17 is not configured and if groupBasedBeamReporting-r17 is not configured, and defined in Table 9.5.4.1-2A for FR2-1 when highSpeedMeasFlagFR2-r17 is not configured and if groupBasedBeamReporting-r17 is configured, and defined in Table 9.5.4.1-3 for FR2 power class 6 UE when highSpeedMeasFlagFR2-r17 is configured, where

-	M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise

-	The value of N in Table 9.5.4.1-2 and Table 9.5.4.1-3.is [reducedRxBeamNum] for UE supporting [faster beam switching capability] and activated with multi-Rx operation, otherwise the value of N in Table 9.5.4.1-2 is 8.

Editor note: FFS how to capture UE is activated with multi-Rx operation.

-	The value of N in Table 9.5.4.1-2A is 8.

For a UE supporting [support for Case 1 requirements] and when concurrent measurement gap(s) with Pre-MG(s) are configured, or a UE supporting [support for Case 2 requirements] and when concurrent measurement gap(s) with NCSG measurement gap(s) are configured, or a UE supporting concurrentMeasGap-r17 or musim-GapPreference-r17 or both concurrent measurement GAP and musim-GapPreference-r17 and when concurrent gaps or periodic MUSIM gaps or both concurrent gaps and periodic MUSIM gaps are configured,

-	an SSB or an SMTC occasion is not considered to be overlapped by a gap occasion if the gap occasion is dropped according to 9.1.8 and 9.1.10,

-	P value for SSB resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

-	For a window W of duration max(TL1,  MGRP_max), where MGRP_max is the maximum MGRP across all -	For a window W of duration max(TL1,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE measurement gaps , MUSIM gap(s) or NCSGs and per-FR measurement gaps or NCSGs, and, in case of Pre-MG, all activated per-UE measurement gaps and per-FR measurement gaps, within the same FR as serving cell, and starting at the beginning of any SSB resource occasion:

Ntotal is the total number of SSB resource occasions within the window, including those overlapped with measurement gap occasions, MUSIM gap occasions or SMTC occasions within the window, and

-	Noutside_MG is the number of SSB resource occasions that are not overlapped with any non-dropped measurement gap occasion nor non-dropped MUSIM gap occasion within the window W, and

-	Navailable is the number of SSB resource occasions that are not overlapped with any non-dropped measurement gap occasion, non-dropped MUSIM gap occasion nor any SMTC occasion within the window W.

-	an SSB or an SMTC occasion is considered to be overlapped with the MUSIM gap if it overlaps a MUSIM gap occasion.

-	TL1 is periodicity of the target SSB.

Otherwise, for a UE neither supporting concurrentMeasGap-r17 nor [support for Case 1 requirements] nor [support for Case 2 requirements] or when neither of the above configurations applies, i.e. concurrent measurement gaps, concurrent measurement gap(s) with Pre-MG(s) and concurrent measurement gap(s) with NCSG measurement gap(s) and UE does not support musim-GapPreference-r17 or when no MUSIM gaps are configured,

For FR1,

-	P=11-TSSBxRP, when in the monitored cell there are GAPs configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the SSB; and

-	P=1 when in the monitored cell there are no GAPs overlapping with any occasion of the SSB.

For FR2,

P1= 11-TSSBTSMTCperiod, when SSB is not overlapped with measurement gap and SSB is partially overlapped with SMTC occasion (TSSB < TSMTCperiod).

-	P1 is PL1_sharing*Psharing factor, when SSB is not overlapped with measurement gap and SSB is fully overlapped with SMTC occasion (TSSB = TSMTCperiod).

-	P1=11-TSSBxRP-TSSBTSMTCperiod, when SSB is partially overlapped with GAP and SSB is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with GAP and

-	TSMTCperiod ≠ xRP or

-	TSMTCperiod = xRP and TSSB < 0.5*TSMTCperiod

-	P is Psharing factor1-TSSBxRP, when SSB is partially overlapped with GAP and SSB is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with GAP and TSMTCperiod = xRP and TSSB = 0.5*TSMTCperiod

-	P1= 11-TSSBmin(TSMTCperiod,xRP), when SSB is partially overlapped with GAP (TSSB < xRP) and SSB is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is partially or fully overlapped with GAP.

-	P is  PL1_sharing*Psharing factor1-TSSBxRP, when SSB is partially overlapped with measurement gap and SSB is fully overlapped with SMTC occasion (TSSB = TSMTCperiod) and SMTC occasion is partially overlapped with GAP (TSMTCperiod < xRP)

-	If SSB resource from the cell with different PCI is configured for L1-RSRP measurement, and P2 is valid accoding to 9.13.4.1, and any symbol of the SSBs from serving cell and cell with different PCI are overlapping or adjacent (in time domain)

-	P = P11-P1*TSSBP2*TSSB_CDP ,   if P1*TSSB < P2*TSSB_CDP.

-	P = P1, if P1*TSSB > P2*TSSB_CDP.

-	P = 2*P1, if P1*TSSB = P2*TSSB_CDP.

-	Otherwise, P = P1

Where:

-	TSSB = ssb-periodicityServingCell of the serving cell

-	TSMTCperiod = the configured SMTC period

-	TSSB_CDP = SSB periodicity of the cell with PCI different from serving cell

-	Psharing factor = 1, if the SSB configured for L1-RSRP measurement outside gap is

-	not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

-	not overlapped by the RSSI symbols indicated by ss-RSSI-Measurement and 1 data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured.

-	Psharing factor = 3, otherwise.

-	PL1_sharing = 2, if SSB resource from the cell with different PCI is configured for L1-RSRP measurement, and Psharing_factor,CDP is used in 9.13.4.1, and any symbol of the SSBs from serving cell and cell with different PCI are overlapping or adjacent (in time domain). PL1_sharing = 1, otherwise.

-	TSSB = ssb-periodicityServingCell

-	TSMTCperiod = the configured SMTC period

-	When a measurement gap is configured and the measurement gap is not NCSG,

-	an SSB or an SMTC occasion is considered to be overlapped with the GAP if it overlaps a measurement gap occasion, and

-	xRP = MGRP

-	If the UE is configured with Pre-MG, an SSB or an SMTC occasion is only considered to be overlapped by the Pre-MG if the Pre-MG is activated.

-	Otherwise, when NCSG measurement gap only is configured,

-	an SSB or an SMTC occasion is considered to be overlapped with the GAP if

-	it overlaps the VIL1 or VIL2 of NCSG, or

-	it overlaps the ML of NCSG in FR2, and there exists a target carrier to be measured within NCSG that is intra-frequency carrier or inter-frequency carrier in the same band as the serving cell, or inter-frequency carrier in different band as the serving cell and UE does not support IBM between the target carrier and the serving cell,

-	and

-	xRP = VIRP

The value of TL1-RSRP_Measurement_Period_SSB is defined in Table 9.5.4.1-4 for UE incapable of [capability of measurement with RTD>CP] in FR1, Table 9.5.4.1-5 for UE capable of [capability of measurement with RTD>CP] in FR1, Table 9.5.4.1-6 for UE incapable of [capability of measurement with RTD>CP] in FR2 and Table 9.5.4.1-7 for UE capable of [capability of measurement with RTD>CP] in FR2 when there is intra-frequency L1-RSRP measurement on neighbor cell(s) to measure, where

-	M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise

-	N= 8 in Table 9.5.4.1-6 and Table 9.5.4.1-7.

-	P value for SSB resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

For a window W of duration max (TL1,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as serving cell, and starting at the beginning of any SSB resource occasion:

-	Ntotal is the total number of SSB resource occasions within the window, including those overlapped with measurement gap occasions or SMTC occasions within the window, and

-	Noutside_MG is the number of SSB resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of SSB resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	TL1 is periodicity of the target SSB.

-	Psharing factor = 1, if the SSB configured for L1-RSRP measurement outside measurement gap is

-	not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

-	not overlapped with the RSSI symbols indicated by ss-RSSI-Measurement and 1data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured,

-	Psharing factor = 3, otherwise.

-	PL1_sharing is defined as

- 	When number of neighboring cells to be measured is 1

-	PL1_sharing = 2, if any symbol of the SSBs from serving cell and neighbor cell are overlapping or adjacent (in time domain)

-	PL1_sharing = 1, otherwise

-	When number of neighboring cells to be measured is more than 1

-	PL1_sharing = 3.

If the high layer in TS 38.331 [2] signaling of smtc2 is configured, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1. TSMTCperiod is the shortest SMTC period among all CCs in the same FR2 band, provided the SMTC offset of all CCs in FR2 have the same offset.

Longer evaluation period would be expected if the combination of SSB, SMTC occasion and GAP configurations does not meet pervious conditions.

When UE is configured with aperiodic MUSIM gap and the aperiodic MUSIM gap is overlapping with SSB resource occasion for L1-RSRP, longer evaluation period would be expected.

When UE is configured with MUSIM gap(s), and SSB resource occasions for L1-RSRP are fully overlapped with MUSIM gap(s) or fully overlapped with the union of MUSIM gap(s) and GAPs, no requirement applies for the SSB based L1-RSRP measurement.

For either an FR1 or FR2 serving cell, longer evaluation period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

For either an FR1 or FR2 serving cell, longer L1 RSRP measurement period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

Table 9.5.4.1-1: Measurement period TL1-RSRP_Measurement_Period_SSB for FR1

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1 when TSSB ≤ 40 ms and highSpeedMeasFlag-r16 or highSpeedMeasCA-Scell-r17 are configured; otherwise K = 1.5. Note 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16. or measurementEnhancementCA-r17 |  |

Table 9.5.4.1-2: Measurement period TL1-RSRP_Measurement_Period_SSB for FR2

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

Table 9.5.4.1-2A: Measurement period TL1-RSRP_Measurement_Period_SSB configured with groupBasedBeamReporting-r17 for FR2-1

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

Table 9.5.4.1-3: Measurement period TL1-RSRP_Measurement_Period_SSB configured with highSpeedMeasFlagFR2-r17 for FR2

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N1Note2)*TSSB) |
| DRX cycle ≤ 80ms | max(TReport, ceil(M*P*N1Note2*M2)*max(TDRX,TSSB)) |
| 80ms< DRX ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: Scaling factor N1 = 2 when highSpeedMeasFlagFR2-r17 = set1 or scaling factor N1 = 6 when highSpeedMeasFlagFR2-r17 = [set2], if UE does not support [simultaneousReceptionFR2HST-r18] or when highSpeedDeploymentTypeFR2-r17 is not configured as bidirectional. Scaling factor N1 = 1.5 when highSpeedMeasFlagFR2-r17 is configured to set1 or scaling factor N1 = 4 when highSpeedMeasFlagFR2-r17 is configured to set2, if UE supports [simultaneousReceptionFR2HST-r18] and when highSpeedDeploymentTypeFR2-r17 is configured as bidirectional. Note 3: M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1 |  |

Table 9.5.4.1-4: Measurement period TL1-RSRP_Measurement_Period_SSB in FR1 for UE incapable of [capability of measurement with RTD>CP]

| Configuration | TL1-RSRP_Measurement_Period_SSB_intra (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1.5. Note 3: No requirements if the actual RTD of serving cell and neighbor cell is larger than CP. |  |

Table 9.5.4.1-5: Measurement period TL1-RSRP_Measurement_Period_SSB in FR1 for UE capable of [capability of measurement with RTD>CP]

| Configuration | TL1-RSRP_Measurement_Period_SSB_intra (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1.5. |  |

Editor notes: The requirements for multiple layers will be updated based on newly achieved agreements.

Table 9.5.4.1-6: Measurement period TL1-RSRP_Measurement_Period_SSB in FR2 for UE incapable of [capability of measurement with RTD>CP]

| Configuration | TL1-RSRP_Measurement_Period_SSB_intra (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*PL1_sharing*N)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*PL1_sharing *N)*max(TDRX,TSSB) ) |
| DRX cycle > 320ms | ceil(1.5*M*P*PL1_sharing*N)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: No requirements if the actual RTD of serving cell and neighbor cell is larger than CP. |  |

Table 9.5.4.1-7: Measurement period TL1-RSRP_Measurement_Period_SSB in FR2 for UE capable of [capability of measurement with RTD>CP]

| Configuration | TL1-RSRP_Measurement_Period_SSB_intra (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*PL1_sharing*N)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*PL1_sharing*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*PL1_sharing*N)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

<< End of change 12>>

<< Start of change 13>>

9.5.5.2	Measurement restriction for CSI-RS based L1-RSRP

The SSB mentioned in this clause can be associated with either the serving cell PCI or a PCI different from serving cell PCI or intra-frequency neighbor cell(s) configured with L1-RSRP measurement.

For both FR1 and FR2, when the CSI-RS for L1-RSRP measurement is in the same OFDM symbol as SSB for RLM, BFD, CBD or L1-RSRP measurement, UE is not required to receive CSI-RS for L1-RSRP measurement in the PRBs that overlap with an SSB.

For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement is within the active BWP and has same SCS than CSI-RS for L1-RSRP measurement, the UE shall be able to perform CSI-RS measurement without restrictions.

For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement is within the active BWP and has different SCS than CSI-RS for L1-RSRP measurement, the UE shall be able to perform CSI-RS measurement with restrictions according to its capabilities:

-	If the UE supports simultaneousRxDataSSB-DiffNumerology the UE shall be able to perform CSI-RS measurement without restrictions.

-	If the UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both CSI-RS for L1-RSRP measurement and SSB. Longer measurement period for CSI-RS based L1-RSRP measurement is expected, and no requirements are defined.

For FR1, when the CSI-RS for L1-RSRP measurement is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement, UE shall be able to measure the CSI-RS for L1-RSRP measurement without any restriction.

For FR2, when the CSI-RS for L1-RSRP measurement on one CC is in the same OFDM symbol as SSB for RLM, BFD or L1-RSRP measurement on the same CC or different CCs in the same band, or in the same symbol as SSB for CBD measurement on the same CC or different CCs in the same band when beam failure is detected, UE is required to measure one of but not both CSI-RS for L1-RSRP measurement and SSB. Longer measurement period for CSI-RS based L1-RSRP measurement is expected, and no requirements are defined.

For FR2, when the CSI-RS for L1-RSRP measurement on one CC is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on the same CC or different CCs in the same band,

-	In the following cases, UE is required to measure one of but not both CSI-RS for L1-RSRP measurement and the other CSI-RS. Longer measurement period for CSI-RS based L1-RSRP measurement is expected, and no requirements are defined.

-	The CSI-RS for L1-RSRP measurement or the other CSI-RS in a resource set configured with repetition ON, or

-	The other CSI-RS is configured in q1 and beam failure is detected, or

-	The two CSI-RS-es are not QCL-ed w.r.t. QCL-TypeD, or the QCL information is not known to UE,

-	Otherwise, UE shall be able to measure the CSI-RS for L1-RSRP measurement without any restriction.

For UE incapable of [capability of measurement with RTD>CP] and for UE capable of [capability of measurement with RTD>CP],

-	For both FR1 and FR2, when the CSI-RS for L1-RSRP measurement fully or partially overlapps with the OFDM symbol as SSB from candidate LTM neighbor cell for intra-frequency L1-RSRP measurement or inter-frequency L1-RSRP measurement without gap, UE is not required to receive CSI-RS for L1-RSRP measurement in the PRBs that overlap with an SSB.

-	For FR1, when the CSI-RS for L1-RSRP measurement fully or partially overlapps with the OFDM symbol as SSB from candidate LTM neighbor cell for intra-frequency L1-RSRP measurement or inter-frequency L1-RSRP measurement without gap, if CSI-RS and SSB have different SCS and UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both CSI-RS for L1-RSRP measurement and SSB. Longer measurement period for CSI-RS based L1-RSRP is expected, and no requirements are defined.

-    For FR2, when the CSI-RS for L1-RSRP measurement on one CC fully or partially overlapps with the OFDM symbol as SSB from candidate LTM neighbor cell for intra-frequency L1-RSRP measurement or inter-frequency L1-RSRP measurement without gap in the same band, UE is required to measure one of but not both CSI-RS for L1-RSRP measurement and SSB. Longer measurement period for CSI-RS based L1-RSRP is expected, and no requirements are defined.

For FR2-1, when the CSI-RS for L1-RSRP measurement on PCell is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on PCell, UE supporting [multi-Rx capability] shall be able to measure the CSI-RS for L1-RSRP measurement without restriction when the following conditions are met:

-	Both the CSI-RS for L1-RSRP and the other CSI-RS are not in any CSI-RS resource set configured with repetition ON, and

-	Resources of the active TCI states for two PDSCHs have been reported as a resource group in Rel-17 group-based RSRP report, and

-	[The two CSI-RSs and both PDSCHs are overlapped on the same OFDM symbol, and]

-	The CSI-RS for L1-RSRP has same QCL source as the active TCI state of one [PDSCH], and the other CSI-RS has same QCL source as the active TCI state of the other [PDSCH], and

Editor’s note 1: FFS the condition CSI-RS(s) and PDSCH(s) are overlapped on the same OFDM symbol(s).

Editor’s note 2: FFS how to capture UE is activated with multi-Rx operation.

<< End of change 13>>

<< Start of change 14>>

9.14	Intra-frequency L1-RSRP measurements for neighbor cell

9.14.1	Introduction

A measurement is defined as a SSB based intra-frequency L1-RSRP measurement provided the centre frequency of the SSB of the serving cell indicated for L1-RSRP measurement and the centre frequency of the SSB of the neighbour cell are the same, and the subcarrier spacing of the two SSBs are also the same.

When configured with [LTM-CSI-ResourceConfig-r18] by the network, the UE shall be able to perform L1-RSRP measurements of configured measurement resources from a neighbor cell, with the measurement resources configured as SSBs of the neighbor cell.

[If the number of resources/cells, including the number of resources/cells configured for serving cell L1-RSRP measurement in 9.14 exceeds the UE capability indicated by [x], UE behaviour is TBD.]

The UE shall report the measurement quantity and send periodic, semi-persistent or aperiodic reports, according to the higher layer parameter [reportConfigType-r18] of each reporting setting [LTM-CSI-ReportConfig-r18].

9.14.2	Requirements Applicability

The requirements in the clause 9.14 are applicable to FR1 and FR2-1 for LTM.

The requirements in clause 9.14 apply, provided for the SSB from the neighbor cell configured for intra-frequency L1-RSRP, the following conditions are met:

-	The cell is known

-	The SSB resources configured for L1-RSRP measurements are measurable.

An SSB resource configured for L1-RSRP for neigbor cell shall be considered measurable when for each relevant SSB the following conditions are met:

-	L1-RSRP related side conditions given in clause [10.1.19] for FR1 and [10.1.20] for FR2-1, respectively, for a corresponding band,

-	SSB_RP and SSB Ês/Iot according to Annex [B.2.4.1] for a corresponding band.

The cell is considered as known if the following conditions are met in this requirement:

-	The UE has performed L3 measurement on the target cell -during the last 5 seconds, and

-	The SSB from the target cell configured for L1 measurement remains detectable according to the cell identification requirements specified in clause 9.2.

Otherwise, the cell is unknown.

9.14.3	Measurement Reporting Requirements

The UE shall report the L1-RSRP value as a 7-bit value in the range [-140, -44] dBm with 1dB step size according to clause [10.1.19] for FR1 and [10.1.20] for FR2 if [noOfReportedRS-PerCell-r18] and [noOfReportedCells-r18] are both configured to one. If [noOfReportedRS-PerCell-r18] and/or [noOfReportedCells-r18] are configured to be larger than one, the UE shall use differential L1-RSRP based reporting as defined in clause [10.1.19] for FR1 and [10.1.20] for FR2. The differential L1-RSRP is quantized to a 4-bit value with 2dB step size. The mapping between the reported L1-RSRP value and the measured quantity is described in 10.1.6.

9.14.3.1	Periodic Reporting

Reported L1-RSRP measurements contained in periodic L1-RSRP measurement reports shall meet the requirements in clause [10.1.19] for FR1 and [10.1.20] for FR2, respectively. The UE shall transmit the periodic L1-RSRP reporting on PUCCH over the air interface according to the periodicity defined in clause 5.2.1.4 in TS 38.214 [26].

9.14.3.2	Semi-Persistent Reporting

Reported L1-RSRP measurements contained in a Semi-Persistent L1-RSRP measurement report shall meet the requirements in clause [10.1.19] for FR1 and [10.1.20] for FR2, respectively. This requirement applies for semi-persistent L1-RSRP reports send on PUSCH or PUCCH.

The UE shall only send semi-persistent L1-RSRP measurement reports on PUSCH, if a DCI request has been received.

The UE shall only send semi-persistent L1-RSRP measurement reports on PUCCH, if an activation command [7] has been received.

The UE shall transmit the semi-persistent L1-RSRP reporting on PUSCH or PUCCH over the air interface according to the periodicity defined in clause 5.2.1.4 in TS 38.214 [26].

9.14.3.3	Aperiodic Reporting

Reported L1-RSRP measurements contained in aperiodic triggered, aperiodic triggered periodic and aperiodic triggered semi-persistent L1-RSRP reports shall meet the requirements in clauses [10.1.19] for FR1 and [10.1.20] for FR2, respectively.

The UE shall only send aperiodic L1-RSRP measurement report if a DCI trigger has been received.

After the UE receives CSI request in DCI, the UE shall transmit the aperiodic L1-RSRP reporting on PUSCH over the air interface at the time specified according to clause 6.1.2.1 in TS 38.214 [26].

9.14.4	Number of cells and number of SSB

For each intra-frequency layer, the UE shall be capable of performing L1-RSRP measurements for at least [[TBD] cells/SSBs based on UE capability]

9.14.5	L1-RSRP measurement requirements without measurement gaps

9.14.5.1	SSB based L1-RSRP Reporting

The requirements specified in this clause are only applicable when

-	highSpeedMeasFlag-r16 is not configured, and

-	highSpeedMeasFlagFR2-r17 is not configured, and

-	highSpeedMeasCA-Scell-r17 is not configured.

-	The SSB from the neighbor cell completely contained in the active BWP of the UE

If a neighbor cell is known according 9.14.2, the UE shall be capable of performing L1-RSRP measurements based on the configured SSB resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB_intra if deriveSSB-IndexFromCell is enabled or UE has reported SSB index in L3 measurement report of the same cell. Otherwise, UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB_intra+TSSB_time_index_intra, where TSSB_time_index_intra is the time period used to acquire the index of the SSB being measured given in Table 9.2.5.1-3.

The value of TL1-RSRP_Measurement_Period_SSB_intra is defined for FR1 in Table 9.14.5.1-1 for UE incapable of [capability of measurement with RTD>CP] and in Table 9.14.5.1-2 for UE capable of [capability of measurement with RTD>CP].  The value of TL1-RSRP_Measurement_Period_SSB_intra is defined for FR2 in Table 9.14.5.1-3 for UE incapable of [capability of measurement with RTD>CP] and in Table 9.14.5.1-4 for UE capable of [capability of measurement with RTD>CP], where

-	M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3  otherwise

-	N= 8.

-	P value for SSB resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

For a window W of duration max (TL1,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as serving cell, and starting at the beginning of any SSB resource occasion:

-	Ntotal is the total number of SSB resource occasions within the window, including those overlapped with measurement gap occasions or SMTC occasions within the window, and

-	Noutside_MG is the number of SSB resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of SSB resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	TL1 is periodicity of the target SSB.

-	Psharing factor = 1, if the SSB configured for L1-RSRP measurement outside measurement gap is

-	not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

-	not overlapped with the RSSI symbols indicated by ss-RSSI-Measurement and 1data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured,

-	Psharing factor = [3], otherwise.

-	PL1_sharing is defined as

-	When number of neighboring cells to be measured is 1

-	PL1_sharing = 2, if any symbol of the SSBs from serving cell and neighbor cell are overlapping or adjacent (in time domain)

-	PL1_sharing = 1, otherwise

-	When number of neighboring cells to be measured is more than 1

-	When TCI state of any intra-frequency or inter-frequency without gap neighbor cell are not in the active TCI state list

-	PL1_sharing = 3*NNeighbor_Cell, where NNeighbor_Cell is the number of neighbor cells to measure on intra-frequency and inter-frequency without gap

-	Otherwise

-	PL1_sharing = 3*NNeighbor_Cell_in_list, where NNeighbor_Cell_in_list  is the number of neigbour cells (including intra-frequency neighbor cells and inter-frequency without gap neighbor cells) whose TCI state(s) are in the active TCI state list.

If the high layer in TS 38.331 [2] signaling of smtc2 is configured, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1. TSMTCperiod is the shortest SMTC period among all CCs in the same FR2 band, provided the SMTC offset of all CCs in FR2 have the same offset.

Longer measurement period would be expected if the combination of SSB, SMTC occasion and measurement gap configurations does not meet pervious conditions.

For either an FR1 or FR2 cell, longer measurement period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

For either an FR1 or FR2 cell, longer L1 RSRP measurement period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

Table 9.14.5.1-1: Intra-frequency L1-RSRP measurement period TL1-RSRP_Measurement_Period_SSB_intra in FR1 for UE incapable of [capability of measurement with RTD>CP]

| Configuration | TL1-RSRP_Measurement_Period_SSB_intra (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB_NBC) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB_NBC)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TSSB_NBC is the periodicity of the neighbor cell SSB-Index configured for intra-frequency L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1.5. Note 3: No requirements if the actual RTD of serving cell and neighbor cell is larger than CP. |  |

Table 9.14.5.1-2: Intra-frequency L1-RSRP measurement period TL1-RSRP_Measurement_Period_SSB_intra in FR1 for UE capable of [capability of measurement with RTD>CP]

| Configuration | TL1-RSRP_Measurement_Period_SSB_intra (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB_NBC*NLayer) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB_NBC) *NLayer) |
| DRX cycle > 320ms | ceil(M*P)*TDRX*NLayer |
| Note 1: TSSB_NBC is the periodicity of the neighbor cell SSB-Index configured for intra-frequency L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. [NLayer = the number of intra-frequency layers configured for L1-RSRP measurement with [LTM-CSI-ResourceConfig-r18] + the number of inter-frequency layers without measurement gaps which are configured for L1-RSRP measurement with [LTM-CSI-ResourceConfig-r18]]. Note 2: K = 1.5. |  |

Table 9.14.5.1-3: Intra-frequency L1-RSRP measurement period TIntra_ L1-RSRP_Measurement_Period_SSB in FR2 for UE incapable of [capability of measurement with RTD>CP]

| Configuration | TL1-RSRP_Measurement_Period_SSB_intra (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*PL1_sharing*N)*TSSB_NBC) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*PL1_sharing *N)*max(TDRX,TSSB_NBC) ) |
| DRX cycle > 320ms | ceil(1.5*M*P*PL1_sharing*N)*TDRX |
| Note 1: TSSB_NBC is the periodicity of the neighbor cell SSB-Index configured for intra-frequency L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: No requirements, if the actual RTD of serving cell and neighbor cell is larger than CP. Note3: When the number of neighbor cells to be measured on intra-frequency and inter-frequency without gap is more than 1 and TCI state(s) of one of these neighbor cells is in the active TCI state, no requirements for other intra-frequency neighbor cells except the one whose TCI state(s) is in the active TCI state. |  |

Table 9.14.5.1-4: Intra-frequency L1-RSRP measurement period TL1-RSRP_Measurement_Period_SSB_intra in FR2 for UE capable of [capability of measurement with RTD>CP]

| Configuration | TL1-RSRP_Measurement_Period_SSB_intra (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*PL1_sharing*N)*TSSB_NBC) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*PL1_sharing*N)*max(TDRX,TSSB_NBC)) |
| DRX cycle > 320ms | ceil(1.5*M*P*PL1_sharing*N)*TDRX |
| Note 1: TSSB_NBC is the periodicity of the neighbor cell SSB-Index configured for intra-frequency L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: When the number of neighbor cells to be measured on intra-frequency and inter-frequency without gap is more than 1 and TCI state(s) of one of these neighbor cells is in the active TCI state, no requirements for other intra-frequency neighbor cells except the one whose TCI state(s) is in the active TCI state. |  |

9.14.6	Measurement restriction for L1-RSRP measurement

Measurement restrictions described in the following clauses apply when UE is performing L1-RSRP measurement on neighbor cell(s) without measurement gap.

Unless explicitly stated, the SSB to be measured for L1-RSRP measurement is transmitted from neighbor cell(s).

9.14.6.1	Measurement restriction for SSB based L1-RSRP

For FR1,

when the SSB for L1-RSRP measurement fully or partially overlapps with the OFDM symbol as SSB transmitted from serving cell(s) for RLM, BFD, CBD or L1-RSRP measurement,

-	UE shall be able to measure the SSB for L1-RSRP measurement without any restriction;

when the SSB for L1-RSRP measurement is in the same OFDM symbol as CSI-RS transmitted from serving cell(s) for RLM, BFD, CBD or L1-RSRP measurement,

-	If SSB and CSI-RS have same SCS, UE shall be able to measure the SSB for L1-RSRP measurement without any restriction;

-	If SSB and CSI-RS have different SCS,

-	If UE supports simultaneousRxDataSSB-DiffNumerology, UE shall be able to measure the SSB for L1-RSRP measurement without any restriction;

-	[If UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both SSB for L1-RSRP measurement and CSI-RS. Longer measurement period for SSB based L1-RSRP measurement is expected, and no requirements are defined].

For FR2, for UE incapable of [capability of measurement with RTD>CP] and for UE capable of [capability of measurement with RTD>CP],

when the SSB for L1-RSRP measurement on one CC fully or partially overlapps with the OFDM symbol as SSB transmitted from serving cell(s) for RLM, BFD, or CBD measurement on the same CC or different CCs in the same band, UE is required to measure one of but not both SSBs. Longer measurement period for SSB based RLM is expected, and no requirements are defined.

when the SSB for L1-RSRP measurement on one CC fully or partially overlapps with the OFDM symbol as CSI-RS transmitted from serving cell(s) for RLM, BFD, or CBD measurement on the same CC or different CCs in the same band, UE is required to measure one of but not both SSB for L1-RSRP measurement and CSI-RS. Longer measurement period for SSB based L1-RSRP measurement is expected, and no requirements are defined.

For FR2, if the network configures same or mixed numerology between SSB for L1-RSRP measurement on one FR2 band and CSI-RS for RLM, BFD, CBD, L1-RSRP or L1-SINR measurement on the other FR2 band, UE shall be able to perform the related SSB based measurements in one band without any measurement restrictions in the other band, provided that UE is capable of independent beam management on this FR2 band pair.

9.14.7	Scheduling availability of UE during L1-RSRP measurement

Scheduling availability restrictions described in the following clauses apply when UE is performing L1-RSRP measurement on neighbor cell(s) without measurement gap.

Unless explicitly stated, the SSB to be measured for L1-RSRP measurement is transmitted from neigbor cell(s).

9.14.7.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1

There are no scheduling restrictions due to L1-RSRP measurement performed on SSB as RS for L1-RSRP measurement with the same SCS as PDSCH/PDCCH in FR1.

9.14.7.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1

For UEs which support simultaneousRxDataSSB-DiffNumerology [14] there are no restrictions on scheduling availability due to L1-RSRP measurement based on SSB as RS for L1-RSRP measurement. For UEs which do not support simultaneousRxDataSSB-DiffNumerology [14] the following restrictions apply due to L1-RSRP measurement based on SSB configured for L1-RSRP measurement.

-	The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on the concerned OFDM symbols, where the concern OFDM symbols are

-	the same and 1 OFDM symbol before or after the OFDM symbols corresponding to the SSB indexes configured for L1-RSRP measurement, if UE supports [capability of measurement with RTD>CP],

-	the same OFDM symbols corresponding to the SSB indexes configured for L1-RSRP measurement, if UE does not support [capability of measurement with RTD>CP],

When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions apply to cell(s) in the same band on the symbols that fully or partially overlap with restricted symbols. When inter-band carrier aggregation within FR1 is configured, there are no scheduling restrictions on FR1 cells configured in other bands than the bands in which the cell where L1-RSRP measurement is performed.

9.14.7.3	Scheduling availability of UE performing L1-RSRP measurement on FR2

The following scheduling restriction applies due to L1-RSRP measurement.

-	The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on the concerned OFDM symbols, where the concern OFDM symbols are

the same and 1 OFDM symbol before or after the OFDM symbols corresponding to the SSB indexes configured for L1-RSRP measurement.

When intra-band carrier aggregation in FR2 is performed, the scheduling restrictions is performed apply to cell(s) in the band on the symbols that fully or partially overlap with restricted symbols.

When inter-band carrier aggregation in FR2 is performed, there are no scheduling restrictions on FR2 cells in the bands due to L1-RSRP measurement performed on FR2 cell(s) in different band(s), provided that UE is capable of independent beam management on this FR2 band pair. Additionally, there is no scheduling restriction if the UE is configured with different numerology between SSB on one FR2 band and data on the other FR2 band provided the UE is configured for IBM operation for the band pair.

If following conditions are met,

-	UE has been notified about system information update through paging,

-	The gap between UE’s reception of PDCCH that UE monitors in the Type 2-PDCCH CSS set and that notifies system information update, and the PDCCH that UE monitors in the Type0-PDCCH CSS set, is greater than 2 slots,

For the SSB and CORESET for RMSI scheduling multiplexing patterns 3, UE is expected to receive the PDCCH that UE monitors in the Type0-PDCCH CSS set, and the corresponding PDSCH, on SSB symbols to be measured for L1-RSRP measurement; and

For the SSB and CORESET for RMSI scheduling multiplexing patterns 2, UE is expected to receive PDSCH that corresponds to the PDCCH that UE monitors in the Type0-PDCCH CSS set, on SSB symbols to be measured for L1-RSRP measurement.

9.14.7.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA

There are no scheduling restrictions on FR1 cell(s) due to L1-RSRP measurement performed on FR2 cell(s).

There are no scheduling restrictions on FR2 cell(s) due to L1-RSRP measurement performed on FR1 cell(s).

9.14.7.5	Scheduling availability of UE performing L1-RSRP measurement in TDD bands on FR1

When UE performs L1-RSRP measurement on neighbor cell in a TDD band, the following restrictions apply due to L1-RSRP measurement

-	The UE is not expected to transmit PUCCH/PUSCH/SRS on the concerned OFDM symbols and restricted symbols may partially or fully overlap with UL symbols, where the concern OFDM symbols are

-	the same and 1 OFDM symbol before or after the OFDM symbols corresponding to the SSB indexes configured for L1-RSRP measurement, if UE supports [capability of measurement with RTD>CP],

-	the same OFDM symbols corresponding to the SSB indexes configured for L1-RSRP measurement, if UE does not support [capability of measurement with RTD>CP].

<< End of change 14>>

<< Start of change 15>>

9.15	NR inter-frequency L1 measurement

9.15.1	Introduction

A L1-RSRP measurement is defined as an inter-frequency L1-RSRP measurement provided it is not defined as an intra- frequency L1-RSRP measurement according to clause [9.15].

When measurement gaps are provided, the UE shall be able to perform inter-frequency L1-RSRP measurements of SSBs from inter-frequency neighbor cells.

The UE shall be able to measure all SSB resources within the CSI-ResourceConfig settings configured for L1-RSRP measurement on inter-frequency neighbor cells with measurement gaps. If the number of resources exceeds the UE capability indicated by [TBD], it is up to UE implementation on how to choose resources to measure.

The UE shall report the measurement quantity (reportQuantity) and send periodic, semi-persistent or aperiodic reports, according to the higher layer parameter reportConfigType of each reporting setting CSI-ReportConfig.

9.15.2	Requirements Applicability

The requirements in clause 9.15 apply if the following conditions are met:

-	The SSB configured for inter-frequency L1-RSRP measurement is on the same carrier frequency of SSB configured for RRM measurement in one of the measurement objects configured in the measObjectNR.

-	The inter-frequency neighbor cell is known.

-	The SSB resources configured for inter-frequency L1-RSRP measurements are measurable.

An SSB resource configured for inter-frequency L1-RSRP shall be considered measurable when for each relevant SSB the following conditions are met:

-	L1-RSRP related side conditions given in [clauses 10.1.19.1 and 10.1.20.1 for FR1 and FR2], respectively, for a corresponding band,

-	SSB_RP and SSB Ês/Iot according to [Annex B.2.4.1] for a corresponding band.

The inter-frequency neighbor cell is considered as known if the following conditions are met in this requirement:

-	The UE has performed L3 measurement on the target cell -during the last 5 seconds, and

-	The SSB from the target cell configured for L1 measurement remains detectable according to the cell identification requirements specified in clause 9.3.

Otherwise, the cell is unknown.

9.15.3	Measurement Reporting Requirements

The UE shall send L1-RSRP reports for report configurations configured for the inter-frequency L1 measurement on neighbor cell.

The UE shall report the L1-RSRP value as a 7-bit value in the range [-140, -44] dBm with 1dB step size according to clause 10.1.6. If nrofReportedRS is configured to be larger than one, or if groupBasedBeamReporting-r17 is enabled, the UE shall use differential L1-RSRP based reporting as defined in clause 10.1.6. The differential L1-RSRP is quantized to a 4-bit value with 2dB step size. The mapping between the reported L1-RSRP value and the measured quantity is described in 10.1.6.

In EN-DC and NE-DC operation, when the UE is configured to perform E-UTRA SRS carrier-based switching an additional delay can be expected in FR1 if the UE is capable of per-FR gap, or an additional delay can be expected in both FR1 and FR2 if the UE is not capable of per-FR gap.

9.15.3.1	Periodic Reporting

Reported L1-RSRP measurements contained in periodic L1-RSRP measurement reports shall meet the requirements in clauses 10.1.19 for FR1 and 10.1.20 for FR2, respectively.

The UE shall transmit the periodic L1-RSRP reporting on PUCCH over the air interface according to the periodicity defined in clause 5.2.1.4 in TS 38.214 [26].

9.15.3.2	Semi-Persistent Reporting

Reported L1-RSRP measurements contained in a Semi-Persistent L1-RSRP measurement report shall meet the requirements in clauses 10.1.19 for FR1 and 10.1.20 for FR2, respectively. This requirement applies for semi-persistent L1-RSRP reports send on PUSCH or PUCCH.

The UE shall only send semi-persistent L1-RSRP measurement reports on PUSCH, if a DCI request has been received.

The UE shall only send semi-persistent L1-RSRP measurement reports on PUCCH, if an activation command [7] has been received.

The UE shall transmit the semi-persistent L1-RSRP reporting on PUSCH or PUCCH over the air interface according to the periodicity defined in clause 5.2.1.4 in TS 38.214 [26].

9.15.3.3	Aperiodic Reporting

Reported L1-RSRP measurements contained in aperiodic triggered, aperiodic triggered periodic and aperiodic triggered semi-persistent L1-RSRP reports shall meet the requirements in clauses 10.1.19 for FR1 and 10.1.20 for FR2, respectively.

The UE shall only send aperiodic L1-RSRP measurement report if a DCI trigger has been received.

After the UE receives CSI request in DCI, the UE shall transmit the aperiodic L1-RSRP reporting on PUSCH over the air interface at the time specified according to clause 6.1.2.1 in TS 38.214 [26].

9.15.4	Number of cells and number of SSB

For inter-frequency layer, during each layer 1 measurement period, the UE shall be capable of performing L1-RSRP measurements for at least [[TBD] cells/SSBs based on UE capability].

9.15.5	inter-frequency L1-RSRP with MG

9.15.5.1	Inter-frequency SSB based L1-RSRP Reporting

The requirements specified in this clause are only applicable when

-	highSpeedMeasFlag-r16 is not configured, and

-	highSpeedMeasFlagFR2-r17 is not configured, and

-	highSpeedMeasCA-Scell-r17 is not configured, and

-	Concurrent gaps are not configured, and

-	SSBs of the neighbor cell configured for inter-frequency L1-RSRP measurement are fully or partially overlapped with measurement gaps.

If an inter-frequency neighbor cell is known according to 9.15.2, the UE shall be capable of performing L1-RSRP measurements based on the configured SSB resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB_Inter if deriveSSB-IndexFromCellInter-r17 is enabled on target frequency layers or UE is indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured) of the same cell. Otherwise, UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB_inter+TSSB_time_index_inter, where TSSB_time_index_inter is the time period used to acquire the index of the SSB being measured given in Table 9.3.4.

The value of TL1-RSRP_Measurement_Period_SSB_Inter is defined in Table 9.15.5.1-1 for FR1. The value of TL1-RSRP_Measurement_Period_SSB_Inter is defined in Table 9.15.5.1-2 for FR2, where

-	M= 2 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M= 4 otherwise

-	N= 8.

-	CSSFinter: it is a carrier specific scaling factor and is determined according to CSSFwithin_gap in clause 9.1.5.15 for L1-RSRP measurement conducted within measurement gaps.

Table 9.15.5.1-1: Inter-frequency L1-RSRP measurement period TL1-RSRP_Measurement_Period_SSB_Inter for known cells in FR1

| Condition | TL1-RSRP_Measurement_Period_SSB_Inter |
| --- | --- |
| No DRX | Max(Treport, Ceil(M * Kgap) Max(MGRP, SSB period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(Treport, Ceil(M 1.5 * Kgap) Max(MGRP, SSB period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | Ceil(M * Kgap) DRX cycle CSSFinter |
| Note 1: The definition of Kgap is the same as L3 measurement which is a scaling factor for a SSB frequency layer to be measured within an associated measurement gap pattern. |  |

Table 9.15.5.1-2: Inter-frequency L1-RSRP measurement period TL1-RSRP_Measurement_Period_SSB_Inter for known cells in FR2

| Condition | TL1-RSRP_Measurement_Period_SSB_Inter |
| --- | --- |
| No DRX | Max(Treport, Ceil(Kgap M*N) Max(MGRP, SSB period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(Treport, Ceil(1.5 * Kgap M*N) Max(MGRP, SSB period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | Ceil(Kgap M*N) DRX cycle CSSFinter |
| Note 1: The definition of Kgap is the same as L3 measurement which is a scaling factor for a SSB frequency layer to be measured within an associated measurement gap pattern. |  |

9.15.6	Inter frequency L1-RSRP measurement without measurement gaps

9.15.6.1	Inter frequency L1-RSRP measurement requirements

9.15.6.1.1	Inter-frequency SSB based L1-RSRP measurement

When configured by the network and the inter-frequency cell’s SSB is completely contained in the DL active BWP, the UE shall be able to perform L1-RSRP measurements based on the configured SSB resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB_inter_withoutGap.

[If a neighbor cell is known according 9.15.2, the UE shall be capable of performing L1-RSRP measurements based on the configured SSB resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB_inter_withoutGap if deriveSSB-IndexFromCellInter-r17 is enabled or UE has reported SSB index in L3 measurement report of the same cell. [Otherwise, UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB_inter_withoutGap+TSSB_time_index_inter, where TSSB_time_index_inter is the time period used to acquire the index of the SSB being measured given in Table 9.3.9.1-3 for FR1 and in Table 9.3.9.1-4 for FR2.

The value of TL1-RSRP_Measurement_Period_SSB_inter_withoutGap is defined for FR1 in Table 9.15.6.1.1-1.  The value of TL1-RSRP_Measurement_Period_SSB_inter_withoutGap  is defined for FR2 in Table 9.15.6.1.1-2, where

-	M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise

-	N= 8.

-	P value for SSB resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

For a window W of duration max(TL1,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as serving cell, and starting at the beginning of any SSB resource occasion:

-	Ntotal is the total number of SSB resource occasions within the window, including those overlapped with measurement gap occasions or SMTC occasions within the window, and

-	Noutside_MG is the number of SSB resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of SSB resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	TL1 is periodicity of the target SSB.

-	Psharing factor = 1, if the SSB configured for L1-RSRP measurement outside measurement gap is

-	not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

-	not overlapped with the RSSI symbols indicated by ss-RSSI-Measurement and 1data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured,

-	Psharing factor = 3, otherwise.

-	PL1_sharing is defined as

-	When number of neighboring cells to be measured is 1

-	PL1_sharing = 2, if any symbol of the SSBs from serving cell and neighbor cell are overlapping or adjacent (in time domain)

-	PL1_sharing = 1, otherwise

-	When number of neighboring cells to be measured is more than 1

-	When TCI state of any intra-frequency or inter-frequency without gap neighbor cells are not in the active TCI state list

-	PL1_sharing = 3*NNeighbor_Cell, where NNeighbor_Cell is the number of neighbor cells to measure on intra-frequency and inter-frequency without gap

-	Otherwise

-	PL1_sharing = 3*NNeighbor_Cell_in_list, where NNeighbor_Cell_in_list  is the number of neigbour cells (including intra-frequency neighbor cells and inter-frequency without gap neighbor cells) whose TCI state(s) are in the active TCI state list.

If the high layer in TS 38.331 [2] signaling of smtc2 is configured, for cells indicated in the pci-List parameter in smtc2, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1.

Longer measurement period would be expected if the combination of SSB, SMTC occasion and measurement gap configurations does not meet previous conditions.

For either an FR1 or FR2 cell, longer measurement period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

For either an FR1 or FR2 cell, longer L1 RSRP measurement period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

Table 9.15.6.1.1-1: Measurement delay for inter frequency L1-RSRP measurement without measurement gaps in FR1 for UE capable of [capability of measurement with RTD>CP]

| Configuration | TL1-RSRP_Measurement_Period_SSB_inter_withoutGap (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB_NBC*NLayer) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB_NBC)*NLayer) |
| DRX cycle > 320ms | ceil(M*P)*TDRX*NLayer |
| Note 1: TSSB_NBC is the periodicity of the neighbor cell SSB-Index configured for inter-frequency L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NLayer = the number of inter-frequency layers which are configured for L1-RSRP measurement with [LTM-CSI-ResourceConfig-r18] and measured without measurement gaps + the number of intra-frequency layers configured for L1-RSRP measurement with [LTM-CSI-ResourceConfig-r18]. Note 2: K = 1.5. Note 3: void |  |

Table 9.15.6.1.1-2: Measurement delay for inter frequency L1-RSRP measurement without measurement gaps in FR2

| Configuration | TL1-RSRP_Measurement_Period_SSB_inter_withoutGap (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*PL1_sharing*N)*TSSB_NBC) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*PL1_sharing *N)*max(TDRX,TSSB_NBC) ) |
| DRX cycle > 320ms | ceil(1.5*M*P*PL1_sharing*N)*TDRX |
| Note 1: TSSB_NBC is the periodicity of the neighbor cell SSB-Index configured for inter-frequency L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: No requirements, if the actual RTD of serving cell and neighbor cell is larger than CP, for UE incapable of [capability of measurement with RTD>CP]. Note3: When the number of neighbor cells to be measured on intra-frequency and inter-frequency without gap is more than 1 and TCI state(s) of one of the neighbor cells is in the active TCI state, no requirements for other intra-frequency and inter-frequency without gap neighbor cells except the one whose TCI state(s) is in the active TCI state. |  |

Table 9.15.6.1.1-3: Measurement delay for inter frequency L1-RSRP measurement without measurement gaps in FR1 for UE incapable of [capability of measurement with RTD>CP]

| Configuration | TL1-RSRP_Measurement_Period_SSB_intra (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB_NBC) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB_NBC)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TSSB_NBC is the periodicity of the neighbor cell SSB-Index configured for inter-frequency L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1.5. Note 3: No requirements if the actual RTD of serving cell and neighbor cell is larger than CP. |  |

9.15.6.2	Measurement restriction for inter frequency L1-RSRP measurement

Measurement restrictions described in the following clauses apply when UE is performing L1-RSRP measurement on neighbor cell(s) without measurement gap.

Unless explicitly stated, the SSB to be measured for L1-RSRP measurement is transmitted from neigbor cell(s).

Editor’s Note: The section may be further updated based any newly achieved agreements.

9.15.6.2.1	Measurement restriction for SSB based L1-RSRP

For FR1,

when the SSB for L1-RSRP measurement fully or partially overlaps with the OFDM symbol

as SSB transmitted from serving cell(s) for RLM, BFD, CBD or L1-RSRP measurement,

- UE shall be able to measure the SSB for L1-RSRP measurement without any restriction;

when the SSB for L1-RSRP measurement fully or partially overlapps with the OFDM symbol

as CSI-RS transmitted from serving cell(s) for RLM, BFD, CBD or L1-RSRP measurement,

-	If SSB and CSI-RS have same SCS, UE shall be able to measure the SSB for L1-RSRP measurement without any restriction;

-	If SSB and CSI-RS have different SCS,

-	If UE supports simultaneousRxDataSSB-DiffNumerology, UE shall be able to measure the SSB for L1-RSRP measurement without any restriction;

-	If UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both SSB for L1-RSRP measurement and CSI-RS. Longer measurement period for SSB based L1-RSRP measurement is expected, and no requirements are defined.

For FR2, for UE incapable of [capability of measurement with RTD>CP] and for UE capable of [capability of measurement with RTD>CP],

when the SSB for L1-RSRP measurement on one CC fully or partially overlapps with the OFDM symbol as SSB transmitted from serving cell(s) for RLM, BFD, or CBD measurement on the same CC or different CCs in the same band, UE is required to measure one of but not both SSBs. Longer measurement period for SSB based RLM is expected, and no requirements are defined.

when the SSB for L1-RSRP measurement on one CC fully or partially overlaps with the OFDM symbol as CSI-RS transmitted from serving cell(s) for RLM, BFD, or CBD measurement on the same CC or different CCs in the same band, UE is required to measure one of but not both SSB for L1-RSRP measurement and CSI-RS. Longer measurement period for SSB based L1-RSRP measurement is expected, and no requirements are defined.

For FR2, if the network configures same or mixed numerology between SSB for L1-RSRP measurement on one FR2 band and CSI-RS for RLM, BFD, CBD, L1-RSRP or L1-SINR measurement on the other FR2 band, UE shall be able to perform the related SSB based measurements in one band without any measurement restrictions in the other band, provided that UE is capable of independent beam management on this FR2 band pair.]

9.15.6.3	Scheduling availability of UE during inter-frequency L1-RSRP measurements

Scheduling availability restrictions described in the following clauses apply when UE is performing L1-RSRP measurement on inter-frequency cell(s), and UE is receiving PDCCH/PDSCH from serving cell.

Unless explicitly stated, the SSB to be measured for L1-RSRP measurement is transmitted from neighbor cell(s).

9.15.6.3.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1

There are no scheduling restrictions due to L1-RSRP measurement performed on SSB as RS for L1-RSRP measurement with the same SCS as PDSCH/PDCCH in FR1.

9.15.6.3.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1

For UEs which support simultaneousRxDataSSB-DiffNumerology [14] there are no restrictions on scheduling availability due to L1-RSRP measurement based on SSB as RS for L1-RSRP measurement. For UEs which do not support simultaneousRxDataSSB-DiffNumerology [14] the following restrictions apply due to L1-RSRP measurement based on SSB configured for L1-RSRP measurement.

-	The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on the concerned OFDM symbols, where the concern OFDM symbols are

-	the same and 1 OFDM symbol before or after the OFDM symbols corresponding to the SSB indexes configured for L1-RSRP measurement, if UE supports [capability of measurement with RTD>CP],

-	the same OFDM symbols corresponding to the SSB indexes configured for L1-RSRP measurement, if UE does not support [capability of measurement with RTD>CP],

When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions apply to cell(s) in the same band on the symbols that fully or partially overlap with restricted symbols. When inter-band carrier aggregation within FR1 is configured, there are no scheduling restrictions on FR1 cells configured in other bands than the bands in which the cell where L1-RSRP measurement is performed is configured.

9.15.6.3.3	Scheduling availability of UE performing L1-RSRP measurement on FR2

The following scheduling restriction applies due to L1-RSRP measurement.

-	The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on the concerned OFDM symbols, where the concern OFDM symbols are

the same and 1 OFDM symbol before or after the OFDM symbols corresponding to the SSB indexes configured for L1-RSRP measurement.

When intra-band carrier aggregation in FR2 is performed, the scheduling restrictions is performed apply to cell(s) in the band on the symbols that fully or partially overlap with restricted symbols.

When inter-band carrier aggregation in FR2 is performed, there are no scheduling restrictions on FR2 cells in the bands due to L1-RSRP measurement performed on FR2 cell(s) in different band(s), provided that UE is capable of independent beam management on this FR2 band pair. Additionally, there is no scheduling restriction if the UE is configured with different numerology between SSB on one FR2 band and data on the other FR2 band provided the UE is configured for IBM operation for the band pair.

If following conditions are met,

-	UE has been notified about system information update through paging,

-	The gap between UE’s reception of PDCCH that UE monitors in the Type 2-PDCCH CSS set and that notifies system information update, and the PDCCH that UE monitors in the Type0-PDCCH CSS set, is greater than 2 slots,

For the SSB and CORESET for RMSI scheduling multiplexing patterns 3, UE is expected to receive the PDCCH that UE monitors in the Type0-PDCCH CSS set, and the corresponding PDSCH, on SSB symbols to be measured for L1-RSRP measurement; and

For the SSB and CORESET for RMSI scheduling multiplexing patterns 2, UE is expected to receive PDSCH that corresponds to the PDCCH that UE monitors in the Type0-PDCCH CSS set, on SSB symbols to be measured for L1-RSRP measurement.

9.15.6.3.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA

There are no scheduling restrictions on FR1 cell(s) due to L1-RSRP measurement performed on FR2 cell(s).

There are no scheduling restrictions on FR2 cell(s) due to L1-RSRP measurement performed on FR1 cell(s).

9.15.6.3.5	Scheduling availability of UE performing L1-RSRP measurement in TDD bands on FR1

When UE performs L1-RSRP measurement on neighbor cell in a TDD band, the following restrictions apply due to L1-RSRP measurement

-	The UE is not expected to transmit PUCCH/PUSCH/SRS on the concerned OFDM symbols and restricted symbols may partially or fully overlap with UL symbols, where the concern OFDM symbols are

-	the same and 1 OFDM symbol before or after the OFDM symbols corresponding to the SSB indexes configured for L1-RSRP measurement, if UE supports [capability of measurement with RTD>CP],

-	the same OFDM symbols corresponding to the SSB indexes configured for L1-RSRP measurement, if UE does not support [capability of measurement with RTD>CP].

<< End of change 15>>