3GPP TSG-RAN WG4 Meeting #110 	R4-2401937

# Athens, GR, 26 Feb – 01 Mar, 2024

Source: 	vivo

Title: 	Discussion on performance requirements for R18 LTM

Agenda Item:	8.16.2.1

Document for:	Discussion

Introduction

In RAN4 #109, the WF [1] is approved.

Based on all above information, we provide our views on performance requirements and test cases for R18 LTM.

Discussion

<On L1 measurement accuracy requirements>

As discussed in [2], for R18 LTM, generally there are two types of UE measurement behaviour. One type is that UE only supports RTD<=CP, while UE may perform L1 measurements within active BWP on multiple layers simultaneously, as long as the RTD of cells to be measured on each layer is less than CP and the MRTD of multiple layers meet the restrictions defined in TS 38.133. For this type of UE behaviour, UE needs to identify the Rx timing for the cells to be measured based on L3 measurements. Based on L3 measurement UE may identify the timing reference cell for L1 measurements. In case the RTD between a cell, namely cell A, and the reference cell is larger than CP, then UE may skip the L1 measurement on cell A. In this case, the UE can ensure the accuracy of the measurement results it reports.

Proposal 1  For UE not supporting RTD>CP, all reported L1 measurements results shall follow legacy L1 measurement accuracy requirements, provided:

- UE shall sense the actual RTD between DL timing reference and any cell/SSB to be measured based on L3 measurements.

- If the actual RTD between the DL timing reference cell and a cell to measure is larger than CP, UE shall skip the L1 measurement on this cell.

- UE shall only report the L1 measurement results of the cells that the actual RTD to Rx timing reference is less than CP.

Another issue to be considered is the side conditions for LTM L1 measurements. Since L1 measurements are based on the SSB periodicity on an SSB frequency layer, rather than SMTC periodicity, in our view UE may not exactly know the instant interference level on each measurement occasion. Therefore, the Iot considered for L1 measurements shall count all the contributor in the measurement bandwidth, including power level from neighbour cell SSB. And the side condition shall be the same as legacy L1 measurement, i.e. -3dB.

Proposal 2  Side conditions in intra-frequency and inter-frequency L1-RSRP measurement accuracy requirements is Es/Iot =-3dB, while the Iot shall count all the contributor in the measurement bandwidth, including power level from SSB of other cells.

<On test case coverage for R18 LTM>

In [1], a table is provided by the Rapporteur for the test case coverage. We generally agree with the view. The only two comments would be:

1. Test cases for UE-based TA shall be introduced if RAN4 agrees to specify requirements for UE-based TA, at least for FR1.

2. The neighbour cell TCI shall also be activated in FR1 intra-f L1-RSRP test cases for RTD < CP

Proposal 3  Test cases for UE-based TA shall be introduced if RAN4 agrees to specify RRM requirements for UE-based TA.

Proposal 4  The neighbour cell TCI shall also be activated in FR1 intra-f L1-RSRP test cases for RTD < CP.

Conclusions

Based on above analysis, we have following observations and proposals.

Proposal 1  For UE not supporting RTD>CP, all reported L1 measurements results shall follow legacy L1 measurement accuracy requirements, provided:

- UE shall sense the actual RTD between DL timing reference and any cell/SSB to be measured based on L3 measurements.

- If the actual RTD between the DL timing reference cell and a cell to measure is larger than CP, UE shall skip the L1 measurement on this cell.

- UE shall only report the L1 measurement results of the cells that the actual RTD to Rx timing reference is less than CP.

Proposal 2  Side conditions in intra-frequency and inter-frequency L1-RSRP measurement accuracy requirements is Es/Iot =-3dB, while the Iot shall count all the contributor in the measurement bandwidth, including power level from SSB of other cells.

Proposal 3  Test cases for UE-based TA shall be introduced if RAN4 agrees to specify RRM requirements for UE-based TA.

Proposal 4  The neighbour cell TCI shall also be activated in FR1 intra-f L1-RSRP test cases for RTD < CP.

- References

[1]  R4-2321399  WF on RRM performance requirements of R18 Further NR mobility enhancement, Apple.  RAN4 #109

[2]  R4-2401936  Discussion on RAN4 UE features for R18 LTM, vivo.  RAN4 #110