# 3GPP TSG-RAN WG4 Meeting # 110                                   R4-2402037

# Athens, Greece, 26 February – 1 March, 2024

# Title: 	Simulation results for UE demodulation performance and CSI requirements for less than 5MHz

# Source: 	ZTE Corporation

# Agenda item:	8.10.6.1

# Document for:	Discussion

# 1 Introduction

This contribution contains initial simulation results for the <5MHz WID [1]. The results are provided to aid in determining where new requirements can/should be introduced[2].

# 2 Discussion

## 2.1 PBCH requirements

The simulation results for punctured PBCH were provided, in accordance with the conclusions of the previous meeting.

| Duplex | Bandwidth (MHz) / Subcarrier spacing (kHz) | SSB/PBCH index | Propagation condition | Antenna configuration and correlation matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-bch (%) | SNR (dB) |
| FDD | 3 / 15 | Unknown | TDLC300-100 | 1 x 2 Low | 1 | -6.1 |
| FDD | 3 / 15 | Unknown | TDLC300-100 | 1 x 4 Low | 1 | -9.2 |

## 2.1 PDSCH requirements

Firstly, we compared the performance between 15PRBs and 52PRBs. From our simulation results show no significant difference in performance between 3MHz and legacy requirements(e.g. 10MHz). Additional, we provided the PDSCH simulation results for 3MHz under non- HST conditions.

Observation 1. Our simulation results shown no significant difference in performance between 3Mhz and legacy requirements.

Table 1. PDSCH simulation results for 52PRB and 15PRB

| Test num. | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
|  |  |  |  |  | 52PRBs | 15PRBs |
| 1 | QPSK, 0.30 | TDLB100-400 | 2x2, ULA Low | 70 | -3.2 | -3.0 |
| 2 | 16QAM,0.48 | TDLC300-100 | 2x2, ULA Low | 70 | 5.4 | 5.1 |
| 3 | 256QAM, 0.82 | TDLA30-10 | 2x2, ULA Low | 70 | 20.7 | 20.5 |

Table 2. PDSCH simulation results for 3MHz in non-HST conditions

| Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Rank | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| R.PDSCH.1-1.1 FDD | 3 / 15 | QPSK, 0.30 | TDLB100-400 | Rank 1 | 2x2, ULA Low | 70 | -3.0 |
|  |  |  |  |  | 2x4, ULA Low | 70 | -6.1 |
| R.PDSCH.1-2.1 FDD | 3 / 15 | 16QAM, 0.48 | TDLC300-100 | Rank 1 | 2x2, ULA Low | 70 | 5.1 |
|  |  |  |  |  | 2x4, ULA Low | 70 | 2.0 |

# 3 Conclusion

In this contribution, we give some discussions on demodulation performance requirements for less than 5MHz demodulation requirements, The conclusions are:

Observation 1. Our simulation results shown no significant difference in performance between 3Mhz and legacy requirements.

# 4 References

- RP-222645, Revised WID: NR support for dedicated spectrum less than 5MHz for FR1, RAN#95e.

- R4-2321064 .WF on NR_FR1_lessthan_5MHz_BW_demod, Nokia, Nokia Shanghai Bell.