# 3GPP TSG- RAN WG4 Meeting # 110	R4-2402049

# Athens, Greece, 26 February – 1 March, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.115-1 | CR | - | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TS38.115-1 on Fixed Reference Channels for NCR-MT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_netcon_repeater-Perf |  |  |  |  |  | Date: |  |  | 2024-02-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Add Fixed Reference Channels of NCR-MT for TS 38.115-1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add Fixed Reference Channels for TS 38.115-1, which include PDSCH, PDCCH, CSI performance parts. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Fixed Reference Channels for TS 38.115-1 are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex F |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< START OF CHANGE>

Annex F (normative):
NCR-MT Reference measurement channels

F.7	NCR-MT Fixed Reference Channels

F.7.1	Fixed Reference Channels for PDSCH performance requirements

#### F.7.1.1	FDD

The parameters for the reference measurement channels are specified in table F.7.1.1.1-1 and table F.7.1.1.2-1 for FR1 FDD 15kHz PDSCH performance requirements.

##### F.7.1.1.1	Fixed Reference Channels for PDSCH performance requirements (QPSK)

Table F.7.1.1.1-1: FRC parameters for FR1 PDSCH performance requirements, 1 transmission layers, QPSK

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | M-FR1-F.7.1.1.1-1 |
| Channel bandwidth | MHz | 10 |
| Subcarrier spacing | kHz | 15 |
| Allocated resource blocks | PRBs | 52 |
| Number of consecutive PDSCH symbols |  | 12 |
| MCS table |  | 64QAM |
| MCS index |  | 4 |
| Modulation |  | QPSK |
| Target Coding Rate |  | 0.3 |
| Number of MIMO layers |  | 1 |
| Number of DMRS REs |  | 12 |
| Overhead for TBS determination |  | 0 |
| Information Bit Payload per Slot |  | 4096 |
| Transport block CRC per Slot |  | 24 |
| Number of Code Blocks per Slot |  | 1 |
| Binary Channel Bits Per Slot |  | 13728 |

##### F.7.1.1.2	Fixed Reference Channels for PDSCH performance requirements (16QAM)

Table F.7.1.1.2-1: FRC parameters for FR1 PDSCH performance requirements, 1 transmission layers, 16QAM

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | M-FR1-F.7.1.1.2-1 |
| Channel bandwidth | MHz | 10 |
| Subcarrier spacing | kHz | 15 |
| Allocated resource blocks | PRBs | 52 |
| Number of consecutive PDSCH symbols |  | 12 |
| MCS table |  | 64QAM |
| MCS index |  | 13 |
| Modulation |  | 16QAM |
| Target Coding Rate |  | 0.48 |
| Number of MIMO layers |  | 1 |
| Number of DMRS REs |  | 12 |
| Overhead for TBS determination |  | 0 |
| Information Bit Payload per Slot |  | 13064 |
| Transport block CRC per Slot |  | 24 |
| Number of Code Blocks per Slot |  | 2 |
| Binary Channel Bits Per Slot |  | 27456 |

#### F.7.1.2	TDD

The parameters for the reference measurement channels are specified in table F.7.1.2.1-1and table F.7.1.2.2-1 for FR1 TDD 30kHz PDSCH performance requirements.

##### F.7.1.2.1	Fixed Reference Channels for PDSCH performance requirements (QPSK)

Table F.7.1.2.1-1: FRC parameters for FR1 PDSCH performance requirements, 1 transmission layers, QPSK

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | M-FR1-F.7.1.2.1-1 |
| Channel bandwidth | MHz | 40 |
| Subcarrier spacing | kHz | 30 |
| Allocated resource blocks | PRBs | 106 |
| Number of consecutive PDSCH symbols |  | 12 |
| MCS table |  | 64QAM |
| MCS index |  | 4 |
| Modulation |  | QPSK |
| Target Coding Rate |  | 0.3 |
| Number of MIMO layers |  | 1 |
| Number of DMRS REs |  | 12 |
| Overhead for TBS determination |  | 0 |
| Information Bit Payload per Slot |  | 8456 |
| Transport block CRC per Slot |  | 24 |
| Number of Code Blocks per Slot |  | 2 |
| Binary Channel Bits Per Slot |  | 27984 |

##### F.7.1.2.2	Fixed Reference Channels for PDSCH performance requirements (16QAM)

Table F.7.1.2.2-1: FRC parameters for FR1 PDSCH performance requirements, 1 transmission layers, 16QAM

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | M-FR1-F.7.1.2.2-1 |
| Channel bandwidth | MHz | 40 |
| Subcarrier spacing | kHz | 30 |
| Allocated resource blocks | PRBs | 106 |
| Number of consecutive PDSCH symbols |  | 12 |
| MCS table |  | 64QAM |
| MCS index |  | 13 |
| Modulation |  | 16QAM |
| Target Coding Rate |  | 0.48 |
| Number of MIMO layers |  | 1 |
| Number of DMRS REs |  | 12 |
| Overhead for TBS determination |  | 0 |
| Information Bit Payload per Slot |  | 26632 |
| Transport block CRC per Slot |  | 24 |
| Number of Code Blocks per Slot |  | 4 |
| Binary Channel Bits Per Slot |  | 55968 |

F.7.2	Fixed Reference Channels for PDCCH performance requirements

#### F.7.2.1	FDD

The parameters for the reference measurement channels are specified in table F.7.2.1-1 for FR1 FDD 15kHz PDCCH performance requirements.

Table F.7.2.1-1 . FR1 PDCCH Reference Channels

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | M-FR1-F.7.2.1-1 | M-FR1-F.7.2.1-2 | M-FR1-F.7.2.1-3 |
| Subcarrier spacing | kHz | 15 | 15 | 15 |
| CORESET frequency domain allocation |  | 24 | 48 | 48 |
| CORESET time domain allocation |  | 2 | 2 | 1 |
| Aggregation level |  | 2 | 4 | 8 |
| DCI Format |  | 1_0 | 1_1 | 1_1 |
| Payload (without CRC) | Bits | 39 | 52 | 52 |

#### F.7.2.2	TDD

The parameters for the reference measurement channels are specified in table F.7.2.2-1 for FR1 TDD 30kHz PDCCH performance requirements.

Table F.7.2.2-1 . FR1 PDCCH Reference Channels

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | M-FR1-F.7.2.2-1 | M-FR1-F.7.2.2-2 | M-FR1-F.7.2.2-3 |
| Subcarrier spacing | kHz | 30 | 30 | 30 |
| CORESET frequency domain allocation |  | 102 | 102 | 102 |
| CORESET time domain allocation |  | 1 | 1 | 1 |
| Aggregation level |  | 2 | 4 | 8 |
| DCI Format |  | 1_0 | 1_1 | 1_1 |
| Payload (without CRC) | Bits | 41 | 53 | 53 |

F.7.3	Fixed Reference Channels for CSI reporting performance

This clause defines the DL signal applicable to the reporting of channel status information.

Tables in this clause specifies the mapping of CQI index to Information Bit payload, which complies with the CQI definition specified in clause 5.2.2.1 of TS 38.214 [11] and with MCS definition specified in clause 5.1.3 of TS 38.214 [11].

Table F.7.3-1.  Fixed Reference Channels for FR1 CSI reporting with CQI table 2

| Reference channel |  |  |  | M-FR1-A.7.3-1 | M-FR1-A.7.3-2 |
| --- | --- | --- | --- | --- | --- |
| Number of allocated PDSCH resource blocks |  |  |  | 52 | 106 |
| Number of consecutive PDSCH symbols |  |  |  | 12 | 12 |
| Number of PDSCH MIMO layers |  |  |  | 1 | 1 |
| Number of DMRS REs (Note 1) |  |  |  | 24 | 24 |
| Overhead for TBS determination |  |  |  | 0 | 0 |
| Available RE-s for PDSCH |  |  |  | 6240 | 12720 |
| CQI index | Spectral efficiency | MCS index | Modulation | Information Bit Payload per Slot |  |
| 0 | OOR | OOR | OOR |  | N/A |
| 1 | 0.1523 | 0 | QPSK | 1480 | 2976 |
| 2 | 0.3770 | 1 |  | 2408 | 4744 |
| 3 | 0.8770 | 3 |  | 5504 | 11016 |
| 4 | 1.4766 | 5 | 16QAM | 9224 | 18960 |
| 5 | 1.9141 | 7 |  | 12040 | 24576 |
| 6 | 2.4063 | 9 |  | 15112 | 30728 |
| 7 | 2.7305 | 11 | 64QAM | 16896 | 34816 |
| 8 | 3.3223 | 13 |  | 20496 | 42016 |
| 9 | 3.9023 | 15 |  | 24576 | 49176 |
| 10 | 4.5234 | 17 |  | 28168 | 57376 |
| 11 | 5.1152 | 19 |  | 31752 | 65576 |
| 12 | 5.5547 | 21 | 256QAM | 34816 | 69672 |
| 13 | 6.2266 | 23 |  | 38936 | 79896 |
| 14 | 6.9141 | 25 |  | 43032 | 88064 |
| 15 | 7.4063 | 27 |  | 46104 | 94248 |

< End OF CHANGE>