3GPP TSG-RAN WG4 Meeting #110	R4-2402072

Athens, Greece, February 19 – March 01, 2024

Source:	Nokia

Title:	Discussion on various correction to MSD values and definitions

Agenda item:	7.1.1.1

Document for:	Discussion and Approval

Introduction

At previous meetings a number of suggested corrections have been proposed. Some of these have not yet been agreed since additional justification have been requested. This Tdoc presents needed justification.

Further, since the last meeting additional errors and point for clarification have been identified. This Tdoc discussed these and provide a suggestion for corrections.

Discussion

Correction to CA_n48-n96 harmonic mixing

A correction CR was presented in RAN4#109 with changes to existing CA combinations with errors [1]. However, the correction CR to CA_n48-n96 was rejected as it was requested to present further justification for the change of the mistake in 38.101-1 of Table 7.3A.4-4. This is presented here, where it is important to note that this is a TDD-TDD combinations with mandatory capability signalling of simultaneous Rx/Tx and it is specified with 2UL as well.

It was requested at RAN4#109 to provide analysis and justification why the harmonic mixing entry in TS 38.101-1 Table 7.3A.4-4 for CA_n48-n96 should be corrected.

Analysis for CA_n48-n96

Table 5.x.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_ n48-n96.

Table 5.x.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n48 | 3550 | 3700 | 3550 | 3700 | 7100 | 7400 | 10650 | 11100 | 14200 | 14800 | 17750 | 18500 |
| n96 | 5925 | 7125 | 5925 | 7125 | 11850 | 14250 | 17775 | 21375 | 23700 | 28500 | 29625 | 35625 |

Table 5.x.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n48 | 3550 | 3700 | 3550 | 3700 | 7100 | 7400 | 10650 | 11100 | 14200 | 14800 | 17750 | 18500 |
| n96 | 5925 | 7125 | 5925 | 7125 | 11850 | 14250 | 17775 | 21375 | 23700 | 28500 | 29625 | 35625 |

As can be seen in the co-existence studies in 5.x.1.3 there are occurrences of uplink harmonic in which the 2nd order uplink of n48 hits the downlink of n96. This has been neglected in 38.101-1 since n96 is an unlicensed band.

It is also seen that the UL1 of n96 hits the downlink DL2 of n48, meaning that a harmonic mixing case of UL1/DL2 must be captured.

Proposed correction for CA_n48-n96

The harmonic issue identified in the analysis also presented in the previous section has wrongly been filed into TS 38.101-1 Table 7.3A.4-4 as captured below.

Table 7.3A.4-4: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n96 | n48 | 5 | 15 | 25 (RBstart=0) | 5 | 5.8 | NOTE 2 | UL2/DL3 |
| n96 | n48 | 20 | 15 | 100 (RBstart=0) | 100 | 0.5 | NOTE 2 | UL2/DL3 |

The corrected entry is shown below:

| n96 | n48 | 5 | 15 | 25 (RBstart=0) | 5 | 5.8 | NOTE 2 | UL1/DL2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n96 | n48 | 20 | 15 | 100 (RBstart=0) | 100 | 0.5 | NOTE 2 | UL1/DL2 |

A draftCR [4] corresponding to the shown correction above have been provided for the Rel-18 basket bigCR.

Agree the corrections for CA_n48-n96 harmonic mixing in the provided draftCR

Since the error exist in previous releases this should also be corrected.

Correction to IMD test points

DC_2A-66A_n77A and DC_2A-n66A_n77A IMD test points

The use of Note 11, referencing the restricted operation of n77 to 3450 – 3550 MHz and 3700 – 3980 MHz and 3450 – 3650 MHz and 3650 – 3980 MHz in Canada, also makes the IMD4 test invalid, since there’s no IMD4 and the n77 test frequency of 3385MHz is out of range. When n77 stops at 3980MHz, it is the IMD2 range that would produce an issue, so the section shown in Table 1 is invalid. Instead, the section can cover the IMD2, which is a result of the restricted n77 operation.

Some of the IMD test points for DC_2A-66A_n77A and DC_2A-n66A_n77A are invalid.

Table 1: Extract of DC using note 11.

| DC_2A-66A_n77A11 DC_2A-66A_n77C11 DC_2A-66A_n77(2A)11 DC_2A-2A-66A_n77A11 DC_2A-2A-66A_n77C11 DC_2A-66A-66A_n77A11 DC_2A-66A-66A_n77C11 DC_2A-2A-66A-66A_n77A11 DC_2A-2A-66A-66A_n77C11 | 2 | N/A | 5 | N/A | 1940 | 9.1 | IMD4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 66 | 1775 | 5 | 25 | 2195 | N/A | N/A |
|  | n77 | 3385 | 5 | 25 | 3385 | N/A | N/A |

Proposal to add IMD2 to requirement section of Note 11 usage of 32.1dB, same as section wo. Note 11 including new test frequency points

Additionally for DC_2A_n66A-n77A with Note 11 the use of Note 4 is proposed to indicate that there’s also IMD5 in this combination.

Proposal to add Note 4 on DC_2A_n66A-n77A11

DC_2A-66A_n78A / DC_2A-n66A_n78A IMD test points

The two cases have been split and similar corrections to IMD and test points to match DC_2-66/_n66_n77 has been applied. Corrections cover that DC_2A_n66A-n78 cannot have IMD2 in band 2A, since there no 2 NR uplinks simultaneously.

DC_2A_n66A-n78 cannot have IMD2 in band 2A.

Proposal to group DC_2A-66A_n78A and DC_2A_n66A-n78A separately and correct IMD2 case in 2A when there’s 2 NR bands

DC_ DC_66A_n2A-n77A IMD test points

Related to the other combinations of this discussion the combination DC_66A_n2A-n77A has missed the IMD2 into n77A.

Proposal to add IMD2 into n77 for DC_66A_n2A-n77A with MSD values from DC_2A_n66A-n77A

DraftCRs [7] and [8] corresponding to the shown correction in section 2.2 have been provided for the Rel-18 baskets bigCRs..

Agree the corrections for IMD test points in the provided draftCRs

Since the error exist in previous releases this should also be corrected.

Missing IMD2 and IMD4 for CA_n25-n66-n78

Comparing the band combination CA_n2-n66-n77, CA_n25A-n66A-n77A and CA_n25A-n66A-n78A it is found that many of the IMD tests have been omitted and there is inconsistency between PC3 and PC2 tables. Checking the IMD analysis the omission of the IMD tests are not simply because n78 has a smaller bandwidth and that IMD4 and IMD5 tests are included in CA_n2-n66-n77 and CA_n25-n66-n77. This discussion is for agreeing to update CA_n25A-n66A-n78A with the missing IMD tests and aligning the method for including IMD4 and IMD5.

Analysis for CA_n25-n66-n78

Table 2 lists Band n25 + Band n66 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE coexistence analysis.

Table 3 lists Band n25 + Band n78 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE coexistence analysis.

Table 4 lists Band n66 + Band n78 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE coexistence analysis.

Table 2: CA_n25-n66 into n78

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 1850 | 1915 | 1710 | 1780 |
| DL Frequency [MHz] | 1930 | 1995 | 2110 | 2200 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 205 | 70 | 3560 | 3695 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1920 | 2120 | 1505 | 1710 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5410 | 5610 | 5270 | 5475 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3770 | 4035 | 3215 | 3490 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 140 | 410 | 7120 | 7390 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7260 | 7525 | 6980 | 7255 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5270 | 4925 | 5950 | 5620 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1640 | 1300 | 2325 | 1990 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8690 | 9035 | 9110 | 9440 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8830 | 9170 | 8970 | 9305 |

Table 3: CA_n25-n78 into n66

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 1850 | 1915 | 3300 | 3800 |
| DL Frequency [MHz] | 1930 | 1995 | 3300 | 3800 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1385 | 1950 | 5150 | 5715 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 100 | 530 | 4685 | 5750 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7000 | 7630 | 8450 | 9515 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1750 | 2445 | 7985 | 9550 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3900 | 2770 | 10300 | 11430 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8850 | 9545 | 11750 | 13315 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13350 | 11285 | 4360 | 3600 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 7700 | 6070 | 855 | 2050 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15050 | 17115 | 10700 | 11460 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13600 | 15230 | 12150 | 13345 |

Table 4: CA_n66-n78 into n25

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 1710 | 1780 | 3300 | 3800 |
| DL Frequency [MHz] | 2110 | 2200 | 3300 | 3800 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1520 | 2090 | 5010 | 5580 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 380 | 260 | 4820 | 5890 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6720 | 7360 | 8310 | 9380 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1330 | 2040 | 8120 | 9690 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 4180 | 3040 | 10020 | 11160 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8430 | 9140 | 11610 | 13180 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13490 | 11420 | 3820 | 3040 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 7980 | 6340 | 1260 | 2470 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14910 | 16980 | 10140 | 10920 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13320 | 14960 | 11730 | 12940 |

- Observation 4: Based on the tables above it can be seen that:

- n25 + n66 IMD2 and IMD4 may affect Rx frequencies of band n78. This test is missing in existing 38.101-1, re-use CA_n25-n66-n77 MSD values.

- n25 + n78 IMD4 may affect Rx frequencies of band n66. This test has been missed. Re-use CA_n25-n66-n77 MSD values.

- n66 + n78 IMD2, IMD4 and IMD5 may affect Rx frequencies of band n25. These tests have been missed. Re-use CA_n25-n66-n77 MSD values.

Proposal to add missing IMD2 and IMD4 cases to CA_n25-n66-n78

A draftCR [6] corresponding to the shown correction above have been provided for the Rel-18 basket bigCR.

Agree the corrections for CA_n25-n66-n78 in the provided draftCR

Since the error exist in previous releases this should also be corrected.

Correction to CA_n3-n7-n8 use of note 11

It has been noted that there is only the entry of CA_n3-n7-n8 in 38.101-1 that uses a summation of IMD. This is the only entry having an MSD definition of IMD2+IMD3 with the single use of note 11, that explains IMD3 can be disregard.

NOTE 11: This MSD requirement apply with both IMD2 and IMD3 products should be generated.

The only combination using a summation of IMD is CA_n3-n7-n8 in 38.101-1.

There is already an entry stating that IMD3 can be disregarded. This is note 4:

NOTE 4:	This band is subject to IMD3 also which MSD is not specified.

There is already another note in 38.101-1 which can be applied instead of Note 11.

Note 4 have been used among other places at the following combinations:

| Band combination | Source of IMD |
| --- | --- |
| CA_n3-n5-n7 | IMD24 |
| CA_n3-n7-n26 | IMD24 |
| CA_n3-n8-n41 | IMD24 |
| CA_n3-n8-n41 | IMD24 |
| CA_n3-n28-n41 | IMD24 |
| CA_n3-n40-n105 | IMD24 |
| CA_n5-n41-n66 | IMD24 |
| CA_n5-n41-n66 | IMD24 |
| CA_n8-n40-n78 | IMD24 |
| CA_n18-n41-n77 | IMD22,4 |
| CA_n18-n41-n77 | IMD24 |
| CA_n18-n41-n77 | IMD21,4 |
| CA_n25-n41-n85 | IMD24 |
| CA_n28-n41-n77 | IMD24 |
| CA_n28-n41-n77 | IMD24 |
| CA_n28-n78-n79 | IMD21,3,4 |
| CA_n28-n78-n79 | IMD23,4 |
| CA_n41-n77-n85 | IMD24.6 |
| CA_n41-n77-n85 | IMD24,6 |

It is worth noting that CA_n3-n8-n41 is among these combinations, since n41 has the same downlink frequencies as n7. For none of the cases that use note 4 has the practice been to write IMD2+IMD34.

CA_n3-n7-n8 implicitly already uses Note 4 since n41 has the same downlink frequencies as n7.

In the cases that define the IMD with notes stating that also IMD4 and IMD5 occurrences will impact the receive band, there are no similar combinations like IMD2+IMD4 and IMD3+IMD5. Therefore, we propose to remove the “+IMD3” that is used only in this single case. We propose to void note 11 and we propose to add note 4 to the combination CA_n3-n7-n8.

Use note 4 and delete IMD2+IMD3, replace with IMD2 only

Void note 11, that is only used in this one CA case

A draftCR [5] corresponding to the proposals above have been provided for the Rel-18 basket bigCR.

Agree the corrections for CA_n3-n7-n8 in the provided draftCR

Correction of MSD values

CA_n1-n77-n79

The two band combinations CA_n1-n77-n79 and CA_n1-n78-n79 are very similar in terms of MSD values, with the only difference being the exact frequencies in use in n77 versus the lesser frequency range of n78. Notice in the extract from 38.101-1 v18.4.0 that the exact same tests are created for MSD in n1, except that CA_n1-n77-n79 has a value of 6dB. This is a mistake and should follow the 15.6dB defined for the same case in CA_n1-n78-n79.

Table 7.3A.5-2: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n77-n79 | n1 | N/A | 5 | N/A | 2140 | 6.0 | FDD | IMD31,2 |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
|  | n79 | 4660 | 40 | 216 | 4660 | N/A | TDD | N/A |
| CA_n1-n78-n79 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n78 | 3410 | 10 | 50 | 3410 | N/A | TDD | N/A |
|  | n79 | N/A | 40 | N/A | 4870 | 15.9 | TDD | IMD31,3 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3490 | 4.6 | TDD | IMD53 |
|  | n79 | 4670 | 40 | 216 | 4670 | N/A | TDD | N/A |
|  | n1 | N/A | 5 | N/A | 2140 | 15.6 | FDD | IMD31,2 |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
|  | n79 | 4660 | 40 | 216 | 4660 | N/A | TDD | N/A |

Reuse the MSD of CA_n1-n78-n79 for CA_n1-n77-n79

CA_n3-n7-n28

This discussion explains an error in CA_n3-n7-n28 of R4-2111807.

The IMD2 value for n3 is wrong and should follow DC_3-n7-n28.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-7A_n28A DC_3A-7C_n28A DC_3C-7A_n28A DC_3C-7C_n28A | 3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |
| DC_3A-7A-7A_n28A | n28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | 7 | N/A | 10 | N/A | 2682 | 16.9 | IMD3 |
|  | 7 | 2543 | 10 | 50 | 2663 | N/A | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | 3 | N/A | 5 | N/A | 1832.5 | 26.0 | IMD2 |

Align MSD between DC_3-n7-n28 and CA_n3-n7-n28

The analysis is added in the follwoing for reference.

UE co-existence studies

The harmonic issues have been already analyzed in 3DL/1UL WI. For inter-modulation issues, the 2nd, 3rd, 4th and 5th order intermodulation products are calculated in Table 5.1.X.3-1, 5.1.X.3-2 and 5.1.X.3-3, respectively.

Table 5.1.X.3-1: IMD analysis for n3+n7

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 1710 | 1785 | 2500 | 2570 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 715 | 860 | 4210 | 4355 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | 850 | 1070 | 3215 | 3430 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 5920 | 6140 | 6710 | 6925 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 2560 | 2855 | 5715 | 6000 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 7630 | 7925 | 9210 | 9495 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | 1720 | 1430 | 8420 | 8710 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | 8570 | 8215 | 4640 | 4270 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 11710 | 12065 | 9340 | 9710 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | 4290 | 3930 | 355 | 10 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 10920 | 11280 | 10130 | 10495 |

Table 5.1.X.3-2: IMD analysis for n7+n28

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 703 | 748 | 2500 | 2570 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 1752 | 1867 | 3203 | 3318 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | 1164 | 1004 | 4252 | 4437 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 3906 | 4066 | 5703 | 5888 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 461 | 256 | 6752 | 7007 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 4609 | 4814 | 8203 | 8458 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | 3734 | 3504 | 6406 | 6636 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | 9577 | 9252 | 492 | 242 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 10703 | 11028 | 5312 | 5562 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | 6304 | 6004 | 2756 | 3031 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 8906 | 9206 | 7109 | 7384 |

Table 5.1.X.3-3: IMD analysis for n3+n28

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 703 | 748 | 1710 | 1785 |
| 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency limit (MHz) | 962 | 1082 | 2413 | 2533 |
| 3rd order IMD products | 2*f1_low – f2_high | 2*f1_high – f2_low | 2*f2_low – f1_high | 2*f2_high – f1_low |
| IMD frequency limit (MHz) | 379 | 214 | 2672 | 2867 |
| 3rd order IMD products | 2*f1_low + f2_low | 2*f1_high + f2_high | 2*f2_low + f1_low | 2*f2_high + f1_high |
| IMD frequency limit (MHz) | 3116 | 3281 | 4123 | 4318 |
| 4th order IMD products | 3*f1_low – f2_high | 3*f1_high – f2_low | 3*f2_low – f1_high | 3*f2_high – f1_low |
| IMD frequency limit (MHz) | 324 | 534 | 4382 | 4652 |
| 4th order IMD products | 3*f1_low + f2_low | 3*f1_high + f2_high | 3*f2_low + f1_low | 3*f2_high + f1_high |
| IMD frequency limit (MHz) | 3819 | 4029 | 5833 | 6103 |
| 4th order IMD products | 2*f1_low – 2*f2_high | 2*f1_high – 2*f2_low | 2*f1_low + 2*f2_low | 2*f1_high + 2*f2_high |
| IMD frequency limit (MHz) | 2164 | 1924 | 4826 | 5066 |
| 5th order IMD products | f1_low – 4*f2_high | f1_high – 4*f2_low | f2_low – 4*f1_high | f2_high – 4*f1_low |
| IMD frequency limit (MHz) | 6437 | 6092 | 1282 | 1027 |
| 5th order IMD products | f1_low + 4*f2_low | f1_high + 4*f2_high | f2_low + 4*f1_low | f2_high + 4*f1_high |
| IMD frequency limit (MHz) | 7543 | 7888 | 4522 | 4777 |
| 5th order IMD products | 2*f1_low – 3*f2_high | 2*f1_high - 3*f2_low | 2*f2_low – 3*f1_high | 2*f2_high – 3*f1_low |
| IMD frequency limit (MHz) | -3949 | -3634 | 1176 | 1461 |
| 5th order IMD products | 2*f1_low + 3*f2_low | 2*f1_high + 3*f2_high | 2*f2_low + 3*f1_low | 2*f2_high + 3*f1_high |
| IMD frequency limit (MHz) | 6536 | 6851 | 5529 | 5814 |

The above IMD studies show that

- The 2nd order IMD generated by dual uplink of n3+n7 may fall into own Rx of n28

- The 4th order IMD generated by dual uplink of n3+n7 may fall into own Rx of n7

- The 2nd order IMD generated by dual uplink of n7+n28 may fall into own Rx of n3

- The 3rd order IMD generated by dual uplink of n3+n28 may fall into own Rx of n7

The IMD issues specific to 3DL/2UL are the cases that IMDs generated by dual uplink fall into the third Rx band; otherwise, IMD issues are already specified in 2DL/2UL CAs.

As these 3DL/2UL IMD issues are similar to E-UTRA CA_3A_n7A-n28A, the corresponding MSDs are reused.

Table 5.1.X.4-1: MSD for the CA configuration

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n3A-n7A-n28A | n3 | 1747 | 5 | 25 | 1842 | N/A | FDD | N/A |
|  | n7 | 2543 | 5 | 25 | 2663 | N/A | FDD | N/A |
|  | n28 | 741 | 5 | 25 | 796 | 20.0 | FDD | IMD2 |
|  | n3 | 1712.5 | 5 | 25 | 1807.5 | N/A | FDD | N/A |
|  | n7 | 2562 | 5 | 25 | 2682 | 17.0 | FDD | IMD3 |
|  | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n3 | N/A | 5 | N/A | 1832.5 | N/A 26 | FDD | IMD2 |
|  | n7 | 2543 | 5 | 25 | 2663 | N/A | FDD | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | FDD | N/A |

Correct CA_n3-n7-n28 MSD value of 26dB at n3

CA_n3-n78-n105

This discussion explains an error in CA_n3-n78-n105 in MSD value for n78. In [2] it was noted that “MSD values have been re-used from DC_3A-28A_n78A for RX band n78.”, however this is an IMD3 value and not an IMD4 value as the analysis finds.

Based on the tables above it can be seen that

- n3 + n105 IMD4 and IMD5 may affect Rx frequencies of band n78

The MSD table:

Table 5.x.2.3-1: MSD due to IMD issue

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n78-n105 | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n78 | 3740 | 10 | 50 | 3740 | 17.3 | TDD | IMD44 |
|  | n105 | 670 | 5 | 25 | 619 | N/A | FDD | N/A |
|  | n3 | 1713 | 5 | 25 | 1808 | 1 | FDD | IMD4 |
|  | n78 | 3795 | 10 | 50 | 3795 | N/A | TDD | N/A |
|  | n105 | 666 | 5 | 25 | 615 | N/A | FDD | N/A |

Below is the extract from 38.101-3 showing it is an IMD3 issue with the value 17.3dB.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-28A_n78A DC_3C-28A_n78A DC_3A-3A-28A_n78A | 3 | N/A | 5 | N/A | 1870 | 17.3 | IMD3 |
|  | 28 | 740 | 5 | 25 | 760 | N/A | N/A |
|  | n78 | 3350 | 10 | 25 | 3350 | N/A | N/A |

Instead, the values from CA_n66-n71-n78 should be used, since this is an IMD4 product is the same order, while n66 uplink frequencies are similar to n3 and n71 uplink frequencies are similar to n105, making the response in n78 downlink similar.

Table 7.3A.5-2: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n66-n71-n78 | n66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n71 | 668 | 5 | 25 | 622 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3724 | 9 | TDD | IMD41 |
|  | n66 | N/A | 5 | N/A | 2160 | 15.5 | FDD | IMD3 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n78 | 3546 | 10 | 50 | 3546 | N/A | TDD | N/A |

Correct CA_n3-n78-n105 to 9dB on IMD4 of n78.

A draftCR [3] corresponding to the proposals in this section have been provided for the Rel-18 basket bigCR.

Agree the corrections for CA_n1-n77-n79, CA_n3-n7-n28 and CA_n3-n78-n105 in the provided draftCR

Conclusion

The paper provides explanation and justification for several corrections submitted via draftCR to respective band combination baskets.

The following Observations and Proposals were made:

Observation 1: It was requested at RAN4#109 to provide analysis and justification why the harmonic mixing entry in TS 38.101-1 Table 7.3A.4-4 for CA_n48-n96 should be corrected.

Proposal 1: Agree the corrections for CA_n48-n96 harmonic mixing in the provided draftCR

Observation 2: Some of the IMD test points for DC_2A-66A_n77A and DC_2A-n66A_n77A are invalid.

Proposal 2: Proposal to add IMD2 to requirement section of Note 11 usage of 32.1dB, same as section wo. Note 11 including new test frequency points

Proposal 3: Proposal to add Note 4 on DC_2A_n66A-n77A11

Observation 3: DC_2A_n66A-n78 cannot have IMD2 in band 2A.

Proposal 4: Proposal to group DC_2A-66A_n78A and DC_2A_n66A-n78A separately and correct IMD2 case in 2A when there’s 2 NR bands

Proposal 5: Proposal to add IMD2 into n77 for DC_66A_n2A-n77A with MSD values from DC_2A_n66A-n77A

Proposal 6: Agree the corrections for IMD test points in the provided draftCRs

Observation 4: Based on the tables above it can be seen that:

	n25 + n66 IMD2 and IMD4 may affect Rx frequencies of band n78. This test is missing in existing 38.101-1, re-use CA_n25-n66-n77 MSD values.

	n25 + n78 IMD4 may affect Rx frequencies of band n66. This test has been missed. Re-use CA_n25-n66-n77 MSD values.

	n66 + n78 IMD2, IMD4 and IMD5 may affect Rx frequencies of band n25. These tests have been missed. Re-use CA_n25-n66-n77 MSD values.

Proposal 7: Proposal to add missing IMD2 and IMD4 cases to CA_n25-n66-n78

Proposal 8: Agree the corrections for CA_n25-n66-n78 in the provided draftCR

Observation 4: The only combination using a summation of IMD is CA_n3-n7-n8 in 38.101-1.

Observation 5: There is already another note in 38.101-1 which can be applied instead of Note 11.

Observation 6: CA_n3-n7-n8 implicitly already uses Note 4 since n41 has the same downlink frequencies as n7.

Proposal 9: Use note 4 and delete IMD2+IMD3, replace with IMD2 only

Proposal 10: Void note 11, that is only used in this one CA case

Proposal 11: Agree the corrections for CA_n3-n7-n8 in the provided draftCR

Proposal 12: Reuse the MSD of CA_n1-n78-n79 for CA_n1-n77-n79

Proposal 13: Align MSD between DC_3-n7-n28 and CA_n3-n7-n28

Proposal 14: Correct CA_n3-n7-n28 MSD value of 26dB at n3

Proposal 15: Correct CA_n3-n78-n105 to 9dB on IMD4 of n78.

Proposal 16: Agree the corrections for CA_n1-n77-n79, CA_n3-n7-n28 and CA_n3-n78-n105 in the provided draftCR

- References

- R4-2319800, Draft for CR 38.101-1 to add missed harmonic mixing MSD, Nokia

- R4-2308094, TP to TR 37.718-03-01: Addition of CA_n3-n78-n105, Nokia, AMX

- R4-2402073, draftCR to 38.101-1 – Correcting MSD value of CA_n1-n77-n79 , CA_n3-n7-n28 , CA_n3-n78-n105, Nokia

- R4-2402074, draftCR to 38.101-1 – Correction to CA_n48-n96 harmonic mixing, Nokia

- R4-2402075, draftCR to 38.101-1 – Correction to IMD2 IMD3 notation for CA_n3-n7-n8, Nokia

- R4-2402076, draftCR to 38.101-1 - Updates to CA_n25-n66-n78 and other editorials, Nokia

- R4-2402077, draftCR to 38.101-3 – Updates to DC_2A-66A-n77An78A, Nokia

- R4-2402078, draftCR to 38.101-3 – Updates to DC_2A-n66A-n77An78A DC_66A_n2A-n77An78A, Nokia