# 3GPP TSG-RAN WG4 Meeting #110		R4-2402079

# Athens, GR, 26 Feb – 01 Mar, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	On corrections for aerial NR UEs

Agenda item:	6.2.6

Document for:	Discussion and Approval

Introduction

The UAV WI, as presented in [1], was completed with the feature CR [2] agreed at RAN4#109 and implemented to the TS after RAN#102. When reviewing the implemented CR in the TS some minor corrections and modifications have been identified which are further discussed and justified in this contribution.

Discussion

On addition of RAN2 Information Elements

In their previous communication to RAN4, in R2-2311287 Error! Reference source not found. that the indication of additional spectrum emission requirements (NS signaling) for aerial UEs would be introduced by a IE called additionalSpectrumEmissionUAV. The implementation of the feature in Error! Reference source not found. was completed using this information as reference, which was reflected by the information previously captured in Clause 6.2.3K  of TS 38.101-1

From [3]: “ RAN2 would like to inform RAN4 that RAN2 has selected Option 2 described in R4-2314700 and will add a new field additionalSpectrumEmissionUAV which shall be applied by the UAV UE. RAN2 will include this field in Rel-18 LTE and NR specifications. “

However, when finally implementing the feature into the Release 18 specification in TS 36.331, RAN2 adopted a different approach. Instead of additionalSpectrumEmissionUAV, RAN2 has provided a new structure of containers that goes as follows:

- A new container in SI for indication of bands for aerial UEs (freqBandIndicatorAerial or multiBandInfoListAerial) ;

  - Inside this container, a new IE has been introduced: NS-PmaxListAerial.

    - AdditionalSpectrumEmission (no “UAV” added to create a new name) is carried within NS-PmaxListAerial

From [4]: 5.2.2.4.2 Actions upon reception of the SIB1 […] 4> if the UE is aerial UE and it supports at least one frequency band in the frequencyBandListAerial, for FDD from frequencyBandListAerial for uplink, or for TDD from frequencyBandListAerial for downlink, for which the UE supports at least one of the additionalSpectrumEmission values in nr-NS-PmaxListAerial, if present: 5> select the first frequency band in the frequencyBandListAerial, for FDD from frequencyBandListAerial for uplink, or for TDD from frequencyBandListAerial for downlink, which the UE supports and for which the UE supports at least one of the additionalSpectrumEmission values in nr-NS-PmaxListAerial;

Therefore, the indication of the NS_UAV now is not derived by a new IE additionalSpectrumEmissionUAV , but for the different type of container that carries a previously known  additionalSpectrumEmission.

RAN2 have agreed to provide the aerial UE specific NS information by providing the IE additionalSpectrumEmission in a container dedicated specifically for Aerial UEs.

Therefore, the description in clause 6.2K.3.1 needs to be adapted to account for this modification in RAN2 design, such that the definition of NS_UAV becomes unequivocally associated to the aerial container, even though a similar parameter name is used.

Besides the name of the aerial capability used in the definition of the aerial UE was also updated by RAN2.

Based on this discussion we propose a draftCR to update the aforementioned remarks [5].

Agree to the draftCR in R4-2402081 to TS 38.101-1 in order to capture the right indication of the network signaling used by RAN2.

On corrections to power back off for aerial NR UEs

When agreeing the A-MPR values for aerial NR UEs it was argued that a MAX function had to be added from some companies. Therefore, the current TS have this function included to NS_UAV_44 and NS_UAV_70.

The current TS 38.101-1 utilizes MAX function in the A-MPR definition for NS_UAV_44 and NS_UAV_70.

The reasoning for adding the MAX function was avoid the possibility of less than zero values of LCRB. However, LCRB, by definition, denotes the length of a contiguous resource allocation in terms of the number of resource blocks, it is always a positive integer.

LCRB is by definition always a positive integer of resource blocks.

Restricting the, LCRB to a positive values by the MAX(0,…) expression is therefore not needed. Theis because, LCRB ³ 1 is an implicit condition that doesn’t need to be written separately.

There is no need to use the MAX function for NS_UAV_44 and NS_UAV_70.

Currently the MAX function have been used in other A-MPR tables, but it can also be questioned if it is needed here. We propose to remove the MAX part in the A-MPR tables for NS_UAV_44 and NS_UAV_70 but otherwise not make any modification to the agreed A-MPR. This is done via the provided CR submitted as [3].

Remove the MAX function in NS_UAV_44 and NS_UAV_70

Proposal 2 is implemented into the draftCR [5].

Agree to the draftCR in R4-2402081 to TS 38.101-1 in order to remove the MAX function in NS_UAV_44 and NS_UAV_70.

Conclusion

In the paper, the following Observations and Proposals were made:

Observation 1: RAN2 have agreed to provide the aerial UE specific NS information by providing the IE additionalSpectrumEmission in a container dedicated specifically for Aerial UEs.

Proposal 1: Agree to the draftCR in R4-2402081 to TS 38.101-1 in order to capture the right indication of the network signaling used by RAN2.

Observation 2: The current TS 38.101-1 utilizes MAX function in the A-MPR definition for NS_UAV_44 and NS_UAV_70.

Observation 3: LCRB is by definition always a positive integer of resource blocks.

Observation 4: There is no need to use the MAX function for NS_UAV_44 and NS_UAV_70.

Proposal 2: Remove the MAX function in NS_UAV_44 and NS_UAV_70

Proposal 3: Agree to the draftCR in R4-2402081 to TS 38.101-1 in order to remove the MAX function in NS_UAV_44 and NS_UAV_70.

- References

- RP-233386, Revised WID: NR Support for UAV (Uncrewed Aerial Vehicles)

- R4-2322011, Running CR to TS 38.101-1 - Introduction of Aerial UEs support, Ericsson, Nokia, Huawei, Murata, Qualcomm

- R2-2311287, Reply LS on the handling of additional regulatory requirements for UAV UEs, Ericsson, RAN2 #123, Xiamen, China, 9-13 October 2023

- TS 36.331. Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification. V18.0.0. 3GPP

- R4-2402081, “CR to 38.101-1 Corrections for aerial NR UEs”, Nokia, Nokia Shanghai Bell, RAN4 #110, Athens, Greece, 26 Feb – 01 Mar, 2024