3GPP RAN WG 4 Meeting #110	R4-2402249

Athens, Greece: 26 Feb- 1 March, 2024

Agenda Item:	14

Source:	Spark NZ, Reliance Jio, Nokia

Title:	Rel-19 New SID proposal: Antenna Models for 5G Macro BS

Document for:	Discussion

# 1	Introduction

Adaptive antennas are an integral part of modern radio base stations today. They are implemented in most (if not all) base stations. This is likely to remain so in the future.

With the introduction of AAS in Rel-13/15 the complexity of the macro-BS has increased a lot. Unlike traditional fixed beams, AAS allows base stations to dynamically focus the power /beam directly to the UE, optimizing the overall performance. The flexibility introduced by incorporation of beamforming may have some impact on simulations related to network performance and coexistence between networks and other services.

Since the early introduction of AAS systems, new features have been introduced in the form of increased MIMO port numbers, improved CSI, array architectures, coherent joint transmission, multiuser MIMO beamforming etc.  Further evolutions of base stations may involve distributed MIMO and cell free MIMO that may result in the increase of channel rank and therefore capacity. To model the performance and coverage improvement of such systems it is essential to have proper and agreed antenna models and their parameters.

Massive MIMO systems and multiuser MIMO all rely on adaptive antennas. 3GPP Rel-18 and Rel-19 both have further improvements to MIMO [1,2,3] as part of agreed work/study items. These advancements are inherently linked to the deployment of AAS systems.

The architecture of Massive MIMO in 3GPP compliant commercial BSs consists of three key components:

(1) Dual-polarized micro-strip (patch) antenna elements.

(2) Sub-array(s); and

(3) An overall antenna array (global array) composing of multiple sub-arrays.

In addition, various forms of beam forming (analog, digital, hybrid) can be applied to this global array to enable MU MIMO.

The discussions about AAS systems exist in many 3GPP documents but there is no single definitive document consolidating a comprehensive summary of  AAS systems in 3GPP documents.

It is proposed to develop a TR under study item on adaptive antennas that will capture what is already there in 3GPP recommendations and also have additional material that captures new developments.

The proposed TR on adaptive antennas can play a pivotal role in addressing the performance improvements and capacity studies in 3GPP and will also be a useful document for international regulatory bodies.

The proposed list of topics in Section 2 outlines the subjects for an TR.

The above work /study may be undertaken as an important topic by 3GPP RAN4.

# 2 	Proposals

It is proposed to develop a Technical Report on AAS systems by RAN4 under a study item

The proposed Technical Report (TR) aims to address the following key aspects:

- An antenna array antenna model, including the sub-array extension [8] using array factor algorithms that are standardised in 3GPP whilst also aligned with fundamental principles [10].

- Techniques for side lobe suppression, including an investigation into their feasibility and undesirable side effects [6,7].

- Techniques for MU MIMO beamforming as implemented in commercial products [4].

- Definition of Antenna element patterns that are more realistic than specified in [5,9] and are aligned with commercial practices.

- Analysing the performance of arrays with frequency separation from the center frequency, in particular how far in frequency separation does one needs to go before the array performance degrades to that of a single element.

- Performance of antennas on a chip, mutual coupling, slant polarised antennas etc .

- Emerging architectures including D-MIMO, cell free MIMO etc.

By addressing above key aspects, the proposed technical report will facilitate a deeper understanding of adaptive antennas, support standardized implementations, and contribute to the international regulatory studies.

The above work /study may be undertaken as an important topic by 3GPP RAN 4.

# References

[1] W. Chen, J. Montojo, J. Lee, M. Shafi and Y. Kim, "The Standardization of 5G-Advanced in 3GPP," in IEEE Communications Magazine, vol. 60, no. 11, pp. 98-104, November 2022, doi: 10.1109/MCOM.005.2200074.

[2] 3GPP TSG RAN Chair, “RAN Chair’s Summary of Rel-19 Workshop”, 3GPP RWS-230488, June 2023.

[3] 3GPP TSG RAN Chair, “Summary for RAN Rel-19 Package: RAN1/2/3-led”, 3GPP RP-232745, Dec 2023.

[4] Huangping Jin, Kunpeng Liu, Min Zhang, Leiming Zhang, Gilwon Lee, Emad N. Farag, Dalin Zhu, Eko Onggosanusi, Mansoor Shafi, and Harsh Tataria, “Massive MIMO Evolution Toward 3GPP Release 18”.   IEEE Journal of Selected areas on Communications, June 2023 pp 1635- 1654.

[5] 3GPP TR 25.996, “Spatial channel Model for MIMO simulations”.

[6] Joannis P. Gravas  Adaptive Beamforming with Side Lobe Suppression by Placing Extra Radiation Pattern Nulls., https://pure.hud.ac.uk/ws/portalfiles/portal/16137171/final_version.pdf

[7] Qiaowei Yuan et al , Adaptive Antenna Array Pattern Synthesis for Suppressed Sidelobe Level and Controlled Null Using Genetic Algorithm, International conference for convergence of Technology 2014.

[8] R4-2108080, “LS to ITU-R and CEPT on extension of IMT array antenna model to support sub-array structures”, Ericsson.

[9] 3GPP TR 37 840

[10] Balanis, Antenna Theory.