3GPP TSG-RAN WG4 Meeting #110	R4-2402277

Athens, Greece Feb 26 – Mar 1, 2024

Source:	Nokia, Nokia Shanghai Bell, BT

Title:	Discussion on 8Rx general demodulation aspects and spatial channel models

Agenda item:	8.1.3.1.1

Document for:	For Information

Introduction

This document extends the discussion on the 8Rx UE demodulation and CSI requirements introduced in RAN4 #105 and summarized in [1], discussions on this topic were continued at RAN4 #106, RAN4 #106-bis-e, RAN4#107, RAN4#108, and RAN4#108-bis, with the way forward documents being available in [3], [4], [5], [8], and [9].

The requirements defined thus far for PDSCH in this work item are based upon TDL channel modelling. Whilst some discussion occurred on the requirements for a spatial channel model in RAN4#106-bis, ultimately no requirements were agreed within this work item in Release-18 with a spatial channel model.

The features introduced with the 8Rx work item and in particular the 2 codeword case, which occurs when the rank is greater than 4, are not exercised in a TDL channel, where no variable rank performance is observed. Within this contribution the case for a spatial channel model is introduced. This is for information to aid future discussions on the topic of spatial channel modelling for features which utilise aspects of the spatial channel.

Discussion

Motivation of spatial channel models/CDL

The motivation behind Spatial Channel Models (SCM), and specifically the Cluster Delay Line (CDL) model of [11][12] is to enable the performance expectation of 5G NR features to be aligned with that experienced in deployment and/or field testing.

This alignment is relevant for (but not limited to) MIMO and beamforming-related coverage enhancement features, where we observe significant discrepancies when performance is evaluated based on non-spatial channel models, such as TDL.

MIMO, which is a key feature of 5G NR provides improvements by leveraging multiple demodulation branches and MIMO layers that each observe independent realisations of a given wireless channel in a spatial domain. Specifically, this is not realised in the extant Tap Delay Line (TDL) models for RAN4 specifications which only provide channels which represent time and power, with no spatial component.

For MNOs, demonstrating the reliability of these gains is crucial for justifying substantial investments in network infrastructure. The ability to accurately model and predict MIMO performance in different spatial scenarios allows MNOs to take calculated risks in deploying and optimising their networks for a given investment budget. Specified minimum performance requirements for the standardised features play an important role in deployment and planning of mobile networks that deliver services using 3GPP-specified functionality. The current RAN4 specifications do not directly enable such comparison and conformance to occur for advanced MIMO functionality in particular.

The introduction of CDL into RAN4 performance requirements will address the limitation of TDL by enabling spatial characteristics of the channel to be observed, ensuring that the full performance of MIMO features is understood and appropriately represented in network planning and optimization efforts.

As presented in [14] the TDL channel models provide exactly the same performance per layer in all scenarios, this is directly in contrast to the performance of a real system as measured by BT in [13], where observable performance differences between layers was measured.

Specifically, within Rel-18 of 3GPP, the introduction of 8Rx MIMO with 2 codewords in order to provide variable performance per codeword and provide optimised spectral efficiency in a realistic deployment requires a spatial channel to create variable per layer performance, creating a performance difference between each codeword.

SCM usage in RAN4

Since the first study release of 5G NR, efforts have been made to try and achieve alignment on RAN4 performance requirements with spatial channel modelling (SCM) [18][19] . Unfortunately, these efforts have failed, as companies were not able to bring and align results using the CDL models following the definitions in TR 38.901 [11]. Due to time pressures to complete specific releases and following the assertion of working assumptions, requirements were henceforth defined using TDL channel models in the RAN4 specifications.

SCM efforts are not new to NR within 3GPP, as they have origins in LTE, where efforts were made pertaining to verification of multi-antenna reception performance of User Equipment in TR 37.977.

The work on SCM for RAN4 has been on hold since 2019, but with an increasing number of SU/MU-MIMO enhancements and beam reliant features coming in the later releases of 5G NR, the RAN4 demodulation session is finding itself more often in situation, where non-spatial channel models neither show the performance observed in RAN1, nor those expected in deployment.

Given the above motivation, both Nokia and BT believe that with the updates provided in Rel-16 to CDL with TR 38.827 [12] which aimed to fix some aspects of the CDL model such that alignment can be made, along with the introduction of TE vendor products to the market that implement TR 38.827, that the concern of it implementation being too complex to align do not hold anymore.

Furthermore, Nokia and BT believe that as we move towards 6G, that Rel-19 is the ideal release to fix these issues, such that the expected enhancements in 6G can start to be specified with representative requirements from the initial release. Such solid basis would enable 3GPP expert community to focus on specifications improving or enabling services, rather than on continuous alignment of modelling approaches.

In the following sections we present results, which demonstrate why spatial modelling is important, using both simulated and measurement results from BT and Nokia, together with additional theoretical considerations.

SCM usage in RAN1 and RAN4 adaptation

It is important to note that RAN1 has been using CDL channel models since (at least) 2016 [15][16] to develop and improve MIMO and beam related features. Proposed RAN1-led MIMO features at RAN plenary are commonly presented using only CDL link level simulation results [17].

Within RAN1 there is no doubt that a SCM, and in particular CDL models, must be used to evaluate the performance and gains of any MIMO feature.

We note here that the TR 38.901 derived CDL channel models have been found sufficient for feature performance gain evaluation, as the relative performance gain, when comparing “feature on vs. off” in the same channel model implementation, is sufficiently stable across different CDL implementation and runs.

However, RAN4 requires reliable and aligned absolute performance of a feature across different channel model implementations, which was found to be difficult using TR 38.901 derived CDL channel models. This was later attributed to too many random, but performance impacting, starting values, which make comparisons between runs and implementations unstable.

However, most of these instabilities have been fixed in 2019 in the work culminating in TR 38.827. This CDL version uses, for example, fixed AOD to AOA angle coupling, fixed beam to RF port mapping, and fixed initial phases for the different polarization combinations, to achieve stable absolute performance outcomes.

It is further worth noting that CDL implementations based on TR 38.827 are also commonly available in TEs from many TE vendors.

The TR 38.827 CDL implementation has been used in this contribution, and we have added the relevant excerpts from TR 38.827 in Appendix C, along with comments, where we feel the original text needs clarification or extension to enable other interested parties to replicate our analysis from this contribution.

Theory

Per layer post-EQ SINR

In this contribution, we denote the per layer post-EQ SINR as being the SINR (or quality) of each MIMO layer measured after the application of a baseband receiver/equalizer on the channel facing receive ports, where the receiver application gives rise to the concept of MIMO layers. In common MIMO principles, the SINR of a layer determines how much TPUT the corresponding layer can carry (maximum achieved for Gaussian signalling).

The receiver is usually derived using the effective channel estimated from (precoded) DM-RS, where the effective channel is the physical channel including the precoding step (i.e., usually physical channel right multiplied with linear precoding matrix).

Applying a zero forcing (ZF) receiver on the effective channel (or CSI-RS estimated physical channel with unitary precoding) results in equal per layer post-EQ SINR, due to inversion of the effective channel. However, from SU-MIMO signal processing theory, it is possible to derive that the ZF solution is only TPUT optimal, when the SNR budget is extremely high and the (effective) channel is very well conditioned:

Figure 1. Sketch of a post-EQ SINR per layer profile for a fully ZF equalized, or also non-spatial, channels.

The required SNR budgets for ZF to be a reasonable receiver choice are impractically high, and potential massive MIMO effects are not relevant in our assumed 8x8 (baseband) MIMO system.

In any practical system and realistic deployment, the commonly used SU-MMSE receivers are operating in between maximum ratio combining (MRC) and ZF receiver extremes. Also, the commonly used “SVD” receivers or other linear receivers working on the eigenspaces, do not result in an inversion of the effective channel, which results in equal per layer SINR.

Rather, it is commonly observed to have stronger and weaker layers (see also section 2.5.1):

Figure 2. Sketch of a practically relevant post-EQ SINR per layer profile.

SU-MIMO processing theory (e.g. [20]) also tells us that the channel capacity can be achieved by using “SVD” precoder and receivers, where the available TX power is spread over sub-spaces of the channel that are accessed via the precoder receiver pairs. The better quality a subspace is, the more power is to be (spatially) focused there (“water filling solution”, when we assume a sum transmit power constraint).

Thus, the resulting post-EQ per layer SINR are not expected to be equal, especially when optimizing for TPUT.

Per CW MCS/CQI choice

Note that the TPUT loading of each layer in 4G/5G is not independent, contrary to what basic theory assumes for optimal TPUT. Instead, 4G/5G can only choose the MCS (i.e., TPUT loading) independently for two codewords (CWs), which split up to 8 layers among themselves.

We further note, increasing the modulation order can also be interpreted as reducing the effective signal power at the receiver (i.e., decreasing the distance between valid constellation points), and a similar concept applies to the coding rate, whereby the distance is increased or decreased between symbols.

The optimal MCS/CQI choice in 1 or 2 CW systems is far from trivial and the resulting performance is implementation dependent. As an example, we can consider needing to choose one MCS/CQI that fits a CW with 4 layers. One approach is to align MCS/CQI with the weakest layer, hence being able to demodulate all layers. However, this leaves the stronger layers with less TPUT than they could support. In another implementation the second weakest layer might be used to decide the MCS, which immediately removes ¼ of the TPUT, as the weakest layer cannot be demodulated, but there is more TPUT on the 3 remaining layers. Implementations would need a threshold to decide when to switch approaches, or they would need to search the full opportunity space.

Additionally, the system then needs to choose precoders that match certain ports/layers to stronger subspaces, hence optimizing/complicating the MCS choice per CW further.

One solution may be to create high quality layers in one high TPUT CW, and having another lower TPUT CW. Also, the handling of short-term rank deficient channels factors into such choices.

In summary, there is a lot of unused optimization potential, if one decides to artificially equalize the two CWs (or their layers) to be of same decoding BLER, or same quality, or same MCS/CQI.

Spatial properties of certain channel models

There is limited set of fading channel models that are commonly used in RAN4 and RAN1. In the following, we will quickly go over the MIMO relevant properties of the ones used in this contribution. As we are focusing on MIMO use cases, we are especially interested in controllability and representativeness of spatial preferences.

Measurement results, showing deployment observed post-EQ SINR profiles for comparison, are given in section 2.5.1.

TDL

TDL is a non-spatial channel model, i.e., it does not contain spatial preferences in the resulting channel structures.

Following the understanding that MIMO SDM precoders change the signal transmission to occur in defined “spatial domains”, or “directions”, we may thus expect that every (equally power normalized and orthogonal) precoder applied to a TDL channel results in the same average post-EQ SINR per layer, due it not being able to represent the spatial domain.

It is a common RAN4 observation in TDL fading based requirements, that random precoding can be used, as both random and fixed precoding result in the same performance.

In summary, any TDL channel model has no spatial preferences and no post-EQ SINR control.

Figure 3. Simulated post-EQ SINR profile of TDL channel, 
with random or fixed wideband TypeI-SP precoder, SU-MIMO MMSE receiver.

The simulations showing post-EQ SINR pdf (see Figure 3), confirm the above observations.

Precoding choice does not matter in TDL. Averaging over randomly chosen precoders, results in same performance as any chosen fixed (normalized) precoder. This is not in line with SINR profiles encountered in MIMO deployments (see also section 2.5.1).

Since all precoder (directions) perform the same, we confirm that there is no spatial component/preference in the TDL model.

TDL + Beamforming

This modification of TDL is used since LTE (CRI requirements). The modification fundamentally consists of applying a sweeping beamformer pattern “on top” of the TDL created channel.

This version still has only a single spatial preference, but the sweep is controllable.

Precoders and receivers, that match the single beam broadside, achieve best performance.

CDL (following TR 38.827)

CDL is inherently a SCM, i.e., the channel coefficients and matrices are created based on input from large scale spatial preferences model. Those preferences are then realised via propagation determining clusters in the model. Notably, also cross-polarization effects are enabled by having the possibility to define the concept of orientations in a spatial model.

By default, this CDL model uses #BasebandTxBranches/2 different Tx beams at the same time (mimicking potential BS beam patterns) to create the desired number of BB visible RF ports. Beams are matched to the strongest clusters of the chosen cluster environment. If required, these beams can be fully configured and don’t need to always point towards strongest clusters.

This channel has post-EQ SINR profiles that match the ones expected from deployment (see section 2.5.1). The usage of different precoding directions via different precoder choice, is visible in performance.

Figure 4. Simulated post-EQ SINR profile of CDL channel, 
with random or fixed wideband TypeI-SP precoder, SU-MIMO MMSE receiver.

For the CDL channel model, and particularly its TR 38.827 version, our observations are also confirmed (see Figure 4).

With a fixed precoder, we see a post-EQ SINR profile that mimics those encountered in practical deployments, as seen in [13]. Thus, correctly modelling the varying, but large scale environmental, precoding, and receiver determined, per layer SINR performance. This generally also results in differing per CW performance in 2CW cases (see section 2.5.2).

Note: With truly random precoding we would expect for all pdfs to be overlapping, as each port is randomly mapped to all directions/precoders. However, in this specific simulation setup, random precoding means a random selection from the TypeI-SP codebook. This codebook cannot map any precoding vector to any layer; subsets of vectors that can show up at each different layer are formed. Hence a pattern emerges, were the layers have slightly different average performances, even using random PMI choice.

Note: It is noticeable that the post-EQ SINR profiles of certain (not always obvious) pairs match their shape on a relative scale. This comes from the TR 38.827 CDL model mapping baseband visible RF ports to specific separate beams. i.e., port0 and port1 are mapped to the pol0 and pol1 of the strongest beam, and so on. The CDL model then has (in its default configuration) fixed initial phases for 2x2 polarization matrices. So, the large-scale gains of pairs of ports are the same, but the cross-polarization power ratios are still asymmetric.

Outcome

The above simulation results lead us to the following summarizing observation.

The PDSCH post-EQ SINR profiles, when using TDL channel models do not match measurements. SDM processing does not impact performance, when using TDL channel models. CDL both shows typical post-EQ SINR profiles and typical deployment spatial components.

Technical Analysis

Sample measured layer characteristics

This section demonstrates the variability of the quality of spatial layers measured in a typical MIMO deployment using a base station with 4 transceiver ports and receiving equipment with 4 ports, forming a 4x4 MIMO system. Measurements were conducted in a 15MHz channel at 2647.9MHz centre frequency, over several locations in a campus-type environment, shown in Figure 5. To demonstrate the variability of spatial layers, results for line-of-sight and non-line-of-sight measurements are presented.

Figure 5 Measurement locations in a campus-type environment.

Figure 6 and Figure 7 show histograms of per-layer SINR for line-of-sight and non-line-of sight conditions respectively. The per layer SINR was derived performing singular value decomposition on the measured channel response of the 4x4 MIMO system. The histograms reflect the spread of corresponding SINR values per layer across the channel bandwidth and the measurement duration. The span of the per layer SINR, which is in excess of 30dB is clearly demonstrated in both cases, with each spatial layer further exhibiting individual loss statistics.

A span of per layer SINR which is in excess of 30dB is clearly demonstrated in both cases of the measurement campaign, with each spatial layer further exhibiting individual loss statistics.

Figure 6 Histogram of per-layer SINR for sample for predominantly line-of sight measurement.

Figure 7. Histogram of per-layer SINR for sample non-line-of sight measurement.

These figures demonstrate that each spatial layer exhibits individual loss in a realistic deployment; this is in direct contrast to the extant TDL simulations, which would provide the same statistics for each layer. This in turn corresponds to no perceivable SDM gains for MIMO systems/precoders/receivers that optimize post-EQ SINR profiles for performance, and as such non-representative performance requirements.

Hence, leading us to the following proposal:

Channel modelling utilised in performance requirement definitions for NR MIMO features should reflect similar variability in the quality of spatial layers as demonstrated in provided sample MIMO channel measurements.

8Rx 2Codeword Issue

Following the per layer post-EQ SINR expectation and impact analysis, that this contribution has carried out until here, we now apply this understanding to 8RX 8layer/2CW performance requirements.

All simulation setup details, and the complete set of results can be found in appendix A.

The implementation details for the CDL-C UMa channel, following TR 38.827, can be found in Appendix C, along with comments, where we feel the original text needs clarification or extension to enable other interested parties to replicate and align with our results.

Setup and Expectations

We are looking to have a requirement and test setup that offers post-EQ SINR layers of varying qualities to mimic deployment. As discussed before, this tests the performance of MIMO receivers, when needing to align spatial preferences of the channel and to correctly prioritize and match layers/directions for realistic sum TPUT. Additionally, in the case of 2CWs the optimal MCS/CQI choice is far from trivial. We focus on the pure DMD case here, which shows receiver performance in cases, where the per CW MCS cannot result in exactly the same operating point per CW.

Hence, we are interested in the demodulation performance for a test setup and that has both, deployment mimicking post-EQ SINR profiles, different per CW performance, and different per CW MCS.

To summarize our simulation setup in Appendix A, we use a FDD type A PDSCH (40MHz/30kHz) setup, with 2 DM-RS positions, and 8Tx8Rx. Rank8/8-layer transmission is configured resulting in 2 codewords (CWs), with one MCS choice each. HARQ retransmission is active (max 4 Tx). Carrier frequency is 3.5GHz and simple SU-MMSE receiver is used.

Precoding is wideband typeI-SP with (N1, N2)=(4, 1) and (O1, O2)=(4, 1). Random precoder selection is updated per slot, with equal probability of each applicable i1, i2 combination. Fixed precoder is “i11=2, i12=0, i2=1 (i12 is fixed 0)”, unless otherwise stated. No impairments are considered.

The SNR definition in the TPUT figures follows the RAN4 definition (“preEQ SNR”).

TDL Low

The following are some representative curves from our simulation results in Appendix A.

Figure 8. Per CW performance, TDLC300-100 (30kph) low, with random or fixed wideband TypeI-SP precoder, SU-MIMO MMSE receiver, MCS0=17, MCS1=17.

As expected from the TDL post-EQ SINR profiles, both CWs exhibit equal performance, in both random and fixed precoding for the same MCS, i.e., SDM precoding is inconsequential. This is not representative of CW choices in a deployment, whereby each CW will have different MCS, as this is needed to achieve reasonable performance in MIMO deployments.

CDL with Random and Fixed Precoder

The following are some representative curves from our simulation results in Appendix A.

Figure 9. Per CW performance, CDLC Uma 30kph, 8 layers,
with random or fixed wideband TypeI-SP precoder, MCS0=17, MCS1=17.
(BS AE pattern as in 38.827, both CDL ray paring and polarization scalar initial phases random.)

This Figure 9 is following what would be expected from a real and/or simulated channel with a spatial component or spatial preferences.

When using a fixed MCS for both CW and using a fixed precoder, the stronger CW (i.e., the one with the better layers mapped to it) achieves 70%TPUT at a lower pre-EQ SNR. Incidentally this is CW2 in our example. The relative performance depends on the specific chosen fixed PMI/precoder and how it maps subspaces of the channel to layers to CWs. For the chosen PMI, the performance gap is significant (9dB), but follows the expectations from the post-EQ SINR profile (same configuration shown in Figure 4).

Trying to choose precoders that equalize the per CW performance, is neither simple (see Figure 10), nor advantageous from a TPUT perspective, as spatial preferences need to be mismatched with precoding vectors to achieve this goal. Such an equalization would also not be representative of real-world MIMO performance and performance scaling.
Furthermore, choosing random precoding in an effort to try and overlap per CW performance is even less advisable, as this will result in frequent mismatched precoder/channel choices, which frequently result in outage layers and lower overall performance.

With random precoder choice the performance of the two CWs is rather similar, but unlike one might have expected, not perfectly overlapping. As we have seen for the CDL SINR profiles (section 2.4.2), the limitations of the typeI CB are visible again. This codebook cannot map any precoding vector to any layer, so subsets of vectors that can show up at the different layers are formed. Hence a pattern emerges, were the layers have slightly different performances, even in random PMI choice.

| PMI i_{11} = 2,3 | PMI i_{11} = 4,5 |
| --- | --- |

Figure 10. Per CW performance, CDLC Uma 30kph, 8 layers,
different fixed wideband TypeI-SP precoders, MCS0=17, MCS1=17.

While not crucial for the viability of a channel model for the 2CW MIMO use case, it is interesting to note that the performance delta between the two CWs is mainly determined by the eight i_{11} typeI precoder parameter choices. The other parameters only change the gap minorly, see examples in Figure 10. This also reinforces the notion that the limited precoding vector to layer mapping choices impose slight, but unavoidable, differences in performance ranges on each layer, which are linked to the spatial channel components.

Figure 11. Per CW performance, CDLC Uma 30kph, 8 layers, MCS0=17, MCS1= variable (22 being right choice),
fixed wideband TypeI-SP precoders.

Before we looked at the different per CW performance using various TPUT vs. SNR results. However, during a RAN4 requirements/test the test setup is physically limited to only creating a single SNR level at the input of the demodulator (“pre-EQ SNR”). This is aligned with how signals would be received in deployment.
To create a feasible test setups and realistic expectations, requirements must thus be defined so that a single pre-EQ SNR level tests both CWs at the same time. This can be achieved by choosing different MCS values per CW, in such a way that the per CW performance overlaps reasonably. Figure 11 visualizes the process.

This also mimics the correct MCS/CQI choices encountered in deployment. As the performance of different receiver implementations naturally differ over the test setups, it is neither possible nor reasonable to overlap the curves under all circumstances. This is aligned with reality and shows/sets the receiver performance in cases, where the per CW MCS cannot result in exactly the same operating point per CW. I.e., a reasonably close to reality performance can be expected from a large range of MCS choices.

Outcome

The above simulation results lead us to the following summarizing observation.

Analysis of SNR requirements for 70%TPUT and 2CW/8RX use case shows that TDL channel modelling cannot recreate post-EQ SINR profiles and different per CW performance/MCS that mimic MIMO deployments. CDL (the TR 38.827 version) mimics the MIMO effects and profiles the most closely and offers stable and repeatable performance levels.

Impacted requirements of CDL solutions

Until now we have discussed how CDL channels improve the validity of requirements of 2CW transmissions, and multi-layer transmissions in general, when compared to expectations in deployment. It is straightforward to extend the useful properties of CDL channel modelling to other use cases. For example, to define MU-MIMO receiver and SU/MU CSI feedback requirements (both CQI and PMI).

MU-MIMO has the same needs as SU-MIMO, when it comes to modelling per layer post-EQ SINR correctly, with the added necessity to include interference from co-scheduled UEs. The CDL channel model only needs minor evident adjustments to cover these modelling needs (see Appendix B, 6.1).
When it comes to CQI feedback requirements, we have made previously made point that it is very ambiguous concerning how an implementation should choose the CQI/MCS for a CW, given different per layer post-EQ SINR estimations. This task becomes still more complicated, when the SINR profile depends on the chosen PMI and when this PMI choice may impact interference to other UEs. Requirements with channels of fitting SINR profiles and realistic precoder impact are need in this use case.

It is straightforward to see how a spatial channel model, and CDL in particular, helps to align the test/requirement setup to match the various MIMO effects in deployment, while TDL cannot.

Way Forward

Given the importance of spatial channel model demonstrated in this contribution, we see it necessary to:

Study the most appropriate method to add spatial channel modelling to performance requirements in Rel-19. The 8Rx/2CW use case analysed in this contribution can serve as alignment goal.

Conclusion

This contribution provides our views on general items relating to 8RX UE demodulation. The proposals are summarized as follows:

Observation 1: The PDSCH post-EQ SINR profiles, when using TDL channel models do not match measurements. SDM processing does not impact performance, when using TDL channel models. CDL both shows typical post-EQ SINR profiles and typical deployment spatial components.

Observation 2: A span of per layer SINR which is in excess of 30dB is clearly demonstrated in both cases of the measurement campaign, with each spatial layer further exhibiting individual loss statistics.

Proposal 1: Channel modelling utilised in performance requirement definitions for NR MIMO features should reflect similar variability in the quality of spatial layers as demonstrated in provided sample MIMO channel measurements.

Observation 3: Analysis of SNR requirements for 70%TPUT and 2CW/8RX use case shows that TDL channel modelling cannot recreate post-EQ SINR profiles and different per CW performance/MCS that mimic MIMO deployments. CDL (the TR 38.827 version) mimics the MIMO effects and profiles the most closely and offers stable and repeatable performance levels.

Proposal 2: Study the most appropriate method to add spatial channel modelling to performance requirements in Rel-19. The 8Rx/2CW use case analysed in this contribution can serve as alignment goal.

References

- RP-221496 “Revised WID Further RF requirements enhancement for NR and EN-DC in frequency range 1 (FR1)”

- R4-2220152, “Topic summary for [105][326] RF_FR1_enh2_Demod”, Huawei, HiSilicon, RAN WG4 #105, Toulouse, Nov. 2022

- R4-2305888, “WF for 8Rx UE performance requirements”, Huawei, HiSilicon, RAN WG4 #106-bis-e, Online, April 17 – April 26, 2023

- R4-2302942, “WF for 8Rx UE performance requirements”, Huawei, HiSilicon, RAN WG4 #106, Athens, Greece, 27 February - 03 March, 2023

- R4-2309806, “WF for 8Rx UE performance requirements”, Huawei, HiSilicon, RAN WG4 #107, Incheon, Korea, 22 – 26 May 2023

- TS 38.101-4 v17.8.0, Technical Specification Group Radio Access Network; NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements (Release 17), March 2023

- R4-2310279, “WF on UE 8Rx requirements” NTT Docomo, Inc, RAN WG4 #107, Incheon, Korea, 22 – 26 May 2023

- R4-2313877, “WF for 8Rx UE performance requirements”, Huawei, HiSilicon, RAN WG4 #108, Toulouse

- R4-2316914, “WF for 8Rx UE performance requirements”, Huawei, HiSilicon, RAN WG4 #108bis, Xiamen

- R4-2321199, “WF for 8Rx UE performance requirements”, Huawei, HiSilicon, RAN WG4 RAN4#109, Chicago, November 2023

- 38.901 “	Study on channel model for frequencies from 0.5 to 100 GHz’, Rel-14, 2018

- 38.827 “Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE)”, Rel-16, 2018

- A. Tukmanov, J. Weng and R. Husbands, "Polarization and Correlation in MIMO Channels," WSA & SCC 2023; 26th International ITG Workshop on Smart Antennas and 13th Conference on Systems, Communications, and Coding, Braunschweig, Germany, 2023.

- R4-2304097, PDSCH Requirements for 8Rx UE performance, Nokia, Nokia Shanghai Bell, RAN4#106bis-e.

- R1-1608661, “Summary of [86-20] email discussion on MIMO calibration for NR”, ZTE, RAN1#86bis

- R1-1610284, “MIMO link level calibration results”, Nokia, Alcatel-Lucent Shanghai Bell, RAN1#86bis

- RWS-210003 “eMBB PHY Enhancements for Rel 18” Qualcomm, RAN Rel-18 Workshop, June 2021

- R4-1809602 “RAN4-AH-1807 Meeting report”, RAN4#88

- R4-1809399 “Way forward on channel model for NR UE demodulation and CSI requirements for FR2”, Qualcomm, Keysight, RAN4#AH18007

- Björnson, Emil, and Özlem Tuğfe Demir. "Introduction to Multiple Antenna Communications and Reconfigurable Surfaces." (2024).

Appendix A – Simulation Setup and Results

All simulations presented in this contribution have used the simulation setup detailed in this Appendix, unless stated otherwise during the presentation of the results.

Simulation setup

Table 1: PDSCH with CDL/TDL simulation setup

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Simulated Channel |  |  | PDSCH |
| Duplex mode |  |  | FDD |
| FR / Carrier frequency |  |  | FR1 / 3.5 GHz |
| UE receiver type |  |  | Simple SU MMSE |
| (BB) Antenna configuration |  |  | 8 Tx / 8 Rx |
| Waveform |  |  | CP-OFDM with normal CP |
| Channel Bandwidth/SCS |  |  | 40MHz/30kHz |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
|  | Transmission Rank |  | 8 (i.e., 2CW) |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 2 (for rank > 4) |
|  | DMRS ports indexes |  | {1000-1007} for Rank8 |
| Codebook configuration | CodebookType |  | typeI-SinglePanel |
|  | CodebookMode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (4,1) for 8Tx |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) |
|  | RI Restriction |  | 10000000 for rank 8 |
| PDSCH & PDSCH DMRS Precoding | Common parameters |  | Wideband (but shouldn’t matter) |
|  | Fixed |  | i11=2, i12=0, i2=1 (i12 is fixed 0 in ULA with this configuration. All indices counting from “0”) |
|  | Random |  | Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with wideband granularity |
| Analog impairments |  |  | None (No TO, no FO, no PA model, no TxEVM, no RxEVM, no IQ imbalance, etc.) |
| Number of HARQ Processes |  |  | 4 |
| Maximum HARQ transmissions |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |

Channel model selection

The following channels should be used for simulation:

TDLC 300-100 (ULA) Low.

CDL UMa NLOS 30kph (“CDLC”) in the TR 38.827 version.

We chose those two, as they are very comparable for all non-spatial characteristics, which is not surprising as TDLC settings were derived/filtered from this CDLC variant in TR 38.901.

Both share the following characteristics:

300/365 ns RMS delay spread,

30kph UE speed/velocity,

NLOS representative, no (intended) LOS components.

Though they are not fully equivalent due to the spatial effects.

Further results

We want to first clarify two common “SNR” values used throughout the presentation of results:

SNR: The RAN4 definition of SNR, which is the average signal over noise power ratio as measured at the BB input branches of the equalizer/demodulator, averaged over each demodulator input branch and RE where the signal exists, i.e., it is the pre-EQ SNR. 
It is most often found as the “simulation setup SNR” and “relative TPUT result SNR”.

SINR: The average per layer post-EQ, i.e., measured after the application of a BB receiver/equalizer on the channel facing receive ports. 
It is most often found as the “simulation result of SINR profiles”.

Secondly, we don’t repeat all results here that have already been shown in the main body of this contribution here, but some repetition is necessary for ease of reading.

Per CW performance and CDL settings

Baseline

Figure 12. Per CW performance, CDLC Uma 30kph, 8 layers,
with random or fixed wideband TypeI-SP precoder, MCS0=17, MCS1=17.
(BS AE pattern as in 38.827, both CDL ray paring and polarization scalar initial phases random.)

CDL ray paring and polarization scalar initial phases

Figure 13. Per CW performance, CDLC Uma 30kph, 8 layers,
with random or fixed wideband TypeI-SP precoder, MCS0=17, MCS1=17.
(BS AE pattern as in 38.827, both CDL ray paring and polarization scalar initial phases fixed.)

BS AE pattern

Figure 14. Per CW performance, CDLC Uma 30kph, 8 layers,
with random or fixed wideband TypeI-SP precoder, MCS0=17, MCS1=17.
(BS AE pattern omnidirectional, both CDL ray paring and polarization scalar initial phases random.)

Precoder selection

| PMI i_{11} = 1,2 | PMI i_{11} = 2,3 |
| --- | --- |
| PMI i_{11} = 4,5 | PMI i_{11} = 6,7 |

Figure 15. Per CW performance, CDLC Uma 30kph, 8 layers,
different fixed wideband TypeI-SP precoders, MCS0=17, MCS1=17.
(BS AE pattern as in 38.827, both CDL ray paring and polarization scalar initial phases random.)

MCS sweeping

| MCS1=20 | MCS1=21 |
| --- | --- |
| MCS1=22 | MCS1=23 |

Figure 16. Per CW performance, CDLC Uma 30kph, 8 layers, 
MCS0=17, MCS1= variable (22 being right choice),
fixed wideband TypeI-SP precoders.
(BS AE pattern as in 38.827, both CDL ray paring and polarization scalar initial phases random.)

Appendix B – Potential CDL extensions

Multi-user handling

This contribution has focused on the SU-MIMO case However, spatial channel models are also indispensable for requirements concerning MU MIMO.

The inability of RAN4 UE demod in Rel-16 (NR_eMIMO) and Rel-17 (NR_feMIMO) to define requirements for MU precoding codebooks, due to the channel models and test setup not supporting inter-UE interference, is indicative of the need for spatial channel models.

Since CDL is a SCM, it is sufficient to offset AoAs between UEs, to achieve spatially separated UEs with correct large scale channel similarities. Such an extension can quickly be quickly agreed if MU MIMO requirements are targeted.

Precoding solution biasing extension

In TR 39.901 section 7.7.5, a comment is made concerning potential precoding solution biasing, when using CDL models as defined there, and how to fix this potential issue:

CDL extension: Scaling of angles The angle values of CDL models are fixed, which is not very suitable for MIMO simulations for several reasons; The PMI statistics can become biased, and a fixed precoder may perform better than open-loop and on par with closed-loop or reciprocity beamforming. Furthermore, a CDL only represents a single channel realization. The predefined angle values in the CDL models can be generalized by introducing angular translation and scaling. By translation, mean angle can be changed to and angular spread can be changed by scaling. The translated and scaled ray angles can be obtained according to the following equation…

While it is good to know, such an extension exists, it does not seem to be relevant for usage of CDL in RAN4 requirements. This may be needed in RAN1, when comparing different features/solutions against each other.
However in RAN4, the group is not concerned with selecting between features, RAN4 is concerned with setting a minimum performance for any and all implementations of a certain feature in specific test conditions.

Appendix C – CDL C UMa implementation

This appendix is mostly an excerpt from TR 38.827.
It summarizes all steps and parameters needed to implement the CDL C UMa channel used for comparisons in this contribution.

We have added comments on top of the text from TR 38.827, where we felt the original description needs clarification or extension to enable other interested parties to replicate our analysis.

## 7.1	General

The different channel models are defined to create corresponding complex multipath radio propagation conditions for FR1 and FR2. The following scenarios are selected for NR MIMO OTA:

FR1 scenarios:

-	For 2x2 MIMO: Urban Macro

-	For 4x4 MIMO: Urban Micro

FR2 static testing scenarios:

-	Urban Micro street canyon and Indoor

In order to describe unambiguously the procedure of generating realizations CDL channel models, various aspects need to be clarified, e.g., details of scaling procedure, inclusion of BS antenna arrays and beams to the model output and removing unwanted randomness of model realizations.

The concept of angular scaling is based on rotating AoDs/ZoDs and scaling CDL model using the methods in TR 38.901 (section 7.7.5.1) to make them fit the median values in TR 38.901 Table 7.5-6 for the accepted scenarios.

For NR MIMO OTA testing, the following channel models are required to be measured: FR1 UMi CDL-A in table 7.2.1-1, FR1 UMa CDL-C in table 7.2.1-8; FR2 InO CDL-A in table 7.2.2-6, FR2 UMi CDL-C in table 7.2.2-3.

For NR FR1 and FR2 MIMO OTA testing, the number of samples for sequence length at each testing point is FFS.

## 7.2	Channel Models

This section describes amendments to the step-wise procedure of the CDL subclause 7.7.1 in TR 38.901 for generating fast fading radio channel realizations. This channel model methodology considers non-Jakes spectrum with the multi-path fading propagation conditions between the gNB emulator and test chamber probe modelled based on Clustered Delay Line (CDL) methodology.

First, the RMS delay spread values of CDL models are normalized first and they must be scaled in delay so that a desired RMS delay spread can be achieved. The scaled delays can be obtained according to the following equation:

,	(7.2-1)

in which

-		is the normalized delay value of the nth cluster in a CDL in Tables 7.7.1.1 – 7.7.1.5 of [2]

-		is the new delay value (in [ns]) of the nth cluster

-		is the target delay spread (in [ns]).

Values of DSdesired for FR1/FR2 and for different model scenarios are specified in Table 7.2-1. Target delay spread values.Table 7.2-1.

Table 7.2-1. Target delay spread values.

| Frequency | Scenario | DSdesired |
| --- | --- | --- |
| FR1 | UMi | 100 ns |
| FR1 | UMa | 365 ns |
| FR2 | UMi | 60 ns |
| FR2 | InO | 30 ns |

Subsequently, the departure and arrival angles (based on subclause 7.7.1 step 1 in TR38.901 are generated by combining 7.7-5 and part of step 7 in subclause 7.5. The arrival angles of azimuth using are generated using the following equation

,	(7.2-2)

where

-	n,AOA and cASA are the cluster AOA and the cluster-wise rms azimuth spread of arrival angles (cluster ASA), respectively, in Tables 7.7.1.1 – 7.7.1.5 of TR38.901

-	m denotes the ray offset angles within a cluster given by Table 7.5-3,

-	 is the mean angle of the original channel model table in NLOS case (equation is specified in Annex A.2 of TR38.901) and the LOS angle  in LOS case,

-	Tables 7.2-2 and 7.2.-3 contain the non-circular angle spread values of the original CDL models of TR38.901 before any angular scaling, ASmodel are the angular spreads derived from the original CDL Tables 7.7.1.1 – 7.7.1.5 of TR38.901. TR25.996 describes μθ:

,	(7.2-3)

The values are calculated for the AOD, AOA, ZOD, and ZOA angles after removing the mean angle following the definition of rms angular spread in TR25.996, without finding the minimum over circular shifts. Here, the calculation is performed after removing the mean angle first and subsequently equation A-2 from Annex A of TR38.901

,	(7.2-4)

is used to rotate  to zero (and also wrap AOAs within +/-180). Equations A-3

,	(7.2-5)

and A-1 of TR 25.996

,	(7.2-6)

are used to calculate the ASmodel. Note that equation A-2 of TR 25.996 is not applied to ASmodel calculations, the following equation is used instead

ASdesired is the target angular spread. Table 7.2-4 specifies ASdesired values for CDL-A,B,C,D,E UMi and UMa at FR1 and Table 7.2-5 specifies the corresponding ASdesired values at FR2. These target values are obtained by determining median angular spreads of Table 7.5-6 of TR38.901.

The angular scaling is applied to the ray angles and no further scaling is performed. The generation of AOD (), ZOA (), and ZOD () follows a procedure similar to AOA as described above. Here, the azimuth angles may need to be wrapped around to be within [0, 360] degrees, while the zenith angles may need to be clipped to be within [0, 180] degrees.

Each CDL parameter table of contains two sets of three rows, i.e., three clusters, with exactly same angular parameters. This is harmful for the statistical properties of the models as they become non-WSS across the ensemble of model realizations. Instead of making the angular parameters non-equal by introducing small offsets to angles of the three rows, the problematic clusters are treated as midpaths as intended when the CDLs, where drawn from statistical distributions which works across all frequency ranges. For the clusters that look like midpaths, e.g., Cluster 2-4 and 5-7 for CDL-A and Cluster 2-4 and 6-8 for CDL-C, the powers for each of the three clusters are added and using the regular midpath power distribution of 0.5, 0.3, and 0.2 specified in Table 7.5-5 of TR 38.901, the powers for the each of the midpaths are calculated. Notice that the intra cluster delay spread in Table 7.5-5 of TR 38.901 is not followed, and the same delays as the original CDL are followed for the midpaths (aka Sub-Cluster). This helps keeping the rms DS of the modified CDL to 1s.

Table 7.2-2: Original (non-circular) angle spreads of CDL models UMi and UMa (K-factor 9 dB)

| Model | ASmodel [deg] |  |  |  |
| --- | --- | --- | --- | --- |
|  | ASD | ASA | ZSD | ZSA |
| CDL-A | 73.6985 | 85.2676 | 28.5575 | 21.0831 |
| CDL-B | 41.5917 | 59.3326 | 5.9633 | 10.3818 |
| CDL-C | 39.0949 | 71.1175 | 4.0666 | 10.4245 |
| CDL-D | 15.6771 | 17.3604 | 2.4462 | 1.5362 |
| CDL-E | 13.1544 | 37.5640 | 1.4577 | 2.4601 |

Table 7.2-3: Original (non-circular) angle spreads of CDL-D and CDL-E models InO (K-factor 7 dB)

| Model | ASmodel [deg] |  |  |  |
| --- | --- | --- | --- | --- |
|  | ASD | ASA | ZSD | ZSA |
| CDL-D | 18.9859 | 21.0747 | 2.9629 | 1.8735 |
| CDL-E | 15.7784 | 45.3434 | 1.7692 | 2.9982 |

Table 7.2-4: Desired AS for UMi and UMa at 3.5 GHz (FR1)

| Model | ASdesiredASdesiredASdesired [deg] |  |  |  |
| --- | --- | --- | --- | --- |
|  | ASD | ASA | ZSD | ZSA |
| UMi NLOS (CDL-A, B, C) | 23.9751 | 57.2457 | 0.7762 | 7.8320 |
| UMi LOS (CDL-D, E) | 15.0432 | 47.6149 | 0.6166 | 4.6204 |
| UMa NLOS (CDL-A, B, C) | 25.7620 | 74.1138 | 4.8978 | 18.2050 |
| UMa LOS (CDL-D, E) | 14.0180 | 64.5654 | 3.4674 | 8.9125 |
| Note: For UMa frequency fc = 6 as stated in [2], and other parameters hUMa = 25, hUMi = 10, hUT = 1.5, and D2D = 100. |  |  |  |  |

Table 7.2-5: Desired AS for UMi and InO at 28 GHz (FR2)

| Model | ASdesiredASdesiredASdesiredASdesiredASdesired [deg] |  |  |  |
| --- | --- | --- | --- | --- |
|  | ASD | ASA | ZSD | ZSA |
| UMi NLOS (CDL-A, B, C) | 15.6188 | 49.3183 | 0.7762 | 7.2695 |
| UMi LOS (CDL-D, E) | 13.7050 | 41.0212 | 0.6166 | 3.8350 |
| InO NLOS (CDL-A, B, C) | 41.6869 | 50.3659 | 12.0226 | 14.7109 |
| InO LOS (CDL-D, E) | 39.8107 | 31.8526 | 1.3702 | 11.4756 |

Subsequently, the AOD angles are coupled to AOA angles within a cluster n. Instead of random procedure, the coupling is performed using the fixed coupling pattern specified in Table 7.2-6. The same fixed coupling pattern is applied for all clusters n.

Table 7.2-6: Fixed coupling pattern of ray angles to be applied for each cluster

|  | m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 6 | 12 | 5 | 10 | 8 | 11 | 16 | 14 | 18 | 9 | 20 | 4 | 2 | 15 | 7 | 13 | 19 | 17 | 3 | 1 |
|  | 20 | 9 | 12 | 1 | 13 | 18 | 10 | 4 | 8 | 2 | 6 | 14 | 11 | 19 | 7 | 3 | 17 | 5 | 15 | 16 |
|  | 2 | 16 | 3 | 11 | 18 | 9 | 5 | 17 | 4 | 19 | 15 | 20 | 13 | 7 | 10 | 1 | 8 | 12 | 6 | 14 |
|  | 15 | 18 | 13 | 1 | 12 | 9 | 6 | 7 | 5 | 3 | 2 | 8 | 14 | 17 | 19 | 16 | 11 | 20 | 10 | 4 |

In the next steps, the linear cross polarization power ratios (XPR) are calculated for each ray m of each cluster n as

,	(7.2-7)

where X is the per-cluster XPR in dB from Tables 7.7.1.1 – 7.7.1.5 of TR38.901.

The gNB beam pattern including the assumptions for gNB antenna for definitions and symbols of subclause 7.3 of TR38.901 for FR1 and FR2 are summarized in Table 7.2-7.

Table 7.2-7: BS Antenna Parameters

| Parameter description | Symbol | Parameter value |  |  |
| --- | --- | --- | --- | --- |
|  |  | FR1 ≤2.5GHz | FR1 >2.5GHz | FR2 |
| Antenna panels in vertical dimension | Mg | 1 | 1 | 1 |
| Antenna panels in horizontal dimension | Ng | 1 | 1 | 1 |
| Elements per panel in vertical dimension | Me | 4 | 8 | 8 |
| Elements per panel in horizontal dimension | Ne | 8 | 8 | 16 |
| Number of polarizations per panel | P | 2 | 2 | 2 |
| Element spacing in horizontal dimension () | dH | 0.5 | 0.5 | 0.5 |
| Element spacing in vertical dimension () | dV | 0.5 | 0.5 | 0.5 |

Antenna element radiation patterns, including orientation of the element main polarization components as well as orientation of the antenna array for both FR1 and FR2 are as in the example pattern in Table 7.3-1 of TR38.901. The antenna element has ±45 polarization components and the radiation pattern parameters are 3dB = 65, 3dB = 65, Amax = 30dB, SLAv = 30dB, GE,max =8 dBi.

Note: The above antenna element (AE) radiation pattern (“F”) has been used in the presented simulations for the BS AEs.

However, it is recommended that for implementation alignment between contributions no AE radiation pattern is assumed (i.e., omnidirectional). In our opinion, the chosen AE pattern impacts the RMS delay spread scaling and necessitates coordinate transformations as defined in TR 38.901 section 7.3.2.

Initial alignment would be more straightforward without BS AE patterns, and neglecting this step does not impact the appropriateness of CDL for RAN4 requirement setting.

It is assumed the co-polarized elements of the array are combined to a single RF port, i.e. they compose an antenna array that can form beams by setting certain weights per element. Weight vector for the first polarization and for the second polarization is

α1,1…αMe,Ne=1MeNeexp-j2πrtx,max T ∙ dtx,me,neλ0∈C1×MeNe,	 (7.2-8)

where dtx,me,ne is the location vector of transmit antenna element me=1,…,Me and ne=1,…,Ne, and rtx,max is a spherical unit vector denoting the target beam direction. Determination of beam directions rtx,max is described in section 7.3.

Note: We understand the intent behind this AE weight definition is to create an abstraction between baseband (BB) visible RF ports, i.e., the Tx antennas in RAN4 requirements, and beamforming capabilities (due to many AEs) behind each BB visible RF port. Hence, one target beam direction is needed for 2 RF ports (2x2 MIMO), 2 directions for 4 ports (4x4), and 4 directions for 8 ports (8x8), with directions determined as in section 7.3.

The normalization of the weight vector by 1/MeNe above seems only fitting for the 2x2 case and only if taken to be the square root 1/MeNe. Furthermore, a deployed gNB DU is unlikely to have to scale the per AE Tx power, when changing using different AE numbers in the first place.

Regardless, since the RAN4 SNR definition is made at the input branches of the demodulator, any scaling factor assumptions here will be normalized out and do ultimately not impact performance.

Note: No antenna parameters (including position and orientation) are specified for the UE in TR 38.827.

We assume the UE Rx BB visible RF ports to be modelled as ULA (N1=4, N2=1) with 0.5 spacing, pointing its broadside to the transmitter, and rotation/slant is matched to the gNB X-pols.

Random initial phase  are not used for the different polarization combinations (θθ, θϕ, ϕθ, ϕϕ). Instead, a fixed and pre-defined set of initial phases Φmθθ,Φmθϕ,Φmϕθ,Φmϕϕ of Table 7.2-8 and a scalar random initial phase term Φn,m~Uni(-π,π) is used for each ray m of each cluster n.

Note: Our reading of “a scalar random initial phase term Φn,m~Uni(-π,π) is used for each ray m of each cluster n” is that a single draw of Φn,m is shared for all BB visible Tx/Rx branch pairs (s, u), of the current cluster and ray combination. This allows for UE antenna correlation to be introduced via UE AE positions.

The set of fixed initial phases can be same for all clusters, i.e. Φn,mθθ=Φmθθ ∀ n=1,…,N etc. for all four polarization combinations. These 20×4 initial phase values can be specified either by a table of values or by setting a random number generator and a fixed seed number. The distribution of scalar initial phases Φn,m is uniform within -π,π. Its purpose is to enable generation of different fading sequences on different uses of the model, but still maintaining the power angular distribution of the model. The scalar initial phases can be fixed (or removed) if completely deterministic process, i.e. exactly same fading sequences at each model use, is aimed at.

Table 7.2-8: Fixed initial phases for 2x2 polarization matrices. These values are drawn from uniform distribution

| m | Φmθθ [rad] | Φmϕθ [rad] | Φmθϕ [rad] | Φmϕϕ [rad] |
| --- | --- | --- | --- | --- |
| 1 | 1.7609 | -0.6928 | -1.6230 | -0.6037 |
| 2 | -2.5356 | -2.3124 | 2.7775 | 2.8660 |
| 3 | 0.4725 | -2.7660 | -1.6664 | -0.9226 |
| 4 | 2.0181 | -3.0448 | -2.8713 | -2.0798 |
| 5 | 0.9369 | 1.4560 | 0.9283 | -0.3084 |
| 6 | 0.2954 | -1.2798 | 1.5375 | -1.9544 |
| 7 | 1.1735 | -1.9886 | -0.8263 | 0.7893 |
| 8 | 1.7607 | -2.6319 | 2.6979 | 1.7324 |
| 9 | -0.0830 | -0.4030 | -0.3344 | -1.2167 |
| 10 | 0.0535 | 0.0677 | 1.9957 | 1.8525 |
| 11 | 0.9068 | -0.7627 | 1.9577 | 0.2062 |
| 12 | -0.9379 | 2.7583 | 2.3621 | 0.3151 |
| 13 | 0.7695 | 0.5469 | -1.8363 | -1.2488 |
| 14 | -0.1827 | -1.6934 | 2.1634 | -1.9179 |
| 15 | -1.7221 | -2.0690 | -1.7111 | -0.4040 |
| 16 | -1.1869 | 2.6602 | -0.4385 | -1.9804 |
| 17 | 2.5439 | 3.0143 | -0.3841 | -2.4434 |
| 18 | -1.5201 | -0.5735 | 0.5962 | -1.4941 |
| 19 | 0.6462 | 1.3271 | -1.7483 | -2.4038 |
| 20 | -1.2775 | -1.1386 | -0.4765 | 0.0494 |

To determine the channel all clusters are treated as "weaker cluster", i.e., no further sub-clusters in delay should be generated. The BS beamforming weights αme,neαme,nedefined in Equation 7.2-8 for antenna elements are used and the BS antenna signals are summed for BS beamforming. The BS transmits downlink signals with S beams. Index s=1,…,S denotes the formed beam index. Each beam may have different and thus the beamforming weight of eq. (7.2-8) becomes specific for index s as αs,me,ne; it should be noted though that there are always two orthogonally polarized beams to the same direction. Here, the random initial phases Φn,m are used for sub-paths, but not for the different polarization combinations (θθ, θϕ, ϕθ, ϕϕ). The channel coefficient for time instant t, Rx antenna/beam u, Tx beam s, and cluster n is defined by the following equations. They apply for the NLOS clusters and the LOS path, respectively:

Hu,s,nNLOSt=PnMm=1Mne=1Neme=1MeFrx,u,θθn,m,ZOA,φn,m,AOAFrx,u,φθn,m,ZOA,φn,m,AOATexpjΦmθθκn,m-1expjΦmθϕκn,m-1expjΦmϕθexpjΦmϕϕFtx,s,me,ne,θθn,m,ZOD,φn,m,AODFtx,s,me,ne,φθn,m,ZOD,φn,m,AOD

αs,me,neexpjΦn,mexpj2πrrx,n,mT.drx,uλ0expj2πrtx,n,mT.dtx,s,me,neλ0expj2πrrx,n,mT.vλ0t,	(7.2-9)

Note: The channel coefficient above is for time instant t. To form a channel impulse response, the absolute cluster delays pertaining to each (per cluster) coefficient, are listed later in section 7.2.1. The sub-cluster approach from TR 38.901 equation 7.5-27 is not applied.

Note: A single draw of Φn,m is shared for all BB visible Tx/Rx branch pairs (s, u), of the current cluster and ray combination. Hence the position vectors of the UE Rx antennas (drx,u) need to be well defined and forms the UE RX antenna correlation. Co-locating them would result in correlation factors of 1 between the same polarizations, and result in a rank 2 limited channel.

The position vectors are not well defined in TR 38.827 and need to be clarified. We assumed the ULA structure noted previously. This results in valid UE Rx antenna correlation modelling for both conducted testing and OTA testing, under the virtual cable (calibration) test setup assumption.

Note: The random initial phase term Φn,m can also be removed to achieve a deterministic but time varying system, as mentioned above. We chose to have them random, but removal should be considered for alignment.

Note: The UE antenna radiation patterns (Frx) are assumed to be omnidirectional. However, the values of Frx,u,θ/φθn,m,ZOA,φn,m,AOA can not be simply set to one. As we cannot perfectly align all X-pols from all clusters and directions at the same time, there will be a different distribution of energy between cross-polarization effects. LCS to GCS transformations are needed to find the polarization alignment mismatches.

LCS to GCS conversions were done using model-1 in TR 38. 901 section 7.3.2 but should be matching model-2 implementations. The collection of intermediate results may be needed in case of simulation alignment problems.

Hu,s,nLOSt=ne=1Neme=1MeFrx,u,θθLOS,ZOA,φLOS,AOAFrx,u,φθLOS,ZOA,φLOS,AOAT100-1Ftx,s,θθLOS,ZOD,φLOS,AODFtx,s,φθLOS,ZOD,φLOS,AODαs,me,ne

expj2πrrx,LOST∙drx,uλ0expj2πrtx,LOST∙dtx,s,me,neλ0expj2πrrx,LOST∙vλ0t ,                        (7.2-10)

where Ftx,s,me,ne,θ, Ftx,s,me,ne,φ, and dtx,s,me,ne are the theta and phi polarized radiation patterns and the position vector of the BS antenna element me,ne of sub-array s, respectively. Symbols Frx,u,θ , Frx,u,ϕ, , , and , are determined as in TR 38.901. UE velocity vector is determined as

(7.2-11)

UE velocity v is defined as follows: 30km/h for FR1 vs 3 km/h (Indoor Office) and 12 km/h (UMi) for FR2. The UE travelling direction (v, v) are as follows for FR1:

-	(135°,90o) for UMi CDL A channel model

-	([127.0455°],90o) for UMi CDL C channel model

-	([182.1659°],90o) for UMa CDL A channel model

-	(65°,90o) for UMa CDL C channel model

The UE travelling direction (v, v) are as follows for FR2:

-	(112.51°,90°) for InO CDL-A channel model

-	(74.11°,90°) for UMi CDL-C channel model

Note: Following the implementation of at least one TE vendor, the [] values above are confirmed to

FR1 UMa: CDL-A = (182.17°, 90°), B = (90°, 90°), C = (65°, 90°).

FR1 UMi: CDL-A = (135°, 90°), B = (90°, 90°). C = (127.05°, 90°).

And the FR2 cases may be inversed, though this has no impact in the CDL-C UMa case we are focusing on.

### 7.2.1	Channel Models for FR1

The Channel model parameter tables for CDL-A, B, C, D, and E for UMa and UMi at 3.5 GHz are presented in this subclause without the effect of base station antenna filtering.

For FR1, the baseline emulated propagation environment for FR1 MIMO OTA is 2D without elevation modelling, i.e., all ZOA are set 90° and ZSA is 0° in the following tables.

Tables 7.2.1-1 - 7.2.1-5 show the model parameters, UMi CDL-A—CDL-E models, respectively. For the determination of desired zenith spread of departure (ZSDdesired) from table 7.5-8 of TR38.901, the following parameters are used hBS = 10 m, hUT = 1.5 m, and d2d = 100 m.

Table 7.2.1-1: Channel model parameters for UMi CDL-A at 3.5 GHz

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -13.4014 | -59.324 | 98.721 | 95.9936 | 90 |
| 2 | 38.19 | 0 | -2.752 | -156.546 | 97.1624 | 90 |
| 3 | 40.25 | -2.2185 | -2.752 | -156.546 | 97.1624 | 90 |
| 4 | 58.68 | -3.9794 | -2.752 | -156.546 | 97.1624 | 90 |
| 5 | 46.1 | -5.9799 | 27.9576 | 115.7066 | 97.9452 | 90 |
| 6 | 53.75 | -8.1984 | 27.9576 | 115.7066 | 97.9452 | 90 |
| 7 | 67.08 | -9.9593 | 27.9576 | 115.7066 | 97.9452 | 90 |
| 8 | 57.5 | -10.5014 | 38.1399 | -55.2369 | 98.7118 | 90 |
| 9 | 76.18 | -7.5014 | -27.9638 | -82.1587 | 96.1295 | 90 |
| 10 | 153.75 | -15.9014 | 50.144 | 127.5226 | 95.3467 | 90 |
| 11 | 189.78 | -6.6014 | -28.3867 | 99.1238 | 98.0648 | 90 |
| 12 | 222.42 | -16.7014 | 42.4666 | -131.84 | 99.2935 | 90 |
| 13 | 217.18 | -12.4014 | -51.1586 | 82.1383 | 98.7444 | 90 |
| 14 | 249.42 | -15.2014 | -57.3396 | 115.7066 | 98.902 | 90 |
| 15 | 251.19 | -10.8014 | -43.6439 | -58.728 | 95.912 | 90 |
| 16 | 305.82 | -11.3014 | -45.6283 | -69.4699 | 95.7272 | 90 |
| 17 | 408.1 | -12.7014 | 52.4212 | -85.7169 | 95.806 | 90 |
| 18 | 445.79 | -16.2014 | 46.8909 | 138.3987 | 99.0271 | 90 |
| 19 | 456.95 | -18.3014 | 41.7835 | 161.2923 | 94.9226 | 90 |
| 20 | 479.66 | -18.9014 | -39.9679 | 168.543 | 95.083 | 90 |
| 21 | 500.66 | -16.6014 | -51.5165 | 132.7593 | 99.2962 | 90 |
| 22 | 530.43 | -19.9014 | 39.7665 | -155.942 | 95.2461 | 90 |
| 23 | 965.86 | -29.7014 | -19.6683 | 101.3393 | 98.5677 | 90 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 1.6266 | 7.385 | 0.0815 | 0 | 10 |  |

Table 7.2.1-2: Channel model parameters for UMi CDL-B at 3.5 GHz

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 3.2594 | -173.655 | 105.6621 | 90 |
| 2 | 10.72 | -2.2185 | 3.2594 | -173.655 | 105.6621 | 90 |
| 3 | 21.55 | -3.9794 | 3.2594 | -173.655 | 105.6621 | 90 |
| 4 | 20.95 | -3.2014 | -21.7581 | 127.2975 | 106.8987 | 90 |
| 5 | 28.7 | -9.8014 | -39.8007 | -91.3552 | 107.4194 | 90 |
| 6 | 29.86 | -1.2 | -8.6729 | 155.8564 | 105.3237 | 90 |
| 7 | 37.52 | -3.4185 | -8.6729 | 155.8564 | 105.3237 | 90 |
| 8 | 50.55 | -5.1794 | -8.6729 | 155.8564 | 105.3237 | 90 |
| 9 | 36.81 | -7.6014 | -40.8383 | -93.0919 | 107.2762 | 90 |
| 10 | 36.97 | -3.0014 | 28.1617 | 133.6653 | 105.1674 | 90 |
| 11 | 57 | -8.9014 | -43.6052 | -87.11 | 104.9592 | 90 |
| 12 | 52.83 | -9.0014 | 40.7281 | 98.1597 | 104.6858 | 90 |
| 13 | 110.21 | -4.8014 | -32.1917 | 106.2642 | 105.3497 | 90 |
| 14 | 127.56 | -5.7014 | -31.2117 | -91.1623 | 105.2325 | 90 |
| 15 | 154.74 | -7.5014 | 33.292 | -95.697 | 105.0893 | 90 |
| 16 | 178.42 | -1.9014 | 15.5376 | -140.658 | 105.2976 | 90 |
| 17 | 201.69 | -7.6014 | -43.8934 | -93.8638 | 104.9071 | 90 |
| 18 | 282.94 | -12.2014 | -54.327 | 62.7505 | 106.8857 | 90 |
| 19 | 302.19 | -9.8014 | -46.8333 | -82.6718 | 104.9722 | 90 |
| 20 | 361.87 | -11.4014 | -49.7155 | 69.6972 | 107.4584 | 90 |
| 21 | 410.67 | -14.9014 | -61.8207 | 57.058 | 107.3413 | 90 |
| 22 | 427.9 | -9.2014 | 41.4774 | 91.7918 | 107.2241 | 90 |
| 23 | 478.34 | -11.3014 | -46.8333 | -64.726 | 106.9508 | 90 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 5.7644 | 21.2262 | 0.3905 | 0 | 8 |  |

Table 7.2.1-3: Channel model parameters for UMi CDL-C at 3.5 GHz

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -4.4215 | -36.1891 | -122.2815 | 98.9242 | 90 |
| 2 | 20.99 | -1.25 | -21.5937 | 125.831 | 99.1915 | 90 |
| 3 | 22.19 | -3.4684 | -21.5937 | 125.831 | 99.1915 | 90 |
| 4 | 23.29 | -5.2294 | -21.5937 | 125.831 | 99.1915 | 90 |
| 5 | 21.76 | -2.5215 | -32.5709 | -143.6126 | 99.5732 | 90 |
| 6 | 63.66 | 0 | -7.4275 | 166.4003 | 99.306 | 90 |
| 7 | 64.48 | -2.2185 | -7.4275 | 166.4003 | 99.306 | 90 |
| 8 | 65.6 | -3.9794 | -7.4275 | 166.4003 | 99.306 | 90 |
| 9 | 65.84 | -7.4215 | 37.2175 | 73.8315 | 100.4513 | 90 |
| 10 | 79.35 | -7.1215 | -47.1664 | 82.7664 | 98.5616 | 90 |
| 11 | 82.13 | -10.7215 | 41.5716 | -79.6999 | 100.6231 | 90 |
| 12 | 93.36 | -11.1215 | -67.1585 | 66.9895 | 98.218 | 90 |
| 13 | 122.85 | -5.1215 | -41.5244 | 84.0543 | 100.165 | 90 |
| 14 | 130.83 | -6.8215 | -47.0437 | -96.2818 | 100.2604 | 90 |
| 15 | 217.04 | -8.7215 | -55.7519 | 94.8406 | 98.1225 | 90 |
| 16 | 271.05 | -13.2215 | 55.3698 | 53.9494 | 100.2604 | 90 |
| 17 | 425.89 | -13.9215 | 53.2234 | 16.0364 | 98.4852 | 90 |
| 18 | 460.03 | -13.9215 | 46.8456 | 32.2963 | 98.1416 | 90 |
| 19 | 549.02 | -15.8215 | -70.1021 | 18.2098 | 97.9698 | 90 |
| 20 | 560.77 | -17.1215 | 48.9306 | 37.0455 | 100.7376 | 90 |
| 21 | 630.65 | -16.0215 | 49.6052 | 33.7452 | 98.1225 | 90 |
| 22 | 663.74 | -15.7215 | 57.7615 | 29.801 | 98.1034 | 90 |
| 23 | 704.27 | -21.6215 | 65.6725 | 11.6092 | 100.4513 | 90 |
| 24 | 865.23 | -22.8215 | -83.5324 | 56.2837 | 100.9476 | 90 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 1.2265 | 12.0742 | 0.5726 | 0 | 7 |  |

Table 7.2.1-4: Channel model parameters for UMi CDL-D at 3.5 GHz

| Cluster # | Cluster PAS | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Specular (LOS path) | 0 | -0.2 | 0 | 180 | 98.5 | 90 |
|  | Laplacian | 0 | -13.8303 | 0 | -180 | 98.5 | 90 |
| 2 | Laplacian | 3.5275 | -19.1289 | 85.5931 | -69.0402 | 95.2232 | 90 |
| 3 | Laplacian | 61.6807 | -21.3474 | 85.5931 | -69.0402 | 95.2232 | 90 |
| 4 | Laplacian | 137.3705 | -23.1083 | 85.5931 | -69.0402 | 95.2232 | 90 |
| 5 | Laplacian | 141.6035 | -18.2289 | 12.4743 | 133.3735 | 98.2479 | 90 |
| 6 | Laplacian | 181.8169 | -20.4474 | 12.4743 | 133.3735 | 98.2479 | 90 |
| 7 | Laplacian | 261.6389 | -22.2083 | 12.4743 | 133.3735 | 98.2479 | 90 |
| 8 | Laplacian | 178.8941 | -23.2303 | 33.2009 | -62.0624 | 98.5 | 90 |
| 9 | Laplacian | 407.3746 | -28.1303 | -61.8919 | -109.358 | 95.9542 | 90 |
| 10 | Laplacian | 799.9337 | -23.9303 | -31.5697 | 36.555 | 96.6852 | 90 |
| 11 | Laplacian | 949.8016 | -25.1303 | 50.4731 | -14.3389 | 99.8359 | 90 |
| 12 | Laplacian | 978.4246 | -30.3303 | -126.759 | -71.2668 | 93.9125 | 90 |
| 13 | Laplacian | 1262.337 | -28.0303 | 74.0784 | 83.851 | 95.4753 | 90 |
| Per-Cluster Parameters |  |  |  |  |  |  |  |
| Parameter |  | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value |  | 4.7978 | 21.9419 | 0.7562 | 0 | 11 |  |

Table 7.2.1-5: Channel model parameters for UMi CDL-E at 3.5 GHz

| Cluster # | Cluster PAS | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Specular (LOS path) | 0 | -0.03 | 0 | 180 | 99.6 | 90 |
|  | Laplacian | 0 | -22.2442 | 0 | -180 | 99.6 | 90 |
| 2 | Laplacian | 49.9706 | -16.0427 | 65.756 | -25.0925 | 101.5457 | 90 |
| 3 | Laplacian | 52.9593 | -18.2612 | 65.756 | -25.0925 | 101.5457 | 90 |
| 4 | Laplacian | 54.8089 | -20.0221 | 65.756 | -25.0925 | 101.5457 | 90 |
| 5 | Laplacian | 52.9593 | -23.1142 | -22.986 | 80.8762 | 99.5154 | 90 |
| 6 | Laplacian | 69.2365 | -22.6142 | 18.526 | 94.9462 | 100.1076 | 90 |
| 7 | Laplacian | 185.8637 | -18.8128 | 10.6353 | -148.945 | 99.2616 | 90 |
| 8 | Laplacian | 187.8204 | -21.0313 | 10.6353 | -148.945 | 99.2616 | 90 |
| 9 | Laplacian | 190.702 | -22.7922 | 10.6353 | -148.945 | 99.2616 | 90 |
| 10 | Laplacian | 257.2613 | -22.5142 | 21.7281 | -133.48 | 100.1076 | 90 |
| 11 | Laplacian | 361.5249 | -25.8142 | 37.3951 | -71.8765 | 98.2465 | 90 |
| 12 | Laplacian | 530.7998 | -20.4142 | 0.5718 | 138.1703 | 99.3039 | 90 |
| 13 | Laplacian | 1168.55 | -30.0142 | 63.9262 | 2.2763 | 97.9081 | 90 |
| 14 | Laplacian | 2009.522 | -29.4142 | 65.8703 | 15.2055 | 101.7149 | 90 |
| Per-Cluster Parameters |  |  |  |  |  |  |  |
| Parameter |  | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value |  | 5.7179 | 13.9432 | 1.2689 | 0 | 8 |  |

Tables 7.2.1-6—7.2.1-10 tabulate channel model parameters for UMa CDL-A—CDL-E models at 3.5 GHz, respectively.

In the determination of desired angle spreads (ASdesired), frequency is set 6 GHz as stated in Table 7.5-6 Part-1 of TR38.901.

For the determination of desired zenith spread of departure (ZSDdesired) from table 7.5-7 of TR38.901, the following parameters are used hUT = 1.5 m, and d2d = 100 m.

Table 7.2.1-6: Channel model parameters for UMa CDL-A at 3.5 GHz

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -13.4014 | -63.5923 | 70.1754 | 89.1998 | 90 |
| 2 | 139.3935 | 0 | -2.804 | -154.231 | 96.5746 | 90 |
| 3 | 146.9125 | -2.2185 | -2.804 | -154.231 | 96.5746 | 90 |
| 4 | 214.182 | -3.9794 | -2.804 | -154.231 | 96.5746 | 90 |
| 5 | 168.265 | -5.9799 | 30.1944 | 92.1659 | 101.5139 | 90 |
| 6 | 196.1875 | -8.1984 | 30.1944 | 92.1659 | 101.5139 | 90 |
| 7 | 244.842 | -9.9593 | 30.1944 | 92.1659 | 101.5139 | 90 |
| 8 | 209.875 | -10.5014 | 41.1356 | -23.0699 | 106.3504 | 90 |
| 9 | 278.057 | -7.5014 | -29.8948 | -57.9245 | 90.0573 | 90 |
| 10 | 561.1875 | -15.9014 | 54.0343 | 107.4637 | 85.1179 | 90 |
| 11 | 692.697 | -6.6014 | -30.3493 | 70.6969 | 102.2686 | 90 |
| 12 | 811.833 | -16.7014 | 45.7847 | -122.245 | 110.0206 | 90 |
| 13 | 792.707 | -12.4014 | -54.8184 | 48.7064 | 106.5562 | 90 |
| 14 | 910.383 | -15.2014 | -61.46 | 92.1659 | 107.551 | 90 |
| 15 | 916.8435 | -10.8014 | -46.7436 | -27.5897 | 88.6853 | 90 |
| 16 | 1116.243 | -11.3014 | -48.8759 | -41.4968 | 87.519 | 90 |
| 17 | 1489.565 | -12.7014 | 56.4812 | -62.5312 | 88.0164 | 90 |
| 18 | 1627.1335 | -16.2014 | 50.5387 | 121.5446 | 108.3399 | 90 |
| 19 | 1667.8675 | -18.3014 | 45.0506 | 151.184 | 82.4424 | 90 |
| 20 | 1750.759 | -18.9014 | -42.7936 | 160.5712 | 83.4543 | 90 |
| 21 | 1827.409 | -16.6014 | -55.2029 | 114.2434 | 110.0378 | 90 |
| 22 | 1936.0695 | -19.9014 | 42.8834 | -153.449 | 84.4834 | 90 |
| 23 | 3525.389 | -29.7014 | -20.9811 | 73.5652 | 105.4414 | 90 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 1.7478 | 9.5611 | 0.5145 | 0 | 10 |  |

Table 7.2.1-7: Channel model parameters for UMa CDL-B at 3.5 GHz

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 3.8721 | -170.785 | 105.7717 | 90 |
| 2 | 39.128 | -2.2185 | 3.8721 | -170.785 | 105.7717 | 90 |
| 3 | 78.6575 | -3.9794 | 3.8721 | -170.785 | 105.7717 | 90 |
| 4 | 76.4675 | -3.2014 | -23.0099 | 112.7684 | 113.5742 | 90 |
| 5 | 104.755 | -9.8014 | -42.3972 | -64.2347 | 116.8595 | 90 |
| 6 | 108.989 | -1.2 | -8.9495 | 149.7425 | 103.6362 | 90 |
| 7 | 136.948 | -3.4185 | -8.9495 | 149.7425 | 103.6362 | 90 |
| 8 | 184.5075 | -5.1794 | -8.9495 | 149.7425 | 103.6362 | 90 |
| 9 | 134.3565 | -7.6014 | -43.5122 | -66.4831 | 115.956 | 90 |
| 10 | 134.9405 | -3.0014 | 30.6303 | 121.0127 | 102.6507 | 90 |
| 11 | 208.05 | -8.9014 | -46.4853 | -58.7386 | 101.3365 | 90 |
| 12 | 192.8295 | -9.0014 | 44.1333 | 75.0448 | 99.6118 | 90 |
| 13 | 402.2665 | -4.8014 | -34.2211 | 85.5375 | 103.8005 | 90 |
| 14 | 465.594 | -5.7014 | -33.1681 | -63.9849 | 103.0613 | 90 |
| 15 | 564.801 | -7.5014 | 36.143 | -69.8558 | 102.1579 | 90 |
| 16 | 651.233 | -1.9014 | 17.0654 | -128.065 | 103.472 | 90 |
| 17 | 736.1685 | -7.6014 | -46.795 | -67.4824 | 101.008 | 90 |
| 18 | 1032.731 | -12.2014 | -58.0062 | 29.2019 | 113.4921 | 90 |
| 19 | 1102.994 | -9.8014 | -49.9539 | -52.9926 | 101.4187 | 90 |
| 20 | 1320.826 | -11.4014 | -53.051 | 38.1956 | 117.1059 | 90 |
| 21 | 1498.946 | -14.9014 | -66.0584 | 21.8321 | 116.3667 | 90 |
| 22 | 1561.835 | -9.2014 | 44.9385 | 66.8006 | 115.6275 | 90 |
| 23 | 1745.941 | -11.3014 | -49.9539 | -29.7588 | 113.9027 | 90 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 6.194 | 27.4808 | 2.464 | 0 | 8 |  |

Table 7.2.1-8: Channel model parameters for UMa CDL-C at 3.5 GHz

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -4.4215 | -37.4195 | -96.4031 | 96.7645 | 90 |
| 2 | 76.6135 | -1.25 | -21.7362 | 118.7405 | 98.4506 | 90 |
| 3 | 80.9935 | -3.4684 | -21.7362 | 118.7405 | 98.4506 | 90 |
| 4 | 85.0085 | -5.2294 | -21.7362 | 118.7405 | 98.4506 | 90 |
| 5 | 79.424 | -2.5215 | -33.5316 | -124.0196 | 100.8594 | 90 |
| 6 | 232.359 | 0 | -6.5142 | 171.2639 | 99.1732 | 90 |
| 7 | 235.352 | -2.2185 | -6.5142 | 171.2639 | 99.1732 | 90 |
| 8 | 239.44 | -3.9794 | -6.5142 | 171.2639 | 99.1732 | 90 |
| 9 | 240.316 | -7.4215 | 41.4581 | 51.4188 | 106.3995 | 90 |
| 10 | 289.6275 | -7.1215 | -49.2149 | 62.9864 | 94.4761 | 90 |
| 11 | 299.7745 | -10.7215 | 46.1367 | -41.2744 | 107.4834 | 90 |
| 12 | 340.764 | -11.1215 | -70.697 | 42.5606 | 92.3083 | 90 |
| 13 | 448.4025 | -5.1215 | -43.1524 | 64.6538 | 104.5929 | 90 |
| 14 | 477.5295 | -6.8215 | -49.0831 | -62.7423 | 105.1951 | 90 |
| 15 | 792.196 | -8.7215 | -58.4403 | 78.6184 | 91.7061 | 90 |
| 16 | 989.3325 | -13.2215 | 60.9633 | 25.6781 | 105.1951 | 90 |
| 17 | 1554.4985 | -13.9215 | 58.6569 | -23.4063 | 93.9944 | 90 |
| 18 | 1679.1095 | -13.9215 | 51.8037 | -2.3553 | 91.8265 | 90 |
| 19 | 2003.923 | -15.8215 | -73.86 | -20.5926 | 90.7426 | 90 |
| 20 | 2046.8105 | -17.1215 | 54.0442 | 3.7933 | 108.2061 | 90 |
| 21 | 2301.8725 | -16.0215 | 54.7691 | -0.4794 | 91.7061 | 90 |
| 22 | 2422.651 | -15.7215 | 63.5332 | -5.5859 | 91.5856 | 90 |
| 23 | 2570.5855 | -21.6215 | 72.0338 | -29.1381 | 106.3995 | 90 |
| 24 | 3158.0895 | -22.8215 | -88.2912 | 28.7003 | 109.5309 | 90 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 1.3179 | 15.632 | 3.6131 | 0 | 7 |  |

Table 7.2.1-9: Channel model parameters for UMa CDL-D at 3.5 GHz

| Cluster # | Cluster PAS | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Specular (LOS path) | 0 | -0.2 | 0 | 180 | 98.5 | 90 |
|  | Laplacian | 0 | -13.8303 | 0 | -180 | 98.5 | 90 |
| 2 | Laplacian | 12.8753 | -19.1289 | 79.7601 | -157.697 | 80.0732 | 90 |
| 3 | Laplacian | 225.1344 | -21.3474 | 79.7601 | -157.697 | 80.0732 | 90 |
| 4 | Laplacian | 501.4023 | -23.1083 | 79.7601 | -157.697 | 80.0732 | 90 |
| 5 | Laplacian | 516.8527 | -18.2289 | 11.6242 | 116.7749 | 97.0826 | 90 |
| 6 | Laplacian | 663.6315 | -20.4474 | 11.6242 | 116.7749 | 97.0826 | 90 |
| 7 | Laplacian | 954.9819 | -22.2083 | 11.6242 | 116.7749 | 97.0826 | 90 |
| 8 | Laplacian | 652.9634 | -23.2303 | 30.9383 | -20.0777 | 98.5 | 90 |
| 9 | Laplacian | 1486.917 | -28.1303 | -57.6741 | 147.6324 | 84.1838 | 90 |
| 10 | Laplacian | 2919.758 | -23.9303 | -29.4182 | -14.5102 | 88.2944 | 90 |
| 11 | Laplacian | 3466.776 | -25.1303 | 47.0334 | 44.6351 | 106.0125 | 90 |
| 12 | Laplacian | 3571.25 | -30.3303 | -118.12 | 95.5983 | 72.7025 | 90 |
| 13 | Laplacian | 4607.53 | -28.0303 | 69.03 | 177.7798 | 81.4907 | 90 |
| Per-Cluster Parameters |  |  |  |  |  |  |  |
| Parameter |  | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value |  | 4.4709 | 29.753 | 4.2523 | 0 | 11 |  |

Table 7.2.1-10: Channel model parameters for UMa CDL-E at 3.5 GHz

| Cluster # | Cluster PAS | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Specular (LOS path) | 0 | -0.03 | 0 | 180 | 99.6 | 90 |
|  | Laplacian | 0 | -22.2442 | 0 | -180 | 99.6 | 90 |
| 2 | Laplacian | 182.3926 | -16.0427 | 61.2748 | -98.1037 | 110.5415 | 90 |
| 3 | Laplacian | 193.3013 | -18.2612 | 61.2748 | -98.1037 | 110.5415 | 90 |
| 4 | Laplacian | 200.0527 | -20.0221 | 61.2748 | -98.1037 | 110.5415 | 90 |
| 5 | Laplacian | 193.3013 | -23.1142 | -21.4195 | 45.5889 | 99.1243 | 90 |
| 6 | Laplacian | 252.7131 | -22.6142 | 17.2635 | 64.6678 | 102.4543 | 90 |
| 7 | Laplacian | 678.4024 | -18.8128 | 9.9105 | -137.889 | 97.6971 | 90 |
| 8 | Laplacian | 685.5446 | -21.0313 | 9.9105 | -137.889 | 97.6971 | 90 |
| 9 | Laplacian | 696.0624 | -22.7922 | 9.9105 | -137.889 | 97.6971 | 90 |
| 10 | Laplacian | 939.0038 | -22.5142 | 20.2473 | -116.92 | 102.4543 | 90 |
| 11 | Laplacian | 1319.566 | -25.8142 | 34.8467 | -33.3854 | 91.9885 | 90 |
| 12 | Laplacian | 1937.419 | -20.4142 | 0.5328 | 123.2792 | 97.935 | 90 |
| 13 | Laplacian | 4265.208 | -30.0142 | 59.5698 | 67.1651 | 90.0856 | 90 |
| 14 | Laplacian | 7334.755 | -29.4142 | 61.3814 | 84.697 | 111.493 | 90 |
| Per-Cluster Parameters |  |  |  |  |  |  |  |
| Parameter |  | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value |  | 5.3282 | 18.9069 | 7.1358 | 0 | 8 |  |

## 7.3	Channel Model emulation of the Base Station beamforming configuration

The basic parameters of NR BS antenna is specified in table 7.2-7. The propagation environment generated in the test zone is channel model defined in section 7.2 with base station antenna filtering effect. For the channel model emulation in the chamber, the beamforming characteristic of the BS pattern is defined as follow:

-	For FR1: A code book of 60 fixed beams is constructed to a grid of five elevation angles from –20 to +20 with 10 steps and 12 azimuth angles from –80 to +80 with ~15 steps；

-	For FR2: A code book of 128 fixed beams is constructed to a grid of eight elevation angles from –25 to +25 with ~7.1 step size and 16 azimuth angles from –60 to +60 with 8 step size；

Note: This 60/128 beam rastering of limited solid angles and step sizes, quantizes usable beam directions to match the strongest clusters. The quantization result of 8x8 is added below.

For NR FR1 MIMO OTA, 2 strongest transmitting beams are selected from the pre-defined beam grid based on their proximity to the strong clusters of each FR1 channel model.

For FR1 4x4 MIMO OTA, two strongest transmitting beams are selected from the pre-defined beam grid based on their proximity to the strong clusters of each FR1 channel model. These beams should have different azimuth directions and can provide the highest receive power for UE.

For FR1 2x2 MIMO OTA, 1 strongest transmitting beam is selected from the pre-defined beam grid which provides the highest received power for UE based on the FR1 channel model.

-	In detail, beam directions for channels model given in Clause 7.2.1 are

-	For UMa CDL-C, the beam directions are:

-	Strongest beam: AoD: -7.27°, ZoD: 100°

-	2nd strongest beam: AoD: -21.82°, ZoD: 100°

-	For UMi CDL-C, the strongest beam direction is: AoD: -7.27°, ZoD: 100°.

For FR1 8x8 MIMO OTA, the four strongest transmitting beams are selected from the pre-defined beam grid based on their proximity to the strong clusters of each FR1 channel model. These beams should have different azimuth directions and can provide the highest receive power for UE.

-	In detail, beam directions for channels model given in Clause 7.2.1 are

-	For UMa CDL-C, the beam directions are:

-	Strongest beam: AoD: -7.27°, ZoD: 100°

-	2nd strongest beam: AoD: -21.82°, ZoD: 100°

-	3rd strongest beam: AoD: -36.36°, ZoD: 100°

-	4th strongest beam: AoD: -50.91°, ZoD: 100°

For NR FR2 MIMO OTA, 1 strongest transmitting beam is generated from BS, the direction of this beam towards the strongest cluster of each FR2 channel model. In detail, the directions in CDL-A InO and CDL-C UMi models are (-4.0°, 93.6°) and (-12.0°, 100.7°), respectively.