3GPP TSG-RAN WG4 Meeting # 110		R4-2402302

# Athens, GR, 26 Feb – 01 Mar, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.871 | CR | 0003 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (FS_NR_FR2_OTA_enh) CR to TR 38.871 for demodulation testing enhancement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_FR2_OTA_enh |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Inverse channel approach was discussed to achieve target minimum isolation for Demod testing but not captured in TR 38.871. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To capture inverse channel aprpach as an alternaive method to improve isolation and corresponding MU. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inverse channel aprpach can not be used as an alternaive method to improve isolation and corresponding MU. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.2.2, Annex D. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

====================Start of the changes==========================

#### 7.2.2.2	Criteria of UE test directions

The test directions and AoA separation for demodulation test can be declared by UE. The UE declared test directions shall satisfy the following 3 criteria:

-	Minimum isolation shall be at least [-12]dB

-	Rank number shall be higher or same as intended rank for a given test case

-	Each direction of AoA pair shall pass legacy REFSENS requirements with [3] dB degradation.

Note: the minimum isolation of [-12]dB is reused from legacy FR2 up to 2 layer demodulation testing. For multi-Rx chain DL reception demodulation testing, RAN4 recommend the target of the required SNR difference between with and without additional interference introduced by chamber, i.e., MU due to non-ideal isolation, is 1dB. The final minimum isolation and corresponding MU defer the discussion to RAN5. RAN4 also investigated the inverse channel approach and concluded that RSRPB reporting based inverse channel approach can ba used to improve isolation and corresponding MU. An example of inverse channel approach is listed in Annex D.

====================End of the changes===========================

====================Start of the changes==========================

Annex D:
Inverse Channel Approach in demodulation testing

# D.1	An example of Inverse Channel Approach

This Annex describes an example of inverse channel approach to minimize the impact of OTA chamber. With OTA testing for demodulation, the received signal can be given by:

y=HOTA Hc x,                                                                             (D.1-1)

where Hc is the target channel efficiency defined in TS 38.101-4 and HOTA is the additional interference introduced by OTA chamber.

It is assumed that the interference between two TRPs and two polarizations as ηijTaRb. Here Ta, a∈1,2, and Rb,b∈1,2, denote the OTA chamber pair of transmit probe Ta and receive probe Rb. Here i, i∈1,2, and j,j∈1,2denote the polarization of transmit and receive probe; in which if i=j they are the same polarization and if i≠j they are different polarization. Here x and y are the transmit signal and received signal respectively.

y1R1y2R1y1R2y2R2=η11T1R1η21T1R1η12T1R1η22T1R1η11T2R1η21T2R1η12T2R1η22T2R1η11T1R2η12T1R2η12T1R2η22T1R2η11T2R2η12T2R2η21T2R2η22T2R2.HC.x1T1x2T1x1T2x2T2                                        (D.1-2)

If the impact of OTA chamber, i.e., HOTA could be removed or minimized in the testing, then the testing for multi-Rx demodulation would be more accurate. The principle of inverse channel approach is to re-construct inverse of channel matrix of OTA chamber to remove or immunize the impact of HOTA.

RSRPB has been used for checking isolation between branches in legacy FR2 demodulation testing. In multi-Rx demodulation testing, RSRPB could also be used to enhance the minimum isolation, i.e., minimize the impact of OTA chamber.

An example of test procedures could be listed as below:

- Step 1: DUT declares AoA pairs which shall satisfy the 3 criteria defined in section 7.2.2.2 of TR 38.871 including [-12dB] minimum isolation requirement.

- Step 2: Test Equipment to check the isolation for all the branches based on UE RSRPB reporting and calculate HOTA by following steps:

  - TE sends SSB on pol. H from TRP1 (T1). UE measures and reports RSRPB of pol. H and pol. V at antenna panel 1 (R1). Similarly, UE measures and reports RSRPB of pol. H and pol. V at antenna panel 2 (R2).

    - To get η by following equations:

      - η12T1R1= RSRPB_V of R1/RSTx_H of T1

      - η11T1R2= RSRPB_H of R2/ RSTx_H of T1

      - η12T1R2= RSRPB_V of R2/ RSTx_H of T1

      - η11T1R1 = RSRPB_H of R1/ RSTx_H of T1

  - TE sends SSB on pol. V from TRP1 (T1). UE measures and reports RSRPB of pol. H from antenna panel 1 (R1). UE measures and reports pol. V and pol. H from antenna panel 2 (R2).

    - To get  η by following equations:

      - η21T1R1= RSRPB_H of R1/RSTx_V of T1

      - η12T1R2= RSRPB_H of R2/RSTx_V of T1

      - η22T1R2= RSRPB_V of R2/RSTx_V of T1

      - η22T1R1 = RSRPB_V of R1/RSTx_V of T1

  - TE sends SSB on pol. H from TRP2 (T2). UE measures and reports RSRPB of pol. V from antenna panel 2 (R2). UE measures pol. V and pol. H from antenna panel 1 (R1).

    - To get  η by following equations:

      - η11T2R1= RSRPB_H of R1/RSTx_H of T2

      - η12T2R1= RSRPB_V of R1/RSTx_H of T2

      - η21T2R2= RSRPB_V of R2/RSTx_H of T2

      - η11T2R2= RSRPB_H of R2/RSTx_H of T2

  - TE sends SSB on pol. V from TRP2 (T2). UE measures and reports RSRPB of pol. H from antenna panel 2 (R2). UE measures and reports pol. V and pol H from antenna panel 1 (R1).

    - To get η by following equations:

      - η21T2R1= RSRPB_H of R1/RSTx_V of T2

      - η22T2R1= RSRPB_V of R2/RSTx_V of T2

      - η12T2R2 = RSRPB_H of R2/RSTx_V of T2

      - η22T2R2= RSRPB_V of R2/RSTx_V of T2

  - Note that here RSRPB_i of Ra means RSRPB value reported by UE on pol. i from antenna panel a, and RSTx_i of Tb means Tx power of reference signal from TRP b. η is the estimated channel coefficients based on RSRPB.

- Step 3: Test equipment to apply the pseudo-inverse/inverse of HOTA to achieve the target of the required SNR difference between with and without additional interference introduced by chamber, i.e., MU due to non-ideal isolation, is 1dB.

y1R1y2R1y1R2y2R2=inverseη11T1R1η21T1R1η12T1R1η22T1R1 η11T2R1 η21T2R1 η12T2R1 η22T2R1η11T1R2η12T1R2η12T1R2η22T1R2 η11T2R2 η12T2R2 η21T2R2 η22T2R2.HOTA.HC.x1T1x2T1x1T2x2T2            (D.1-3)

====================End of the changes===========================