3GPP TSG-RAN4 Meeting #110	R4-2402331

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

Source:				Ericsson

Title:					NTN Enhancement – VSAT spurious requirement

Agenda item:		8.18.5.1

Document for:		Approval

# Introduction

In last RAN4#109 meeting, the drafting of NTN-FR2 UE specifications made little progress, many issues remained open. One of those issues is related to the NTN VSAT spurious requirements and their specification.

This contribution is discussing this aspect, focusing on band n512 and proposing a way forward to progress on spurious requirements drafting.

# Discussion

## CEPT and ETSI

### Background

CEPT has published ERC 74-01 ([1]) which is a CEPT Recommendation on unwanted emissions in the spurious domain. This document is an important reference to the ETSI Harmonized Standards which are supposed to be compliant with this Recommendation.

In the scope of the NTN enhancement WI and the specification of NTN band n512, ETSI has published several Harmonized Standards which should be used as input for 3GPP:

- EN 303 978 ([2]): covering requirements for ESOMP in the 27.5 - 30.0 GHz.

- EN 301 360 ([3]): covering requirements for terminals in the 29.5 – 30.0 GHz (GSO satellite).

- EN 301 459 ([4]): covering requirements for terminals in the 27.5 – 29.5 GHz (GSO satellite).

### General

Most of the spurious limits are specified in dBpW in both CEPT and ETSI documents. We propose to convert those limits in dBm (=dBpW – 90) as this is a unit which is more used in RAN4.

### Off-axis spurious

The off-axis spurious are requirements applicable for all off-axis angles greater than 7°.

The requirements are depending on the VSAT radio states, if the VSAT is transmitting or not.

#### 2.2.2.1	ERC 74-01

To our understanding, ERC 74-01 ([1]) only specifies requirements for this off-axis case (table 10, note 3).

Also, in transmission state, there are exceptions (table 10, note 4ter) depending on where in the band the VSAT is transmitting.

In teh following subsections, for each EN, we are comparing the Off-axis spurious requirements with ERC 74-01 ([1]) which is supposed to be the input from CEPT Administrations for all ENs.

#### 2.2.2.2	EN 303 978 (mobile VSAT)

EN 303 978 ([2]) specifies the following off-axis spurious requirements:

Comparing to ERC 74-01 ([1]), we noticed the following:

- The frequency range 1.0-2.0 GHz has been added comparing to ERC 74-01. As the limit for that range is more stringent than the one for 1.0-3.4 GHz in ERC 74-01, this is not an issue.

- The measurement bandwidth for the ”normal” requirements (not the exceptions) is 1 MHz in this EN, while it’s 100 kHz in ERC 74-01. The EIRP limits have been scaled accordingly. With this larger measurement bandwidth, those requirements are slightly relaxed comparing ERC 74-01.

- In transmission state, the exceptions (85 dBpW) have been generalized to the all band, not considering the specific frequency range where each exception is applicable (ERC 74-01, table 10, note 4ter). The requirements have then been relaxed, allowing exceptions for the all band while it should have been done for part of the band only.

- In transmission state, the limit of 85 dBpW has been specified for the frequency range 29.5-30.0 GHz while, according the ERC 74-01, table 10, note 4ter, there is no limit set for that frequency range.

#### 2.2.2.3	EN 301 360 (fixed VSAT operating in 29.5 – 30.0 GHz, GSO satellite)

EN 301 360 ([3]) specifies the following off-axis spurious requirements:

Comparing to ERC 74-01 ([1]), we noticed the following:

- The frequency range 1.0-2.0 GHz has been added comparing to ERC 74-01. As the limit for that range is more stringent than the one for 1.0-3.4 GHz in ERC 74-01, this is not an issue.

- The spurious requirements are specified up to 40.0 GHz only while, according to ERC-74-01 table 1, they should be specified up to the 2nd harmonic, i.e. 60 GHz for the Ka-band.

#### 2.2.2.4	EN 301 459 (fixed VSAT operating in 27.5 – 29.5 GHz, GSO satellite)

#### EN 301 459 ([4]) specifies the following off-axis spurious requirements:

Comparing to ERC 74-01 ([1]), we noticed the following:

- The frequency range 1.0-2.0 GHz has been added comparing to ERC 74-01. As the limit for that range is more stringent than the one for 1.0-3.4 GHz in ERC 74-01, this is not an issue.

- The spurious requirements are specified up to 40.0 GHz only while, according to ERC-74-01 table 1, they should be specified up to the 2nd harmonic, i.e. 60 GHz for the Ka-band.

#### Conclusion

Based on the previous observations, trying to comply with ERC 74-01 (with priority) and ETSI Harmonized Standards, we would propose the following:

Proposal1: Specify NTN VSAT Off-axis requirements for band n512 according to the following:

- Convert dBpW values in dBm (=dBpW – 90)

- Consider the same Off-axis spurious requirements for both fixed and mobile VSAT.

- Keep the frequency range 1.0-2.0 GHz (ETSI Harmonized Standards).

- Consider 100 kHz measurement bandwidth for the ”normal” limits (ERC 74-01, EN 301 360 and EN 301 459)

- Consider the 85 dBpW exceptions for the relevant frequency range (as specified in ERC 74-01, EN 301 360 and EN 301 459).

Based on this proposal, the following On-axis spurious requirement should then be specified, see ([5]).

Table: Radiated field strength limits at 10m

| Frequency range | EIRP Limit | Measurement bandwidth |
| --- | --- | --- |
| 30 MHz - 230 MHz | 30 dBμV/m | 120 kHz |
| 230 MHz - 1000 MHz | 37 dBμV/m | 120 kHz |

Table: Off-axis spurious limits in “Emissions disabled” state

| Frequency range | EIRP Limit | Measurement bandwidth |
| --- | --- | --- |
| 1.0 GHz – 2.0 GHz | 42 dBpW | 100 kHz |
| 2.0 GHz – 10.7 GHz | 48 dBpW | 100 kHz |
| 10.7 GHz – 21.2 GHz | 54 dBpW | 100 kHz |
| 21.2 GHz – 60.0 GHz | 60 dBpW | 100 kHz |

Table: Off-axis spurious limits in “Carrier-on” and “Carrier-off” states

| Frequency range | EIRP Limit | Measurement bandwidth |
| --- | --- | --- |
| 1.00 GHz – 3.40 GHz | 49 dBpW | 100 kHz |
| 3.40 GHz – 10.70 GHz | 55 dBpW | 100 kHz |
| 10.70 GHz – 21.20 GHz | 61 dBpW | 100 kHz |
| 21.20 GHz – 27.35 GHz | 67 dBpW | 100 kHz |
| 27.35 GHz – 27.50 GHz | 85 dBpW (note 1) | 1 MHz |
|  | 67 dBpW (note 2) | 100 kHz |
| 27.50 GHz – 29.35 GHz | 85 dBpW (note 1) | 1 MHz |
|  | 67 dBpW (note 2) | 100 kHz |
| 29.35 GHz – 29.50 GHz | 85 dBpW | 1 MHz |
| 30.00 GHz – 30.15 GHz | 85 dBpW | 1 MHz |
| 30.15 GHz – 60.00 GHz | 67 dBpW | 100 kHz |
| Note 1: For fixed VSAT transmitting in the frequency range 29.5 – 30.0 GHz Note 2: For fixed VSAT transmitting in the frequency range 27.5 – 29.5 GHz |  |  |

If RAN4 would like to simplify the limits table in ”Carrier-on” and ”Carrier-off” states, following alternative could also be considered:

Table: Off-axis spurious limits in “Carrier-on” and “Carrier-off” states

| Frequency range | EIRP Limit | Measurement bandwidth |
| --- | --- | --- |
| 1.00 GHz – 3.40 GHz | 49 dBpW | 100 kHz |
| 3.40 GHz – 10.70 GHz | 55 dBpW | 100 kHz |
| 10.70 GHz – 21.20 GHz | 61 dBpW | 100 kHz |
| 21.20 GHz – 29.35 GHz | 67 dBpW | 100 kHz |
| 29.35 GHz – 29.50 GHz | 85 dBpW | 1 MHz |
| 30.00 GHz – 30.15 GHz | 85 dBpW | 1 MHz |
| 30.15 GHz – 60.00 GHz | 67 dBpW | 100 kHz |

Observation1: Based on the above proposal1, the off-axis spurious requirement in “Carrier-on” and “Carrier-off” states might be further simplified not considering the exceptions for certain frequency range.

### On-axis spurious

According to the ETSI Harmonized Standards, the on-axis spurious requirement is only specified for the frequency range of the considered band. For example, looking at EN 303 978, note 2 of sub-clause 4.2.2.2.1 (”carrier-on” state):

And in sub-clause 4.2.2.2.2 (”Carrier-off” and ”Emissions disabled” states):

In the 3GPP context, the EN On-axis spurious requirement should be considered as a SEM requirement and a spurious requirement.

Observation2: The On-axis spurious requirements are specified in a similar way in EN 301 978, EN 301 360 and EN 301 459.

Proposal2: Specify the NTN VSAT On-axis spurious requirements for band n512 for both mobile VSAT and fixed VSAT based on EN 303 0978.

# Conclusion

In this contribution, we discussed the NTN VSAT spurious requirement and analyzed existing Regulations and published Harmonized standards. We made the following proposals:

Proposal1: Specify NTN VSAT Off-axis requirements for band n512 according to the following:

- Convert dBpW values in dBm (=dBpW – 90)

- Consider the same Off-axis spurious requirements for both fixed and mobile VSAT.

- Keep the frequency range 1.0-2.0 GHz (ETSI Harmonized Standards).

- Consider 100 kHz measurement bandwidth for the ”normal” limits (ERC 74-01, EN 301 360 and EN 301 459)

- Consider the 85 dBpW exceptions for the relevant frequency range (as specified in ERC 74-01, EN 301 360 and EN 301 459).

Observation1: Based on the above proposal1, the off-axis spurious requirement in “Carrier-on” and “Carrier-off” states might be further simplified not considering the exceptions for certain frequency range.

Observation2: The On-axis spurious requirements are specified in a similar way in EN 301 978, EN 301 360 and EN 301 459.

Proposal2: Specify the NTN VSAT On-axis spurious requirements for band n512 for both mobile VSAT and fixed VSAT based on EN 303 0978.

Our companion draft CR ([5]) is based on these proposals

# Reference

- ERC 74-01,  Unwanted emissions in the spurious domain, 1 October 2021 update

- EN 303 978, Satellite Earth Stations and Systems (SES); Harmonised Standard for Earth Stations on Mobile Platforms (ESOMP)  transmitting towards satellites in geostationary orbit, operating in the 27,5 GHz to 30,0 GHz frequency bands covering the essential requirements of article 3.2 of the Directive 2014/53/EU, v2.1.2

- EN 301 360, Satellite Earth Stations and Systems (SES); Harmonised Standard for Satellite Interactive Terminals (SIT) and Satellite  User Terminals (SUT) transmitting towards satellites in geostationary orbit, operating in the 27,5 GHz to 29,5 GHz frequency bands covering the essential requirements of article 3.2 of the Directive 2014/53/EU, v2.1.1

- EN 301 459, Satellite Earth Stations and Systems (SES); Harmonised Standard for Satellite Interactive Terminals (SIT) and Satellite User Terminals (SUT) transmitting towards satellites in geostationary orbit, operating in the 29,5 GHz to 30,0 GHz frequency bands covering the essential requirements of article 3.2 of the Directive 2014/53/EU, v2.1.1

- R4-2402332, Draft CR to TS 38.101-5: NTN Ka-band introduction – sub-clause 9.5.5, Ericsson, Thales