TSG-RAN Working Group 4 (Radio) meeting #110	R4-2402483

Athens, EU, 26th February to 1st  March 2024

Source:	Ericsson

Title:	Draft LS on Parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27 (4.4-4.8 GHz)

Agenda item:	12

Document for:	Information

# Introduction

At the last ITU-R WP 5D a LS requesting information on IMT parameters relevant for frequency ranges 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz was sent to 3GPP RAN4 [7]. In the LS ITU-R WP 5D is seeking information on terrestrial component IMT-2030 parameters to be used for sharing and compatibility studies in preparation for WRC-27.

In this contribution we give an overview on information currently available in RAN4 and studies to be conducted to be able to respond to requested information. At the end of the contribution a draft LS to RAN is attached.

# Discussion

There is a need to start work to define sharing studies and compatibility parameters, like WP 5D has previously done for terrestrial component of IMT-2020 and IMT-Advanced technologies for bands below 6 GHz, which are contained in Annex 4.4 to Document 5D/716 (June 2021) and in Report ITU-R M.2292, respectively.

WRC-23 agreed a WRC-27 agenda item 1.7 related to IMT (Administrative Circular CA/270). WP 5D is therefore responsible for preparing the characteristics of terrestrial component of IMT for sharing and compatibility studies in the frequency regions listed in Table 2-1.

Table 2-1: Overview

| Range | Frequency range (MHz) |
| --- | --- |
| 1 | 4400 to 4800 |
| 2 | 7125 to 8400 |
| 3 | 14800 to 15350 |

With the LS request to 3GPP RAN4, ITU-R WP 5D expect to receive relevant RF parameters and well as relevant antenna geometry parameters. This time the LS request information related to RF characteristics and antenna characteristics for 3 different frequency ranges and additional information relevant for modelling M-MIMO systems. It’s reasonable to believe that IMT BS products will be designed considering different RF characteristics and antenna characteristics for the 3 different frequency ranges.

In Rel-16 RAN4 conducted a study to evaluate technology feasibility for the frequency range 7 to 24 GHz. The outcome is captured in TR 38.820 [3]. In Rel-17 RAN4 conducted a study to collect RF parameters for 6425 to 7125 MHz and 10000 to 10500 MHz. The outcome is captured in TR 38.921 [4] and created the foundation for the LS requesting information relevant for IMT-2020 and IMT-Advanced technologies.

Some of the information requested by ITU-R WP 5D can be extracted from previous studies in RAN4. Unfortunately, all information is not currently available in existing RAN4 technical reports. Therefore, RAN4 need to conduct additional studies with the goal to evaluate technology feasibility aspects for AAS BS operation within the Range 2 and Range 3.

A fundamental aspect that will have impact on an implementation is the size of the antenna required to achieve acceptable network coverage. Assuming similar intercell distance as current deployed networks would indicate that the physical antenna area remains similar as the size used for 3.5 GHz. This would consequently lead to array antennas architecture where the number of elements scale a factor 4 every time the frequency is doubled. Assuming that parameter assumption for Range 1 listed in TR 38.803 with 96 dual polarized elements is considered. The number of elements required for Range 2 and Range 3 would be 384 and 1536. The large number of antenna elements for Range 2 and Range 3 will put very different design challenges radio electronics compared with Range 1.

According to TR 38.820, the BS architecture may support array antennas with sub-array geometries. RAN4 needs to decide on specific details for the antenna characteristics relevant for Range 2 and Range 3.

In Table 2-2 an overview of currently available information within RAN4 and required studies are listed.

Table 2-2: Overview

| Range | Frequency range (MHz) | References to previous studies | Required studies |
| --- | --- | --- | --- |
| 1 | 4400 to 4800 | RF parameters can be extracted from BS RF core specification TS 38.104 [1]. Antenna parameters can be extracted from TR 38.803, subclause 5.2.3.2.4 [2] | None |
| 2 | 7125 to 8400 | Technology feasibility have been studied in TR 38.820 [3]. RF parameters for 6 and 10 GHz is provided in TR 38.921 [4] and can be used indicative starting point. | Evaluate relevance of information in TR 38.820 [3] using larger array antenna and TR 38.921 [4]. Decide on relevant antenna parameters. |
| 3 | 14800 to 15350 | Technology feasibility have been studied in TR 38.820 [3]. | Evaluate relevance in TR 38.820 [3] using larger array antenna. Decide on relevant antenna parameters. Based on relevant RF parameters and antenna parameters decide if adjacent channel coexistence evaluation is needed. |

The RF technology feasibility evaluation captured in TR 38.820 and corresponding RF parameter study in TR 38.921 assumes an BS array antenna with single element geometry. It’s clear that the array size and considered geometry need to be re-considered for Range 2 and Range 3.

Is shall also be noted that parameters for Range 1 was originally assumed to be based on single element structures. Later the antenna parameters were updated to better reflect real deployments using sub-array structures was provided by RAN4 [5].

When re-evaluating adjacent channel coexistence characteristics for Range 3 not only relevant parameters for BS should be considered also relevant UE parameters should be considered. It would be reasonable to consider directional UE antenna characteristics rather than omnidirectional.

In parallel to RAN4 discussions, RAN1 will conduct studies on propagation models for large antennas operating with the frequency range 7 to 24 GHz [6].

In addition to RF parameters and antenna parameters, ITU-R ask for information related to description of antenna model parameters, advancements in MIMO architectures, UE simulation assumptions and adjacent band antenna gain roll-off characteristics.

# Conclusion

In Rel-16, RAN4 studied feasibility of technology for the frequency range 7 to 24 GHz. The finding was captured in TR 38.820.

It can be concluded that information relevant for the frequency range 4400 to 4800 MHz is available in RAN4 and can be provided to ITU-R. The RF characteristics can be extracted from current Rel-18 BS RF core specification [1] and antenna modelling parameters can be extracted from TR 38.803 [2]. A draft LS to RAN is attached at the end of this contribution with parameters applicable for 4400 to 4800 MHz.

However, for the frequency range 7125 to 8400 MHz and 14800 to 15350 MHz relevant information related to RF parameters and antenna parameters is not available in RAN4. Some indicative information relevant for 7125 to 8400 MHz and 14800 to 15350 MHz can be found in TR 38.820 [3] and TR 38.921 [4]. Therefore, RAN4 needs to discuss what information can be re-used and what information needs to be further looked after. Higher frequencies would drive the size of the array antenna in terms of elements. The size of the antenna will impact many of the RF parameters listed by ITU-R. RAN4 need to initiate work with the goal to create a collection of parameters seen relevant for 7125 to 8400 MHz and 14800 to 15350 MHz and planned sharing studies. For frequency ranges 7125 to 8400 MHz and 14800 to 15350 MHz relevant parameters needs to be established latest April/May 2025.

This means that RAN4 need to prepare the Rel-19 BS RF agenda to include work related to find proper and relevant RF parameters and antenna assumptions for requested frequency regions. If not, RAN4 will not be able to respond to information requested by ITU-R WP 5D in time. A SID is provided in a companion contribution in [8].

# References

[1]	TS 38.104, “NR; Base Station (BS) radio transmission and reception”, V18.4.0, 3GPP

[2] 	TR 38.803, “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects”, V14.3.0, 3GPP

[3]	TR 38.820, “Study on the 7 to 24 GHz frequency range for NR”, V16.1.0, 3GPP

[4]	TR 38.921, “Study on International Mobile Telecommunications (IMT) parameters for 6.425 - 7.025 GHz, 7.025 - 7.125 GHz and 10.0 - 10.5 GHz”, V17.1.0, 3GPP

[5]	R4-2108080, “LS to ITU-R and CEPT on extension of IMT array antenna model to support sub-array structures”, Ericsson, Nokia, Huawei, Qualcomm

[6]	RP-234018, “New SID: Study on channel modelling enhancements for 7-24GHz for NR”, Nokia

[7]	R4-2400333, “Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35 GHz”, ITU-R Working Party 5D

[8]	R4-2402484, “New SI proposal: Study on IMT parameters for 7 125-8 400 MHz and 14.8-15.35 GHz”, Ericsson

DRAFT LS:

3GPP TSG RAN WG4 Meeting #110													R4-240xxx

Athens, Greece, 26 February – 1 March, 2024

Title:	LS on Parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-24 (4.4-4.8 GHz)

Response to:	LS R4-2400333 on Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35 GHz

Release:	-

Work Item:	-

Source:	TSG RAN WG4

To:	RAN

Cc:

Contact person:	Johan Sköld

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

RAN WG4 received the incoming LS from ITU-R Working Party 5D on Parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27 (Att. 7.4 to 5D/134) and would like to thank for the opportunity to give input in this topic.

The bands within 4 400 MHz to 4 800 MHz is part of what in 3GPP is defined as Frequency Range 1 (FR1) and the 5G RF parameters for the bands are specified in 3GPP specifications TS 38.104 for the BS and TS 38.101-1 for the UE. The recommended IMT-2020 technology related parameters are given in Annex 1 of this LS with references to those two specifications. The following should be noted:

- Where AAS and non-AAS limits may be expressed differently, there are separate entries in table 1. AAS limits always apply Over-the-Air (OTA).

- In the BS specification TS 38.104, non-AAS BS are identified as BS Type 1-C, while AAS BS are identified as BS Type 1-O for the bands.

- Deployment related parameters are documented in ITU-R M.2292.

The recommended IMT-2020 antenna characteristics are given in Annex 2 of this LS. The following should be noted:

- Parameters are interdependent and derived as a package, based on deployment scenarios and other requirements.

- There is no beam forming assumed for the UE in the frequency ranges covered. UEs are therefore not included in the table.

- For fixed beam antennas, antenna parameters in ITU-R M.2292 apply.

The information as attached for the frequency range 4 400 to 4 800 MHz can be sent to ITU-R WP5D immediately. For frequency ranges 7 125 to 8 400 MHz and 14 800 to 15 350 MHz, RAN4 need to study aspects related to coexistence and feasibility before sending answers to ITU-R WP 5D. For these frequency ranges relevant parameters can be established earliest April/May 2025 assuming a study starting in March 2023.

# 2	Actions

To ITU-R WP5D

ACTION: 	3GPP RAN4 asks 3GPP RAN to take the above information on 4 400 to 4 800 MHz system parameters and time scale estimate for other frequency ranges in considering when responding to ITU-R WP 5D with information related to RF and antenna parameters and work plan.

# 3	Dates of next TSG RAN WG 4 meetings

TSG-RAN4 Meeting #110bis	Changsha, China			15-19 April

TSG-RAN4 Meeting #111	Fukuoka, Japan				20-25 May

ANNEX 1

IMT-2020 technology-related and deployment-related parameters for bands between 4400 and 4800 MHz

Table 1: IMT-2020 technology related parameters in 4400 – 4800 MHz

|  |  | IMT |  |  |
| --- | --- | --- | --- | --- |
| No. | Parameter | Base station (non-AAS) | Base station (AAS) | Mobile station |
| 1 | Duplex Method | FDD / TDD See [1], § 5.2. |  | FDD / TDD See [2], § 5.2. |
| 2 | Channel bandwidth (MHz) | See [1], § 5.3.5. |  | See [2], § 5.3.5. |
| 3 | Signal bandwidth (MHz) | See [1], § 5.3.2. Signal bandwidth = NRB x SCS x 12. |  | See [2], § 5.3.2. Signal bandwidth = NRB x SCS x 12. |
| 4 | Transmitter characteristics |  |  |  |
| 4.1 | Power dynamic range (dB) | See [1], § 6.3.3. |  | See [2], § 6.2.1 (UE max output power) and §6.3.1 (UE min output power). |
| 4.2 | Spectral mask (dB) | See [1], § 6.6.4. | See [1], § 9.7.4. | See [2], § 6.5.2.2. |
| 4.3 | ACLR | See [1], § 6.6.3. |  | See [2], § 6.5.2.4. |
| 4.4 | Spurious emissions | See [1], § 6.6.4. |  | See [2], § 6.5.3. |
| 4.5 | Maximum output power | See [1], § 6.2. | See [1], § 9.3. | See [2], § 6.2.1. |
| 5 | Receiver characteristics |  |  |  |
| 5.1 | Noise figure (dB) | 5 dB (Wide Area BS) 10 dB (Medium Range BS) 13 dB (Local Area BS) For BS class definitions, see [1], § 4.4 |  | 9 dB |
| 5.2 | Sensitivity (dBm) | See [1], § 7.2.2. | See [1], § 10.3.2. | See [2], § 7.3. |
| 5.3 | Blocking response | See [1], § 7.5 and § 7.4.2. | See [1], § 10.6 and § 10.5.2. | See [2], § 7.6 and § 7.7. |
| 5.4 | ACS | See [1], § 7.4.1. | See [1], § 10.5.1. | See [2], § 7.5. |
| 5.5 | SINR operating range (dB) | See below “SINR operating range and mapping function” |  |  |

References used in the Table:

[1]	3GPP TS 38.104 v.16.3.0, “NR; Base Station (BS) radio transmission and reception”.

[2]	3GPP TS 38.101-1 v.16.3.0. “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

## SINR operating range and mapping function

The following equations approximate the throughput over a channel with a given SNIR, when using link adaptation:

Throughput SNIR, bps/Hz =0                                  for SNIR                                         ∝∙SSNIR                     for SNIRMIN≤SNIR<SNIRMAX ∝∙SSNIRMAX               for SNIR ≥ SNIRMAX

Where:

S(SNIR)	Shannon bound, S(SNIR) =log2(1+SNIR) [bps/Hz]
		Attenuation factor, representing implementation losses
SNIRMIN  	Minimum SNIR of the code set, dB
SNIRMAX 	Maximum SNIR of the code set, dB

The parameters α, SNIRMIN and SNIRMAX can be chosen to represent different modem implementations and link conditions. The parameters proposed in table 2 represent a baseline case, which assumes:

- 1:1 antenna configurations

- AWGN channel model

- Link Adaptation (see table 2 for details of the highest and lowest rate codes)

- No HARQ

Table 2: Parameters describing baseline Link Level performance for 5G NR

| Parameter | DL | UL | Notes |
| --- | --- | --- | --- |
| α | 0.6 | 0.4 | Represents implementation losses |
| SNIRMIN, dB | -10 | -10 | Based on QPSK, 1/8 rate (DL) & 1/5 rate (UL) |
| SNIRMAX, dB | 30 | 22 | Based on 256QAM 0.93(DL) & 64QAM 0.93 (UL) |

_____________

ANNEX 2

Antenna characteristics for IMT-2020 AAS base stations 
for bands between 4400 and 4800 MHz

An extended version of the AAS array antenna model is created to support vertical sub-array geometries with fixed sub-array down-tilt. The model equations are summarized in Table 3.

Table 3: Extended AAS model

| Description | Equation |
| --- | --- |
| Peak normalized element radiation pattern | Aθ,φ=-min--min12φφ3dB2,Am-min12θ-90θ3dB2,SLAv ,Am |
| Peak gain normalized element radiation pattern | AEθ,φ=GE,max+Aθ,φ |
| Sub-array excitation | wm=1Msubexpj2πm-1dv,subλsinθsubtilt |
| Sub-array radiation pattern | Asubθ,φ=AEθ,φ+10log10m=1Msubwmvm2 , where vm=expj2πm-1dv,subλcosθ |
| Array excitation | wm,n=1MNexpj2πm-1dvλsinθetilt-n-1dhλcosθetiltsinφescan Where M and N is corresponding to (Row × Column) in Table 4, row 1.6. |
| Composite array radiation pattern | AAθ,φ=Asubθ,φ+10log10m=1Mn=1Nwm,nvm,n2 , where vm,n=expj2πm-1dvλcosθ+n-1dhλsinθsinφ Where M and N is corresponding to (Row × Column) in Table 4, row 1.6. |

Table 4: Beamforming antenna characteristics for IMT in 4400 – 4800 MHz

|  |  | Rural | Macro suburban | Macro urban | Small cell outdoor/ Micro urban | Small cell indoor/ Indoor urban |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Base station Antenna Characteristics |  |  |  |  |  |  |  |
| 1.1 | Antenna pattern | Table 3 |  |  |  |  | N/A |  |
| 1.2 | Element gain (dBi) (Note 2) | 6.4 | 6.4 | 6.4 | 6.4 | N/A |  |
| 1.3 | Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 65º for V | 90º for H 65º for V | 90º for H 65º for V | 90º for H 65º for V | N/A |  |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V | 30 for both H/V | 30 for both H/V | 30 for both H/V | N/A |  |
| 1.5 | Antenna polarization | Linear ±45º | Linear ±45º | Linear ±45º | Linear ±45º | N/A |  |
| 1.6 | Antenna sub-array configuration (Row × Column) (Note 4) | 4 × 8 elements | 4 × 8 elements | 4 × 8 elements | 8 × 8 elements | N/A |  |
| 1.7 | Horizontal/Vertical radiating sub-array spacing (Note 5) | 0.5 of wavelength for H, 2.1 of wavelength for V | 0.5 of wavelength for H, 2.1 of wavelength for V | 0.5 of wavelength for H, 2.1 of wavelength for V | 0.5 of wavelength for H, 0.7 of wavelength for V | N/A |  |
| 1.7a | Number of element rows in sub-array | 3 | 3 | 3 | 1 | N/A |  |
| 1.7b | Vertical element separation in sub-array (dv,sub) | 0.7 of wavelength of V | 0.7 of wavelength of V | 0.7 of wavelength of V | 0.7 of wavelength of V | N/A |  |
| 1.7c | Pre-set sub-array down-tilt (degrees) (Note 6) | 3 | 3 | 3 | 0 | N/A |  |
| 1.8 | Array Ohmic loss (dB) (Note 2) | 2 | 2 | 2 | ## 2 | N/A |  |
| 1.9 | Conducted power (before Ohmic loss) per sub-array (dBm) (Note 3) | 28 | 28 | 28 | 16 | N/A |  |
| 1.10 | Base station horizontal coverage range (degrees) | +/-60 | +/-60 | +/-60 | +/-60 | N/A |  |
| 1.11 | Base station vertical coverage range (degrees) (Note 1) | 90-100 | 90-100 | 90-100 | 90-120 | N/A |  |
| 1.12 | Mechanical down-tilt (degrees) | 3 | 6 | 6 | N/A | N/A |  |

Note 1:	The vertical coverage range is given for the elevation angle θ, defined between 0° and 180° as 
in ITU-R M.2101.

Note 2:	The element gain in row 1.2 includes the loss given in row 1.8 and is per polarization.

Note 3:	The conducted power per sub-array assumes 4x8x2 sub-arrays (i.e., power per H/V polarized element).

Note 4:	4 × 8 means there are 4 vertical and 8 horizontal radiating sub-arrays.

Note 5:   For the case of 3 elements per sub array, dv will be 2.1 wavelengths.

Note 6:	Pre-set sub-array down-tilt is a fixed design parameter used to optimize coverage. The parameter value relates to the vertical steering range and element beam widths.

_____________