3GPP TSG-WG4 Meeting #110 	R4-2402558

# Athens, Greece, February 26 - March 1, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draft | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR to TS 38.133 on TCI State Switch, UL Timing and RTD Test Case for HST FR2 Enhanced |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | HST_FR2_enh-Perf |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | R-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | A need to test new RRM requirement for PC6 UEs supporting [highSpeedTCISwitchEnhMAC-CE-FR2-r18] and simultaneousReceptionFR2HST-r18]. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction a new clause in section A.7.5.8. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The enhanced requirements are not tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.7.5.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The changes are shown on top of TC A.7.5.8.3 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <Start of Change #1>

#### A.7.5.8.x	MAC-CE based active TCI state switch for PC6 UE supporting [simultaneousReceptionFR2HST-r18] in HST FR2 scenario

##### A.7.5.8.x.1	NR PCell FR2 HST active TCI state switch for a known TCI state

###### A.7.5.8.x.1.1	Test Purpose and Environment

The purpose of this test is to verify the active TCI state switch delay requirement defined in clause 8.10.3A for FR2 power class 6 UE indicating the support of [highSpeedTCISwitchEnhMAC-CE-FR2-r18]. Supported test configuration is shown in Table A.7.5.8.x.1.1-1. Furthermore, the purpose of this test is also to verify the one shot large timing adjustment requirement specified in clause 7.1.2.3 provided highSpeedMeasFlagFR2-r17 is configured and highSpeedLargeOneStepUL-TimingFR2-r17 is enabled for UE supporting FR2 power class 6 and largeOneStepUL-timingFR2-r17 capability. Finally, the purpose of the test is also to verify the maximum receive timing difference requirement for power class 6 UE supporting [simultaneousReceptionFR2HST-r18] specified in clause 7.6.7.

The test scenario comprises of one NR PCell (Cell 1) as given in Table A.7.5.8.x.1.1-2. Cell-specific parameters of NR PCell are specified in Table A.7.5.8.x.1.1-3 below. The OTA related test parameters for FR2 are shown in Table A.7.5.8.x.1.1-4. During the test, highSpeedMeasFlagFR2-r17 is configured to be set2 and broadcast to UE.

When the UE also supports [highSpeedTCISwitchEnhMAC-CE-FR2-r18], [R18 enhanced MAC-CE indication] is used and is indicated as ‘1’ for the TCI state switch.

PDCCHs indicating new transmissions shall be sent continuously on PCell to ensure that the UE would have ACK/NACK sending.

Before the test starts,

-	UE is connected to Cell 1 (PCell) on radio channel 1 (PCC).

-	UE is configured with 3 different TCI states for PCell, PDCCH TCI state 0 (QCL’d to SSB0), TCI state 1 (QCL’d to SSB1), and TCI state 2 (QCL’d to SSB2), in Cell 1 before starting the test.

-	UE is indicated TCI state 0 and TCI state 1 as the active PDCCH TCI states

The test consists of two time periods, T1 and T2. Figure A.7.5.8.x.1.1-1 and Figure A.7.5.8.x.1.1-2 show the Time multiplexed (allocation in Frequency is symbolic) downlink transmissions from each Angle of Arrival.

During T1 SSBs to which PDCCH-TCI-state 0 is QCL’d and SSBs to which PDCCH-TCI-state 1 is QCL’d are transmitted. The transmission with TCI state 0 has the relative timing delay compared to TCI state 1 by the absolute value of the one-way differential propagation time of [2.33 us].

At the beginning of T2, the SSB corresponding to TCI state 2 starts transmitting, which has the relative timing delay compared to TCI state 1 by the absolute value of the one-way differential propagation time Told-Tnew=2.33 μs and [0 μs] compared to TCI state 0. The UE is configured to provide periodic L1-RSRP reports. In slot n which is within 1280ms of UE providing L1-RSRP report with results for SSB0, SSB1, and SSB2, UE receives a MAC-CE command indicating a switch from TCI state 1 to TCI state 2. tci-PresentInDCI is not configured in the PDSCH configuration, i.e. TCI state for the PDSCH is identical to the PDCCH TCI state. After the TCI state switch, the UE transmit timing accuracy shall be measured by the test equipment by using the SRS defined in Table A.7.5.8.x.1-5.

The test equipment verifies that

-	UE can be scheduled on PCell on TCI state 0 during the whole period of the test.

-	UE can be scheduled on PCell on TCI state 1 till n+ THARQ +3 ms.

-	the TCI state switch time in PCell by scheduling the UE on TCI state 2 after slot n + THARQ + 3 ms + Tfirst-SSB + TSSB-proc + Trs + Trs-proc.

-	the UE transmission timing immediately after TCI state switch shall follow the requirements as specified in clause 7.1.2.3.

Table A.7.5.8.3.1.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.5.8.3.1.1-2: General test parameters for TCI state switch

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| T1 | s | 0.2 |  |
| T2 | s | 0.2 |  |

Table A.7.5.8.3.1.1-3: NR Cell specific test parameters for TCI state switch

| Parameter | Unit | Cell 1 |
| --- | --- | --- |
| Frequency Range |  | FR2 |
| Duplex mode |  | TDD |
| TDD configuration |  | TDDConf.3.1 |
| BWchannel |  | 100 MHz: NRB,c = 66 |
| Data RBs allocated |  | 66 |
| Initial DL BWP Configuration |  | DLBWP.0.2 |
| Dedicated DL BWP Configuration |  | DLBWP.1.1 |
| Initial UL BWP Configuration |  | ULBWP.0.2 |
| Dedicated UL BWP Configuration |  | ULBWP.1.1 |
| PDSCH Reference measurement channel |  | SR.3. 2 TDD |
| RMSI CORESET parameters |  | CR.3.1 TDD |
| Dedicated CORESET parameters |  | CCR.3.1 TDD |
| OCNG Patterns |  | OP. 5 |
| SSB Configuration |  | SSB.1 FR2 |
| SMTC Configuration |  | SMTC.1 |
| TCI State 0 |  | TCI.State.0 |
| TCI State 1 |  | TCI.State.1 |
| TCI State 2 |  | TCI.State.2 |
| TRS Configuration |  | TRS.2.1 TDD |
| Correlation Matrix and Antenna Configuration |  | 1x2 |
| EPRE ratio of PSS to SSS | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |
| Propagation Condition |  | AOA1: AWGN AOA2: AWGN with 9722 Hz frequency offset |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |

Table A.7.5.8.3.1.1-4: OTA related test parameters for TCI state switch

| Parameter | Unit |  |  | Cell 1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | SSB0 |  |  |  | SSB1 |  |  | SSB2 |  |  |
|  |  |  |  | T1 |  | T2 | T1 |  |  | T2 | T1 |  | T2 |
| Angle of arrival configuration |  |  |  | Setup 3 according to clause A.3.15.3 Note 8 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | AoA1 |  |  |  | AoA2 |  |  | AoA2 |  |  |
| Assumption for UE beams Note 6 |  |  |  | Rough |  |  |  |  |  |  |  |  |  |
| Ês |  |  | dBm/SCS | -80.6 |  | -80.6 | -80.6 |  |  | -80.6 | -Infinity |  | -80.6 |
| SS B_RP Note 2 |  |  | dBm/ SCS | -80.6 |  | -80.6 | -80.6 |  |  | -80.6 | -Infinity |  | -80.6 |
| BB Note 7 |  |  | dB | 8.3 |  | 8.3 | 8.3 |  |  | 8.3 | -Infinity |  | 8.3 |
| IoNote2 |  |  | dBm/95.04 MHz Note4 | -56.0 |  | -56.0 | -56.0 |  |  | -56.0 | - Infinity |  | -56.0 |
| Note 1: Void Note 2: SS B_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the center of the quiet zone. Note 6: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. Note 7: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.6-4. Note 8: 150 relative angular offsets between active probes AoA1 and AoA2 to be used in this test. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.7.5.8.3.1.1-5: Sounding Reference Symbol Configuration

| Field | Value | Comment |
| --- | --- | --- |
| c-SRS | 16 | Frequency hopping is disabled |
| b-SRS | 0 |  |
| b-hop | 0 |  |
| freqDomainPosition | 0 | Frequency domain position of SRS |
| freqDomainShift | 0 |  |
| groupOrSequenceHopping | neither | No group or sequence hopping |
| SRS-PeriodicityAndOffset | sl5=0 | Once every 5 slots |
| pathlossReferenceRS | ssb-Index=2 | SSB #2 is used for SRS path loss estimation |
| usage | Codebook | Codebook based UL transmission |
| startPosition | 0 | resourceMapping setting. SRS on last symbol of slot, and 1symbols for SRS without repetition. |
| nrofSymbols | n1 |  |
| repetitionFactor | n1 |  |
| combOffset-n2 | 0 | transmissionComb setting |
| cyclicShift-n2 | 0 |  |
| nrofSRS-Ports | port1 | Number of antenna ports used for SRS transmission |
| Note: For further information see clause 6.3.2 in TS 38.331 [2]. |  |  |

Figure A.7.5.8.3.1.1-1: Time multiplexed downlink transmissions during T1

Figure A.7.5.8.3.1.1-2: Time multiplexed downlink transmissions during T2

###### A.7.5.8.3.1.2	Test Requirements

UE shall be able to receive on TCI stat 0 during T1 and T2.

During T1, UE shall send L1-RSRP report with results for both SSB0 and SSB1.

During T2, UE shall send L1-RSRP report with results for SSB0, SSB1 and SSB2.

After the TCI state switch, the UE transmission timing immediately after TCI state switch shall follow the requirements as specified in clause 7.1.2.3.

After receiving MAC-CE command in slot n, UE shall:

-	be able to continue to receive on TCI state 1 till   n+ THARQ +3 ms

-	be able to start receiving on TCI state 2 after n+ THARQ +7 ms + Tfirst-SSB+ Trs

## <End of Change #1>