3GPP TSG-RAN WG4 Meeting # 110															R4-2402598

# Athens, GR, 26 Feb – 01 Mar, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.113 | CR | 0093 | rev |  | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (LTE-RF) CR to TS 36.113: clarification on the limit value for the step frequency case, Rel-15 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE-RF |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In this CR we provide clarification for the selection of the limit values for the step frequency. The proposed approach is reused from CISPR 32, annex I. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarification on the limit value selection for the step frequency. - Other editorials corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity on the limit value would remain in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.3, 8.3.3, 8.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

### 8.2.2	Radiated emission, Ancillary equipment

This test is applicable to ancillary equipment. This test shall be performed on a representative configuration of the ancillary equipment.

#### 8.2.2.1	Definition

This test assesses the ability of ancillary equipment to limit unwanted emission from the enclosure port.

#### 8.2.2.2	Test method

The test method shall be in accordance with CISPR 32 25.

#### 8.2.2.3	Limits

The ancillary equipment shall meet the limits according to CISPR 32 25 shown in Table 8.2.2.3-1 and Table 8.2.2.3-2.

Table 8.2.2.3-1: Limits for radiated emissions from ancillary equipment, measured on a stand-alone basis (10 m measuring distance)

| Frequency range | Quasi-peak |
| --- | --- |
| 30 MHz-230 MHz | 30 dBµV/m |
| 230 MHz-1000 MHz | 37 dBµV/m |
| NOTE: Where there is a step in the relevant limit, the lower value should be applied at the transition frequency, as per CISPR 32 [25]. |  |

Table 8.2.2.3-2: Limits for radiated emissions from ancillary equipment, measured on a stand-alone basis 
(3 m measuring distance)

| Frequency range GHz | Average limit dBµV/m | Peak limit dBµV/m |
| --- | --- | --- |
| 1 to 3 | 50 | 70 |
| 3 to 6 | 54 | 74 |
| NOTE: Where there is a step in the relevant limit, the lower value should be applied at the transition frequency, as per CISPR 32 [25]. |  |  |

------------------------------ Next modified section -------------------------

### 8.3.3	Limits

The equipment shall meet the limits below (including the average limit and the quasi-peak limit) when using, respectively, an average detector receiver and a quasi-peak detector receiver and measured in accordance with the method described in \clause 8.3.2 above. If the average limit is met when using a quasi-peak detector, the equipment shall be deemed to meet both limits and measurement with the average detector receiver is not necessary.

The equipment shall meet the limits given in Table 8.3.3-1.

Table 8.3.3-1: Limits for conducted emissions

| Frequency range | Quasi-peak | Average |
| --- | --- | --- |
| >0,15-0,5MHz | 79dBµV | 66dBµV |
| >0,5-30 MHz | 73dBµV | 60dBµV |
| NOTE: Where there is a step in the relevant limit, the lower value should be applied at the transition frequency, as per CISPR 32 [25]. |  |  |

------------------------------ Next modified section -------------------------

### 8.4.3	Limits

The equipment shall meet the limits below (including the average limit and the quasi-peak limit) when using, respectively, an average detector receiver and a quasi-peak detector receiver and measured in accordance with the method described in clause 8.4.2 above. If the average limit is met when using a quasi-peak detector, the equipment shall be deemed to meet both limits and measurement with the average detector receiver is not necessary.

Table 8.4.3-1: Limits for conducted emissions

| Frequency range | Quasi-peak | Average |
| --- | --- | --- |
| > 0,15-0,5 MHz | 66 - 56 dBµV | 56 - 46 dBµV |
| > 0.5- 5 MHz | 56 dBµV | 46 dBµV |
| > 5-30 MHz | 60 dBµV | 50 dBµV |
| NOTE 1: The limit decreases linearly with the logarithm of the frequency in the range 0,15 MHz to 0,50 MHz. NOTE 2: Where there is a step in the relevant limit, the lower value should be applied at the transition frequency, as per CISPR 32 [25]. |  |  |

Alternatively, for equipment intended to be used in telecommunication centres the limits given in Table 8.4.3-2 shall be used.

Table 8.4.3-2: Limits for conducted emissions

| Frequency range | Quasi-peak | Average |
| --- | --- | --- |
| >0,15-0,5MHz | 79dBµV | 66dBµV |
| >0,5-30 MHz | 73dBµV | 60dBµV |
| NOTE: Where there is a step in the relevant limit, the lower value should be applied at the transition frequency, as per CISPR 32 [25]. |  |  |

------------------------------ End of modified section -------------------------