3GPP TSG-RAN WG4 Meeting #110	R4-2402608

# Athens, Greece, 26th Feb – 1st Mar, 2024

Source: 	Huawei, HiSilicon

Title:	TP for TR 37.718-11-21: DC_3(n)AA_n8A

Agenda Item:	7.6.3

Document for:	Approval

# Introduction

This contribution provides a text proposal on DC band combination of LTE band B3 with NR bands n3 and n8 for TR 37.718-11-21[1]. Although the EN-DC band combination counterpart (DC_(n)3AA-n8A) is already specified in TS 38.101-3, but as an MSD test point is needed for this band combination, based on the rules, the TP needs to be issued.

# Reference

- Rel-18 DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL)

# Text Proposal

----- Start of TP -----

## 6.x	DC_3(n)_n8

### 6.x.1	Operating bands for DC

Table 6.x.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3(n)AA_n8A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |

### 6.x.2	Channel bandwidths per operating band for DC

Table 6.x.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth For DL [MHz] |
| DC_3(n)AA_n8A | DC_3(n)AA1 DC_3A_n8A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 70 |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE1: Only single switched UL is supported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.x.3	Co-existence studies

The studies are already performed for DC_3(n)AA and DC_3A_n8A, in TR 37.717-11-11 and TR 37.716-11-11, respectively.

### 6.x.4	∆TIB and ∆RIB values

Same values as DC_(n)3AA_n8A are used.

### 6.x.5	MSD

Similar values as for DC_(n)3AA_n8A is proposed.

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3(n)AA_n8A | 3 | N/A | 5 | N/A | 1837.5 | 4.5 | IMD5 |
|  | n3 | 1747.5 | 5 | 25 | 1842.5 | 6.4 | IMD5 |
|  | n8 | 897.5 | 5 | 25 | 942.5 | N/A | N/A |

----- End of TP -----