3GPP TSG-RAN4 Meeting #110	R4-2402624

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 4244 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | BigCR for 38.133 on RRM core requirements for NR support for dedicated spectrum less than 5MHz for FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR1_lessthan_5MHz_BW-Core |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Less than 5MHz (NR_FR1_lessthan_5MHz_BW) core requirements maintenance CR. - Merging R4-2402849: Applicability of the requirements - Adding missing Handover requirements based on the agreement: Handover delay (Es/Iot≥-2 dB) àAgreed during last meeting - Unknown intra-frequency target cell: [1+2]*Trs ms - Unknown inter-frequency target cell: [3+2]*Trs ms - Removing duplicate sentence from RLM intro - Table 8.1.2.1-3, <Agreement>: Use Non-Distributed in the table, and add a note to clarify that Distributed mapping may lead to unexpected out-of-sync indication. Updating tables: Table 8.1.2.1-3, Table 8.1.2.1-4: <Agreement>: Channel BW 3MHz 3MHz 5MHz [DL Transmission BW] 12 PRB 15 PRB 20 PRB - Same agreement as in 4. for table Table 8.5.2.1-2: - Voiding duplicate tables. - 9.2.5.1-16 - 9.2.5.1-22 - 9.2.6.2-11 - Table 9.3.9.1-5 update Table 9.3.4-11 update |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding applicability rule RAN4#110 - Adding missing Handover requirements - Update RLM intro - Updating Table 8.1.2.1-3 and Table 8.1.2.1-4 based on RAN4#110 - Updating Table 8.5.2.1-2 based on RAN4#110 - Voiding duplicate tables. - 9.2.5.1-16 - 9.2.5.1-22 - 9.2.6.2-11 - Table 9.3.9.1-5 update - Table 9.3.4-11 update |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR_FR1_lessthan_5MHz_BW is only partially specified and remains unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | RAN#102 approved Big CR: R4-2321648 |  |  |  |  |  |  |  |  |

# <Start of change 1>

### 3.6.X1	Applicability of requirements for a UE operating on a cell with less than 5MHz BW]

For a UE operating on a cell with less than 5MHz BW in FR1 FDD bands, the requirements corresponding to NR single carrier operation are applicable unless specific requirements corresponding to reduced BW are specified.

<End of change 1>

# <Start of change 2>

#### 6.1.1.2	NR FR1 - NR FR1 Handover

##### 6.1.1.2.2	Interruption time

The interruption time is the time between end of the last TTI containing the RRC command on the old PDSCH and the time the UE starts transmission of the new PRACH, excluding the RRC procedure delay.

When intra-frequency or inter-frequency handover is commanded, the interruption time shall be less than Tinterrupt

Tinterrupt = Tsearch + TIU + Tprocessing  + T∆ + Tmargin ms

Where:

Tsearch is the time required to search the target cell when the target cell is not already known when the handover command is received by the UE. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times.

Editor’s Note: P1 is to be updated based on simulation results.

-	If the target cell is a known intra-frequency cell, then Tsearch = 0ms.

-	If the target cell is an unknown intra-frequency cell operating with 12 PRB SSB bandwidth, then Tsearch = [3]*Trs ms.

-	If the target cell is a known inter-frequency cell, then

-	if the measured SSB is the target SSB for handover of the target cell, Tsearch = 0ms;

-	if the measured SSB and the target SSB for handover belong to the same NR target cell and if the UE supports [UE capability IE for FG 53-3], Tsearch = Trs ms provided any one of the following conditions is satisfied:

-	CD-SSB in initial DL BWP is the measured SSB and NCD-SSB in first active DL BWP is the target SSB for handover

-	NCD-SSB in a DL BWP is the measured SSB and CD-SSB in initial DL BWP is the target SSB for handover

-	Both measured SSB and the target SSB for handover are NCD-SSB within different DL BWPs

-	If the target cell is an unknown inter-frequency cell operating with 12 PRB SSB bandwidth, then Tsearch = [5] *Trs ms.

-	If the target cell is an unknown inter-frequency cell and Tsearch = 3* Trs ms if the target cell Es/Iot≥-2 dB.

<End of change 2>

# <Start of change 3>

## 8.1	Radio Link Monitoring

### 8.1.1	Introduction

The requirements in clause 8.1 apply for radio link monitoring on:

-	PCell in SA NR, NR-DC and NE-DC operation mode,

-	PCell operating with less than 5MHz BW in SA NR (single carrier) operation mode,

-	PSCell in NR-DC and EN-DC operation mode,

-	Deactivated PSCell in NR-DC and EN-DC operation mode, when configured.

<End of change 3>

# <Start of change 4>

### 8.1.2	Requirements for SSB based radio link monitoring

#### 8.1.2.1	Introduction

Table 8.1.2.1-3: PDCCH transmission parameters for out-of-sync evaluation for a UE operating on a cell with less than 5MHz BW

| Attribute | Value for BLER Configuration #0 |  |  |  |
| --- | --- | --- | --- | --- |
| Channel BW | 3MHz |  |  | 5MHz |
| [DL Transmission BW] | 12 PRBs |  | 15 PRBs | 20 PRBs |
| DCI format | 1-0 |  |  |  |
| Number of control OFDM symbols | [2] |  | [3] | [3] |
| Aggregation level (CCE) | [4] |  | [8] | [8] |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 4dB |  |  |  |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 4dB |  |  |  |
| Bandwidth (PRBs) | 12 |  | 15 | 20 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |  |  |  |
| DMRS precoder granularity | REG bundle size |  |  |  |
| REG bundle size | 6 |  |  |  |
| CP length | Normal |  |  |  |
| Mapping from REG to CCE |  | Distributed | Non-DistributedNote 1 |  | Distributed |
| NOTE 1: For 15 PRB Transmission BW, distributed mapping may lead to unexpected out-of-sync indication. |  |  |  |  |

Table 8.1.2.1-4: PDCCH transmission parameters for in-sync evaluation for a UE operating on a cell with less than 5MHz BW

| Attribute | Value for BLER Configuration #0 |  |  |
| --- | --- | --- | --- |
| Channel BW | 3MHz |  | 5MHz |
| [DL Transmission BW] | 12 PRBs | 15 PRBs | 20 PRBs |
| DCI format | 1-0 |  |  |
| Number of control OFDM symbols | [2] | [3] | [3] |
| Aggregation level (CCE) | [2] | [4] | [4] |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 0dB |  |  |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 0dB |  |  |
| Bandwidth (PRBs) | 12 | 15 | 20 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |  |  |
| DMRS precoder granularity | REG bundle size |  |  |
| REG bundle size | 6 |  |  |
| CP length | Normal |  |  |
| Mapping from REG to CCE | Distributed | Non-Distributed | Distributed |

<End of change 4>

# <Start of change 5>

### 8.5.2	Requirements for SSB based beam failure detection

#### 8.5.2.1	Introduction

Table 8.5.2.1-2: PDCCH transmission parameters for beam failure instance for a UE operating on a cell with less than 5MHz BW

| Attribute | Value for BLER |  |  |
| --- | --- | --- | --- |
| Channel BW | 3MHz |  | 5MHz |
| [DL Transmission BW] | 12 PRBs | 15 PRBs | 20 PRBs |
| DCI format | 1-0 |  |  |
| Number of control OFDM symbols | [2] | [3] | [3] |
| Aggregation level (CCE) | [4] | [8 Note 1] | [8] |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 0dB |  |  |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 0dB |  |  |
| Bandwidth (PRBs) | 12 | 15 | 20 |
| Sub-carrier spacing (kHz) | Same as the SCS of RMSI CORESET |  |  |
| DMRS precoder granularity | REG bundle size |  |  |
| REG bundle size | 6 |  |  |
| CP length | Normal |  |  |
| Mapping from REG to CCE | Distributed | Non-DistributedNote2 | [Distributed] |
| NOTE 1: PDCCH puncturing as defined in 38.214 [26] applies. NOTE 2: For 15PRB Transmission BW, distributed mapping may lead to unexpected beam failure detection. |  |  |  |

<End of change 5>

# <Start of change 6>

### 9.2.5	Intrafrequency measurements without measurement gaps

#### 9.2.5.1	Intrafrequency cell identification

Table 9.2.5.1-16: Void

Table 9.2.5.1-17: Time period for PSS/SSS detection for UE indicating [no gap with interruption], (Frequency range FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max( 600ms, 5 x max (80ms, SMTC period ))Note 1 x CSSFintra |
| [DRX cycle≤ 320ms] | max( 600ms, ceil(M2 Note 2x 5) x [max(80ms, SMTC period,DRX cycle)]) x CSSFintra |
| [DRX cycle>320ms] | 5 x [DRX cycle x] CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1. NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured and UE supports measurementEnhancementCA-r17, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1. |  |

Editor’s note: RAN4 has to decide the UE behaviour when DRX is condifured whether interruptions are allowed.

Table 9.2.5.1-18: Time period for PSS/SSS detection for UE indicating [no gap with interruption], (Frequency range FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, ceil(Mpss/sss_sync_w/o_gaps x KFR x Klayer1_measurement) x max (80ms, SMTC period ))Note 1 x CSSFintra |
| [DRX cycle≤ 320ms] | max(600ms, ceil(1.5 x Mpss/sss_sync_w/o_gaps x KFR x Klayer1_measurement) x [max(80ms, SMTC period,DRX cycle)]) x CSSFintra |
| [DRX cycle>320ms] | [ceil(Mpss/sss_sync_w/o_gaps x KFR x Klayer1_measurement) x DRX cycle] x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: KFR is a scaling factor depending on the frequency range and the SSB SCS. For FR2-1, KFR = 1. For FR2-2: KFR = 1 if the SCS of the SSB of the cell being detected is 120 kHz, KFR = 2 if the SCS of the SSB of the cell being detected is 480 kHz, and KFR = 3 if the SCS of the SSB of the cell being detected is 960 kHz. |  |

Editor’s note: RAN4 has to decide the UE behaviour when DRX is condifured whether interruptions are allowed.

Table 9.2.5.1-19: Time period for time index detection for UE indicating [no gap with interruption] (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120ms, 3 x max (80ms, SMTC period ))Note 1 x CSSFintra |
| [DRX cycle≤ 320ms] | max(120ms, ceil (M2 Note 2 x 3) x [max(80ms, SMTC period,DRX cycle)]) x CSSFintra |
| [DRX cycle>320ms] | 3 x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1 NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured and UE supports measurementEnhancementCA-r17, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1 |  |

Editor’s note: RAN4 has to decide the UE behaviour when DRX is condifured whether interruptions are allowed.

Table 9.2.5.1-20: Time period for time index detection  for UE indicating [no gap with interruption] (Frequency range FR2)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(200ms, ceil(MSSB_index_intra x Kp x max(80ms, SMTC period )) x CSSFintra |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5 x MSSB_index_intra x Kp) x max(80ms, SMTC period, DRX cycle) x CSSFintra) |
| DRX cycle>320ms | Ceil(MSSB_index_intra x Kp )x DRX cycle x CSSFintra |

Editor’s note: RAN4 has to decide the UE behaviour when DRX is condifured whether interruptions are allowed.

Table 9.2.5.1-21: Time period for PSS/SSS detection when highSpeedMeasFlagFR2-r17 is configured, (Frequency range FR2) when SMTC period <= 40ms, UE indicating [no gap with interruption]

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, ceil(M1Note 2 x Klayer1_measurement) x max (80ms, SMTC period))Note 1 x CSSFintra |
| [DRX cycle≤ 80ms] | max(600ms, ceil(M1Note 2 x Kp x Klayer1_measurement) x [max(80ms,SMTC period,DRX cycle)]) x CSSFintra |
| [80ms< DRX cycle≤ 320ms] | ceil(1.5 x Mpss/sss_sync_w/o_gaps Note 3 x Klayer1_measurement) x max(80ms, SMTC period,DRX cycle) x CSSFintra |
| [DRX cycle>320ms] | ceil(Mpss/sss_sync_w/o_gaps Note 3 x Klayer1_measurement) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: For UE supporting power class 6, M1 = 6 if highSpeedMeasFlagFR2-r17 = set1 or M1 = 18 if highSpeedMeasFlagFR2-r17 = set2 NOTE 3: Void |  |

Editor’s note: RAN4 has to decide the UE behaviour when DRX is condifured whether interruptions are allowed.

Table 9.2.5.1-22: Void

Table 9.2.5.1-23: Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (Frequency range FR1)

| DRX cycle | TSSB_time_index_intra_less_than_5Mhz |
| --- | --- |
| No DRX | max(120ms, 7 x max(MGRP, SMTC period)) x CSSFintra_less_than_5Mhz |
| DRX cycle≤ 320ms | max(120ms, ceil(1.5 x 7) x max(MGRP, SMTC period,DRX cycle) x CSSFintra_less_than_5Mhz) |
| DRX cycle>320ms | 7 x max(MGRP, DRX cycle) x CSSFintra_less_than_5Mhz |
| NOTE 1: FFS When highSpeedMeasInterFreq-r17 |  |

<End of change 6>

# <Start of change 7>

### 9.2.6	Intra-frequency measurements with measurement gaps

#### 9.2.6.1	Void

#### 9.2.6.2	Intra-frequency cell identification

Table 9.2.6.2-11: Void

Table 9.2.6.1-12: Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (Frequency range FR1) (Frequency range FR1)

| DRX cycle | TSSB_time_index_intra_less_than_5Mhz |
| --- | --- |
| No DRX | max(120ms, [7] x max(MGRP, SMTC period)) x CSSFintra_less_than_5Mhz |
| DRX cycle≤ 320ms | max(120ms, ceil(1.5 x [7]) x max(MGRP, SMTC period,DRX cycle) x CSSFintra_less_than_5Mhz) |
| DRX cycle>320ms | [7] x max(MGRP, DRX cycle) x CSSFintra_less_than_5Mhz |
| Note 1: CSSFintra_less_than_5Mhz is 1 NOTE 2: FFS When highSpeedMeasInterFreq-r17 |  |

<End of change 7>

# <Start of change 8>

### 9.3.9	Inter frequency measurements without measurement gaps

#### 9.3.9.1	Inter frequency Cell identification

Table 9.3.9.1-5: Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (FR1)

| DRX cycle | TSSB_time_index_inter_less_than_5MHz |
| --- | --- |
| No DRX | max(120ms, ceil( [6] x Kp ) x SMTC period)Note 1 x CSSFinter |
| DRX cycle≤ 320ms | max(120ms, ceil (M2 x [6] x Kp) x max(SMTC period,DRX cycle)) x CSSFinter |
| DRX cycle>320ms | Ceil( [6] x Kp) x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: FFS When highSpeedMeasInterFreq-r17 |  |

<End of change 8>

# <Start of change 9>

### 9.3.4	Inter-frequency measurement with measurement gaps

Table 9.3.4-11: Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (Frequency range FR1)

| Condition NOTE1 | TSSB_time_index_inter_less_than_5Mhz |
| --- | --- |
| No DRX | Max(120ms, Ceil([6] * Kgap) Max(MGRP , SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(120ms, Ceil([6] 1.5 * Kgap) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | Ceil([6]* Kgap) DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: FFS When highSpeedMeasInterFreq-r17 |  |

<End of change 9>