3GPP TSG-RAN4 Meeting #110	R4-2402633

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR for 38.133 on SCell-SCG setup delay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Editorial changes to the section 4.7 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Editorial changes to the section 4.7 - Adding capability to 4.7.2 [solution based on existing measurement] and EMR. - Removing brackets from existing measurement requirements - Editing the description of what UE shall not report to what UE is allowed to report. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification remains unclear |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of change 1>

## 4.7	Measurement report for fast CA/DC setup

### 4.7.1	Introduction

A UE supporting [solution based on existing measurement] shall report the idle mode measurement results on the inter-frequency CA and DC candidate frequencies/cells indicated by higher layers and meet the requirements specified in this clause. The UE shall perform idle mode measurements provided that the serving cell support early measurement and is within the validity area. The idle mode measurement requirements apply to a configured carrier frequency and the serving cell are among the supported band combination of the UE.

### 4.7.2	Measurement Requirements

A UE supporting [solution based on existing measurement] and idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16 shall support the idle mode CA measurements on the serving cell, and carriers configured for idle mode CA/DC measurement and meet corresponding measurement requirements defined in clause 4.4.2.

A UE supporting [solution based on existing measurement] but not idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16 shall support the idle mode CA measurements on the serving cell, and carriers configured for idle mode CA/DC measurement and meet corresponding measurement requirements defined in clause 4.2.2. The UE physical layer shall be capable of reporting SS-RSRP and SS-RSRQ measurements of the carriers configured to higher layers, with measurement accuracy as specified in clauses 10.1.4B and 10.1.5B and 10.1.9B and 10.1.10B, respectively

### 4.7.3	Measurement Report Requirements

A UE supporting [solution based on existing measurement] shall be able to report valid measurement results upon, or after, RRC setup complete. The measurement results are considered valid if the following conditions are met:

-	the measurements are performed within the last [X] seconds before msg1 transmission for RRC resume/setup request, where [X] is configured by [TBD], and

-	the measurement results satisfy measurement accuracy requirement at the measurement instance.

If the X value is not configured, UE may report measurement results given that the measurement results satisfy measurement accuracy requirements as specified in the section 4.7.2 at the measurement instance.

<End of change 1>

# <Start of change 2>

## 5.8	Measurement report for fast CA/DC setup

### 5.8.1	Introduction

A UE supporting [solution based on existing measurement] shall perform the inactive mode measurement on the inter-frequency CA and DC candidate frequencies/cells indicated by higher layers and meet the requirement specified in this clause. The UE shall perform inactive mode measurements provided that the serving cell support early measurement and is within the validity area. The inactive mode measurement requirements apply to a configured carrier frequency and the serving cell are among the supported band combination of the UE.

### 5.8.2	Measurement Requirements

The requirements in clause 4.7.2 shall apply.

### 5.8.3	Measurement Report Requirements

The requirements in clause 4.7.3 shall apply.

<End of change 2>