3GPP TSG-WG4 Meeting # 110	R4-2402692

# Athens, Greece,  26 February – 1 March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.133 Update to measurement report mapping for positioning measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Perf |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To implement report mapping for positioning measurements with PRS_SRS bandwidth aggregation |  |  |  |  |  |  |  |  |
| Summary of change: |  | Report mapping tables for k = {-1, -2, -3, -4, -5, -6} are defined for: - RSTD measurements - UE Rx-Tx time difference measurements - UL-RTOA measurements - gNB Rx-Tx measurements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Report mapping tables for positioning measurements with PRS/SRS badnwidth aggregation are not defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.23.3, 10.1.25.3, 13.1, and 13.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<START OF CHANGE>

#### 10.1.23.3	Report mapping

##### 10.1.23.3.1	Absolute DL RSTD Measurement Reporting

The reporting range for the DL RSTD measurement is defined from -985024Tc to 985024Tc with the resolution step of 2kTc, where

Tc is defined in TS 38.211 [6],

kmin≤k≤kmax,

kmin = -6 and kmax = 5, when configured PRS resource for the reference cell and neighbor cell measured for the RSTD measurement are in FR1 and/or FR2,

k≥ timingReportingGranularityFactor [34] configured by LMF via LPP for the RSTD measurement.

The measurement report mapping for different k values are specified in Tables 10.1.23.3.1-1  10.1.23.3.1-12.

Table 10.1.23.3.1-1: Report mapping for k=0

| Reported Quantity Value, | Measured Quantity Value, | Unit |
| --- | --- | --- |
| RSTD_i | RSTD |  |
| RSTD_0000000 | RSTD < -985024 | Tc |
| RSTD_0000001 | -985024 RSTD < -985023 | Tc |
| RSTD_0000002 | -985023 RSTD < -985022 | Tc |
|  |  | … |
| RSTD_0985024 | -1 RSTD < 0 | Tc |
| RSTD_0985025 | 0 RSTD < 1 | Tc |
| … | … | … |
| RSTD_1970047 | 985022 RSTD < 985023 | Tc |
| RSTD_1970048 | 985023 RSTD < 985024 | Tc |
| RSTD_1970049 | 985024 RSTD | Tc |

Table 10.1.23.3.1-2: Report mapping for k=1

| Reported Quantity Value, | Measured Quantity Value, | Unit |
| --- | --- | --- |
| RSTD_i | RSTD |  |
| RSTD_000000 | RSTD < -985024 | Tc |
| RSTD_000001 | -985024 RSTD < -985022 | Tc |
| RSTD_000002 | -985022 RSTD < -985020 | Tc |
|  |  | … |
| RSTD_492512 | -2 RSTD < 0 | Tc |
| RSTD_492513 | 0 RSTD < 2 | Tc |
| … | … | … |
| RSTD_985023 | 985020 RSTD < 985022 | Tc |
| RSTD_985024 | 985022 RSTD < 985024 | Tc |
| RSTD_985025 | 985024 RSTD | Tc |

Table 10.1.23.3.1-3: Report mapping for k=2

| Reported Quantity Value, | Measured Quantity Value, | Unit |
| --- | --- | --- |
| RSTD_i | RSTD |  |
| RSTD_000000 | RSTD < -985024 | Tc |
| RSTD_000001 | -985024 RSTD < -985020 | Tc |
| RSTD_000002 | -985020 RSTD < -985016 | Tc |
|  |  | … |
| RSTD_246256 | -4 RSTD < 0 | Tc |
| RSTD_246257 | 0 RSTD < 4 | Tc |
| … | … | … |
| RSTD_492511 | 985016 RSTD < 985020 | Tc |
| RSTD_492512 | 985020 RSTD < 985024 | Tc |
| RSTD_492513 | 985024 RSTD | Tc |

Table 10.1.23.3.1-4: Report mapping for k=3

| Reported Quantity Value | Measured Quantity Value, | Unit |
| --- | --- | --- |
| RSTD_i | RSTD |  |
| RSTD_000000 | RSTD < -985024 | Tc |
| RSTD_000001 | -985024 RSTD < -985016 | Tc |
| RSTD_000002 | -985016 RSTD < -985008 | Tc |
|  |  | … |
| RSTD_123128 | -8 RSTD < 0 | Tc |
| RSTD_123129 | 0 RSTD < 8 | Tc |
| … | … | … |
| RSTD_246255 | 985008 RSTD < 985016 | Tc |
| RSTD_246256 | 985016 RSTD < 985024 | Tc |
| RSTD_246257 | 985024 RSTD | Tc |

Table 10.1.23.3.1-5: Report mapping for k=4

| Reported Quantity Value, | Measured Quantity Value, | Unit |
| --- | --- | --- |
| RSTD_i | RSTD |  |
| RSTD_000000 | RSTD < -985024 | Tc |
| RSTD_000001 | -985024 RSTD < -985008 | Tc |
| RSTD_000002 | -985008 RSTD < -984992 | Tc |
|  |  | … |
| RSTD_061564 | -16 RSTD < 0 | Tc |
| RSTD_061565 | 0 RSTD < 16 | Tc |
| … | … | … |
| RSTD_123127 | 984992 RSTD < 985008 | Tc |
| RSTD_123128 | 985008 RSTD < 985024 | Tc |
| RSTD_123129 | 985024 RSTD | Tc |

Table 10.1.23.3.1-6: Report mapping for k=5

| Reported Quantity Value, | Measured Quantity Value, | Unit |
| --- | --- | --- |
| RSTD_i | RSTD |  |
| RSTD_00000 | RSTD < -985024 | Tc |
| RSTD_00001 | -985024 RSTD < -984992 | Tc |
| RSTD_00002 | -984992 RSTD < -984960 | Tc |
|  |  | … |
| RSTD_30782 | -32 RSTD < 0 | Tc |
| RSTD_30783 | 0 RSTD < 32 | Tc |
| … | … | … |
| RSTD_61563 | 984960 RSTD < 984992 | Tc |
| RSTD_61564 | 984992 RSTD < 985024 | Tc |
| RSTD_61565 | 985024 RSTD | Tc |

Table 10.1.23.3.1-7 Report mapping for k = -1.

| Reported Quantity Value, | Measured Quantity Value, | Unit |
| --- | --- | --- |
| RSTD_i | RSTD |  |
| RSTD_0000000 | RSTD < -985024 | Tc |
| RSTD_0000001 | -985024 RSTD < -985023.5 | Tc |
| RSTD_0000002 | -985023.5 RSTD < -985023 | Tc |
|  |  | … |
| RSTD_1970048 | -0.5 RSTD < 0 | Tc |
| RSTD_1970049 | 0 RSTD < 0.5 | Tc |
| … | … | … |
| RSTD_3940095 | 985023 RSTD < 985023.5 | Tc |
| RSTD_3940096 | 985023.5 RSTD < 985024 | Tc |
| RSTD_3940097 | 985024 RSTD | Tc |

Table 10.1.23.3.1-8 Report mapping for k = -2.

| Reported Quantity Value, | Measured Quantity Value, | Unit |
| --- | --- | --- |
| RSTD_i | RSTD |  |
| RSTD_0000000 | RSTD < -985024 | Tc |
| RSTD_0000001 | -985024 RSTD < -985023.75 | Tc |
| RSTD_0000002 | -985023.75 RSTD < -985023.5 | Tc |
|  |  | … |
| RSTD_3940096 | -0.25 RSTD < 0 | Tc |
| RSTD_3940097 | 0 RSTD < 0.25 | Tc |
| … | … | … |
| RSTD_7880191 | 985023.5 RSTD < 985023.75 | Tc |
| RSTD_7880192 | 985023.75 RSTD < 985024 | Tc |
| RSTD_7880193 | 985024 RSTD | Tc |

Table 10.1.23.3.1-9 Report mapping for k = -3.

| Reported Quantity Value, | Measured Quantity Value, | Unit |
| --- | --- | --- |
| RSTD_i | RSTD |  |
| RSTD_00000000 | RSTD < -985024 | Tc |
| RSTD_00000001 | -985024 RSTD < -985023.875 | Tc |
| RSTD_00000002 | -985023.875 RSTD < -985023.75 | Tc |
|  |  | … |
| RSTD_07880192 | -0.125 RSTD < 0 | Tc |
| RSTD_07880193 | 0 RSTD < 0.125 | Tc |
| … | … | … |
| RSTD_15760383 | 985023.75 RSTD < 985023.875 | Tc |
| RSTD_15760384 | 985023.875 RSTD < 985024 | Tc |
| RSTD_15760385 | 985024 RSTD | Tc |

Table 10.1.23.3.1-10 Report mapping for k = -4.

| Reported Quantity Value, | Measured Quantity Value, | Unit |
| --- | --- | --- |
| RSTD_i | RSTD |  |
| RSTD_00000000 | RSTD < -985024 | Tc |
| RSTD_00000001 | -985024 RSTD < -985023.9375 | Tc |
| RSTD_00000002 | -985023.9375 RSTD < -985023.875 | Tc |
|  |  | … |
| RSTD_15760384 | -0. 0625 RSTD < 0 | Tc |
| RSTD_15760385 | 0 RSTD < 0.0625 | Tc |
| … | … | … |
| RSTD_31520767 | 985023.875 RSTD < 985023.9375 | Tc |
| RSTD_31520768 | 985023.9375 RSTD < 985024 | Tc |
| RSTD_31520769 | 985024 RSTD | Tc |

Table 10.1.23.3.1-11 Report mapping for k = -5.

| Reported Quantity Value, | Measured Quantity Value, | Unit |
| --- | --- | --- |
| RSTD_i | RSTD |  |
| RSTD_00000000 | RSTD < -985024 | Tc |
| RSTD_00000001 | -985024 RSTD < -985023.9688 | Tc |
| RSTD_00000002 | -985023.9688 RSTD < -985023.9375 | Tc |
|  |  | … |
| RSTD_31520768 | -0.0312 RSTD < 0 | Tc |
| RSTD_31520769 | 0 RSTD < 0.0312 | Tc |
| … | … | … |
| RSTD_63041535 | 985023.9375 RSTD < 985023.9688 | Tc |
| RSTD_63041536 | 985023.9688 RSTD < 985024 | Tc |
| RSTD_63041537 | 985024 RSTD | Tc |

Table 10.1.23.3.1-12 Report mapping for k = -6.

| Reported Quantity Value, | Measured Quantity Value, | Unit |
| --- | --- | --- |
| RSTD_i | RSTD |  |
| RSTD_000000000 | RSTD < -985024 | Tc |
| RSTD_000000001 | -985024 RSTD < -985023.9844 | Tc |
| RSTD_000000002 | -985023.9844 RSTD < -985023.9688 | Tc |
|  |  | … |
| RSTD_063041536 | -0.0156 RSTD < 0 | Tc |
| RSTD_063041537 | 0 RSTD < 0.0156 | Tc |
| … | … | … |
| RSTD_126083071 | 985023.9688 RSTD < 985023.9844 | Tc |
| RSTD_126083072 | 985023.9844 RSTD < 985024 | Tc |
| RSTD_126083073 | 985024 RSTD | Tc |

##### 10.1.23.3.2	Differential Reporting for DL RSTD Measurement

A first DL RSTD measurement is reported by means of differential reporting, i.e. as RSTD, relative to a second DL RSTD measurement (RSTD2), provided that:

-	the absolute measured quantity value of the second DL RSTD measurement (RSTD2) is not larger than the absolute measured quantity value of the first DL RSTD measurement (RSTD1), i.e., RSTD=RSTD1-RSTD2≥0, and

-	the absolute value of the second DL RSTD measurement (RSTD2) is reported together with RSTD for the first DL RSTD measurement.

The reporting range for differential reporting RSTD of the first DL RSTD measurement is defined from 0 up to 8191Tc with the resolution step of 2kTc, where

Tc is defined in TS 38.211 [6],

kmin≤k≤kmax,

kmin = -6 and kmax = 5, when configured PRS resource for the reference cell and neighbor cell measured for the RSTD measurement are in FR1 and/or FR2,

k≥ timingReportingGranularityFactor [34] configured by LMF via LPP for the RSTD measurement.

The measurement report mapping for different k values are specified in Tables 10.1.23.3.2-1  10.1.23.3.2-12.

Table 10.1.23.3.2-1: Report mapping for k=0

| Reported Quantity Value, DIFFRSTD_i | RSTD = RSTD1 RSTD2 | Unit |
| --- | --- | --- |
| DIFFRSTD_0000 | 0 RSTD < 1 | Tc |
| DIFFRSTD_0001 | 1 RSTD < 2 | Tc |
| DIFFRSTD_0002 | 2 RSTD < 3 | Tc |
|  |  | … |
| DIFFRSTD_8189 | 8189 RSTD < 8190 | Tc |
| DIFFRSTD_8190 | 8190 RSTD < 8191 | Tc |
| DIFFRSTD_8191 | 8191 RSTD | Tc |

Table 10.1.23.3.2-2: Report mapping for k=1

| Reported Quantity Value, DIFFRSTD_i | RSTD = RSTD1 RSTD2 | Unit |
| --- | --- | --- |
| DIFFRSTD_0000 | 0 RSTD < 2 | Tc |
| DIFFRSTD_0001 | 2 RSTD < 4 | Tc |
| DIFFRSTD_0002 | 4 RSTD < 6 | Tc |
|  |  | … |
| DIFFRSTD_4093 | 8186 RSTD < 8188 | Tc |
| DIFFRSTD_4094 | 8188 RSTD < 8190 | Tc |
| DIFFRSTD_4095 | 8190 RSTD | Tc |

Table 10.1.23.3.2-3: Report mapping for k=2

| Reported Quantity Value, DIFFRSTD_i | RSTD = RSTD1 RSTD2 | Unit |
| --- | --- | --- |
| DIFFRSTD_0000 | 0 RSTD < 4 | Tc |
| DIFFRSTD_0001 | 4 RSTD < 8 | Tc |
| DIFFRSTD_0002 | 8 RSTD < 12 | Tc |
|  |  | … |
| DIFFRSTD_2045 | 8180 RSTD < 8184 | Tc |
| DIFFRSTD_2046 | 8184 RSTD < 8188 | Tc |
| DIFFRSTD_2047 | 8188 RSTD | Tc |

Table 10.1.23.3.2-4: Report mapping for k=3

| Reported Quantity Value, DIFFRSTD_i | RSTD = RSTD1 RSTD2 | Unit |
| --- | --- | --- |
| DIFFRSTD_0000 | 0 RSTD < 8 | Tc |
| DIFFRSTD_0001 | 8 RSTD < 16 | Tc |
| DIFFRSTD_0002 | 16 RSTD < 24 | Tc |
|  |  | … |
| DIFFRSTD_1021 | 8168 RSTD < 8176 | Tc |
| DIFFRSTD_1022 | 8176 RSTD < 8184 | Tc |
| DIFFRSTD_1023 | 8184 RSTD | Tc |

Table 10.1.23.3.2-5: Report mapping for k=4

| Reported Quantity Value, DIFFRSTD_i | RSTD = RSTD1 RSTD2 | Unit |
| --- | --- | --- |
| DIFFRSTD_000 | 0 RSTD < 16 | Tc |
| DIFFRSTD_001 | 16 RSTD < 32 | Tc |
| DIFFRSTD_002 | 32 RSTD < 48 | Tc |
|  |  | … |
| DIFFRSTD_509 | 8144 RSTD < 8160 | Tc |
| DIFFRSTD_510 | 8160 RSTD < 8176 | Tc |
| DIFFRSTD_511 | 8176 RSTD | Tc |

Table 10.1.23.3.2-6: Report mapping for k=5

| Reported Quantity Value, DIFFRSTD_i | RSTD = RSTD1 RSTD2 | Unit |
| --- | --- | --- |
| DIFFRSTD_000 | 0 RSTD < 32 | Tc |
| DIFFRSTD_001 | 32 RSTD < 64 | Tc |
| DIFFRSTD_002 | 64 RSTD < 96 | Tc |
|  |  | … |
| DIFFRSTD_253 | 8096 RSTD < 8128 | Tc |
| DIFFRSTD_254 | 8128 RSTD < 8160 | Tc |
| DIFFRSTD_255 | 8160 RSTD | Tc |

Table 10.1.23.3.2-7: Report mapping for k = -1.

| Reported Quantity Value, DIFFRSTD_i | RSTD = RSTD1 RSTD2 | Unit |
| --- | --- | --- |
| DIFFRSTD_00000 | 0 RSTD < 0.5 | Tc |
| DIFFRSTD_00001 | 0.5 RSTD < 1 | Tc |
| DIFFRSTD_00002 | 1 RSTD < 1.5 | Tc |
|  |  | … |
| DIFFRSTD_16380 | 8190 RSTD < 8190.5 | Tc |
| DIFFRSTD_16381 | 8190.5 RSTD < 8191 | Tc |
| DIFFRSTD_16382 | 8191 RSTD | Tc |

Table 10.1.23.3.2-8: Report mapping for k = -2.

| Reported Quantity Value, DIFFRSTD_i | RSTD = RSTD1 RSTD2 | Unit |
| --- | --- | --- |
| DIFFRSTD_00000 | 0 RSTD < 0.25 | Tc |
| DIFFRSTD_00001 | 0.25 RSTD < 0.5 | Tc |
| DIFFRSTD_00002 | 0.5 RSTD < 0. 75 | Tc |
|  |  | … |
| DIFFRSTD_32762 | 8190.5 RSTD < 8190.75 | Tc |
| DIFFRSTD_32763 | 8190.75 RSTD < 8191 | Tc |
| DIFFRSTD_32764 | 8191 RSTD | Tc |

Table 10.1.23.3.2-9: Report mapping for k = -3.

| Reported Quantity Value, DIFFRSTD_i | RSTD = RSTD1 RSTD2 | Unit |
| --- | --- | --- |
| DIFFRSTD_00000 | 0 RSTD < 0.125 | Tc |
| DIFFRSTD_00001 | 0.125 RSTD < 0.25 | Tc |
| DIFFRSTD_00002 | 0.25 RSTD < 0. 375 | Tc |
|  |  | … |
| DIFFRSTD_65526 | 8190.75 RSTD < 8190.875 | Tc |
| DIFFRSTD_65527 | 8190.875 RSTD < 8191 | Tc |
| DIFFRSTD_65528 | 8191 RSTD | Tc |

Table 10.1.23.3.2-10: Report mapping for k = -4.

| Reported Quantity Value, DIFFRSTD_i | RSTD = RSTD1 RSTD2 | Unit |
| --- | --- | --- |
| DIFFRSTD_000000 | 0 RSTD < 0.0625 | Tc |
| DIFFRSTD_000001 | 0.0625 RSTD < 0.125 | Tc |
| DIFFRSTD_000002 | 0.125 RSTD < 0.1875 | Tc |
|  |  | … |
| DIFFRSTD_131054 | 8190.875 RSTD < 8190.9375 | Tc |
| DIFFRSTD_131055 | 8190.9375 RSTD < 8191 | Tc |
| DIFFRSTD_131056 | 8191 RSTD | Tc |

Table 10.1.23.3.2-11: Report mapping for k = -5.

| Reported Quantity Value, DIFFRSTD_i | RSTD = RSTD1 RSTD2 | Unit |
| --- | --- | --- |
| DIFFRSTD_000000 | 0 RSTD < 0.0312 | Tc |
| DIFFRSTD_000001 | 0.0312 RSTD < 0.0625 | Tc |
| DIFFRSTD_000002 | 0.0625 RSTD < 0.0938 | Tc |
|  |  | … |
| DIFFRSTD_262110 | 8190.9375 RSTD < 8190.9688 | Tc |
| DIFFRSTD_262111 | 8190.9688 RSTD < 8191 | Tc |
| DIFFRSTD_262112 | 8191 RSTD | Tc |

Table 10.1.23.3.2-12: Report mapping for k = -6.

| Reported Quantity Value, DIFFRSTD_i | RSTD = RSTD1 RSTD2 | Unit |
| --- | --- | --- |
| DIFFRSTD_000000 | 0 RSTD < 0. 0156 | Tc |
| DIFFRSTD_000001 | 0. 0156 RSTD < 0. 0312 | Tc |
| DIFFRSTD_000002 | 0. 0312 RSTD < 0.0469 | Tc |
|  |  | … |
| DIFFRSTD_524222 | 8190.9688 RSTD < 8190. 9844 | Tc |
| DIFFRSTD_524223 | 8190. 9844 RSTD < 8191 | Tc |
| DIFFRSTD_524224 | 8191 RSTD | Tc |

##### 10.1.23.3.3	Additional Path Report Mapping for DL RSTD

The reporting range for the additional path reporting for an RSTD measurement is defined up to the range from -8175Tc to 8175Tc with the resolution step of 2kTc, where

Tc is defined in TS 38.211 [6],

kmin≤k≤kmax,

kmin = -6 and kmax = 5, when configured PRS resource for the reference cell and neighbor cell measured for the RSTD measurement are in FR1 and/or FR2,

k≥ timingReportingGranularityFactor [34] configured by LMF via LPP for the RSTD measurement.

The UE can report the timing of up to two additional paths with respect to the path timing determining the RSTD measurement.

A UE capable of  additionalPathsExtSupport-r17 can report the timing for a number additional paths, up to its capability, with respect to the path timing determining the RSTD measurement.

The report mappings for different k values are specified in Tables 10.1.23.3.3-1  10.1.23.3.3-12.

Table 10.1.23.3.3-1: Report mapping for k=0

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_00000 | path < -8175 | Tc |
| path_00001 | -8175 path < -8174 | Tc |
| path_00002 | -8174 path < -8173 | Tc |
|  |  | … |
| path_08175 | -1 path < 0 | Tc |
| path_08176 | 0 path < 1 | Tc |
| … | … | … |
| path_16349 | 8173 path < 8174 | Tc |
| path_16350 | 8174 path < 8175 | Tc |
| path_16351 | 8175 path | Tc |

Table 10.1.23.3.3-2: Report mapping for k=1

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000 | path < -8175 | Tc |
| path_0001 | -8175 path < -8173 | Tc |
| path_0002 | -8173 path < -8171 | Tc |
|  |  | … |
| path_4088 | -1 path < 1 | Tc |
| … | … | … |
| path_8174 | 8171 path < 8173 | Tc |
| path_8175 | 8173 path < 8175 | Tc |
| path_8176 | 8175 path | Tc |

Table 10.1.23.3.3-3: Report mapping for k=2

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000 | path < -8174 | Tc |
| path_0001 | -8174 path < -8170 | Tc |
| path_0002 | -8170 path < -8166 | Tc |
|  |  | … |
| path_2044 | -2 path < 2 | Tc |
| … | … | … |
| path_4086 | 8166 path < 8170 | Tc |
| path_4087 | 8170 path < 8174 | Tc |
| path_4088 | 8174 path | Tc |

Table 10.1.23.3.3-4: Report mapping for k=3

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000 | path < -8172 | Tc |
| path_0001 | -8172 path < -8164 | Tc |
| path_0002 | -8164 path < -8156 | Tc |
|  |  | … |
| path_1022 | -4 path < 4 | Tc |
| … | … | … |
| path_2042 | 8156 path < 8164 | Tc |
| path_2043 | 8164 path < 8172 | Tc |
| path_2044 | 8172 path | Tc |

Table 10.1.23.3.3-5: Report mapping for k=4

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000 | path < -8168 | Tc |
| path_0001 | -8168 path < -8152 | Tc |
| path_0002 | -8152 path < -8136 | Tc |
|  |  | … |
| path_511 | -8 path < 8 | Tc |
| … | … | … |
| path_1020 | 8136 path < 8152 | Tc |
| path_1021 | 8152 path < 8168 | Tc |
| path_1022 | 8168 path | Tc |

Table 10.1.23.3.3-6: Report mapping for k=5

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_000 | path < -8160 | Tc |
| path_001 | -8160 path < -8128 | Tc |
| path_002 | -8128 path < -8096 | Tc |
|  |  | … |
| path_256 | 0 path < 32 | Tc |
| … | … | … |
| path_509 | 8096 path < 8128 | Tc |
| path_510 | 8128 path < 8160 | Tc |
| path_511 | 8160 path | Tc |

Table 10.1.23.3.3-7 Report mapping for k = -1.

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_00000 | path < -8175 | Tc |
| path_00001 | -8175 path < -8174.5 | Tc |
| path_00002 | -8174.5 path < -8174 | Tc |
|  |  | … |
| path_16350 | -0.5 path < 0 | Tc |
| path_16351 | 0 path < 0.5 | Tc |
| … | … | … |
| path_32699 | 8174 path < 8174.5 | Tc |
| path_32700 | 8174.5 path < 8175 | Tc |
| path_32701 | 8175 path | Tc |

Table 10.1.23.3.3-8 Report mapping for for k = -2.

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_00000 | path < -8175 | Tc |
| path_00001 | -8175 path < -8174.75 | Tc |
| path_00002 | -8174.75 path < -8174.5 | Tc |
|  |  | … |
| path_32700 | -0.25 path < 0 | Tc |
| path_32701 | 0 path < 0.25 | Tc |
| … | … | … |
| path_65399 | 8174.5 path < 8174.75 | Tc |
| path_65400 | 8174.75 path < 8175 | Tc |
| path_65401 | 8175 path | Tc |

Table 10.1.23.3.3-9 Report mapping for for k = -3.

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_00000 | path < -8175 | Tc |
| path_00001 | -8175 path < -8174.8750 | Tc |
| path_00002 | -8174.8750 path < -8174.7500 | Tc |
|  |  | … |
| path_065400 | -0.25 path < 0 | Tc |
| path_065401 | 0 path < 0.25 | Tc |
| … | … | … |
| path_130799 | 8174.7500 path < 8174.8750 | Tc |
| path_130800 | 8174.8750 path < 8175 | Tc |
| path_130801 | 8175 path | Tc |

Table 10.1.23.3.3-10 Report mapping for for k = -4.

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_000000 | path < -8175 | Tc |
| path_000001 | -8175 path < -8174.9375 | Tc |
| path_000002 | -8174.9375 path < -8174.8750 | Tc |
|  |  | … |
| path_130800 | -0.25 path < 0 | Tc |
| path_130801 | 0 path < 0.25 | Tc |
| … | … | … |
| path_261599 | 8174.8750 path < 8174.9375 | Tc |
| path_261600 | 8174.9375 path < 8175 | Tc |
| path_261601 | 8175 path | Tc |

Table 10.1.23.3.3-11 Report mapping for for k = -5.

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000000 | path < -8175 | Tc |
| path_0000001 | -8175 path < -8174.9688 | Tc |
| path_0000002 | -8174.9688 path < -8174.9375 | Tc |
|  |  | … |
| path_0261600 | -0.0312 path < 0 | Tc |
| path_0261601 | 0 path < 0.0312 | Tc |
| … | … | … |
| path_523199 | 8174.9375 path < 8174.9688 | Tc |
| path_523200 | 8174.9688 path < 8175 | Tc |
| path_523201 | 8175 path | Tc |

Table 10.1.23.3.3-12 Report mapping for for k = -6.

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000000 | path < -8175 | Tc |
| path_0000001 | -8175 path < -8174.9844 | Tc |
| path_0000002 | -8174.9844 path < -8174.9688 | Tc |
|  |  | … |
| path_0523200 | -0.0156 path < 0 | Tc |
| path_0523201 | 0 path < 0.0156 | Tc |
| … | … | … |
| path_1046399 | 8174.9688 path < 8174.9844 | Tc |
| path_1046400 | 8174.9844 path < 8175 | Tc |
| path_1046401 | 8175 path | Tc |

<END OF CHANGE #1>

<START OF CHANGE #2>

#### 10.1.25.3	Report mapping

Absolute UE Rx-Tx measurement reporting in clause 10.1.25.3.1, differential reporting for UE Rx-Tx measurement in clause 10.1.25.3.2, and additional path report mapping for UE Rx-Tx measurement in clause 10.1.25.3.3 applies, regardless of number of samples used to measure PRS, to report:

-	TEG based measurement corresponding to UE reported Rx TEG in nr-UE-Rx-TEG-ID-r17 [34],

-	gap-based UE Rx-Tx measurement,

-	gapless UE Rx-Tx measurement,

-	UE Rx-Tx in RRC_INACTIVE state.

##### 10.1.25.3.1	Absolute UE Rx-Tx Measurement Report Mapping

The reporting range for the absolute UE Rx-Tx time difference measurement (TUE Rx-Tx) is defined from -985024Tc to 985024Tc with the resolution step of 2kTc, where:

Tc is defined in TS 38.211 [6],

kmin≤k≤kmax,

kmin = -6 and kmax = 5, when configured PRS and SRS resource for TUE Rx-Tx measurement are in FR1 and/or FR2,

k≥ timingReportingGranularityFactor [34] configured by LMF via LPP for the UE Rx-Tx time difference measurement.

The TUE Rx-Tx report mapping for k = {0, 1, 2, 3, 4, 5, -1, -2, -3, -4, -5, -6} are specified in Tables 10.1.25.3.1-1, 10.1.25.3.1-2, 10.1.25.3.1-3, 10.1.25.3.1-4, 10.1.25.3.1-5, 10.1.25.3.1-6, 10.1.25.3.1-7, 10.1.25.3.1-8, 10.1.25.3.1-9, 10.1.25.3.1-10, 10.1.25.3.1-11, and 10.1.25.3.1-12 respectively.

Table 10.1.25.3.1-1: Absolute UE Rx-Tx time difference measurement report mapping for k=0

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_0000000 | TUE Rx-Tx < -985024 | Tc |
| RX-TX_TIME_DIFFERENCE_0000001 | -985024  TUE Rx-Tx < -985023 | Tc |
| RX-TX_TIME_DIFFERENCE_0000002 | -985023  TUE Rx-Tx < -985022 | Tc |
|  |  | … |
| RX-TX_TIME_DIFFERENCE_0985024 | -1  TUE Rx-Tx < 0 | Tc |
| RX-TX_TIME_DIFFERENCE_0985025 | 0  TUE Rx-Tx < 1 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_1970047 | 985022  TUE Rx-Tx < 985023 | Tc |
| RX-TX_TIME_DIFFERENCE_1970048 | 985023  TUE Rx-Tx < 985024 | Tc |
| RX-TX_TIME_DIFFERENCE_1970049 | 985024  TUE Rx-Tx | Tc |

Table 10.1.25.3.1-2: Absolute UE Rx-Tx time difference measurement report mapping for k=1

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_000000 | TUE Rx-Tx < -985024 | Tc |
| RX-TX_TIME_DIFFERENCE_000001 | -985024  TUE Rx-Tx < -985022 | Tc |
| RX-TX_TIME_DIFFERENCE_000002 | -985022  TUE Rx-Tx < -985020 | Tc |
|  |  | … |
| RX-TX_TIME_DIFFERENCE_492512 | -2  TUE Rx-Tx < 0 | Tc |
| RX-TX_TIME_DIFFERENCE_492513 | 0  TUE Rx-Tx < 2 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_985023 | 985020  TUE Rx-Tx < 985022 | Tc |
| RX-TX_TIME_DIFFERENCE_985024 | 985022  TUE Rx-Tx < 985024 | Tc |
| RX-TX_TIME_DIFFERENCE_985025 | 985024  TUE Rx-Tx | Tc |

Table 10.1.25.3.1-3: Absolute UE Rx-Tx time difference measurement report mapping for k=2

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_000000 | TUE Rx-Tx < -985024 | Tc |
| RX-TX_TIME_DIFFERENCE_000001 | -985024  TUE Rx-Tx < -985020 | Tc |
| RX-TX_TIME_DIFFERENCE_000002 | -985020  TUE Rx-Tx < -985016 | Tc |
|  |  | … |
| RX-TX_TIME_DIFFERENCE_246256 | -4  TUE Rx-Tx < 0 | Tc |
| RX-TX_TIME_DIFFERENCE_246257 | 0  TUE Rx-Tx < 4 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_492511 | 985016  TUE Rx-Tx < 985020 | Tc |
| RX-TX_TIME_DIFFERENCE_492512 | 985020  TUE Rx-Tx < 985024 | Tc |
| RX-TX_TIME_DIFFERENCE_492513 | 985024  TUE Rx-Tx | Tc |

Table 10.1.25.3.1-4: Absolute UE Rx-Tx time difference measurement report mapping for k=3

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_000000 | TUE Rx-Tx < -985024 | Tc |
| RX-TX_TIME_DIFFERENCE_000001 | -985024  TUE Rx-Tx < -985016 | Tc |
| RX-TX_TIME_DIFFERENCE_000002 | -985016  TUE Rx-Tx < -985008 | Tc |
|  |  | … |
| RX-TX_TIME_DIFFERENCE_123128 | -8  TUE Rx-Tx < 0 | Tc |
| RX-TX_TIME_DIFFERENCE_123129 | 0  TUE Rx-Tx < 8 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_246255 | 985008  TUE Rx-Tx < 985016 | Tc |
| RX-TX_TIME_DIFFERENCE_246256 | 985016  TUE Rx-Tx < 985024 | Tc |
| RX-TX_TIME_DIFFERENCE_246257 | 985024  TUE Rx-Tx | Tc |

Table 10.1.25.3.1-5: Absolute UE Rx-Tx time difference measurement report mapping for k=4

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_000000 | TUE Rx-Tx < -985024 | Tc |
| RX-TX_TIME_DIFFERENCE_000001 | -985024  TUE Rx-Tx < -985008 | Tc |
| RX-TX_TIME_DIFFERENCE_000002 | -985008  TUE Rx-Tx < -984992 | Tc |
|  |  | … |
| RX-TX_TIME_DIFFERENCE_061564 | -16  TUE Rx-Tx < 0 | Tc |
| RX-TX_TIME_DIFFERENCE_061565 | 0  TUE Rx-Tx < 16 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_123127 | 984992  TUE Rx-Tx < 985008 | Tc |
| RX-TX_TIME_DIFFERENCE_123128 | 985008  TUE Rx-Tx < 985024 | Tc |
| RX-TX_TIME_DIFFERENCE_123129 | 985024  TUE Rx-Tx | Tc |

Table 10.1.25.3.1-6: Absolute UE Rx-Tx time difference measurement report mapping for k=5

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_00000 | TUE Rx-Tx < -985024 | Tc |
| RX-TX_TIME_DIFFERENCE_00001 | -985024  TUE Rx-Tx < -984992 | Tc |
| RX-TX_TIME_DIFFERENCE_00002 | -984992  TUE Rx-Tx < -984960 | Tc |
|  |  | … |
| RX-TX_TIME_DIFFERENCE_30782 | -32  TUE Rx-Tx < 0 | Tc |
| RX-TX_TIME_DIFFERENCE_30783 | 0  TUE Rx-Tx < 32 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_61563 | 984960  TUE Rx-Tx < 984992 | Tc |
| RX-TX_TIME_DIFFERENCE_61564 | 984992  TUE Rx-Tx < 985024 | Tc |
| RX-TX_TIME_DIFFERENCE_61565 | 985024  TUE Rx-Tx | Tc |

Table 10.1.25.3.1-7: Absolute UE Rx-Tx time difference measurement report mapping for k = -1.

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_0000000 | TUE Rx-Tx < -985024 | Tc |
| RX-TX_TIME_DIFFERENCE_0000001 | -985024  TUE Rx-Tx < -985023.5 | Tc |
| RX-TX_TIME_DIFFERENCE_0000002 | -985023.5  TUE Rx-Tx < -985023 | Tc |
|  |  | … |
| RX-TX_TIME_DIFFERENCE_1970048 | -0.5  TUE Rx-Tx < 0 | Tc |
| RX-TX_TIME_DIFFERENCE_1970049 | 0  TUE Rx-Tx < 0.5 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_3940095 | 985023  TUE Rx-Tx < 985023.5 | Tc |
| RX-TX_TIME_DIFFERENCE_3940096 | 985023.5  TUE Rx-Tx < 985024 | Tc |
| RX-TX_TIME_DIFFERENCE_3940097 | 985024  TUE Rx-Tx | Tc |

Table 10.1.25.3.1-8: Absolute UE Rx-Tx time difference measurement report mapping for k = -2.

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_0000000 | TUE Rx-Tx < -985024 | Tc |
| RX-TX_TIME_DIFFERENCE_0000001 | -985024  TUE Rx-Tx < -985023.75 | Tc |
| RX-TX_TIME_DIFFERENCE_0000002 | -985023.75  TUE Rx-Tx < -985023.5 | Tc |
|  |  | … |
| RX-TX_TIME_DIFFERENCE_3940096 | -0.25  TUE Rx-Tx < 0 | Tc |
| RX-TX_TIME_DIFFERENCE_3940097 | 0  TUE Rx-Tx < 0.25 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_7880191 | 985023.5  TUE Rx-Tx < 985023.75 | Tc |
| RX-TX_TIME_DIFFERENCE_7880192 | 985023.75  TUE Rx-Tx < 985024 | Tc |
| RX-TX_TIME_DIFFERENCE_7880193 | 985024  TUE Rx-Tx | Tc |

Table 10.1.25.3.1-9: Absolute UE Rx-Tx time difference measurement report mapping for k = -3.

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_00000000 | TUE Rx-Tx < -985024 | Tc |
| RX-TX_TIME_DIFFERENCE_00000001 | -985024  TUE Rx-Tx < -985023.875 | Tc |
| RX-TX_TIME_DIFFERENCE_00000002 | -985023.875  TUE Rx-Tx < -985023.75 | Tc |
|  |  | … |
| RX-TX_TIME_DIFFERENCE_07880192 | -0.125  TUE Rx-Tx < 0 | Tc |
| RX-TX_TIME_DIFFERENCE_07880193 | 0  TUE Rx-Tx < 0.125 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_15760383 | 985023.75  TUE Rx-Tx < 985023.875 | Tc |
| RX-TX_TIME_DIFFERENCE_15760384 | 985023.875  TUE Rx-Tx < 985024 | Tc |
| RX-TX_TIME_DIFFERENCE_15760385 | 985024  TUE Rx-Tx | Tc |

Table 10.1.25.3.1-10: Absolute UE Rx-Tx time difference measurement report mapping for k = -4.

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_00000000 | TUE Rx-Tx < -985024 | Tc |
| RX-TX_TIME_DIFFERENCE_00000001 | -985024  TUE Rx-Tx < -985023.9375 | Tc |
| RX-TX_TIME_DIFFERENCE_00000002 | -985023.9375  TUE Rx-Tx < -985023.875 | Tc |
|  |  | … |
| RX-TX_TIME_DIFFERENCE_15760384 | -0.0625  TUE Rx-Tx < 0 | Tc |
| RX-TX_TIME_DIFFERENCE_15760385 | 0  TUE Rx-Tx < 0.0625 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_31520767 | 985023.875  TUE Rx-Tx < 985023.9375 | Tc |
| RX-TX_TIME_DIFFERENCE_31520768 | 985023.9375  TUE Rx-Tx < 985024 | Tc |
| RX-TX_TIME_DIFFERENCE_31520769 | 985024  TUE Rx-Tx | Tc |

Table 10.1.25.3.1-11: Absolute UE Rx-Tx time difference measurement report mapping for k = -5.

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_00000000 | TUE Rx-Tx < -985024 | Tc |
| RX-TX_TIME_DIFFERENCE_00000001 | -985024  TUE Rx-Tx < -985023.9688 | Tc |
| RX-TX_TIME_DIFFERENCE_00000002 | -985023.9688  TUE Rx-Tx < -985023.9375 | Tc |
|  |  | … |
| RX-TX_TIME_DIFFERENCE_31520768 | -0.0312  TUE Rx-Tx < 0 | Tc |
| RX-TX_TIME_DIFFERENCE_31520769 | 0  TUE Rx-Tx < 0.0312 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_63041535 | 985023.9375  TUE Rx-Tx < 985023.9688 | Tc |
| RX-TX_TIME_DIFFERENCE_63041536 | 985023.9688  TUE Rx-Tx < 985024 | Tc |
| RX-TX_TIME_DIFFERENCE_63041537 | 985024  TUE Rx-Tx | Tc |

Table 10.1.25.3.1-12: Absolute UE Rx-Tx time difference measurement report mapping for k = -6.

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_00000000 | TUE Rx-Tx < -985024 | Tc |
| RX-TX_TIME_DIFFERENCE_00000001 | -985024  TUE Rx-Tx < -985023.9844 | Tc |
| RX-TX_TIME_DIFFERENCE_00000002 | -985023.9844  TUE Rx-Tx < -985023.9688 | Tc |
|  |  | … |
| RX-TX_TIME_DIFFERENCE_63041536 | -0.0156  TUE Rx-Tx < 0 | Tc |
| RX-TX_TIME_DIFFERENCE_63041537 | 0  TUE Rx-Tx < 0.0156 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_126083071 | 985023.9688  TUE Rx-Tx < 985023.9844 | Tc |
| RX-TX_TIME_DIFFERENCE_126083072 | 985023.9844  TUE Rx-Tx < 985024 | Tc |
| RX-TX_TIME_DIFFERENCE_126083073 | 985024  TUE Rx-Tx | Tc |

##### 10.1.25.3.2	Differential UE Rx-Tx Measurement Report Mapping

The reporting range for differential UE Rx-Tx time difference measurement (TUE Rx-Tx) is defined from 0 up to 8191Tc where:

TUE Rx-Tx = TUE Rx-Tx1 - TUE Rx-Tx2; where:

TUE Rx-Tx1 > TUE Rx-Tx2,

TUE Rx-Tx1 is the first absolute UE Rx-Tx time difference measurement,

TUE Rx-Tx1 is the second absolute UE Rx-Tx time difference measurement,

Tc is defined in TS 38.211 [6],

kmin≤k≤kmax,

kmin = -6 and kmax = 5, when PRS and SRS resource configured for TUE Rx-Tx measurement are in FR1 and/or FR2,

k≥ timingReportingGranularityFactor [34] configured by LMF via LPP for the UE Rx-Tx time difference measurement.

The TUE Rx-Tx report mapping for k = {0, 1, 2, 3, 4, 5, -1, -2, -3, -4, -5, -6} are specified in Tables 10.1.25.3.2-1, 10.1.25.3.2-2, 10.1.25.3.2-3, 10.1.25.3.2-4, 10.1.25.3.2-5, 10.1.25.3.2-6, 10.1.25.3.2-7, 10.1.25.3.2-8, 10.1.25.3.2-9, 10.1.25.3.2-10, 10.1.25.3.2-11, and 10.1.25.3.2-12 respectively.

Table 10.1.25.3.2-1: Differential UE Rx-Tx time difference measurement report mapping for k=0

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| DIFF_RX-TX_TIME_DIFFERENCE_0000 | 0  TUE Rx-Tx < 1 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_0001 | 1  TUE Rx-Tx < 2 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_0002 | 2  TUE Rx-Tx < 3 | Tc |
|  |  | … |
| DIFF_RX-TX_TIME_DIFFERENCE_8189 | 8189  TUE Rx-Tx < 8190 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_8190 | 8190  TUE Rx-Tx < 8191 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_8191 | 8191  TUE Rx-Tx | Tc |

Table 10.1.25.3.2-2: Differential UE Rx-Tx time difference measurement report mapping for k=1

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| DIFF_RX-TX_TIME_DIFFERENCE_0000 | 0  TUE Rx-Tx < 2 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_0001 | 2  TUE Rx-Tx < 4 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_0002 | 4  TUE Rx-Tx < 6 | Tc |
|  |  | … |
| DIFF_RX-TX_TIME_DIFFERENCE_4093 | 8186  TUE Rx-Tx < 8188 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_4094 | 8188  TUE Rx-Tx < 8190 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_4095 | 8190  TUE Rx-Tx | Tc |

Table 10.1.25.3.2-3: Differential UE Rx-Tx time difference measurement report mapping for k=2

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| DIFF_RX-TX_TIME_DIFFERENCE_0000 | 0  TUE Rx-Tx < 4 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_0001 | 4  TUE Rx-Tx < 8 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_0002 | 8  TUE Rx-Tx < 12 | Tc |
|  |  | … |
| DIFF_RX-TX_TIME_DIFFERENCE_2045 | 8180  TUE Rx-Tx < 8184 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_2046 | 8184  TUE Rx-Tx < 8188 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_2047 | 8188  TUE Rx-Tx | Tc |

Table 10.1.25.3.2-4: Differential UE Rx-Tx time difference measurement report mapping for k=3

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| DIFF_RX-TX_TIME_DIFFERENCE_0000 | 0  TUE Rx-Tx < 8 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_0001 | 8  TUE Rx-Tx < 16 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_0002 | 16  TUE Rx-Tx < 24 | Tc |
|  |  | … |
| DIFF_RX-TX_TIME_DIFFERENCE_1021 | 8168  TUE Rx-Tx < 8176 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_1022 | 8176  TUE Rx-Tx < 8184 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_1023 | 8184  TUE Rx-Tx | Tc |

Table 10.1.25.3.2-5: Differential UE Rx-Tx time difference measurement report mapping for k=4

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| DIFF_RX-TX_TIME_DIFFERENCE_000 | 0  TUE Rx-Tx < 16 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_001 | 16  TUE Rx-Tx < 32 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_002 | 32  TUE Rx-Tx < 48 | Tc |
|  |  | … |
| DIFF_RX-TX_TIME_DIFFERENCE_509 | 8144  TUE Rx-Tx < 8160 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_510 | 8160  TUE Rx-Tx < 8176 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_511 | 8176  TUE Rx-Tx | Tc |

Table 10.1.25.3.2-6: Differential UE Rx-Tx time difference measurement report mapping for k=5

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| DIFF_RX-TX_TIME_DIFFERENCE_000 | 0  TUE Rx-Tx < 32 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_001 | 32  TUE Rx-Tx < 64 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_002 | 64  TUE Rx-Tx < 96 | Tc |
|  |  | … |
| DIFF_RX-TX_TIME_DIFFERENCE_253 | 8096  TUE Rx-Tx < 8128 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_254 | 8128  TUE Rx-Tx < 8160 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_255 | 8160  TUE Rx-Tx | Tc |

Table 10.1.25.3.2-7: Differential UE Rx-Tx time difference measurement report mapping for k = -1.

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| DIFF_RX-TX_TIME_DIFFERENCE_00000 | 0  TUE Rx-Tx < 0.5 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_00001 | 0.5  TUE Rx-Tx < 1 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_00002 | 1  TUE Rx-Tx < 1.5 | Tc |
|  |  | … |
| DIFF_RX-TX_TIME_DIFFERENCE_16380 | 8190  TUE Rx-Tx < 8190.5 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_16381 | 8190.5  TUE Rx-Tx < 8191 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_16382 | 8191  TUE Rx-Tx | Tc |

Table 10.1.25.3.2-8: Differential UE Rx-Tx time difference measurement report mapping for k = -2.

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| DIFF_RX-TX_TIME_DIFFERENCE_00000 | 0  TUE Rx-Tx < 0.25 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_00001 | 0.25  TUE Rx-Tx < 0.5 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_00002 | 0.5  TUE Rx-Tx < 0.75 | Tc |
|  |  | … |
| DIFF_RX-TX_TIME_DIFFERENCE_32762 | 8190.5  TUE Rx-Tx < 8190.75 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_32763 | 8190.75  TUE Rx-Tx < 8191 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_32764 | 8191  TUE Rx-Tx | Tc |

Table 10.1.25.3.2-9: Differential UE Rx-Tx time difference measurement report mapping for k = -3.

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| DIFF_RX-TX_TIME_DIFFERENCE_00000 | 0  TUE Rx-Tx < 0.125 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_00001 | 0.125  TUE Rx-Tx < 0.25 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_00002 | 0.25  TUE Rx-Tx < 0.375 | Tc |
|  |  | … |
| DIFF_RX-TX_TIME_DIFFERENCE_65526 | 8190.75  TUE Rx-Tx < 8190.875 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_65527 | 8190.875  TUE Rx-Tx < 8191 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_65528 | 8191  TUE Rx-Tx | Tc |

Table 10.1.25.3.2-10: Differential UE Rx-Tx time difference measurement report mapping for k = -4.

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| DIFF_RX-TX_TIME_DIFFERENCE_000000 | 0  TUE Rx-Tx < 0.0625 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_000001 | 0.0625  TUE Rx-Tx < 0.125 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_000002 | 0.125  TUE Rx-Tx < 0.1875 | Tc |
|  |  | … |
| DIFF_RX-TX_TIME_DIFFERENCE_131054 | 8190.875  TUE Rx-Tx < 8190.9375 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_131055 | 8190.9375  TUE Rx-Tx < 8191 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_131056 | 8191  TUE Rx-Tx | Tc |

Table 10.1.25.3.2-11: Differential UE Rx-Tx time difference measurement report mapping for k = -5.

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| DIFF_RX-TX_TIME_DIFFERENCE_000000 | 0  TUE Rx-Tx < 0.0312 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_000001 | 0.0312  TUE Rx-Tx < 0.0625 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_000002 | 0.0625  TUE Rx-Tx < 0.0938 | Tc |
|  |  | … |
| DIFF_RX-TX_TIME_DIFFERENCE_262110 | 8190.9375  TUE Rx-Tx < 8190.9688 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_262111 | 8190.9688  TUE Rx-Tx < 8191 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_262112 | 8191  TUE Rx-Tx | Tc |

Table 10.1.25.3.2-12: Differential UE Rx-Tx time difference measurement report mapping for k = -6.

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| DIFF_RX-TX_TIME_DIFFERENCE_000000 | 0  TUE Rx-Tx < 0.0156 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_000001 | 0.0156  TUE Rx-Tx < 0.0312 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_000002 | 0.0312  TUE Rx-Tx < 0.0469 | Tc |
|  |  | … |
| DIFF_RX-TX_TIME_DIFFERENCE_524222 | 8190.9688  TUE Rx-Tx < 8190.9844 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_524223 | 8190.9844  TUE Rx-Tx < 8191 | Tc |
| DIFF_RX-TX_TIME_DIFFERENCE_524224 | 8191  TUE Rx-Tx | Tc |

##### 10.1.25.3.3	Additional Path Report Mapping for UE Rx-Tx Time Difference

The reporting range for the additional path reporting for an UE Rx-Tx time difference measurement is defined up to the range from -8175Tc to 8175Tc with the resolution step of 2kTc, where

Tc is defined in TS 38.211 [6],

kmin≤k≤kmax,

kmin = -6 and kmax = 5, when PRS and SRS resource configured for the UE Rx-Tx time difference measurement are in FR1 and/or FR2,

k≥ timingReportingGranularityFactor [34] configured by LMF via LPP for the UE Rx-Tx time difference measurement.

The UE can report the timing of up to two additional paths with respect to the path timing determining the UE Rx-Tx time difference measurement.

The UE capable of  additionalPathsExtSupport-r17 can report the timing of up to its supported number of additional paths with respect to the path timing determining the UE Rx-Tx measurement.

The report mappings for different k values are specified in Tables 10.1.25.3.3-1  10.1.25.3.3-12.

Table 10.1.25.3.3-1: Report mapping for k=0

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_00000 | path < -8175 | Tc |
| path_00001 | -8175 path < -8174 | Tc |
| path_00002 | -8174 path < -8173 | Tc |
|  |  | … |
| path_08175 | -1 path < 0 | Tc |
| path_08176 | 0 path < 1 | Tc |
| … | … | … |
| path_16349 | 8173 path < 8174 | Tc |
| path_16350 | 8174 path < 8175 | Tc |
| path_16351 | 8175 path | Tc |

Table 10.1.25.3.3-2: Report mapping for k=1

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000 | path < -8175 | Tc |
| path_0001 | -8175 path < -8173 | Tc |
| path_0002 | -8173 path < -8171 | Tc |
|  |  | … |
| path_4088 | -1 path < 1 | Tc |
| … | … | … |
| path_8174 | 8171 path < 8173 | Tc |
| path_8175 | 8173 path < 8175 | Tc |
| path_8176 | 8175 path | Tc |

Table 10.1.25.3.3-3: Report mapping for k=2

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000 | path < -8174 | Tc |
| path_0001 | -8174 path < -8170 | Tc |
| path_0002 | -8170 path < -8166 | Tc |
|  |  | … |
| path_2044 | -2 path < 2 | Tc |
| … | … | … |
| path_4086 | 8166 path < 8170 | Tc |
| path_4087 | 8170 path < 8174 | Tc |
| path_4088 | 8174 path | Tc |

Table 10.1.25.3.3-4: Report mapping for k=3

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000 | path < -8172 | Tc |
| path_0001 | -8172 path < -8164 | Tc |
| path_0002 | -8164 path < -8156 | Tc |
|  |  | … |
| path_1022 | -4 path < 4 | Tc |
| … | … | … |
| path_2042 | 8156 path < 8164 | Tc |
| path_2043 | 8164 path < 8172 | Tc |
| path_2044 | 8172 path | Tc |

Table 10.1.25.3.3-5: Report mapping for k=4

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000 | path < -8168 | Tc |
| path_0001 | -8168 path < -8152 | Tc |
| path_0002 | -8152 path < -8136 | Tc |
|  |  | … |
| path_511 | -8 path < 8 | Tc |
| … | … | … |
| path_1020 | 8136 path < 8152 | Tc |
| path_1021 | 8152 path < 8168 | Tc |
| path_1022 | 8168 path | Tc |

Table 10.1.25.3.3-6: Report mapping for k=5

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_000 | path < -8160 | Tc |
| path_001 | -8160 path < -8128 | Tc |
| path_002 | -8128 path < -8096 | Tc |
|  |  | … |
| path_256 | 0 path < 32 | Tc |
| … | … | … |
| path_509 | 8096 path < 8128 | Tc |
| path_510 | 8128 path < 8160 | Tc |
| path_511 | 8160 path | Tc |

Table 10.1.25.3.3-7: Report mapping for k = -1.

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_00000 | path < -8175 | Tc |
| path_00001 | -8175 path < -8174.5 | Tc |
| path_00002 | -8174.5 path < -8174 | Tc |
|  |  | … |
| path_16350 | -0.5 path < 0 | Tc |
| path_16351 | 0 path < 0.5 | Tc |
| … | … | … |
| path_32699 | 8174 path < 8174.5 | Tc |
| path_32700 | 8174.5 path < 8175 | Tc |
| path_32701 | 8175 path | Tc |

Table 10.1.25.3.3-8: Report mapping for k = -2.

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_00000 | path < -8175 | Tc |
| path_00001 | -8175 path < -8174.75 | Tc |
| path_00002 | -8174.75 path < -8174.5 | Tc |
|  |  | … |
| path_32700 | -0.25 path < 0 | Tc |
| path_32701 | 0 path < 0.25 | Tc |
| … | … | … |
| path_65399 | 8174.5 path < 8174.75 | Tc |
| path_65400 | 8174.75 path < 8175 | Tc |
| path_65401 | 8175 path | Tc |

Table 10.1.25.3.3-9: Report mapping for k = -3.

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_00000 | path < -8175 | Tc |
| path_00001 | -8175 path < -8174.875 | Tc |
| path_00002 | -8174.875 path < -8174.75 | Tc |
|  |  | … |
| path_65400 | -0.125 path < 0 | Tc |
| path_65401 | 0 path < 0.125 | Tc |
| … | … | … |
| path_130799 | 8174.75 path < 8174.875 | Tc |
| path_130800 | 8174.875 path < 8175 | Tc |
| path_130801 | 8175 path | Tc |

Table 10.1.25.3.3-10: Report mapping for k = -4.

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_00000 | path < -8175 | Tc |
| path_00001 | -8175 path < -8174.9375 | Tc |
| path_00002 | -8174.9375 path < -8174.875 | Tc |
|  |  | … |
| path_130800 | -0.0625 path < 0 | Tc |
| path_130801 | 0 path < 0.0625 | Tc |
| … | … | … |
| path_261599 | 8174.875 path < 8174.9375 | Tc |
| path_261600 | 8174.9375 path < 8175 | Tc |
| path_261601 | 8175 path | Tc |

Table 10.1.25.3.3-11: Report mapping for k = -5.

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_000000 | path < -8175 | Tc |
| path_000001 | -8175 path < -8174.9688 | Tc |
| path_000002 | -8174.9688 path < -8174.9375 | Tc |
|  |  | … |
| path_261600 | -0.0312 path < 0 | Tc |
| path_261601 | 0 path < 0.0312 | Tc |
| … | … | … |
| path_523199 | 8174.9375 path < 8174.9688 | Tc |
| path_523200 | 8174.9688 path < 8175 | Tc |
| path_ 523201 | 8175 path | Tc |

Table 10.1.25.3.3-12: Report mapping for k = -6.

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000000 | path < -8175 | Tc |
| path_0000001 | -8175 path < -8174.9844 | Tc |
| path_0000002 | -8174.9844 path < -8174.9688 | Tc |
|  |  | … |
| path_0523200 | -0.0156 path < 0 | Tc |
| path_0523201 | 0 path < 0.0156 | Tc |
| … | … | … |
| path_1046399 | 8174.9688 path < 8174.9844 | Tc |
| path_1046400 | 8174.9844 path < 8175 | Tc |
| path_1046401 | 8175 path | Tc |

<END OF CHANGE #2>

<START OF CHANGE #3>

## 13.1	UL-RTOA

### 13.1.1	Report mapping

The reporting range of UL Relative Time of Arrival (UL-RTOA), as defined in Clause 5.2.2 of TS 38.215 [4], is defined from -985024Tc to +985024Tc. The reporting resolution is uniform across the reporting range and is defined as T = Tc2k where k is selected by gNB from the set {-6, -5, -4, -3, -2, -1, 0, 1, 2, 3, 4, 5}.

Tc is defined in TS 38.211 [6].

LMF provides a recommended resolution parameter, timingReportingGranularityFactor [35]. gNB selects parameter k based on timingReportingGranularityFactor [35] and informs the LMF.

The mapping of measured quantity for each reporting resolution (k) is defined in Table 13.1.1-1 to Table 13.1.1-12.

Table 13.1.1-1: Measurement report mapping for k = 0.

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| UL_RTOA_0000000 | -985024 > UL_RTOA | Tc |
| UL_RTOA_0000001 | -985024 UL_RTOA < -985023 | Tc |
| UL_RTOA_0000002 | -985023 UL_RTOA < -985022 | Tc |
|  |  | … |
| UL_RTOA_0985023 | -2 UL_RTOA < -1 | Tc |
| UL_RTOA_0985024 | -1 UL_RTOA 0 | Tc |
| UL_RTOA_0985025 | 0 < UL_RTOA 1 | Tc |
| UL_RTOA_0985026 | 1 < UL_RTOA 2 | Tc |
| UL_RTOA_0985027 | 2 < UL_RTOA 3 | Tc |
| … | … | … |
| UL_RTOA_1970048 | 985023 < UL_RTOA 985024 | Tc |
| UL_RTOA_1970049 | 985024 < UL_RTOA | Tc |

Table 13.1.1-2: Measurement report mapping for k = 1.

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| UL_RTOA_000000 | -985024 > UL_RTOA | Tc |
| UL_RTOA_000001 | -985024 UL_RTOA < -985022 | Tc |
| UL_RTOA_000002 | -985022 UL_RTOA < -985020 | Tc |
|  |  | … |
| UL_RTOA_492511 | -4 UL_RTOA < -2 | Tc |
| UL_RTOA_492512 | -2 UL_RTOA 0 | Tc |
| UL_RTOA_492513 | 0 < UL_RTOA 2 | Tc |
| UL_RTOA_492514 | 2 < UL_RTOA 4 | Tc |
| UL_RTOA_492515 | 4 < UL_RTOA 6 | Tc |
| … | … | … |
| UL_RTOA_985024 | 985022 < UL_RTOA 985024 | Tc |
| UL_RTOA_985025 | 985024 < UL_RTOA | Tc |

Table 13.1.1-3: Measurement report mapping for k = 2.

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| UL_RTOA_000000 | -985024 > UL_RTOA | Tc |
| UL_RTOA_000001 | -985024 UL_RTOA < -985020 | Tc |
| UL_RTOA_000002 | -985020 UL_RTOA < -985018 | Tc |
|  |  | … |
| UL_RTOA_246255 | -8 UL_RTOA < -4 | Tc |
| UL_RTOA_246256 | -4 UL_RTOA 0 | Tc |
| UL_RTOA_246257 | 0 < UL_RTOA 4 | Tc |
| UL_RTOA_246258 | 4 < UL_RTOA 8 | Tc |
| UL_RTOA_246259 | 8 < UL_RTOA 12 | Tc |
| … | … | … |
| UL_RTOA_492512 | 985020 < UL_RTOA 985024 | Tc |
| UL_RTOA_492513 | 985024 < UL_RTOA | Tc |

Table 13.1.1-4: Measurement report mapping for k = 3.

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| UL_RTOA_000000 | -985024 > UL_RTOA | Tc |
| UL_RTOA_000001 | -985024 UL_RTOA < -985016 | Tc |
| UL_RTOA_000002 | -985016 UL_RTOA < -985008 | Tc |
|  |  | … |
| UL_RTOA_123127 | -16 UL_RTOA < -8 | Tc |
| UL_RTOA_123128 | -8 UL_RTOA 0 | Tc |
| UL_RTOA_123129 | 0 < UL_RTOA 8 | Tc |
| UL_RTOA_123130 | 8 < UL_RTOA 16 | Tc |
| UL_RTOA_123131 | 16 < UL_RTOA 24 | Tc |
| … | … | … |
| UL_RTOA_246256 | 985016 < UL_RTOA 985024 | Tc |
| UL_RTOA_246257 | 985024 < UL_RTOA | Tc |

Table 13.1.1-5: Measurement report mapping for k = 4.

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| UL_RTOA_000000 | -985024 > UL_RTOA | Tc |
| UL_RTOA_000001 | -985024 UL_RTOA < -985008 | Tc |
| UL_RTOA_000002 | -985008 UL_RTOA < -984992 | Tc |
|  |  | … |
| UL_RTOA_061563 | -32 UL_RTOA < -16 | Tc |
| UL_RTOA_061564 | -16 UL_RTOA 0 | Tc |
| UL_RTOA_061565 | 0 < UL_RTOA 16 | Tc |
| UL_RTOA_061566 | 16 < UL_RTOA 32 | Tc |
| UL_RTOA_061567 | 32 < UL_RTOA 48 | Tc |
| … | … | … |
| UL_RTOA_123128 | 985008 < UL_RTOA 985024 | Tc |
| UL_RTOA_123129 | 985024 < UL_RTOA | Tc |

Table 13.1.1-6: Measurement report mapping for k = 5.

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| UL_RTOA_00000 | -985024 > UL_RTOA | Tc |
| UL_RTOA_00001 | -985024 UL_RTOA < -984992 | Tc |
| UL_RTOA_00002 | -984992 UL_RTOA < -984960 | Tc |
|  |  | … |
| UL_RTOA_30781 | -64 UL_RTOA < -32 | Tc |
| UL_RTOA_30782 | -32 UL_RTOA 0 | Tc |
| UL_RTOA_30783 | 0 < UL_RTOA 32 | Tc |
| UL_RTOA_30784 | 32 < UL_RTOA 64 | Tc |
| UL_RTOA_30785 | 64 < UL_RTOA 96 | Tc |
| … | … | … |
| UL_RTOA_61564 | 984992 < UL_RTOA 985024 | Tc |
| UL_RTOA_61565 | 985024 < UL_RTOA | Tc |

Table 13.1.1-7: Measurement report mapping for k = -1.

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| UL_RTOA_0000000 | -985024 > UL_RTOA | Tc |
| UL_RTOA_0000001 | -985024 UL_RTOA < -985023.5 | Tc |
| UL_RTOA_0000002 | -985023.5 UL_RTOA < -985023 | Tc |
|  |  | … |
| UL_RTOA_1970048 | -0.5 UL_RTOA < 0 | Tc |
| UL_RTOA_1970049 | 0 UL_RTOA < 0.5 | Tc |
| … | … | … |
| UL_RTOA_3940095 | 985023 UL_RTOA < 985023.5 | Tc |
| UL_RTOA_3940096 | 985023.5 UL_RTOA < 985024 | Tc |
| UL_RTOA_3940097 | 985024 UL_RTOA | Tc |

Table 13.1.1-8: Measurement report mapping for k = -2.

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| UL_RTOA_0000000 | -985024 > UL_RTOA | Tc |
| UL_RTOA_0000001 | -985024 UL_RTOA < -985023.75 | Tc |
| UL_RTOA_0000002 | -985023.75 UL_RTOA < -985023.5 | Tc |
|  |  | … |
| UL_RTOA_3940096 | -0.25 UL_RTOA < 0 | Tc |
| UL_RTOA_3940097 | 0 UL_RTOA < 0.25 | Tc |
| … | … | … |
| UL_RTOA_7880191 | 985023.5 UL_RTOA < 985023.75 | Tc |
| UL_RTOA_7880192 | 985023.75 UL_RTOA < 985024 | Tc |
| UL_RTOA_7880193 | 985024 UL_RTOA | Tc |

Table 13.1.1-9: Measurement report mapping for k = -3.

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| UL_RTOA_00000000 | -985024 > UL_RTOA | Tc |
| UL_RTOA_00000001 | -985024 UL_RTOA < -985023.875 | Tc |
| UL_RTOA_00000002 | -985023.875 UL_RTOA < -985023.75 | Tc |
|  |  | … |
| UL_RTOA_07880192 | -0.125 UL_RTOA < 0 | Tc |
| UL_RTOA_07880193 | 0 UL_RTOA < 0.125 | Tc |
| … | … | … |
| UL_RTOA_15760383 | 985023.75 UL_RTOA < 985023.875 | Tc |
| UL_RTOA_15760384 | 985023.875 UL_RTOA < 985024 | Tc |
| UL_RTOA_15760385 | 985024 UL_RTOA | Tc |

Table 13.1.1-10: Measurement report mapping for k = -4.

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| UL_RTOA_0000000 | -985024 > UL_RTOA | Tc |
| UL_RTOA_0000001 | -985024 UL_RTOA < -985023.9375 | Tc |
| UL_RTOA_0000002 | -985023.9375 UL_RTOA < -985023.875 | Tc |
|  |  | … |
| UL_RTOA_15760384 | -0.0625 UL_RTOA < 0 | Tc |
| UL_RTOA_15760385 | 0 UL_RTOA < 0.0625 | Tc |
| … | … | … |
| UL_RTOA_31520767 | 985023.875 UL_RTOA < 985023.9375 | Tc |
| UL_RTOA_31520768 | 985023.9375 UL_RTOA < 985024 | Tc |
| UL_RTOA_31520769 | 985024 UL_RTOA | Tc |

Table 13.1.1-11: Measurement report mapping for k = -5.

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| UL_RTOA_00000000 | -985024 > UL_RTOA | Tc |
| UL_RTOA_00000001 | -985024 UL_RTOA < -985023.9688 | Tc |
| UL_RTOA_00000002 | -985023.9688 UL_RTOA < -985023.9375 | Tc |
|  |  | … |
| UL_RTOA_31520768 | -0.0312 UL_RTOA < 0 | Tc |
| UL_RTOA_31520769 | 0 UL_RTOA < 0.0312 | Tc |
| … | … | … |
| UL_RTOA_63041535 | 985023.9375 UL_RTOA < 985023.9688 | Tc |
| UL_RTOA_63041536 | 985023.9688 UL_RTOA < 985024 | Tc |
| UL_RTOA_63041537 | 985024 UL_RTOA | Tc |

Table 13.1.1-12: Measurement report mapping for k = -6.

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| UL_RTOA_000000000 | -985024 > UL_RTOA | Tc |
| UL_RTOA_000000001 | -985024 UL_RTOA < -985023.9844 | Tc |
| UL_RTOA_000000002 | -985023.9844 UL_RTOA < -985023.9688 | Tc |
|  |  | … |
| UL_RTOA_063041536 | -0.0156 UL_RTOA < 0 | Tc |
| UL_RTOA_063041537 | 0 UL_RTOA < 0.0156 | Tc |
| … | … | … |
| UL_RTOA_126083071 | 985023.9688 UL_RTOA < 985023.9844 | Tc |
| UL_RTOA_126083072 | 985023.9844 UL_RTOA < 985024 | Tc |
| UL_RTOA_126083073 | 985024 UL_RTOA | Tc |

### 13.1.1A	Additional Path Report Mapping for UL-RTOA

The reporting range of additional path reporting for UL Relative Time of Arrival (UL-RTOA), as defined in Clause 5.2.2 of TS 38.215 [4], is defined from -8175Tc to +8175Tc. The reporting resolution is uniform across the reporting range and is defined as T = Tc2k where k is selected by gNB from the set {-6, -5, -4, -3, -2, -1, 0, 1, 2, 3, 4, 5}.

Tc is defined in TS 38.211 [6].

LMF provides a recommended resolution parameter, timingReportingGranularityFactor [35]. gNB selects parameter k based on timingReportingGranularityFactor [35] and informs the LMF.

The mapping of measured quantity for each reporting resolution (k) is defined in Table 13.1.1A-1 to Table 13.1.1A-12.

Table 13.1.1A-1: Measurement report mapping for k = 0.

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_00000 | path < -8175 | Tc |
| path_00001 | -8175 path < -8174 | Tc |
| path_00002 | -8174 path < -8173 | Tc |
|  |  | … |
| path_08175 | -1 path < 0 | Tc |
| path_08176 | 0 path < 1 | Tc |
| … | … | … |
| path_16349 | 8173 path < 8174 | Tc |
| path_16350 | 8174 path < 8175 | Tc |
| path_16351 | 8175 path | Tc |

Table 13.1.1A-2: Measurement report mapping for k = 1.

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000 | path < -8175 | Tc |
| path_0001 | -8175 path < -8173 | Tc |
| path_0002 | -8173 path < -8171 | Tc |
|  |  | … |
| path_4088 | -1 path < 1 | Tc |
| … | … | … |
| path_8174 | 8171 path < 8173 | Tc |
| path_8175 | 8173 path < 8175 | Tc |
| path_8176 | 8175 path | Tc |

Table 13.1.1A-3: Measurement report mapping for k = 2.

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000 | path < -8174 | Tc |
| path_0001 | -8174 path < -8170 | Tc |
| path_0002 | -8170 path < -8166 | Tc |
|  |  | … |
| path_2044 | -2 path < 2 | Tc |
| … | … | … |
| path_4086 | 8166 path < 8170 | Tc |
| path_4087 | 8170 path < 8174 | Tc |
| path_4088 | 8174 path | Tc |

Table 13.1.1A-4: Measurement report mapping for k = 3.

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000 | path < -8172 | Tc |
| path_0001 | -8172 path < -8164 | Tc |
| path_0002 | -8164 path < -8156 | Tc |
|  |  | … |
| path_1022 | -4 path < 4 | Tc |
| … | … | … |
| path_2042 | 8156 path < 8164 | Tc |
| path_2043 | 8164 path < 8172 | Tc |
| path_2044 | 8172 path | Tc |

Table 13.1.1A-5: Measurement report mapping for k = 4.

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000 | path < -8168 | Tc |
| path_0001 | -8168 path < -8152 | Tc |
| path_0002 | -8152 path < -8136 | Tc |
|  |  | … |
| path_0511 | -8 path < 8 | Tc |
| … | … | … |
| path_1020 | 8136 path < 8152 | Tc |
| path_1021 | 8152 path < 8168 | Tc |
| path_1022 | 8168 path | Tc |

Table 13.1.1A-6: Measurement report mapping for k = 5.

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_000 | path < -8160 | Tc |
| path_001 | -8160 path < -8128 | Tc |
| path_002 | -8128 path < -8096 | Tc |
|  |  | … |
| path_256 | 0 path < 32 | Tc |
| … | … | … |
| path_509 | 8096 path < 8128 | Tc |
| path_510 | 8128 path < 8160 | Tc |
| path_511 | 8160 path | Tc |

Table 13.1.1A-7: Measurement report mapping for k = -1.

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_00000 | path < -8175 | Tc |
| path_00001 | -8175 path < -8174.5 | Tc |
| path_00002 | -8174.5 path < -8174 | Tc |
|  |  | … |
| path_16350 | -0.5 path < 0 | Tc |
| path_16351 | 0 path < 0.5 | Tc |
| … | … | … |
| path_32699 | 8174 path < 8174.5 | Tc |
| path_32700 | 8174.5 path < 8175 | Tc |
| path_32701 | 8175 path | Tc |

Table 13.1.1A-8: Measurement report mapping for k = -2.

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_00000 | path < -8175 | Tc |
| path_00001 | -8175 path < -8174.75 | Tc |
| path_00002 | -8174.75 path < -8174.5 | Tc |
|  |  | … |
| path_32700 | -0.25 path < 0 | Tc |
| path_32701 | 0 path < 0.25 | Tc |
| … | … | … |
| path_65399 | 8174.5 path < 8174.75 | Tc |
| path_65400 | 8174.75 path < 8175 | Tc |
| path_65401 | 8175 path | Tc |

Table 13.1.1A-9: Measurement report mapping for k = -3.

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_000000 | path < -8175 | Tc |
| path_000001 | -8175 path < -8174.875 | Tc |
| path_000002 | -8174.875 path < -8174.75 | Tc |
|  |  | … |
| path_065400 | -0.125 path < 0 | Tc |
| path_065401 | 0 path < 0.125 | Tc |
| … | … | … |
| path_130799 | 8174.75 path < 8174.875 | Tc |
| path_130800 | 8174.875 path < 8175 | Tc |
| path_130801 | 8175 path | Tc |

Table 13.1.1A-10: Measurement report mapping for k = -4.

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_000000 | path < -8175 | Tc |
| path_000001 | -8175 path < -8174.9375 | Tc |
| path_000002 | -8174.9375 path < -8174.875 | Tc |
|  |  | … |
| path_130800 | -0.0625 path < 0 | Tc |
| path_130801 | 0 path < 0.0625 | Tc |
| … | … | … |
| path_261599 | 8174.875 path < 8174.9375 | Tc |
| path_261600 | 8174.9375 path < 8175 | Tc |
| path_261601 | 8175 path | Tc |

Table 13.1.1A-11: Measurement report mapping for k = -5.

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_000000 | path < -8175 | Tc |
| path_000001 | -8175 path < -8174.9688 | Tc |
| path_000002 | -8174.9688 path < -8174.9375 | Tc |
|  |  | … |
| path_261600 | -0.0312 path < 0 | Tc |
| path_261601 | 0 path < 0.0312 | Tc |
| … | … | … |
| path_523199 | 8174.9375 path < 8174.9688 | Tc |
| path_523200 | 8174.9688 path < 8175 | Tc |
| path_523201 | 8175 path | Tc |

Table 13.1.1A-12: Measurement report mapping for k = -6.

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000000 | path < -8175 | Tc |
| path_0000001 | -8175 path < -8174.9844 | Tc |
| path_0000002 | -8174.9844 path < -8174.9688 | Tc |
|  |  | … |
| path_0523200 | -0.0156 path < 0 | Tc |
| path_0523201 | 0 path < 0.0156 | Tc |
| … | … | … |
| path_1046399 | 8174.9688 path < 8174.9844 | Tc |
| path_1046400 | 8174.9844 path < 8175 | Tc |
| path_1046401 | 8175 path | Tc |

## 13.2	gNB Rx-Tx time difference

### 13.2.1	Report mapping

The reporting range of gNB Rx-Tx time difference, as defined in Clause 5.2.3 of TS 38.215 [4], is defined from -985024Tc to +985024Tc. The reporting resolution is uniform across the reporting range and is defined as T = Tc2k where k is selected by gNB from the set {-6, -5, -4, -3, -2, -1, 0, 1, 2, 3, 4, 5}.

Tc is defined in TS 38.211 [6].

LMF provides a recommended resolution parameter, timingReportingGranularityFactor [35]. gNB selects parameter k based on timingReportingGranularityFactor [35] and informs the LMF.

The mapping of measured quantity for each reporting resolution (k) is defined in Table 13.2.1-1 to Table 13.2.1-12.

Table 13.2.1-1:Measurement report mapping for k = 0.

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_0000000 | -985024 > RX-TX | Tc |
| RX-TX_0000001 | -985024 RX-TX < -985023 | Tc |
| RX-TX_0000002 | -985023 RX-TX < -985022 | Tc |
|  |  | … |
| RX-TX_0985023 | -2 RX-TX < -1 | Tc |
| RX-TX_0985024 | -1 RX-TX 0 | Tc |
| RX-TX_0985025 | 0 < RX-TX 1 | Tc |
| RX-TX_0985026 | 1 < RX-TX 2 | Tc |
| RX-TX_0985027 | 2 < RX-TX 3 | Tc |
| … | … | … |
| RX-TX_1970048 | 985023 < RX-TX 985024 | Tc |
| RX-TX_1970049 | 985024 < RX-TX | Tc |

Table 13.2.1-2: Measurement report mapping for k = 1.

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_000000 | -985024 > RX-TX | Tc |
| RX-TX_000001 | -985024 RX-TX < -985022 | Tc |
| RX-TX_000002 | -985022 RX-TX < -985020 | Tc |
|  |  | … |
| RX-TX_492511 | -4 RX-TX < -2 | Tc |
| RX-TX_492512 | -2 RX-TX 0 | Tc |
| RX-TX_492513 | 0 < RX-TX 2 | Tc |
| RX-TX_492514 | 2 < RX-TX 4 | Tc |
| RX-TX_492515 | 4 < RX-TX 6 | Tc |
| … | … | … |
| RX-TX_985024 | 985022 < RX-TX 985024 | Tc |
| RX-TX_985025 | 985024 < RX-TX | Tc |

Table 13.2.1-3: Measurement report mapping for k = 2.

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_000000 | -985024 > RX-TX | Tc |
| RX-TX_000001 | -985024 RX-TX < -985020 | Tc |
| RX-TX_000002 | -985020 RX-TX < -985018 | Tc |
|  |  | … |
| RX-TX_246255 | -8 RX-TX < -4 | Tc |
| RX-TX_246256 | -4 RX-TX 0 | Tc |
| RX-TX_246257 | 0 < RX-TX 4 | Tc |
| RX-TX_246258 | 4 < RX-TX 8 | Tc |
| RX-TX_246259 | 8 < RX-TX 12 | Tc |
| … | … | … |
| RX-TX_492512 | 985020 < RX-TX 985024 | Tc |
| RX-TX_492513 | 985024 < RX-TX | Tc |

Table 13.2.1-4: Measurement report mapping for k = 3.

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_000000 | -985024 > RX-TX | Tc |
| RX-TX_000001 | -985024 RX-TX < -985016 | Tc |
| RX-TX_000002 | -985016 RX-TX < -985008 | Tc |
|  |  | … |
| RX-TX_123127 | -16 RX-TX < -8 | Tc |
| RX-TX_123128 | -8 RX-TX 0 | Tc |
| RX-TX_123129 | 0 < RX-TX 8 | Tc |
| RX-TX_123130 | 8 < RX-TX 16 | Tc |
| RX-TX_123131 | 16 < RX-TX 24 | Tc |
| … | … | … |
| RX-TX_246256 | 985016 < RX-TX 985024 | Tc |
| RX-TX_246257 | 985024 < RX-TX | Tc |

Table 13.2.1-5: Measurement report mapping for k = 4.

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_000000 | -985024 > RX-TX | Tc |
| RX-TX_000001 | -985024 RX-TX < -985008 | Tc |
| RX-TX_000002 | -985008 RX-TX < -984992 | Tc |
|  |  | … |
| RX-TX_061563 | -32 RX-TX < -16 | Tc |
| RX-TX_061564 | -16 RX-TX 0 | Tc |
| RX-TX_061565 | 0 < RX-TX 16 | Tc |
| RX-TX_061566 | 16 < RX-TX 32 | Tc |
| RX-TX_061567 | 32 < RX-TX 48 | Tc |
| … | … | … |
| RX-TX_123128 | 985008 < RX-TX 985024 | Tc |
| RX-TX_123129 | 985024 < RX-TX | Tc |

Table 13.2.1-6: Measurement report mapping for k = 5.

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_00000 | -985024 > RX-TX | Tc |
| RX-TX_00001 | -985024 RX-TX < -984992 | Tc |
| RX-TX_00002 | -984992 RX-TX < -984960 | Tc |
|  |  | … |
| RX-TX_30781 | -64 RX-TX < -32 | Tc |
| RX-TX_30782 | -32 RX-TX 0 | Tc |
| RX-TX_30783 | 0 < RX-TX 32 | Tc |
| RX-TX_30784 | 32 < RX-TX 64 | Tc |
| RX-TX_30785 | 64 < RX-TX 96 | Tc |
| … | … | … |
| RX-TX_61564 | 984992 < RX-TX 985024 | Tc |
| RX-TX_61565 | 985024 < RX-TX | Tc |

Table 13.2.1-7: Measurement report mapping for k = -1.

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_0000000 | -985024 > RX-TX | Tc |
| RX-TX_0000001 | -985024  RX-TX < -985023.5 | Tc |
| RX-TX_0000002 | -985023.5  RX-TX < -985023 | Tc |
|  |  | … |
| RX-TX_1970048 | -0.5  RX-TX < 0 | Tc |
| RX-TX_1970049 | 0  RX-TX < 0.5 | Tc |
| … | … | … |
| RX-TX_3940095 | 985023  RX-TX < 985023.5 | Tc |
| RX-TX_3940096 | 985023.5  RX-TX < 985024 | Tc |
| RX-TX_3940097 | 985024  RX-TX | Tc |

Table 13.2.1-8: Measurement report mapping for k = -2.

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_0000000 | -985024 > RX-TX | Tc |
| RX-TX_0000001 | -985024  RX-TX < -985023.75 | Tc |
| RX-TX_0000002 | -985023.75  RX-TX -985023.5 | Tc |
|  |  | … |
| RX-TX_3940096 | -0.25  RX-TX < 0 | Tc |
| RX-TX_3940097 | 0  RX-TX < 0.25 | Tc |
| … | … | … |
| RX-TX_7880191 | 985023.5  RX-TX < 985023.75 | Tc |
| RX-TX_7880192 | 985023.75  RX-TX < 985024 | Tc |
| RX-TX_7880193 | 985024  RX-TX | Tc |

Table 13.2.1-9: Measurement report mapping for k = -3.

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_0000000 | -985024 > RX-TX | Tc |
| RX-TX_0000001 | -985024  RX-TX < -985023.875 | Tc |
| RX-TX_0000002 | -985023.875  RX-TX -985023.75 | Tc |
|  |  | … |
| RX-TX_7880192 | -0.125  RX-TX < 0 | Tc |
| RX-TX_7880193 | 0  RX-TX < 0.125 | Tc |
| … | … | … |
| RX-TX_15760383 | 985023.75  RX-TX < 985023.875 | Tc |
| RX-TX_15760384 | 985023.875  RX-TX < 985024 | Tc |
| RX-TX_15760385 | 985024  RX-TX | Tc |

Table 13.2.1-10: Measurement report mapping for k = -4.

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_0000000 | -985024 > RX-TX | Tc |
| RX-TX_0000001 | -985024  RX-TX < -985023.9375 | Tc |
| RX-TX_0000002 | -985023.9375  RX-TX -985023.875 | Tc |
|  |  | … |
| RX-TX_15760384 | -0.0625  RX-TX < 0 | Tc |
| RX-TX_15760385 | 0  RX-TX < 0.0625 | Tc |
| … | … | … |
| RX-TX_31520767 | 985023.875  RX-TX < 985023.9375 | Tc |
| RX-TX_31520768 | 985023.9375  RX-TX < 985024 | Tc |
| RX-TX_31520769 | 985024  RX-TX | Tc |

Table 13.2.1-11: Measurement report mapping for k = -5.

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_00000000 | -985024 > RX-TX | Tc |
| RX-TX_00000001 | -985024  RX-TX < -985023.9688 | Tc |
| RX-TX_00000002 | -985023.9688  RX-TX -985023.9375 | Tc |
|  |  | … |
| RX-TX_31520768 | -0.0312  RX-TX < 0 | Tc |
| RX-TX_31520769 | 0  RX-TX < 0.0312 | Tc |
| … | … | … |
| RX-TX_63041535 | 985023.9375  RX-TX < 985023.9688 | Tc |
| RX-TX_63041536 | 985023.9688  RX-TX < 985024 | Tc |
| RX-TX_63041537 | 985024  RX-TX | Tc |

Table 13.2.1-12: Measurement report mapping for k = -6.

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_00000000 | -985024 > RX-TX | Tc |
| RX-TX_00000001 | -985024  RX-TX < -985023.9844 | Tc |
| RX-TX_00000002 | -985023.9844  RX-TX -985023.9688 | Tc |
|  |  | … |
| RX-TX_63041536 | -0.0156  RX-TX < 0 | Tc |
| RX-TX_63041537 | 0  RX-TX < 0.0156 | Tc |
| … | … | … |
| RX-TX_126083071 | 985023.9688  RX-TX < 985023.9844 | Tc |
| RX-TX_126083072 | 985023.9844  RX-TX < 985024 | Tc |
| RX-TX_126083073 | 985024  RX-TX | Tc |

### 13.2.1A		Additional Path Report Mapping for gNB Rx-Tx

The reporting range of additional path for gNB Rx-Tx time difference, as defined in Clause 5.2.3 of TS 38.215 [4], is defined from -8175Tc to 8175Tc. The reporting resolution is uniform across the reporting range and is defined as T = Tc2k where k is selected by gNB from the set {-6, -5, -4, -3, -2, -1, 0, 1, 2, 3, 4, 5}.

Tc is defined in TS 38.211 [6].

LMF provides a recommended resolution parameter, timingReportingGranularityFactor [35]. gNB selects parameter k based on timingReportingGranularityFactor [35] and informs the LMF.

The mapping of measured quantity for each reporting resolution (k) is defined in Table 13.2.1A-1 to Table 13.2.1A-12.

Table 13.2.1A-1: Measurement report mapping for k = 0.

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_00000 | path < -8175 | Tc |
| path_00001 | -8175 path < -8174 | Tc |
| path_00002 | -8174 path < -8173 | Tc |
|  |  | … |
| path_08175 | -1 path < 0 | Tc |
| path_08176 | 0 path < 1 | Tc |
| … | … | … |
| path_16349 | 8173 path < 8174 | Tc |
| path_16350 | 8174 path < 8175 | Tc |
| path_16351 | 8175 path | Tc |

Table 13.2.1A-2: Measurement report mapping for k = 1.

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000 | path < -8175 | Tc |
| path_0001 | -8175 path < -8173 | Tc |
| path_0002 | -8173 path < -8171 | Tc |
|  |  | … |
| path_4088 | -1 path < 1 | Tc |
| … | … | … |
| path_8174 | 8171 path < 8173 | Tc |
| path_8175 | 8173 path < 8175 | Tc |
| path_8176 | 8175 path | Tc |

Table 13.2.1A-3: Measurement report mapping for k = 2.

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000 | path < -8174 | Tc |
| path_0001 | -8174 path < -8170 | Tc |
| path_0002 | -8170 path < -8166 | Tc |
|  |  | … |
| path_2044 | -2 path < 2 | Tc |
| … | … | … |
| path_4086 | 8166 path < 8170 | Tc |
| path_4087 | 8170 path < 8174 | Tc |
| path_4088 | 8174 path | Tc |

Table 13.2.1A-4: Measurement report mapping for k = 3.

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000 | path < -8172 | Tc |
| path_0001 | -8172 path < -8164 | Tc |
| path_0002 | -8164 path < -8156 | Tc |
|  |  | … |
| path_1022 | -4 path < 4 | Tc |
| … | … | … |
| path_2042 | 8156 path < 8164 | Tc |
| path_2043 | 8164 path < 8172 | Tc |
| path_2044 | 8172 path | Tc |

Table 13.2.1A-5: Measurement report mapping for k = 4.

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000 | path < -8168 | Tc |
| path_0001 | -8168 path < -8152 | Tc |
| path_0002 | -8152 path < -8136 | Tc |
|  |  | … |
| path_511 | -8 path < 8 | Tc |
| … | … | … |
| path_1020 | 8136 path < 8152 | Tc |
| path_1021 | 8152 path < 8168 | Tc |
| path_1022 | 8168 path | Tc |

Table 13.2.1A-6: Measurement report mapping for k = 5.

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_000 | path < -8160 | Tc |
| path_001 | -8160 path < -8128 | Tc |
| path_002 | -8128 path < -8096 | Tc |
|  |  | … |
| path_256 | 0 path < 32 | Tc |
| … | … | … |
| path_509 | 8096 path < 8128 | Tc |
| path_510 | 8128 path < 8160 | Tc |
| path_511 | 8160 path | Tc |

Table 13.2.1A-7: Measurement report mapping for k = -1.

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_00000 | path < -8175 | Tc |
| path_00001 | -8175 path < -8174.5 | Tc |
| path_00002 | -8174.5 path < -8174 | Tc |
|  |  | … |
| path_16350 | -0.5 path < 0 | Tc |
| path_16351 | 0 path < 0.5 | Tc |
| … | … | … |
| path_32699 | 8174 path < 8174.5 | Tc |
| path_32700 | 8174.5 path < 8175 | Tc |
| path_32701 | 8175 path | Tc |

Table 13.2.1A-8: Measurement report mapping for k = -2.

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_00000 | path < -8175 | Tc |
| path_00001 | -8175 path < -8174.75 | Tc |
| path_00002 | -8174.75 path < -8174.5 | Tc |
|  |  | … |
| path_32700 | -0.25 path < 0 | Tc |
| path_32701 | 0 path < 0.25 | Tc |
| … | … | … |
| path_65399 | 8174.5 path < 8174.75 | Tc |
| path_65400 | 8174.75 path < 8175 | Tc |
| path_65401 | 8175 path | Tc |

Table 13.2.1A-9: Measurement report mapping for k = -3.

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_000000 | path < -8175 | Tc |
| path_000001 | -8175 path < -8174.875 | Tc |
| path_000002 | -8174.875 path < -8174.75 | Tc |
|  |  | … |
| path_065400 | -0.125 path < 0 | Tc |
| path_065401 | 0 path < 0.125 | Tc |
| … | … | … |
| path_130799 | 8174.75 path < 8174.875 | Tc |
| path_130800 | 8174.875 path < 8175 | Tc |
| path_130801 | 8175 path | Tc |

Table 13.2.1A-10: Measurement report mapping for k = -4.

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_000000 | path < -8175 | Tc |
| path_000001 | -8175 path < -8174.9375 | Tc |
| path_000002 | -8174.9375 path < -8174.875 | Tc |
|  |  | … |
| path_130800 | -0.0625 path < 0 | Tc |
| path_130801 | 0 path < 0.0625 | Tc |
| … | … | … |
| path_261599 | 8174.875 path < 8174.9375 | Tc |
| path_261600 | 8174.9375 path < 8175 | Tc |
| path_261601 | 8175 path | Tc |

Table 13.2.1A-11: Measurement report mapping for k = -5.

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_000000 | path < -8175 | Tc |
| path_000001 | -8175 path < -8174.9688 | Tc |
| path_000002 | -8174.9688 path < -8174.9375 | Tc |
|  |  | … |
| path_261600 | -0.0312 path < 0 | Tc |
| path_261601 | 0 path < 0.0312 | Tc |
| … | … | … |
| path_523199 | 8174.9375 path < 8174.9688 | Tc |
| path_523200 | 8174.9688 path < 8175 | Tc |
| path_523201 | 8175 path | Tc |

Table 13.2.1A-12: Measurement report mapping for k = -6.

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_000000 | path < -8175 | Tc |
| path_000001 | -8175 path < -8174.9844 | Tc |
| path_000002 | -8174.9844 path < -8174.9688 | Tc |
|  |  | … |
| path_523200 | -0.0156 path < 0 | Tc |
| path_523201 | 0 path < 0.0156 | Tc |
| … | … | … |
| path_1046399 | 8174.9688 path < 8174.9844 | Tc |
| path_1046400 | 8174.9844 path < 8175 | Tc |
| path_1046401 | 8175 path | Tc |

<END OF CHANGE #3>