3GPP TSG-RAN WG4 Meeting #110	R4-2402697

Athens, Greece, February 19 – March 01, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	Maintenance on Discontinuous Coverage for NB-IoT/eMTC in NTN

Agenda item:	6.2.2.3

Document for:	Discussion

Introduction

In Rel-17, support for NB-IoT and eMTC over NTN was studied in [1] and defined in RAN1, RAN2, and RAN3 under the Work Item described in [2]. RAN4 requirements will be defined in a release-independent manner in Rel-18. The work was initiated in the RAN4 #104 meeting and must be continued following the WF from previous meetings and the WID [3].

In meeting RAN4 #108, the maintenance discussion on the WI has progressed and some issues were identified and left for further studies on the Way-Forward approved in that meeting [4].

In this document, Nokia’s views regarding the following topics are presented regarding discontinuous coverage enhancements.

Discussion

Requirements on Discontinuous Coverage

The current specification is lacking some further definitions on the requirements that are applicable to the UE under discontinuous coverage. In clause 4.7A.2, the following is specified:

TS 36.133 clause 4.7A.2 If SystemInformationBlockType32 [2] has been received and if the UE has determined that it is out of coverage, the UE is not required to perform cell measurements from the last slot of SI transmission which indicates that UE will be in out of coverage after t-Service [2] when the serving cell stop serving the area. Editor’s note: FFS on how to capture following agreement: When the UE is provided with t-serviceStart-r17 and has discontinuous coverage capabilities, then after t-service-r17 is reached and the UE is out of coverage, the UE may delay or resume cell measurements/search till when the UE is in coverage. Definition of in coverage is FFS.

There are two problems to be addressed here. First of all, the sentence marked in green above, is rather unclear of when the requirement applicability starts.  Observe, the example provided in Figure 1, the SIB information is repeated across multiple SI-Windows (that might be very long in time each). This includes the information about discontinuous coverage (provided in SIB32) and t-service (provided in SIB31).  Imagine that the UE reads the information in the first SI-Window below. From the specification text, it might be inferred that the UE is not required to perform measurements, which is the one “indicating” the UE will be out of coverage after t-service.

Figure 1. Example where multiple consecutive SI-Windows provide information about discontinuous coverage (SIB31) and t-service (SIB32).  Specification text is unclear about at which point the UE is no longer required to perform cell measurements.

In current phrasing, it might be understood that the UE can skip measurements several seconds before t-service.

Clearly, this was not the intent of companies when agreeing that the UE is not required to perform measurements in discontinuous coverage.

Therefore, we propose:

In TS 36.133, rephrase: “If SystemInformationBlockType32 [2] has been received and if the UE has determined that it is will be out of coverage after t-Service, the UE is not required to perform cell measurements from the last slot of the last SI transmission which indicates that UE will be in out of coverage after before t-Service [2]”

Now, for the second part, the  exact wording and what entails the specification intent under the FFS have been matter of thorough discussion among companies for many meetings. One important observation is that the same sentence is not under FFS in the clauses for NB-IoT.

The sentence “UE may delay or resume cell measurements/search till when the UE is in coverage” is unclear.

It is unclear what “delay or resume” cell “measurements/search”. The concept of delaying or resuming measurements indicate that the UE would be camped in the same cell which is not possible in discontinuous coverage.

This sentence needs to be modified to be more clear. We propose that it is rephrased as:

In TS 36.133, rephrase: “When the UE is provided with t-serviceStart-r17 and has discontinuous coverage capabilities, then after t-service-r17 is reached and the UE is out of coverage, the UE may not perform idle mode measurements until t-serviceStart-r17 is reached.”

The other part is related to the ongoing discussion about the definition of “in-coverage” for the continuation of the measurements. No consensus has been reached yet. But, even in the absence of a current agreement, the FFS in the specification shall be addressed, as this cannot remain open as an editor note in the final version of the spec.

There are two main opposing views:

- The UE is expected to be in coverage when, based on received parameters from SIB32, the UE is within the estimated coverage area. OR

- The UE is in coverage after the UE has already started to performing measurements and found a suitable cell.

We think that the complicating factor in this discussion is the “in-coverage” definition. The “in-coverage” implies that the UE is within radio coverage range, which can only be determined after the UE has already starting the measurements. Consequently, this leads to a conundrum: saying that the UE needs to restart measurements after in-coverage, means that the UE needs to measure first, until it determines it is in coverage, for then to be required to be measured.

To avoid this, from RAN4 point of view, we can simplify the requirements:

In order to remove the FFS from the specification text, we propose to maintain the generic description but in line with RAN4 intention:

In TS 36.133, do not not define “in-coverage” status.

Another relevant aspect to be considered is that the FFS on the spec only refers for the cases where the UE is configured with t-serviceStart. But in certain cases and more importantly for earth-moving cell scenarios, SIB32 will be configured with footprint information, not only with t-serviceStart.

When the UE is provided with footprintInfo and has discontinuous coverage capabilities, then the UE shall perform idle mode measurements when the UE is within the area provided by footprintInfo.

Conclusion

In this paper we discussed maintenance aspects related to the discontinuous coverage specification.

The following Observations and Proposals were made:

Observation 1: In current phrasing, it might be understood that the UE can skip measurements several seconds before t-service.

Proposal 1: In TS 36.133, rephrase: “If SystemInformationBlockType32 [2] has been received and if the UE has determined that it is will be out of coverage after t-Service, the UE is not required to perform cell measurements from the last slot of the last SI transmission which indicates that UE will be in out of coverage after before t-Service [2]”

Observation 2: It is unclear what “delay or resume” cell “measurements/search”. The concept of delaying or resuming measurements indicate that the UE would be camped in the same cell which is not possible in discontinuous coverage.

Proposal 2: In TS 36.133, rephrase: “When the UE is provided with t-serviceStart-r17 and has discontinuous coverage capabilities, then after t-service-r17 is reached and the UE is out of coverage, the UE may not perform idle mode measurements until t-serviceStart-r17 is reached.”

Proposal 3: In TS 36.133, do not not define “in-coverage” status.

Proposal 4: When the UE is provided with footprintInfo and has discontinuous coverage capabilities, then the UE shall perform idle mode measurements when the UE is within the area provided by footprintInfo.

- References

- TR 36.763, “Study on Narrow-Band Internet of Things (NB-IoT) / enhanced Machine Type Communication (eMTC) support for Non-Terrestrial Networks (NTN)”

- RP-211601 – NB-IoT/eMTC support for Non-Terrestrial Networks

- RP-220938 – New WID on NB-IoT/eMTC core & performance requirements for NTN

- R4-2310098, WF on NB-IoT/eMTC NTN core & perf. RRM requirements maintenance, Mediatek, RAN WG4 #108, Toulouse