3GPP TSG-RAN4 Meeting #110 	R4-2402712

# Athens, Greece, 26 February - 01 March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.106 | CR | draft | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on Propagation Condition for NCR-MT for 38.106 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_netcon_repeater-Perf |  |  |  |  |  | Date: |  |  | 2023-02-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Need of Propagation Condition for NCR-MT in 38.106 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding Appendix C on Propagation Condition in 38.106 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No propagation condition for NCR-MT |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2 (Reference), Appendix C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.115-1, 38.115-2 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[unchanged parts removed]

[27]	3GPP TS 38.306: “NR; User Equipment (UE) radio access capabilities”.

[28]	3GPP TR 38.901: "Study on channel model for frequencies from 0.5 to 100 GHz"

<End of Change 1>

<Start of Change 2>

Annex C (normative): 
Propagation conditions

## C.1		Static propagation condition

The propagation for the static performance measurement is an Additive White Gaussian Noise (AWGN) environment. No fading or multi-paths exist for this propagation model.

### C.1.1	NCR-MT receiver with 2RX

For 1 port transmission the channel matrix is defined in the frequency domain by:

.

For 2 port transmission the channel matrix is defined in the frequency domain by:

.

For 4 port transmission the channel matrix is defined in the frequency domain by:

For 8 port transmission the channel matrix is defined in the frequency domain by:

## C.2	Multi-path fading propagation conditions

### C.2.1	General

The multipath propagation conditions consist of several parts:

-	A delay profile in the form of a "tapped delay-line", characterized by a number of taps at fixed positions on a sampling grid. The profile can be further characterized by the r.m.s. delay spread and the maximum delay spanned by the taps.

-	A combination of channel model parameters that include the Delay profile and the Doppler spectrum that is characterized by a classical spectrum shape and a maximum Doppler frequency.

-	Different models are used for FR1 (410 MHz - 7.125 GHz) and FR2 (24.25 GHz – 52.6 GHz).

### C.2.2	Delay profiles

#### C.2.2.1	General

The delay profiles are simplified from the TR 38.901 [28] TDL models. The simplification steps are shown below for information. These steps are only used when new delay profiles are created. Otherwise, the delay profiles specified in C.2.2.1 can be used as such.

-	Step 1: Use the original TDL model from TR 38.901 [28].

-	Step 2: Re-order the taps in ascending delays.

-	Step 3: Perform delay scaling according to the procedure described in clause 7.7.3 in TR 38.901 [28].

-	Step 4: Apply the quantization to the delay resolution 5 ns. This is done simply by rounding the tap delays to the nearest multiple of the delay resolution.

-	Step 5: If multiple taps are rounded to the same delay bin, merge them by calculating their linear power sum.

-	Step 6: If there are more than 12 taps in the quantized model, merge the taps as follows:

-	Find the weakest tap from all taps (both merged and unmerged taps are considered):

-	If there are two or more taps having the same value and are the weakest, select the tap with the smallest delay as the weakest tap.

-	When the weakest tap is the first delay tap, merge taps as follows:

-	Update the power of the first delay tap as the linear power sum of the weakest tap and the second delay tap.

-	Remove the second delay tap.

-	When the weakest tap is the last delay tap, merge taps as follows:

-	Update the power of the last delay tap as the linear power sum of the second-to-last tap and the last tap.

-	Remove the second-to-last tap.

-	Otherwise:

-	For each side of the weakest tap, identify the neighbour tap that has the smaller delay difference to the weakest tap.

-	When the delay difference between the weakest tap and the identified neighbour tap on one side equals the delay difference between the weakest tap and the identified neighbour tap on the other side.

-	Select the neighbour tap that is weaker in power for merging.

-	Otherwise, select the neighbour tap that has smaller delay difference for merging.

-	To merge, the power of the merged tap is the linear sum of the power of the weakest tap and the selected tap.

-	When the selected tap is the first tap, the location of the merged tap is the location of the first tap. The weakest tap is removed.

-	When the selected tap is the last tap, the location of the merged tap is the location of the last tap. The weakest tap is removed.

-	Otherwise, the location of the merged tap is based on the average delay of the weakest tap and selected tap. If the average delay is on the sampling grid, the location of the merged tap is the average delay. Otherwise, the location of the merged tap is rounded towards the direction of the selected tap (e.g. 10 ns & 20 ns  15 ns, 10 ns & 25 ns  20 ns, if 25 ns had higher or equal power; 15 ns, if 10 ns had higher power). The weakest tap and the selected tap are removed.

-	Repeat step 6 until the final number of taps is 12.

-	Step 7: Round the amplitudes of taps to one decimal (e.g. -8.78 dB  -8.8 dB)

-	Step 8: If the delay spread has slightly changed due to the tap merge, adjust the final delay spread by increasing or decreasing the power of the last tap so that the delay spread is corrected.

-	Step 9: Re-normalize the highest tap to 0 dB.

Note 1:	Some values of the delay profile created by the simplification steps may differ from the values in tables C.2.2.2-2, C.2.2.2-3, and C.2.1.1-4 for the corresponding model.

Note 2:	For Step 5 and Step 6, the power values are expressed in the linear domain using 6 digits of precision. The operations are in the linear domain.

#### C.2.2.2	Delay profiles for FR1

The delay profiles for FR1 are selected to be representative of low, medium and high delay spread environment. The resulting model parameters are specified in C.2.2.2-1 and the tapped delay line models are specified in tables C.2.2.2-2 ~ table C.2.2.2-4.

Table C.2.2.2-1: Delay profiles for NR channel models

| Model | Number of channel taps | Delay spread (r.m.s.) | Maximum excess tap delay (span) | Delay resolution |
| --- | --- | --- | --- | --- |
| TDLA30 | 12 | 30 ns | 290 ns | 5 ns |
| TDLB100 | 12 | 100 ns | 480 ns | 5 ns |
| TDLC300 | 12 | 300 ns | 2595 ns | 5 ns |

Table C.2.2.2-2: TDLA30 (DS = 30 ns)

| Tap # | Delay (ns) | Power (dB) | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -15.5 |  |
| 2 | 10 | 0 |  |
| 3 | 15 | -5.1 |  |
| 4 | 20 | -5.1 |  |
| 5 | 25 | -9.6 |  |
| 6 | 50 | -8.2 | Rayleigh |
| 7 | 65 | -13.1 |  |
| 8 | 75 | -11.5 |  |
| 9 | 105 | -11.0 |  |
| 10 | 135 | -16.2 |  |
| 11 | 150 | -16.6 |  |
| 12 | 290 | -26.2 |  |

Table C.2.2.2-3: TDLB100 (DS = 100ns)

| Tap # | Delay (ns) | Power (dB) | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | 0 |  |
| 2 | 10 | -2.2 |  |
| 3 | 20 | -0.6 |  |
| 4 | 30 | -0.6 |  |
| 5 | 35 | -0.3 |  |
| 6 | 45 | -1.2 | Rayleigh |
| 7 | 55 | -5.9 |  |
| 8 | 120 | -2.2 |  |
| 9 | 170 | -0.8 |  |
| 10 | 245 | -6.3 |  |
| 11 | 330 | -7.5 |  |
| 12 | 480 | -7.1 |  |

Table C.2.2.2-4: TDLC300 (DS = 300 ns)

| Tap # | Delay (ns) | Power (dB) | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -6.9 |  |
| 2 | 65 | 0 |  |
| 3 | 70 | -7.7 |  |
| 4 | 190 | -2.5 |  |
| 5 | 195 | -2.4 |  |
| 6 | 200 | -9.9 | Rayleigh |
| 7 | 240 | -8.0 |  |
| 8 | 325 | -6.6 |  |
| 9 | 520 | -7.1 |  |
| 10 | 1045 | -13.0 |  |
| 11 | 1510 | -14.2 |  |
| 12 | 2595 | -16.0 |  |

### C.2.3	Combinations of channel model parameters

The propagation conditions used for the performance measurements in multi-path fading environment are indicated as a combination of a channel model name and a maximum Doppler frequency, i.e., TDLA<DS>-<Doppler>, TDLB<DS>-<Doppler> or TDLC<DS>-<Doppler> where '<DS>' indicates the desired delay spread and '<Doppler>' indicates the maximum Doppler frequency (Hz).

Table C.2.3-1 show the propagation conditions that are used for the performance measurements in multi-path fading environment for low, medium and high Doppler frequencies for FR1.

Table C.2.3-1: Channel model parameters for FR1

| Combination name | Model | Maximum Doppler frequency |
| --- | --- | --- |
| TDLA30-5 | TDLA30 | 5 Hz |
| TDLA30-10 | TDLA30 | 10 Hz |
| TDLB100-400 | TDLB100 | 400 Hz |
| TDLC300-100 | TDLC300 | 100 Hz |

### C.2.4	MIMO channel correlation matrices

#### C.2.4.1	General

The MIMO channel correlation matrices defined in annex C.2.4 apply for the antenna configuration using uniform linear arrays at both gNB and NCR-MT and for the antenna configuration using cross polarized antennas.

#### C.2.4.2	MIMO correlation matrices using Uniform Linear Array

##### C.2.4.2.1	General

The MIMO channel correlation matrices defined in annex C.2.4.2 apply for the antenna configuration using uniform linear array (ULA) at both gNB and NCR-MT.

##### C.2.4.2.2	Definition of MIMO correlation matrices

Table C.2.4.2.2-1 defines the correlation matrix for the gNB.

Table C.2.4.2.2-1: gNB correlation matrix

|  | gNB correlation |
| --- | --- |
| One antenna |  |
| Two antennas |  |
| Four antennas |  |
| Eight antennas |  |
| Note: The matrix applies to the gNB for NCR-MT requirements. |  |

Table C.2.4.2.2-2 defines the correlation matrix for the NCR-MT:

Table I.2.4.2.2-2: NCR-MT correlation matrix

|  | One antenna | Two antennas | Four antennas |
| --- | --- | --- | --- |
| NCR-MT correlation |  |  |  |
| Note: The correlation matrix RUE applies to NCR-MT for NCR-MT requirements. |  |  |  |

Table C.2.4.2.2-3 defines the channel spatial correlation matrix. The parameters, α and β in table C.2.4.2.2-3 defines the spatial correlation between the antennas at the gNB and NCR-MT respectively.

Table C.2.4.2.2-3: correlation matrices

| 1x2 case |  |
| --- | --- |
| 1x4 case |  |
| 2x2 case |  |
| 2x4 case |  |
| 4x4 case |  |
| NOTE 1: RgNB refers to the correlation matrix of gNB for NCR-MT requirements. NOTE 2: RUE refers to the correlation matrix of NCR-MT for NCR-MT requirements |  |

For cases with more antennas at either gNB or NCR-MT or both, the channel spatial correlation matrix can still be expressed as the Kronecker product of  and according to.

##### C.2.4.2.3	MIMO correlation matrices at high, medium and low level

The  and  for different correlation types are given in table C.2.4.2.3-1.

Table C.2.4.2.3-1: Correlation for high, medium and low level

| Low correlation |  | Medium correlation |  | High correlation |  |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0.9 | 0.3 | 0.9 | 0.9 |

The correlation matrices for high, medium and low correlation are defined in table C.2.4.2.3-2, C.2.4.2.3-3 and C.2.4.2.3-4 as below.

The values in table C.2.4.2.3-2 have been adjusted for the 2x4 and 4x4 high correlation cases to ensure the correlation matrix is positive semi-definite after round-off to 4 digit precision. This is done using the equation:

Where the value "a" is a scaling factor such that the smallest value is used to obtain a positive semi-definite result. For the 2x4 high correlation case, a = 0.00010. For the 4x4 high correlation case, a = 0.00012.

The same method is used to adjust the 4x4 medium correlation matrix in table C.2.4.2.3-3 to ensure the correlation matrix is positive semi-definite after round-off to 4 digit precision with a = 0.00012.

Table C.2.4.2.3-2: MIMO correlation matrices for high correlation

| 1x2 case |  |
| --- | --- |
| 2x2 case |  |
| 2x4 case |  |
| 4x4 case |  |

Table C.2.4.2.3-3: MIMO correlation matrices for medium correlation

| 1x2 case | [N/A] |
| --- | --- |
| 2x2 case |  |
| 2x4 case |  |
| 4x4 case |  |

Table C.2.4.2.3-4: MIMO correlation matrices for low correlation

| 1x2 case |  |
| --- | --- |
| 1x4 case |  |
| 1x8 case |  |
| 2x2 case |  |
| 2x4 case |  |
| 2x4 case |  |
| 4x4 case |  |

In table C.2.4.2.3-4,  is a  identity matrix.

NOTE:	For completeness, the correlation matrices were defined for high, medium and low correlation but performance requirements exist only for low correlation.

#### C.2.4.3	Multi-antenna channel models using cross polarized antennas

##### C.2.4.3.1	General

The MIMO channel correlation matrices defined in annex C.2.4.3 apply to two cases as presented below:

-	One TX antenna and multiple RX antennas case, with cross polarized antennas used at gNB

-	Multiple TX antennas and multiple RX antennas case, with cross polarized antennas used at NCR-MT

The cross-polarized antenna elements with +/-45 degrees polarization slant angles are deployed at gNB. For one TX antenna case, antenna element with +90 degree polarization slant angle is deployed at NCR-MT. For multiple TX antennas case, cross-polarized antenna elements with +90/0 degrees polarization slant angles are deployed at NCR-MT.

For the cross-polarized antennas, the N antennas are labelled such that antennas for one polarization are listed from 1 to N/2 and antennas for the other polarization are listed from N/2+1 to N, where N is the number of TX or RX antennas.

##### C.2.4.3.2	Definition of MIMO correlation matrices using cross polarized antennas

For the channel spatial correlation matrix, the following is used:

Where

-	 is the spatial correlation matrix at the NCR-MT (NCR-MT requirements) with same polarization,

-	 is the spatial correlation matrix at the gNB (NCR-MT requirements) with same polarization,

-	 is a polarization correlation matrix,

-	 is a permutation matrix, and

-	denotes transpose.

Table C.2.4.3.2-1 defines the polarization correlation matrix.

Table C.2.4.3.2-1: Polarization correlation matrix

|  | One TX antenna | Multiple TX antennas |
| --- | --- | --- |
| Polarization correlation matrix |  |  |

The matrixis defined as

where  and  is the number of TX and RX antennas respectively, and  is the ceiling operator.

The matrix  is used to map the spatial correlation coefficients in accordance with the antenna element labelling system described in C.2.4.3.

##### C.2.4.3.3	Spatial correlation matrices at NCR-MT and gNB sides

C.2.4.3.3.1	Spatial correlation matrices at NCR-MT side

In this subclause, RUE refers to an NCR-MT for NCR-MT requirements.

For 1-antenna transmitter, .

For 2-antenna transmitter using one pair of cross-polarized antenna elements, .

For 4-antenna transmitter using two pairs of cross-polarized antenna elements, .

I.2.4.3.3.2	Spatial correlation matrices at gNB side

In this subclause, RgNB refers to a gNB for NCR-MT requirements.

For 2-antenna receiver using one pair of cross-polarized antenna elements, .

For 4-antenna receiver using two pairs of cross-polarized antenna elements,.

For 8-antenna receiver using four pairs of cross-polarized antenna elements,.

##### C.2.4.3.4	MIMO correlation matrices using cross polarized antennas

The values for parameters α, β and γ for low spatial correlation are given in table C.2.4.3.4-1.

Table C.2.4.3.4-1: Values for parameters α, b and γ

| Low spatial correlation |  |  |
| --- | --- | --- |
| α | b | γ |
| 0 | 0 | 0 |
| Note 1: Value of α applies when more than one pair of cross-polarized antenna elements at gNB side. Note 2: Value of β applies when more than one pair of cross-polarized antenna elements at NCR-MT side. |  |  |

The correlation matrices for low spatial correlation are defined in table C.2.4.3.4-2 as below.

Table C.2.4.3.4-2: MIMO correlation matrices for low spatial correlation

| 1x8 case |  |
| --- | --- |
| 2x8 case |  |

In table C.2.4.3.4-2,  is a  identity matrix.

##### C.2.4.3.5	Beam steering approach

For the 2D cross-polarized antenna array at gNB, given the channel spatial correlation matrix in C.2.4.3.2, C.2.4.3.3 and C.2.4.3.4, the corresponding random channel matrix H can be calculated. The signal model for the k-th slot is denoted as

And the steering matrix is further expressed as following:

where

-	H is the Nr×Nt channel matrix per subcarrier.

-	 is the steering matrix,

-	 is the steering matrix in first dimension with same polarization,

-	 is the steering matrix in second dimension with same polarization,

-	 is the number of antenna elements in first dimension with same polarization,

-	 is the number of antenna elements in second dimension with same polarization,

-	For antenna array with only one direction, number of antenna element in second direction equals 1.

For 1 antenna element with the same polarization in one direction,

.

For 2 antenna elements with the same polarization in one direction,

.

For 3 antenna elements with the same polarization in one direction,

.

For 4 antenna elements with the same polarization in one direction,

.

where the index  stands for first dimension and second dimension respectively.

-	 controls the phase variation in first dimension and second dimension respectively, and the phase for k-th subframe is denoted by, where is the random start value with the uniform distribution, i.e., ,  is the step of phase variation, which is defined in Table C.2.4.3.5-1, and k is the linear increment of 2-μ for every slot throughout the simulation, the index  stands for first dimension and second dimension respectively.

-	 is the precoding matrix for Nt transmission antennas,

-	y is the received signal, x is the transmitted signal, and n is AWGN.

-	 corresponds to subcarrier spacing configuration,

For the 1D cross-polarized antenna array at gNB, the corresponding random channel matrix H can be calculated by letting N2=1, i.e.,

Table C.2.4.3.5-1: The step of phase variation

| Variation Step | Value (rad/ms) |
| --- | --- |
|  | 1.2566×10-3 |

## C.3	Physical signals, channels mapping and precoding

### C.3.1	General

Unless otherwise stated, the transmission on antenna port(s) p=p0, p0+1,..., p0+Np-1 is defined by using a precoder matrix  of size NANT×Np, where NANT is the number of physical transmit antenna elements configured per test , Np is the number of ports for a reference signal or physical channel configured per test, and  p0 is the first port for that reference signal or physical channel as defined in clauses 7.3 and 7.4 in TS 38.211 [8]. This precoder takes as an input a block of signals for antenna port(s) p=p0,p0+1,...,p0+Np-1, ypi=yp0i  yp0+1i …   yp0+Np-1iT, , with  being the number of modulation symbols per antenna port including the reference signal symbols, and generates a block of signals ybfqi=ybf0i  ybf1i …   ybfNANT-1iT the elements of which are to be mapped onto the frequency-time index pair as per the test configuration but transmitted on different physical antenna elements:

ybfqi=Wiyp(i)

For [void - <Reserved for related Clauses on CSI Reporting (for both conducted and radiated)>], the transmission of PDCCH and PDCCH DMRS on antenna port p=p0 is defined by using a precoder matrix  of size 2x1. This precoder takes as an input a block of signals for antenna port(s) p=p0, ypi=yp0i and generates a block of signals ybfqi=ybf0i  ybfNANT2iT the elements of which are to be mapped onto the frequency-time index pair as per the test configuration but transmitted on different physical antenna elements:

ybfqi=Wiyp(i)

The precoder matrix is specific to the test case configuration.  is defined in Clause 5.2.2.2 of TS 38.214 [11].

The transimison on PT-RS antenna port is associated (using same precoder) with the lowest indexed DM-RS antenna port among the DM-RS antenna ports assigned for the PDSCH.

The physical antenna elements are identified by indices, where  is the number of physical antenna elements configured per test.

Modulation symbols yp(i) with p∈4000 (i.e. PSS, SSS, PBCH and DM-RS for PBCH) are directly mapped to first physical antenna element.

Modulation symbols  for CSI-RS resources which configured for tracking with one port are directly mapped to first physical antenna element.

Modulation symbols  for CSI-RS resources which configured for beam refinement with one port are directly mapped to first physical antenna element.

Modulation symbols  for NZP CSI-RS which configured for CSI acquisition with    are mapped to the physical antenna index  where is the number of NZP CSI-RS ports configured per test.

<End of Change 2>