3GPP TSG-RAN4 Meeting #110	R4-2402715

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.115-2 | CR | 0013 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Big CR on Performance Requirements for NCR-MT in 38.115-2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_netcon_repeater-Perf |  |  |  |  |  | Date: |  |  | 2024-03-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Adding performance requirements of NCR-MT into 38.115-2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following are added to 38.115-2: the needed references in Section 2, the PDSCH+PDCCH+CQI Requirements in Section 7, the required Appendices for Testing (E), FRC (H) and Propagation Condition (I). Those additions are coming from the following endorsed draft CRs: - R4-2402999 Draft CR for TS38115-2 on Radiated performance requirements(PDSCH, PDCCH) for NCR-MT. This draft CR contributes to PDSCH+PDCCH Requirements in Section 7. - R4-2403002 Draft CR to 38.115-2: Addition of test procedures for NCR demodulation. This draft CR contributes to CQI Requirements in Section 7 and Test Setup in Appendix E. - R4-2403005 Draft CR on FRC for NCR-MT for 38.115-2. This draft CR contributes to FRC in Appendix H. - R4-2402996 Draft CR on propagation conditions for NCR-MT (TS38.115-2, Rel-18). This draft CR contributes to Propagation Condition in Appendix I. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There are no requirements for radiated conformance testing for NCR-MT |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 7, E, H, I |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | AI 8.20.6 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[unchanged parts removed]

[14]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[15]	3GPP TS 38.214: "NR; Physical layer procedures for data".

[16]	3GPP TS 38.101-4: “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

[17]	3GPP TR 38.901: "Study on channel model for frequencies from 0.5 to 100 GHz"

[18]	3GPP TS 38.211: "NR; Physical channels and modulation"

<End of Change 1>

<Start of Change 2 – [R4-2402999]>

# 7	Radiated performance requirements

## 7.1	NCR-MT performance requirements

### 7.1.1	General

#### 7.1.1.1	Scope and definitions

Radiated performance requirements specify the ability of the NCR-MT type 2-O to correctly demodulate signals in various conditions and configurations. Radiated performance requirements are specified at the RIB.

Radiated performance requirements for the NCR-MT are specified for the fixed reference channels defined in annex H and the propagation conditions in annex I. The requirements only apply to those FRCs that are supported by the NCR-MT.

The radiated performance requirements for NCR-MT type 2-O are limited to two OTA demodulations branches as described in clause 7.1.1.2. Conformance requirements can only be tested for 1 or 2 demodulation branches depending on the number of polarizations supported by the NCR-MT, with the required SNR applied separately per polarization.

NOTE: 	NCR-MT can support more than 2 demodulation branches, however OTA conformance testing can only be performed for 1 or 2 demodulation branches.

The SNR used in this clause is specified based on a single carrier and defined as:

SNR = S / N

Where:

S	is the total signal energy in the slot on a single TAB connector (for NCR-MT type 1-C).

N	is the noise energy in a bandwidth corresponding to the transmission bandwidth over the duration of a slot on a single TAB connector (for NCR-MT type 1-C).

#### 7.1.1.2	OTA demodulation branches

Radiated performance requirements are only specified for up to 2 demodulation branches.

If the NCR-MT type 2-O uses polarization diversity and has the ability to maintain isolation between the signals for each of the demodulation branches, then radiated performance requirements can be tested for up to two demodulation branches (i.e. 1RX or 2RX test setups). When tested for two demodulation branches, each demodulation branch maps to one polarization.

If the NCR-MT type 2-O does not use polarization diversity then radiated performance requirements can only be tested for a single demodulation branch (i.e. 1RX test setup).

### 7.1.2	Demodulation performance requirements

#### 7.1.2.1	General

##### 7.1.2.1.1	Applicability rule for NCR-MT

7.1.2.1.1.1	General

Unless otherwise stated, for an NCR-MT declared to support more than 2 demodulation branches (for NCR-MT type 2-O), the performance requirement tests for 2 demodulation branches shall apply, and demodulation branches is up to NCR-MT implementation.

The tests requiring more than [20] dB SNR level are set to N/A in the test requirements.

7.1.2.1.1.2	Applicability of requirements for different subcarrier spacings

Unless otherwise stated, the tests shall apply only for each subcarrier spacing declared to be supported (see D.7 in table 4.6-1).

7.1.2.1.1.3	Applicability of requirements for TDD with different UL-DL patterns

Unless otherwise stated, for each subcarrier spacing declared to be supported, if NCR-MT supports multiple TDD UL-DL patterns, only one of the supported TDD UL-DL patterns shall be used for all tests.

#### 7.1.2.2	Performance requirements for PDSCH

##### 7.1.2.2.1	Definition and applicability

The performance requirement of PDSCH is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in annex H.

Which specific test(s) are applicable to NCR-MT is based on the test applicability rules defined in clause 7.1.2.1.1.

##### 7.1.2.2.2	Minimum requirements

For NCR-MT type 2-O, the minimum requirement is in TS 38.106 [2], clause 9.3.1.1.

##### 7.1.2.2.3	Test purpose

The test shall verify the receiver's ability to achieve throughput under multipath fading propagation conditions for a given SNR.

##### 7.1.2.2.4	Method of test

7.1.2.2.4.1	Initial conditions

Test environment: Normal, see annex B.2.

RF channels to be tested for single carrier: M, see clause 4.9.1.

RF channels to be tested for carrier aggregation: MBW Channel CA; see clause 4.9.1.

Direction to be tested: OTA REFSENS receiver target reference direction (see D.x in table 4.6-1).

7.1.2.2.4.2	Test procedure

1)	Place the NCR-MT with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.6.

2)	Align the manufacturer declared coordinate system orientation of the NCR-MT with the test system.

3)	Set the NCR-MT in the declared direction to be tested.

4)	Connect the NCR-MT tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex E.6. Each of the demodulation branch signals should be transmitted on one polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to the corresponding DL reference measurement channel defined in annex I, and according to additional test parameters listed in table 7.1.2.2.4.2-1.

Table: 7.1.2.2.4.2-1 Test parameters for testing PDSCH

| Parameter |  | NCR-MT type 2-O |
| --- | --- | --- |
| Cyclic prefix |  | Normal |
| Default TDD UL-DL pattern (Note) |  | 3D1S1U, S=10D:2G:2U |
| HARQ | Maximum number of HARQ transmissions | 4 for Table 7.1.2.2.5.2-1 1 for Table 7.1.2.2.5.2-2 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | DM-RS position (l0) | 2 |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 1 |
|  | DM-RS port(s) | {1000} |
|  | DM-RS sequence generation | NID0=0 |
| Time domain resource assignment | PDSCH mapping type | A |
|  | Start symbol | 1 |
|  | Allocation length | 13 |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
| PT-RS configuration | Frequency density (KPT-RS) | 2 |
|  | Time density (LPT-RS) | 1 |
| PRB bundling size |  | 2 |
| VRB-to-PRB mapping type |  | Not interleaved |
| PDSCH & PDSCH DMRS Precoding configuration |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex I.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the NCR-MT receiver is as specified in clause 7.1.2.2.5.1 for NCR-MT type 2-O respectively, and that the SNR at the NCR-MT receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 7.1.2.2.4.2-2.

Table 7.1.2.2.4.2-2: AWGN power level at the NCR-MT input

| NCR-MT type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| NCR-MT type 2-O | 120 | 100 | EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm / 95.04 MHz |
| NOTE 2: ΔFR2_REFSENS = -3 dB as described in clause 6.1, since the OTA REFSENS reference direction (as declared in D.x in table 4.6-1) is used for testing. NOTE 3: EISREFSENS_50M as declared in D.y in table 4.6-1. |  |  |  |

8)	For reference channels applicable to the NCR-MT, measure the throughput and BLER.

##### 7.1.2.2.5	Test requirements

7.1.2.2.5.1	Test requirement for NCR-MT type 2-O

The throughput or BLER shall be equal to or larger than the fraction of maximum throughput of BLER for the FRCs stated in Tables 7.1.2.2.5.2-1 and 7.1.2.2.5.2-2 at the given SNR with the test parameters stated in Table 7.1.2.2.4.2-1.

Table 7.1.2.2.5.2-1: Minimum requirements for PDSCH Type A with Rank 1

| Test number | FRC (Annex H) | Bandwidth (MHz) / Subcarrier spacing (kHz) | Propagation conditions (Annex I) | Antenna configuration | Fraction of maximum throughput (%) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1-1 | M-FR2-H.1-1 | 100/120 | TDLA30-75 | 2x2, ULA Low | 70 | -0.1 |

Table 7.1.2.2.5.2-2: Minimum requirements for PDSCH Type A with Rank 1

| Test number | FRC (Annex H) | Bandwidth (MHz) / Subcarrier spacing (kHz) | Propagation conditions (Annex I) | Antenna configuration | Fraction of maximum BLER (%) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1-1 | M-FR2-H.1-1 | 100/120 | TDLA30-75 | 2x2, ULA Low | 1 | 3.6 |

<End of Change 2 – [R4-2402999]>

<Start of Change 3 – [R4-2402999]>

#### 7.1.2.3	Demodulation performance requirements for PDCCH

##### 7.1.2.3.1	Definition and applicability

The receiver characteristics of the PDCCH are determined by the probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg).

Which specific test(s) are applicable to NCR-MT is based on the test applicability rules defined in clause 7.1.2.1.1.

##### 7.1.2.3.2	Minimum requirement

For NCR-MT type 2-O, the minimum requirement is in TS 38.106 [2], clause 9.3.1.2.

##### 7.1.2.3.3	Test purpose

The test shall verify the receiver's ability to detect the Downlink Scheduling Grant (Pm-dsg) under multipath fading propagation conditions for a given SNR.

##### 7.1.2.3.4	Method of test

7.1.2.3.4.1	Initial conditions

Test environment: Normal, see annex B.2.

RF channels to be tested for single carrier: M, see clause 4.9.1.

RF channels to be tested for carrier aggregation: MBW Channel CA; see clause 4.9.1.

Direction to be tested: OTA REFSENS receiver target reference direction (see D.x in table 4.6-1).

7.1.2.3.4.2	Test procedure

1)	Place the NCR-MT with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.6.

2)	Align the manufacturer declared coordinate system orientation of the NCR-MT with the test system.

3)	Set the NCR-MT in the declared direction to be tested.

4)	Connect the NCR-MT tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex E.6. Each of the demodulation branch signals should be transmitted on one polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to the corresponding DL reference measurement channel defined in annex H, and according to additional test parameters listed in table 7.1.2.3.4.2-1.

Table: 7.1.2.3.4.2-1 Test parameters for testing PDCCH

| Parameter | NCR-MT type 2-O |
| --- | --- |
| Cyclic prefix | Normal |
| Default TDD UL-DL pattern (Note) | 3D1S1U, S=10D:2G:2U |
| DM-RS sequence generation | NID=0 |
| Frequency domain resource allocation for CORESET | Start from RB = 0 with contiguous RB allocation |
| CCE to REG mapping type | Interleaved |
| Interleaver size | 3 for test with aggregation level 2, 8 2 for test with aggregation level 4 |
| REG bundle size | 2 for test with aggregation level 2, 8 6 for test with aggregation level 4 |
| Shift Index | 0 |
| Slots for PDCCH monitoring | Each slot |
| Number of PDCCH candidates for the tested aggregation level | 1 |
| PDCCH Precoding configuration | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination with REG bundling granularity for number of Tx larger than 1 |
| Note: The same requirements are applicable to TDD with different UL-DL patterns. |  |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex I.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the NCR-MT receiver is as specified in clause 7.1.2.3.5.1 for NCR-MT type 2-O respectively, and that the SNR at the NCR-MT receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 7.1.2.3.4.2-2.

Table 7.1.2.3.4.2-2: AWGN power level at the NCR-MT input

| NCR-MT type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| NCR-MT type 2-O | 120 | 100 | EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm / 95.04 MHz |
| NOTE 1: ΔFR2_REFSENS = -3 dB as described in clause 7.1, since the OTA REFSENS reference direction (as declared in D.x in table 4.6-1) is used for testing. NOTE 2: EISREFSENS_50M as declared in D.y in table 4.6-1. |  |  |  |

8)	For reference channels applicable to the NCR-MT, measure the miss-detection of the Downlink Scheduling Grant (Pm-dsg).

##### 7.1.2.3.5	Test requirements

7.1.2.3.5.1	Test requirement for NCR-MT type 2-O

The Pm-dsg shall be equal to or smaller than 1%, for the cases stated in Table 7.1.2.3.5.1-1 at the given SNR with the test parameters stated in Table 7.1.2.3.4.2-1.

Table 7.1.2.3.5.2-1: Minimum requirements for PDCCH

| Test number | Bandwidth (MHz) / Subcarrier spacing (kHz) | CORESET RB | CORESET duration | Aggregation level | FRC (Annex H) | Propagation conditions (Annex I) | Antenna configuration | Pm-dsg (%) | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | 100/120 | 60 | 1 | 2 | M-FR2-H.2-1 | TDLA30-75 | 1x2, ULA Low | 1 | 8.1 |
| 1-2 | 100/120 | 60 | 1 | 4 | M-FR2-H.2-2 | TDLA30-75 | 1x2, ULA Low | 1 | 4.6 |
| 1-3 | 100/120 | 60 | 1 | 8 | M-FR2-H.2-3 | TDLA30-75 | 2x2, ULA Low | 1 | 1.9 |

<End of Change 3 – [R4-2402999]>

<Start of Change 4 – [R4-2403002]>

7.1.3	CSI reporting requirements

#### 7.1.3.1	General

##### 7.1.3.1.1	Applicability of requirements

Unless otherwise stated, the tests shall apply only for each subcarrier spacing declared to be supported.

Unless otherwise stated, for each subcarrier spacing declared to be supported, if NCR-MT supports multiple TDD UL-DL patterns, only one of the supported TDD UL-DL patterns shall be used for all tests.

##### 7.1.3.1.2	Common test parameters

Parameters specified in Table 7.1.3.1.2-1 are applied for all test cases in this clause unless otherwise stated.

Table 7.1.3.1.2-1: Test parameters for CSI test cases

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| PDSCH transmission scheme |  |  |  | Transmission scheme 1 |
| Duplex mode |  |  |  | TDD |
| PTRS epre-Ratio |  |  |  | 0 |
| Actual carrier configuration | Offset between Point A and the lowest usable subcarrier on this carrier (Note 3) |  | RBs | 0 |
|  | Subcarrier spacing |  | kHz | 120 |
| DL BWP configuration #1 | Cyclic prefix |  |  | Normal |
|  | RB offset |  | RBs | 0 |
|  | Number of contiguous PRB |  | PRBs | 66 |
| Active DL BWP index |  |  |  | 1 |
| PDSCH configuration |  | Mapping type |  | Type A |
|  |  | k0 |  | 0 |
|  |  | Starting symbol (S) |  | 2 |
|  |  | Length (L) |  | 12 |
|  |  | PDSCH aggregation factor |  | 1 |
|  |  | PRB bundling type |  | Static |
|  |  | PRB bundling size |  | 2 |
|  |  | Resource allocation type |  | type 0 |
|  |  | RBG size |  | Config 2 |
|  |  | VRB-to-PRB mapping type |  | Non-interleaved |
|  |  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration |  | DMRS Type |  | Type 1 |
|  |  | Number of additional DMRS |  | 1 |
|  |  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
|  |  | DMRS ports indexes |  | {1000} for Rank1 {1000,1001} for Rank2 |
|  |  | Number of PDSCH DMRS CDM group(s) without data |  | 2 |
| PTRS configuration |  | Frequency density (KPT-RS) |  | 2 |
|  |  | Time density (LPT-RS) |  | 1 |
|  |  | Resource Element Offset |  | 2 |
| NZP CSI-RS for CSI acquisition |  | Frequency Occupation |  | Start PRB 0 Number of PRB = BWP size |
| Redundancy version coding sequence |  |  |  | {0,2,3,1} |

#### 7.1.3.2	Reporting of Channel Quality Indicator (CQI)

##### 7.1.3.2.1	Definition and applicability

The performance requirement of CSI reporting is determined by the reporting variance and the BLER performance using the transport format indicated by the reported CQI. The purpose is to verify that the reported CQI values are in accordance with the CQI definition given in TS 38.214 [15]. To account for sensitivity of the input SNR the reporting definition is considered to be verified if the reporting accuracy is met for at least one of two SNR levels separated by an offset of 1 dB.

##### 7.1.3.2.2	Minimum requirement

The minimum requirement for NCR-MT type 2-O is in TS 38.106 [2] clause 9.4.1.

##### 7.1.3.2.3	Test purpose

The test shall verify the receiver's ability to report correct median CQI and expected BLER performance under AWGN conditions.

##### 7.1.3.2.4	Method of test

###### 7.1.3.2.4.1	Initial conditions

Test environment:	Normal, see annex B.2.

RF channels to be tested for single carrier:	M; see clause 4.9.1.

RF channels to be tested for carrier aggregation: MBW Channel CA; see clause 4.9.1.

Direction to be tested: OTA REFSENS receiver target reference direction (see D.x in table 4.6-1).

###### 7.1.3.2.4.2	Procedure

1)	Place the NCR-MT with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.7.

2)	Align the manufacturer declared coordinate system orientation of the NCR-MT with the test system.

3)	Set the NCR-MT in the declared direction to be tested.

4)	Connect the NCR-MT tester generating the wanted signal and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex E.7. Each of the demodulation branch signals should be transmitted on one polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to the corresponding DL reference measurement channel defined in annex H, and according to additional test parameters listed in table 7.1.3.2.4.2-1.

Table 7.1.3.2.4.2-1: Test parameters for testing CQI reporting requirements for FR2

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 100 |  |  |  |
| Subcarrier spacing |  |  | kHz | 120 |  |  |  |
| Duplex Mode |  |  |  | TDD |  |  |  |
| SNRBB |  |  | dB | 8 | 9 | 14 | 15 |
| Propagation channel |  |  |  | AWGN |  |  |  |
| Antenna configuration |  |  |  | 2×2 with static channel specified in Annex I.1 |  |  |  |
| Beamforming Model |  |  |  | As specified in Annex I.3.1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 2 |  |  |  |
|  | CDM Type |  |  | fd-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | 6 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 5/1 |  |  |  |
| ReportConfigType |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  | Table 1 |  |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  | RB | 8 |  |  |  |
| csi-ReportingBand |  |  |  | 111111111 |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | 5/4 |  |  |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  | 010000 |  |  |  |
|  |  | RI Restriction |  | N/A |  |  |  |
| CQI/RI/PMI delay |  |  | ms | 1.75 |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  | As specified in Table H.3-1, M-FR2-H.3-1 |  |  |  |
| Note 1: The same requirements are applicable for TDD with different UL-DL pattern. Note 2: SSB, TRS, CSI-RS and/or other unspecified test parameters with respect to TS 38.101-4 [16] are left up to test implementation, if transmitted or needed. Note 3: If the NCR-MT reports in an available uplink reporting instance at slot #n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). |  |  |  |  |  |  |  |

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the NCR-MT receiver is as specified in clause 7.1.3.2.5.1, and that the SNR at the NCR-MT receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 7.1.3.2.4.2-2.

Table 7.2.3.2.4.2-2: AWGN power level at the NCR-MT input

| NCR-MT type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| NCR-MT type 2-O | 120 | 100 | EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm / 95.04 MHz |
| NOTE 1: ΔFR2_REFSENS = -3 dB as described in clause 7.1, since the OTA REFSENS reference direction (as declared in D.x in table 4.6-1) is used for testing. NOTE 2: EISREFSENS_50M as declared in D.y in table 4.6-1. |  |  |  |

8)	For reference channels applicable to the NCR-MT, measure the median CQI and the BLER at (median CQI +1) and (median CQI -1).

##### 7.1.3.2.5	Test requirement

###### 7.1.3.2.5.1	Test requirement for NCR type 2-O

For the parameters specified in Table 7.1.3.2.4.2-1, the minimum requirements are specified by the following:

a)	the reported CQI value shall be in the range of ±1 of the reported median more than 90% of the time;

b)	if the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, the BLER using the transport format indicated by the (median CQI + 1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, the BLER using transport format indicated by (median CQI – 1) shall be less than or equal to 0.1.

<End of Change 4 – [R4-2403002]>

<Start of Change 5 – [R4-2403002]>

## E.6 	NCR-MT type 2-O PDSCH and PDCCH testing

Figure E.6-1: Functional set-up for PDCCH, PDSCH performance requirements with Rx diversity (2 Rx case shown)

The OTA chamber shown in Figure E.6-1 is intended to be generic and can be replaced with any suitable OTA chamber (e.g. far field anechoic chamber, CATR, etc.).

NOTE 1:	The feedback could be done as an RF feedback, either using NR channels or using other means, or as a digital feedback. The HARQ Feedback should be error free.

NOTE 2:	In tests performed with signal generators, a synchronization signal may be provided between the NCR node and the signal generator, or a common (e.g., GNSS) source may be provided to both NCR node and the signal generator, to enable correct timing of the wanted signal. The method of synchronization with the TE is left to test implementation.

NOTE 3:	It is left up to implementation how L1/L2 is configured for testing.

## E.7	NCR-MT type 2-O CQI testing

Figure E.7-1: Functional set-up for PUSCH, PRACH, CSI reporting performance requirements in static AWGN channel with Rx diversity (2 Rx case shown)

The OTA chamber shown in Figure E.7-1 is intended to be generic and can be replaced with any suitable OTA chamber (e.g. far field anechoic chamber, CATR, etc.).

NOTE 1:	The feedback could be done as an RF feedback, either using NR channels or using other means, or as a digital feedback. The HARQ Feedback should be error free.

NOTE 2:	In tests performed with signal generators, a synchronization signal may be provided between the IAB node and the signal generator, or a common (e.g., GNSS) source may be provided to both IAB node and the signal generator, to enable correct timing of the wanted signal. The method of synchronization with the TE is left to test implementation.

NOTE 3:	It is left up to implementation how L1/L2 is configured for testing.

<End of Change 5 – [R4-2403002]>

<Start of Change 6 – [R4-2403005]>

Annex H (normative): 
NCR-MT Fixed Reference Channels

## H.1	Fixed Reference Channels for PDSCH performance requirements (QPSK)

The parameters for the reference measurement channels are specified in table H.1-1 for FR2 PDSCH performance requirements.

Table H.1-1: Fixed Reference Channels for FR2-1 PDSCH (QPSK)

| Parameter | Unit | M-FR2-H.1-1 |
| --- | --- | --- |
| Channel bandwidth | MHz | 100 |
| Subcarrier spacing | kHz | 120 |
| Allocated resource blocks | PRBs | 66 |
| Number of consecutive PDSCH symbols |  | 13 |
| MCS table |  | 64QAM |
| MCS index |  | 4 |
| Modulation |  | QPSK |
| Target Coding Rate |  | 0.30 |
| Number of MIMO layers |  | 1 |
| Number of DMRS REs |  | 12 |
| Overhead for TBS determination |  | 6 |
| Information Bit Payload per Slot |  | 5504 |
| Transport block CRC per Slot |  | 24 |
| Number of Code Blocks per Slot |  | 1 |
| Binary Channel Bits Per Slot |  | 18282 |

## H.2	Fixed Reference Channels for PDCCH performance requirements

The parameters for the reference measurement channels are specified in table H.2-1 for FR2-1 PDCCH performance requirements.

Table H.2-1: Fixed Reference Channels for FR2-1 PDCCH

| Reference channel | M-FR2-H.2-1 | M-FR2-H.2-2 | M-FR2-H.2-3 |
| --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 120 | 120 | 120 |
| CORESET frequency domain allocation | 60 | 60 | 60 |
| CORESET time domain allocation | 1 | 1 | 1 |
| Aggregation level | 2 | 4 | 8 |
| DCI Format | 1_0 | 1_1 | 1_1 |
| Payload (without CRC) (bits) | 40 | 56 | 56 |

## H.3	Fixed Reference Channels for CQI reporting requirements

The parameters for the reference measurement channels are specified in table H.3-1 for FR2-1 CQI reporting requirements with the CQI definition specified in clause 5.2.2.1 of TS 38.214 [15] and with the MCS definition specified in clause 5.1.3 of TS 38.214 [15].

Table H.3-1: FRC for FR2-1 CQI reporting with CQI table 1 and MCS table 1

| Reference channel |  |  |  | M-FR2-H.3-1 |
| --- | --- | --- | --- | --- |
| Number of allocated PDSCH resource blocks |  |  |  | 66 |
| Number of consecutive PDSCH symbols |  |  |  | 12 |
| Number of PDSCH MIMO layers |  |  |  | 1 |
| Number of DMRS REs (Note 1) |  |  |  | 24 |
| Overhead for TBS determination |  |  |  | 6 |
| Available RE-s for PDSCH |  |  |  | 7590 |
| CQI index | Spectral efficiency | MCS index | Modulation | Information Bit Payload per Slot |
| 0 | OOR | OOR | OOR | N/A |
| 1 | 0.1523 | 0 | QPSK | 1800 |
| 2 | 0.2344 | 0 |  | 1800 |
| 3 | 0.3770 | 2 |  | 2856 |
| 4 | 0.6016 | 4 |  | 4480 |
| 5 | 0.8770 | 6 |  | 6528 |
| 6 | 1.1758 | 8 |  | 8712 |
| 7 | 1.4766 | 11 | 16QAM | 11016 |
| 8 | 1.9141 | 13 |  | 14343 |
| 9 | 2.4063 | 15 |  | 17928 |
| 10 | 2.7305 | 18 | 64QAM | 20496 |
| 11 | 3.3223 | 20 |  | 25104 |
| 12 | 3.9023 | 22 |  | 29192 |
| 13 | 4.5234 | 24 |  | 33816 |
| 14 | 5.1152 | 26 |  | 38936 |
| 15 | 5.5547 | 28 |  | 42016 |
| NOTE 1: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data NOTE 2: PDSCH is only scheduled on slots which are full DL |  |  |  |  |

<End of Change 6 – [R4-2403005]>

<Start of Change 7 – [R4-2402996]>

Annex I (normative): 
Propagation conditions

## I.1	Static propagation condition

The propagation for the static performance measurement is an Additive White Gaussian Noise (AWGN) environment. No fading or multi-paths exist for this propagation model.

## I.1.1	NCR-MT Receiver with 2 Rx

For 1 port transmission the channel matrix is defined in the frequency domain by:

.

For 2 port transmission the channel matrix is defined in the frequency domain by:

.

For 4 port transmission the channel matrix is defined in the frequency domain by:

For 8 port transmission the channel matrix is defined in the frequency domain by:

## I.1.2	UE Receiver with 4Rx

For 1 port transmission the channel matrix is defined in the frequency domain by

.

For 2 port transmission the channel matrix is defined in the frequency domain by

.

For 4 port transmission the channel matrix is defined in the frequency domain by

.

For 8 port transmission the channel matrix is defined in the frequency domain by

## I.2	Multi-path fading propagation conditions

The multipath propagation conditions consist of several parts:

-	A delay profile in the form of a "tapped delay-line", characterized by a number of taps at fixed positions on a sampling grid. The profile can be further characterized by the r.m.s. delay spread and the maximum delay spanned by the taps.

-	A combination of channel model parameters that include the Delay profile and the Doppler spectrum that is characterized by a classical spectrum shape and a maximum Doppler frequency.

-	Different models are used for FR1 (410 MHz - 7.125GHz) and FR2-1 (24.25 GHz – 52.6 GHz).

I.2.1	Delay profiles

The delay profiles are simplified from the TR 38.901 [17] TDL models. The simplification steps are shown below for information. These steps are only used when new delay profiles are created. Otherwise, the delay profiles specified in annex I.2.1.1 and I.2.1.2 can be used as such.

Step 1: Use the original TDL model from TR 38.901 [17].

Step 2: Re-order the taps in ascending delays.

Step 3: Perform delay scaling according to the procedure described in clause 7.7.3 in TR 38.901 [17].

Step 4: Apply the quantization to the delay resolution 5 ns. This is done simply by rounding the tap delays to the nearest multiple of the delay resolution.

Step 5: If multiple taps are rounded to the same delay bin, merge them by calculating their linear power sum.

Step 6: If there are more than 12 taps in the quantized model, merge the taps as follows:

-	Find the weakest tap from all taps (both merged and unmerged taps are considered)

-	If there are two or more taps having the same value and are the weakest, select the tap with the smallest delay as the weakest tap.

-	When the weakest tap is the first delay tap, merge taps as follows:

-	Update the power of the first delay tap as the linear power sum of the weakest tap and the second delay tap.

-	Remove the second delay tap.

-	When the weakest tap is the last delay tap, merge taps as follows:

-	Update the power of the last delay tap as the linear power sum of the second-to-last tap and the last tap.

-	Remove the second-to-last tap.

-	Otherwise:

-	For each side of the weakest tap, identify the neighbour tap that has the smaller delay difference to the weakest tap.

-	When the delay difference between the weakest tap and the identified neighbour tap on one side equals the delay difference between the weakest tap and the identified neighbour tap on the other side.

-	Select the neighbour tap that is weaker in power for merging.

-	Otherwise, select the neighbour tap that has smaller delay difference for merging.

-	To merge, the power of the merged tap is the linear sum of the power of the weakest tap and the selected tap.

-	When the selected tap is the first tap, the location of the merged tap is the location of the first tap. The weakest tap is removed.

-	When the selected tap is the last tap, the location of the merged tap is the location of the last tap. The weakest tap is removed.

-	Otherwise, the location of the merged tap is based on the average delay of the weakest tap and selected tap. If the average delay is on the sampling grid, the location of the merged tap is the average delay. Otherwise, the location of the merged tap is rounded towards the direction of the selected tap (e.g. 10 ns & 20 ns  15 ns, 10 ns & 25 ns  20 ns, if 25 ns had higher or equal power; 15 ns, if 10 ns had higher power). The weakest tap and the selected tap are removed.

-	Repeat step 6 until the final number of taps is 12.

Step 7: Round the amplitudes of taps to one decimal (e.g. -8.78 dB  -8.8 dB)

Step 8: If the delay spread has slightly changed due to the tap merge, adjust the final delay spread by increasing or decreasing the power of the last tap so that the delay spread is corrected.

Step 9: Re-normalize the highest tap to 0 dB.

NOTE 1:	Some values of the delay profile created by the simplification steps may differ from the values in tables I.2.1.1-2, I.2.1.1-3, I.2.1.1-4, and I.2.1.2-2 for the corresponding model.

NOTE 2:	For Step 5 and Step 6, the power values are expressed in the linear domain using 6 digits of precision. The operations are in the linear domain.

#### I.2.1.1	Delay profiles for FR2-1

The delay profiles for FR2-1 are specified in I.2.1.1-1 and the tapped delay line models are specified in table I.2.1.1-2.

Table I.2.1.1-1: Delay profiles for NR channel models

| Model | Number of channel taps | Delay spread (r.m.s.) | Maximum excess tap delay (span) | Delay resolution |
| --- | --- | --- | --- | --- |
| TDLA30 | 12 | 30 ns | 290 ns | 5 ns |

Table I.2.1.1-2: TDLA30 (DS = 30 ns)

| Tap # | Delay (ns] | Power (dB) | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -15.5 | Rayleigh |
| 2 | 10 | 0 |  |
| 3 | 15 | -5.1 |  |
| 4 | 20 | -5.1 |  |
| 5 | 25 | -9.6 |  |
| 6 | 50 | -8.2 |  |
| 7 | 65 | -13.1 |  |
| 8 | 75 | -11.5 |  |
| 9 | 105 | -11.0 |  |
| 10 | 135 | -16.2 |  |
| 11 | 150 | -16.6 |  |
| 12 | 290 | -26.2 |  |

I.2.2	Combinations of channel model parameters

The propagation conditions used for the performance measurements in multi-path fading environment are indicated as a combination of a channel model name and a maximum Doppler frequency, i.e., TDLA<DS>-<Doppler>, TDLB<DS>-<Doppler> or TDLC<DS>-<Doppler> where '<DS>' indicates the desired delay spread and '<Doppler>' indicates the maximum Doppler frequency (Hz).

Table I.2.2-1 and I.2.2-2 show the propagation conditions that are used for the performance measurements in multi-path fading environment for low, medium and high Doppler frequencies for FR1 and FR2, respectively.

Table I.2.2-1: Channel model parameters for FR1

| Combination name | Model | Maximum Doppler frequency |
| --- | --- | --- |
| TDLA30-10 | TDLA30 | 10 Hz |

Table I.2.2-2: Channel model parameters for FR2

| Combination name | Model | Maximum Doppler frequency |
| --- | --- | --- |
| TDLA30-75 | TDLA30 | 75 Hz |

I.2.3	MIMO channel correlation matrices

The MIMO channel correlation matrices defined in I.2.3 apply for the antenna configuration using uniform linear arrays at both gNB and NCR-MT and for the antenna configuration using cross polarized antennas.

#### I.2.3.1	MIMO correlation matrices using Uniform Linear Array (ULA)

The MIMO channel correlation matrices defined in I.2.3.1 apply for the antenna configuration using uniform linear array (ULA) at both gNB and NCR-MT.

##### I.2.3.1.1	Definition of MIMO correlation matrices

Table I.2.3.1.1-1 defines the correlation matrix for the gNB.

Table I.2.3.1.1-1: gNB correlation matrix

|  | gNB correlation |
| --- | --- |
| One antenna |  |
| Two antennas |  |
| Four antennas |  |
| Eight antennas |  |

Table I.2.3.1.1-2 defines the correlation matrix for the NCR-MT:

Table I.2.3.1.1-2: NCR-MT correlation matrix

|  | One antenna | Two antennas | Four antennas |
| --- | --- | --- | --- |
| NCR-MT correlation |  |  |  |

Table I.2.3.1.1-3 defines the channel spatial correlation matrix. The parameters, α and β in table I.2.3.1.1-3 defines the spatial correlation between the antennas at the gNB and NCR-MT respectively.

Table I.2.3.1.1-3: correlation matrices

| 1x2 case |  |
| --- | --- |
| 1x4 case |  |
| 2x2 case |  |
| 2x4 case |  |
| 4x4 case |  |

For cases with more antennas at either gNB or NCR-MT or both, the channel spatial correlation matrix can still be expressed as the Kronecker product of  and according to.

##### I.2.3.1.2	MIMO correlation matrices at high, medium and low level

The  and  for different correlation types are given in table I.2.3.1.2-1.

Table I.2.3.1.2-1: Correlation for high, medium and low level

| Low correlation |  | Medium correlation |  | High correlation |  |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0.9 | 0.3 | 0.9 | 0.9 |

The correlation matrices for high, medium and low correlation are defined in tables I.2.3.1.2-2, I.2.3.1.2-3 and I.2.3.1.2-4 as below.

The values in table I.2.3.1.2-2 have been adjusted for the 2x4 and 4x4 high correlation cases to ensure the correlation matrix is positive semi-definite after round-off to 4 digit precision. This is done using the equation:

Where the value "a" is a scaling factor such that the smallest value is used to obtain a positive semi-definite result. For the 2x4 high correlation case, a=0.00010. For the 4x4 high correlation case, a=0.00012.

The same method is used to adjust the 4x4 medium correlation matrix in table I.2.3.1.2-3 to ensure the correlation matrix is positive semi-definite after round-off to 4 digit precision with a =0.00012.

Table I.2.3.1.2-2: MIMO correlation matrices for high correlation

| 1x2 case |  |
| --- | --- |
| 2x2 case |  |
| 2x4 case |  |
| 4x4 case |  |

Table I.2.3.1.2-3: MIMO correlation matrices for medium correlation

| 1x2 case | N/A |
| --- | --- |
| 2x2 case |  |
| 2x4 case |  |
| 4x4 case |  |

Table I.2.3.1.2-4: MIMO correlation matrices for low correlation

| 1x2 case |  |
| --- | --- |
| 1x4 case |  |
| 1x8 case |  |
| 2x2 case |  |
| 2x4 case |  |
| 2x8 case |  |
| 4x4 case |  |

In table I.2.3.1.2-4,  is a  identity matrix.

NOTE:	For completeness, the correlation matrices were defined for high, medium and low correlation but performance requirements exist only for low correlation.

#### I.2.3.2	Multi-antenna channel models using cross polarized antennas

The MIMO channel correlation matrices defined in I.2.3.2 apply to two cases as presented below:

-	One TX antenna and multiple RX antennas case, with cross polarized antennas used at gNB

-	Multiple TX antennas and multiple RX antennas case, with cross polarized antennas used at both UE and gNB

The cross-polarized antenna elements with +/-45 degrees polarization slant angles are deployed at gNB. For one TX antenna case, antenna element with +90 degree polarization slant angle is deployed at UE. For multiple TX antennas case, cross-polarized antenna elements with +90/0 degrees polarization slant angles are deployed at UE.

For the cross-polarized antennas, the N antennas are labelled such that antennas for one polarization are listed from 1 to N/2 and antennas for the other polarization are listed from N/2+1 to N, where N is the number of TX or RX antennas.

##### I.2.3.2.1	Definition of MIMO correlation matrices using cross polarized antennas

For the channel spatial correlation matrix, the following is used:

Where

-	 is the spatial correlation matrix at the UE with same polarization,

-	 is the spatial correlation matrix at the gNB with same polarization,

-	 is a polarization correlation matrix,

-	 is a permutation matrix, and

-	 denotes transpose.

Table I.2.3.2.1-1 defines the polarization correlation matrix.

Table I.2.3.2.1-1 : Polarization correlation matrix

|  | One TX antenna | Multiple TX antennas |
| --- | --- | --- |
| Polarization correlation matrix |  |  |

The matrixis defined as

where  and  is the number of TX and RX antennas respectively, and  is the ceiling operator.

The matrix  is used to map the spatial correlation coefficients in accordance with the antenna element labelling system described in I.2.3.2.

##### I.2.3.2.2	Spatial correlation matrices at NCR-MT and gNB sides

###### I.2.3.2.2.1	Spatial correlation matrices at NCR-MT side

For 1-antenna transmitter, .

For 2-antenna transmitter using one pair of cross-polarized antenna elements, .

For 4-antenna transmitter using two pairs of cross-polarized antenna elements, .

###### I.2.3.2.2.2	Spatial correlation matrices at gNB side

For 2-antenna receiver using one pair of cross-polarized antenna elements, .

For 4-antenna receiver using two pairs of cross-polarized antenna elements,.

For 8-antenna receiver using four pairs of cross-polarized antenna elements,.

##### I.2.3.2.3	MIMO correlation matrices using cross polarized antennas

The values for parameters α, β and γ for low spatial correlation are given in table I.2.3.2.3-1.

Table I.2.3.2.3-1: Values for parameters α,  and γ

| Low spatial correlation |  |  |
| --- | --- | --- |
| Α |  | γ |
| 0 | 0 | 0 |
| Note 1: Value of α applies when more than one pair of cross-polarized antenna elements at gNB side. Note 2: Value of β applies when more than one pair of cross-polarized antenna elements at UE side. |  |  |

The correlation matrices for low spatial correlation are defined in table I.2.3.2.3-2 as below.

Table I.2.3.2.3-2: MIMO correlation matrices for low spatial correlation

| 1x8 case |  |
| --- | --- |
| 2x8 case |  |

In table I.2.3.2.3-2,  is a  identity matrix.

##### I.2.3.2.4	Beam steering approach

For the 2D cross-polarized antenna array at gNB, given the channel spatial correlation matrix in I.2.3.2.1, I.2.3.2.2 and I.2.3.2.3, the corresponding random channel matrix H can be calculated. The signal model for the k-th slot is denoted as

And the steering matrix is further expressed as following:

where

-	H is the Nr×Nt channel matrix per subcarrier.

-	 is the steering matrix,

-	 is the steering matrix in first dimension with same polarization,

-	 is the steering matrix in second dimension with same polarization,

-	 is the number of antenna elements in first dimension with same polarization,

-	 is the number of antenna elements in second dimension with same polarization,

-	For antenna array with only one direction, number of antenna element in second direction equals 1.

For 1 antenna element with the same polarization in one direction,

.

For 2 antenna elements with the same polarization in one direction,

.

For 3 antenna elements with the same polarization in one direction,

.

For 4 antenna elements with the same polarization in one direction,

.

where the index  stands for first dimension and second dimension respectively.

-	 controls the phase variation in first dimension and second dimension respectively, and the phase for k-th subframe is denoted by, where is the random start value with the uniform distribution, i.e., ,  is the step of phase variation, which is defined in Table I.2.3.2.4-1, and k is the linear increment of 2-μ for every slot throughout the simulation, the index  stands for first dimension and second dimension respectively.

-	 is the precoding matrix for Nt transmission antennas,

-	y is the received signal, x is the transmitted signal, and n is AWGN.

-	 corresponds to subcarrier spacing configuration,

For the 1D cross-polarized antenna array at gNB, the corresponding random channel matrix H can be calculated by letting N2=1, i.e.,

Table I.2.3.2.4-1: The step of phase variation

| Variation Step | Value (rad/ms) |
| --- | --- |
|  | 1.2566×10-3 |

## I.3	Physical signals, channels mapping and precoding

I.3.1	General

Unless otherwise stated, the transmission on antenna port(s)  is defined by using a precoder matrix  of size , where is the number of physical transmit antenna elements configured per test ,  is the number of ports for a reference signal or physical channel configured per test, and   is the first port for that reference signal or physical channel as defined in clauses 7.3 and 7.4 in TS 38.211 [18]. This precoder takes as an input a block of signals for antenna port(s) , , , with  being the number of modulation symbols per antenna port including the reference signal symbols, and generates a block of signals  the elements of which are to be mapped onto the frequency-time index pair as per the test configuration but transmitted on different physical antenna elements:

For Clause 6 and 8, the transmission of PDCCH and PDCCH DMRS on antenna port  is defined by using a precoder matrix  of size 2x1. This precoder takes as an input a block of signals for antenna port(s) ,  and generates a block of signals  the elements of which are to be mapped onto the frequency-time index pair as per the test configuration but transmitted on different physical antenna elements:

The precoder matrix is specific to the test case configuration  is defined in Clause 5.2.2.2 of TS 38.214 [15].

The transmission on PT-RS antenna port is associated (using same precoder) with the lowest indexed DM-RS antenna port among the DM-RS antenna ports assigned for the PDSCH.

The physical antenna elements are identified by indices, where  is the number of physical antenna elements configured per test.

Modulation symbols  with  (i.e. PSS, SSS, PBCH and DM-RS for PBCH) are directly mapped to first physical antenna element.

Modulation symbols  for CSI-RS resources which configured for tracking with one port are directly mapped to first physical antenna element.

Modulation symbols  for CSI-RS resources which configured for beam refinement with one port are directly mapped to first physical antenna element.

Modulation symbols  for NZP CSI-RS which configured for CSI acquisition with    are mapped to the physical antenna index  where is the number of NZP CSI-RS ports configured per test.

<End of Change 7 – [R4-2402996]>

<Start of Change 8 – [R4-2402996]>

Annex J (informative):
Change history

<End of Change 8 – [R4-2402996]>