3GPP TSG-RAN WG4 Meeting #110 	R4-2402716

Athens, Greece, February 26 – March 01, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	HST FR2 Enhanced: UE Demodulation PDSCH Requirements with Carrier Aggregation

Agenda item:	8.8.3.2

Document for:	Discussion

Introduction

In RAN4#109 meeting, the discussions on HST FR2 Enhanced UE Demodulation performance requirements were held, in which the way forward (WF) is documented in [1]. There is one open issue related to HST FR2 PDSCH with CA to be discussed by the company during RAN4#110, namely:

Issue 4-1-1: UE feature lists for FR2 PDSCH requirements with CA WF - Companies can further check in the next meeting

Discussion

In the following we provide our views on the remaining open issue on the UE feature list as in below:

Issue 4-1-1: UE feature lists for FR2 PDSCH requirements with CA WF - Companies can further check in the next meeting Features; Index; Feature group; Components; Prerequisite feature groups; Need for the gNB to know if the feature is supported; Applicable to the capability signalling exchange between UEs (V2X WI only)”.; Consequence if the feature is not supported by the UE; Type; Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Capability interpretation for mixture of FDD/TDD and/or FR1/FR2; Note; Mandatory/Optional. 34. NR_HST_FR2_enh; 34-2; Enhanced demodulation requirements for CA HST FR2; 1. Support of the enhanced demodulation requirements specified for CA for HST FR2 UE; 22-1; Yes; NA; UE does not support enhanced demodulation requirements for CA; Per BC; No; FR2 only; N/A; # Component 1 candidate value: true/false; Optional without capability signalling.

The Work Item Description (WID) of Rel-18 Enhanced HST FR2 is documented in [2], in which the objective performance part is described.

The WID of Rel-18 Enhanced HST FR2 stated that for CA, only intra-band CA is considered.

RAN4 to include “intra-band” to the Feature group: “Enhanced demodulation requirements for intra-band CA HST FR2”

RAN4 to include “intra-band” to the Components: “1. Support of the enhanced demodulation requirements specified for intra-band CA for HST FR2 UE”

Conclusion

In this contribution, we provide our views on the proposed UE feature list for Rel-18 HST FR2 PDSCH with CA. They are summarized in the following Observations and Proposals:

Observation 1: The WID of Rel-18 Enhanced HST FR2 stated that for CA, only intra-band CA is considered.

Proposal 1: RAN4 to include “intra-band” to the Feature group: “Enhanced demodulation requirements for intra-band CA HST FR2”

Proposal 2: RAN4 to include “intra-band” to the Components: “1. Support of the enhanced demodulation requirements specified for intra-band CA for HST FR2 UE”

- References

- R4-2321063, WF for FR2 HST demodulation requirements, Samsung, RAN4#109, Chicago, USA, November 13-17, 2023.

- RP-233151, Revised WID on enhanced NR support for high speed train scenario in frequency range 2 (FR2), 3GPP TSG-RAN Meeting #102, Edinburgh, Scotland, December 11 - 15, 2023