3GPP TSG-RAN WG4 Meeting #110	R4-2402720

Athens, Greece, February 26 – March 01, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	Enhancement of NR dynamic spectrum sharing: UE Demodulation Performance Requirements

Agenda item:	8.27.2

Document for:	Discussion

Introduction

In RAN4#109, several agreements and way forwards (WFs) from the discussions on Enhancement of NR Dynamic Spectrum Sharing (eDSS) have been made and documented in [1]. The discussions themselves were only about PDCCH, as it was already agreed in RAN4#108bis that RAN4 will only define requirements for PDCCH of eDSS [2]. The agreements and WFs in [1] are summarized below:

CRS rate matching pattern assumption Agreement: - Single LTE CRS rate matching pattern UE receiver assumption Agreement: - Leave the receiver design to implementation for evaluation purposes. - Further check submitted results. If there is a large variation in results, then discuss receiver assumptions. Common parameters Agreement: - DCI format: 1_0 - CCE to REG mapping: Non-interleaved - REG bundle size: 6 PRB - Precoder: Random precoder with precoder cycling per REG bundle - LTE CRS: 4 Ports - TX assumption for overlapping REs: Puncture PDCCH and DMRS REs overlapping with LTE CRS - LTE PDCCH, PDSCH: No transmission - Slots with LTE PBCH/PSS/SSS: No PDCCH transmission - Channel estimation: PDCCH CE only on clean symbols Payload bits Agreement: - 39 bits Antenna configuration Way forward: - Option 1: 2x2, 2x4, 4x2, 4x4 - Option 2: 4x2, 4x4 only Channel bandwidth Agreement: - For FDD, consider 10MHz/15kHz Way forward: - For TDD, - Option 1: 10MHz/15kHz - Option 2: 20MHz/15kHz Aggregation level Agreement: - 4, 8 (for evaluation purpose) Channel model Agreement: - TDLA30-10 and TDLC300-100 - Discuss whether to down-select in the next meeting Power ratio of LTE-CRS RE/NR PDCCH-DMRS RE Agreement: - 0dB

In addition to the agreements and WFs as mentioned above, it was also discussed among companies in RAN4#109 to conduct preliminary simulations for the discussions during RAN4#110.

Discussion

In this contribution, we will address the remaining open issues from RAN4#109, including the simulation results. Our simulation results can be found in [3] for further information.

The first open issue to be addressed here is about antenna configuration, as below:

Antenna configuration Way forward: - Option 1: 2x2, 2x4, 4x2, 4x4 - Option 2: 4x2, 4x4 only

The 8 test cases defined in [1] for preliminary simulations have already taken into account the 4 possible antenna configurations as in option 1. Hence, RAN4 can consider them for the requirements, unless there are issues from the collected simulation results from companies.

The 8 test cases for the simulations have covered all antenna configurations.

RAN4 to consider antenna configurations of 2x2, 2x4, 4x2 and 4x4.

The next open issue is about channel bandwidth as described below:

Channel bandwidth Agreement: - For FDD, consider 10MHz/15kHz Way forward: - For TDD, - Option 1: 10MHz/15kHz - Option 2: 20MHz/15kHz

RAN4 to consider the smaller bandwidth size in defining requirements for TDD, i.e., 10MHz/15 KHz.

There are two related open issues to be discussed together below:

Aggregation level Agreement: - 4, 8 (for evaluation purpose) Channel model Agreement: - TDLA30-10 and TDLC300-100 - Discuss whether to down-select in the next meeting

As agreed for the test cases, Aggregation Level 4 (AL4) is to be evaluated with TDLA30-10, while AL8 with TDLC300-100.

With the current setup for the test cases, Aggregation Level 4 (AL4) is for less challenging channel and AL8 for more challenging one.

Both AL4 and AL8 are using the same low payload.

RAN4 to consider both Aggregation Level 4 and 8, with the channel model TDLA30-10 and TDLC300-100, respectively.

RAN4 to discuss the case of using a higher payload for AL8.

Conclusion

In this contribution, we provide our views on UE demodulation performance requirements of NR eDSS. The following Observations and Proposals were made:

Observation 1: The 8 test cases for the simulations have covered all antenna configurations.

Proposal 1: RAN4 to consider antenna configurations of 2x2, 2x4, 4x2 and 4x4.

Proposal 2: RAN4 to consider the smaller bandwidth size in defining requirements for TDD, i.e., 10MHz/15 KHz.

Observation 2: With the current setup for the test cases, Aggregation Level 4 (AL4) is for less challenging channel and AL8 for more challenging one.

Observation 3: Both AL4 and AL8 are using the same low payload.

Proposal 3: RAN4 to consider both Aggregation Level 4 and 8, with the channel model TDLA30-10 and TDLC300-100, respectively.

Proposal 4: RAN4 to discuss the case of using a higher payload for AL8.

- References

- R4-2321139, WF for Enhancement of Dynamic Spectrum Sharing demodulation requirements, 3GPP RAN4#109, Ericsson, Chicago, USA, November 13 ‒ 17, 2023

- R4-2317000 WF for Enhancement of Dynamic Spectrum Sharing demodulation requirements, 3GPP RAN4#108bis, Ericsson, Xiamen, China, Oct. 9 ‒ Oct. 13, 2023

- R4-2402721, On PDCCH of Enhancement of NR Dynamic Spectrum Sharing: Simulation Results, Nokia, Nokia Shanghai Bell, RAN4#110, Athens, Greece, February 26 – March 01, 2024